3GPP TSG-RAN WG2 #119-bis-e	Tdoc R2-2210692

Electronical meeting, 10 October – 19 October 2022

Agenda Item:	8.5.3.1

Source:	Ericsson

Title:	Discussion on solutions for DRX cycle mismatch and jitter

Document for:	Discussion, Decision

# 1	Introduction

In [1], RAN2 reached the following agreements regarding XR-specific power saving:

1: RAN2 to focus on the following issues for power saving, as well necessary parameters XR-awareness to support such enhancements, i.e.:

- -	DRX enhancements to address the issues of DRX cycle mismatch and jitter

- -	Identify necessary parameters from CN for XR-awareness for power saving

Enhancements to Rel-17 PDCCH adaptation can be discussed based on RAN1 feedback, if they have any RAN2 impact

RAN2-specific aspects can be studied based on contributions (e.g. multiple XR traffic flows with different periodicities, SFN wrap-around, RAN2-specific CDRX aspects, …).

In this paper we focus on enhancements to address the issues of DRX cycle mismatch and jitter. Furthermore, we summarize the parameters necessary from CN for XR-awareness for power saving.

# 2	Discussion on Power Saving Enhancements for XR

## 2.2 C-DRX enhancements

We discuss C-DRX enhancements to address the DRX cycle mismatch and jitter. Extensive simulation results for these enhancements are presented in Annex A. Our results show overall that, depending on the network load and the traffic generation rate, different C-DRX solutions need to be applied, to maximize power savings and also achieve a high fraction of satisfied UEs. We discuss each solution in the subsequent sections.

Depending on the network load and the traffic generation rate, different C-DRX solutions need to be applied, to maximize power savings and also achieve a high fraction of satisfied UEs.

Furthermore, the parameter values of the enhanced features are selected by the network assuming there is information about the XR traffic, e.g. from the application: application packet periodicity (generation rate) and periodicity changes for each application traffic flow; application packet jitter information, e.g. range, per application flow; delay budget (PDB) of the application packet for radio interface, per application flow; application packet sequence number in each constituent PDU; and application packet size.

We note that we adopt the term “application packet”, consistent with the term “packet” in [2], where “a packet models the set of IP packets belong to the same video frame”. Nonetheless, an application packet refers also to other types XR flows and not only video.

In order to enhance and/or configure C-DRX power saving features, the network needs per-XR flow information: 
- application packet periodicity and periodicity changes; 
- application packet jitter information; 
- delay budget or remaining PDB of the application packet for radio interface; 
- application packet sequence number carried in each constituent PDU; and
- application packet size.

### 2.2.1 Align DRX to non-integer traffic periodicity

The mismatch between the DRX cycle length and the non-integer video traffic periodicity can result in additional traffic delay and capacity decrease. Due to this, it is beneficial to enhance DRX to match non-integer periodicities, such that the drx-onDurationTimer can be started when traffic is actually expected, in order to limit the delay. Additionally, the drx-onDurationTimer value could be set to cover only the time when traffic is expected, to make best use of the sleeping opportunities and save power.

Aligning the DRX should be done in sync with the network to avoid further issues. Thus, we propose a simple solution to match the DRX cycle with the traffic period based on adding a time offset after a number of cycles. The network provides these two parameters to the UE:

- traffic_time_offset: indicates a fixed time shift for the start of drx-onDurationTimer;

- drx_offset: a number of cycles after which the new shift traffic_time_offset should be added.

With this solution, the DRX cycle length would be approximated to the next lower supported cycle value w.r.t. the non-integer traffic periodicity. The formula and conditions to start the drx-onDurationTimer would then be modified to:

- initialize the following variables when DRX starts the first time: tmp_cycle=0 and drx_cycle=-1;

- check the condition:

[(SFN × 10) + subframe number] modulo (drx-LongCycle) =

(drx-StartOffset + n × traffic_time_offset) modulo (drx-LongCycle);

- if the condition in (ii) is true and (tmp_cycle+1) modulo drx_offset=0, increment n and tmp_cycle;

- otherwise, if the condition in (ii) is true, start drx-onDurationTimer and increment drx_cycle and assign tmp_cycle=drx_cycle.

The required changes to the 3GPP MAC specifications for this solution are shown in Annex C.

Below we show an example for video traffic with generation rate 60 fps and no jitter. We set drx_offset=3, traffic_time_offset=2 ms, drx-LongCycle=16 ms, and drx-onDurationTimer=10 ms. In other words, after the  3rd DRX cycle, a time offset of 2 ms is added.

In Table 1, the midle columns show how the difference between the data arrival and start of DRX cycle increases as a result of the non-integer periodicities. The right-side column shows the result after the proposed solution is applied.

Table 1

| Application packet arrival [ms] |  | Problem with standard formula |  |  | Solution with enhanced formula |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | DRX cycle start [ms] | Difference (packet arrival – DRX start) [ms] |  | DRX cycle start [ms] | Difference (packet arrival – DRX start) [ms] |
| 0 |  | 0 | 0 |  | 0 | 0 |
| 16.66 |  | 16 | 0.66 |  | 16 | 0.66 |
| 33.33 |  | 32 | 1.33 |  | 32 | 1.33 |
| 50 |  | 48 | 2 |  | 50 | 0 |
| 66.66 |  | 64 | 2.66 |  | 66 | 0.66 |
| 83.33 |  | 80 | 3.33 |  | 82 | 1.33 |
| 100 |  | 96 | 4 |  | 100 | 0 |
| 116.66 |  | 112 | 4.66 |  | 116 | 0.66 |
| 133.33 |  | 128 | 5.33 |  | 132 | 1.33 |
| 150 |  | 144 | 6 |  | 150 | 0 |
| 166.66 |  | 160 | 6.66 |  | 166 | 0.66 |
| 183.33 |  | 176 | 7.33 |  | 182 | 1.33 |
| 200 |  | 192 | 8 |  | 200 | 0 |
| 216.66 |  | 208 | 8.66 |  | 216 | 0.66 |
| 233.33 |  | 224 | 9.33 |  | 232 | 1.33 |
| 250 |  | 240 | 10 |  | 250 | 0 |
| 266.66 |  | 256 | 10.66 |  | 266 | 0.66 |
| 283.33 |  | 272 | 11.33 |  | 282 | 1.33 |
| 300 |  | 288 | 12 |  | 300 | 0 |
| 316.66 |  | 304 | 12.66 |  | 316 | 0.66 |

Annex A presents simulation results for our alignment solution to match C-DRX to the non-integer video period. This solution is superior to both R15/16 Long DRX (in terms of PSG) and Short DRX (in terms of fraction of satisfied UEs). Thus, matching DRX to the video traffic period is beneficial, since it maintains a short traffic delay and thus a large fraction of satisfied UEs, while saving UE power.

Matching the DRX cycle with the non-integer video periodicity is a good solution to maintain a low delay, while saving UE power, for high network loads and high traffic generation rates.

Moreover, our solution to enhance the formula for Long DRX cycles has several advantages:

- It is flexible and supports numerous cycle lengths and alignments by means of only two additional parameters (traffic_time_offset and drx_offset) with integer values in ms. Thus, it can easily achieve DRX alignment with currently known and, additionally, future XR non-integer periodicities.

- It is expected that only few values need to be introduced for these two parameters, corrsponding to short of alignment offsets (traffic_time_offset) that need to applied frequently in time (i.e. every few cycles drx_offset).

- The two new parameters traffic_time_offset and drx_offset are configured at RRC and require no L1/L2 dynamic signaling, thus having a low signaling overhead.

- This solution has a small impact in terms of specification changes and can be easily configured to work as current DRX with integer cycle lengths, by not including the IE which would mean simply setting traffic_time_offset=0.

- Can be applied to work with Short DRX.

In contrast to our alignment solution, another solution modifying the CDRX formula by adding new rational cycle values for each non-integer traffic periodicity would require more specification changes. Namely, the new rational and existing integer DRX cycle values would have to be treated differently and different formulas or operations would be applied to start the drx-onDurationTimer. Additionally, this other solution is less future-proof, since the list of supported DRX cycle lengths at RRC would have to contain and be updated with all the exact rational traffic periodicities of any current/future XR application supported by the system. Furthermore, our results in Annex A show that our alignment solution achieves a nearly equal PSG and fraction of satisfied users as a DRX formula with rational cycle values. Consequently, this other formula is not superior to our solution.

Enhance Long DRX formula to match non-integer XR traffic periods as described in this section, by adding two new parameters: (i) a fixed time shift for the start of drx-onDurationTimer; and (ii) a number of DRX cycles after which the new shift should be added.

New integer values in ms for Long DRX cycle lengths (e.g. {8, 9, 11, …, 16, …33, …} ms), close to non-integer XR traffic periodicities are introduced.

### 2.2.2 Handling traffic jitter

In Release 17, RAN1 agreed that video traffic suffers from random jitter around its mean period (typically ±4 ms), covering a long range of 8 ms [2]. If this jitter range is not taken into account when configuring DRX, the video traffic may arrive randomly either before or after the onDuration. This traffic would be subsequently buffered until the next DRX cycle, which would then increase the delay.

A simple solution, using existing DRX functionality, is to configure the drx-onDurationTimer to run during the entire jitter range. However, given the long jitter range of 8 ms, the UE would monitor the PDCCH for a long time, which would lead to limited power saving gains, especially for video periods shorter than 16.66 ms.

It is necessary to enhance C-DRX to cope with traffic jitter, in order to save more UE power, while not increasing the traffic delay significantly.

To solve this, we propose to introduce a two-stage DRX solution. The two-stage DRX concept is illustrated in Figure 2. In principle, the UE does not need to continuously monitor PDCCH over the whole jitter range. It could be sufficient that the UE monitors a few short times during that range. In this case, existing DRX functionality could be used by configuring a Long DRX cycle as stated above, with the difference that the UE would monitor PDCCH only intermittently during that “OnDuration” time.

To achieve this behavior, two-stage DRX introduces Inner DRX cycles within an Outer DRX cycle. The outer onDurationTimer determines the period over which shorter Inner DRX cycles run. In other words, the inner onDurationTimer is started only as long as the outer onDurationTimer is running. The UE monitors the PDCCH only during the inner onDuration time of the Inner cycles.

Figure 2. Illustration of two-stage DRX.

The two-stage DRX solution has the following advantages when handling jitter:

- Its parameters (timers, cycle lengths, and offsets) are configured by RRC. This reduces the need for L1/L2 dynamic signaling and thus the resulting overhead, as well as potential network-UE synchronization loss due to dynamic signaling misdetection. By contrast, using  L1 PDCCH skipping instead two-stage DRX, to create the intermittent gaps in the onDuration, would require a significant amount of fast signalling. Namely a scheduling DCI with a skipping indication would have to be sent before each gap (every couple of ms), even if there is no data to be scheduled. Furthermore, the wanted skipping duration(s) would have to be included in the set of 2 or 3 duration options, thus leaving very few other duration options for other skipping indications, e.g. after all data has been transmitted.

- Can work jointly with DRX alignment solutions for non-integer traffic periodicities. For instance, the outer DRX cycle can be aligned with the traffic periodicity.

- It is a simple solution, since both the Outer and Inner DRX cycles are overall based on the concept of Long/Short DRX, which is an existing functionality. Only minor dependency enhancements would be needed.

Annex A shows simulation results for two-stage DRX, where the outer DRX cycle is configured to match the video traffic periodicity and the outer onDurationTimer covers the jitter range. Two-stage DRX saves significant UE power in all simulated cases, while achieving a short delay and thus a high percentage of satisifed UEs for low/moderate network loads.

Two-stage DRX is suitable for low/moderate network loads, where it saves significant UE power, while also achieving a short delay (and thus many satisfied UEs).

Based on the simulation study and observations above, the following is proposed.

Adopt the two-stage DRX solution to handle jitter for quasi-periodic XR traffic flows.

## 2.3 Discussion on PDCCH monitoring adaptation for jitter handling

Existing PDCCH monitoring adaptation features (SSSG switching and PDCCH skipping) are promising for saving power, but they could also introduce significant signaling overhead, since they are based on dynamic DCI signaling. Furthermore, SSSG switching also adds a latency to apply the new configuration. C-DRX based solutions could manage with less signaling overhead and less or null added switching delay, due to the overall (quasi-) deterministic characteristics of the XR traffic, for which semi-static PDCCH monitoring patterns with C-DRX are suitable.

Nonetheless, some minor SSSG switching enhancements, that may have some impact on CDRX, can be very useful to handle jitter. These enhancements work jointly with Long DRX and aim to reduce the SSSG switching delay and amount of DCI signalling. Specifically, during the onDuration, instead of continuously monitoring, the UE could apply a given SSSG with sparse PDCCH monitoring. This SSSG would implicitly apply when the drx-onDurationTimer starts. If and when traffic arrives, the UE would implicitly switch to a different SSSG allowing dense PDCCH monitoring. This requires that the NW indicates the SSSG group associated to the DRX cycle start and also the SSSG group to apply when data arrives.

Additionally, the PDCCH monitoring formula (for search space sets) needs to take into account the current DRX cycle start, so that the PDCCH monitoring pattern is aligned with the current DRX cycle and thus the same pattern offset occurs in every DRX cycle. This ensures that the same delay statistics are expected in every DRX cycle.

Enhance SSSG switching to address jitter so that:
- an SSSG applies at the start of drx-OnDuration and another SSSG applies when PDCCH indicates data; 
- align the search space set monitoring pattern w.r.t. the DRX cycle.

# 3 Text proposal for TR 38.835

Adopt the text proposal below for Section 5.2.2 of TR 38.835.

“Aligning DRX to the non-integer traffic periodicity is studied and is identified to be beneficial for XR video traffic. A simple solution for this is to enhance the DRX formula by adding two new parameters: (i) a fixed time shift for the start of drx-onDurationTimer; and (ii) a number of DRX cycles after which the new shift should be added. Additionally, introducing new integer values in ms for Long DRX cycle lengths (e.g. {8, 9, 11, …, 16, …33, …} ms), close to non-integer XR traffic periodicities is beneficial to ensure a tighter alignment of DRX with XR traffic.

Handling XR traffic jitter is needed and several solutions are studied. It is concluded that the following solutions are suitable:

- DRX enhancement supporting two stages of cycles (i.e., inner cycles within outer cycles);

- Joint SSSG switching and DRX enhancement: implicitly applying a specific SSSG at the start of drx-onDurationTimer and another SSSG when PDCCH indicates data; and aligning the search space set monitoring pattern w.r.t. the DRX cycle.”

# Conclusion

In the previous sections we made the following observations:

Observation 1	Depending on the network load and the traffic generation rate, different C-DRX solutions need to be applied, to maximize power savings and also achieve a high fraction of satisfied UEs.

Observation 2	In order to enhance and/or configure C-DRX power saving features, the network needs per-XR flow information:  - application packet periodicity and periodicity changes;  - application packet jitter information;  - delay budget or remaining PDB of the application packet for radio interface;  - application packet sequence number carried in each constituent PDU; and - application packet size.

Observation 3	Matching the DRX cycle with the non-integer video periodicity is a good solution to maintain a low delay, while saving UE power, for high network loads and high traffic generation rates.

Observation 4	It is necessary to enhance C-DRX to cope with traffic jitter, in order to save more UE power, while not increasing the traffic delay significantly.

Observation 5	Two-stage DRX is suitable for low/moderate network loads, where it saves significant UE power, while also achieving a short delay (and thus many satisfied UEs).

Based on the discussion in the previous sections we propose the following:

Proposal 1	Enhance Long DRX formula to match non-integer XR traffic periods as described in this section, by adding two new parameters: (i) a fixed time shift for the start of drx-onDurationTimer; and (ii) a number of DRX cycles after which the new shift should be added.

Proposal 2	New integer values in ms for Long DRX cycle lengths (e.g. {8, 9, 11, …, 16, …33, …} ms), close to non-integer XR traffic periodicities are introduced.

Proposal 3	Adopt the two-stage DRX solution to handle jitter for quasi-periodic XR traffic flows.

Proposal 4	Enhance SSSG switching to address jitter so that: - an SSSG applies at the start of drx-OnDuration and another SSSG applies when PDCCH indicates data;  - align the search space set monitoring pattern w.r.t. the DRX cycle.

Proposal 5	Adopt the text proposal below for Section 5.2.2 of TR 38.835.

# References

R2-2208702, “Report on LTE/NR17 legacy, XR, MUSIM, WUS, QoE and NW energy”, TSG RAN WG2 meeting #119-e, Online, 17 - 26 August, 2022.

3GPP TR 38.838, “Study on XR (Extended Reality) Evaluations for NR (Release 17)”, V17.0.0, Dec. 2021.

3GPP TR 38.840, “Study on User Equipment (UE) power saving in NR”, V16.0.0, June 2019.

3GPP TS 38.321, “Medium Access Control (MAC) protocol specification”, V17.0.0, Mar. 2022.

R1-2205054, “Power Saving Techniques for XR”, Qualcomm Incorporated, RAN WG1 #109-e, May 9–20, 2022.

R2-1903001, “Report of 3GPP TSG RAN WG2 meeting #105 Athens, Greece, 25 February - 1 March, 2019”.

# Annex A: Simulation results

In this annex we summarize our simulation results, based on the simulation methodology in [2] and the assumptions in Annex B. The UE power saving gain (PSG) is estimated vs. Always On and the capacity is estimated according to the definition adopted in [2], i.e. the maximum number of users per cell with at least 90% of UEs being satisfied, where a UE is satisfied if all its flows meet their own PDB requirements (99% of the application packets transmitted within their PDB).

Table 2: Results for CDRX, for FR1, high load, Dense Urban scenario, and VR multi-stream traffic: DL video (60 fps, 30 Mbps, ±4 ms jitter, 10 ms PDB), DL audio (10 ms periodicity, 30 ms PDB), UL pose (4 ms periodicity, 10 ms PDB)

| Tdoc # | Power Saving Scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | Load H/L | avg # UEs/Cell | floor (Capacity) | % of satisfied UE | Mean PSG of all UEs | Mean PSG of satisfied UEs |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| R1-2208401 | Always On | - | - | - | H | 8 | 8 | 90.1% | - | - |
| R1-2208401 | R15/16 DRX (Long DRX) | 10 | 8 | 4 | H | 8 | 7 | 82.6% | 4.2% | 4.1% |
| R1-2208401 | R15/16 DRX (Short DRX) | 4 | 2 | 4 | H | 8 | 5 | 69.8% | 9.8% | 9.7% |
| R1-2208401 | Matched CDRX (with our solution in Section 2.2.1) | 16.6 | 10 | 4 | H | 8 | 7 | 82.5% | 9.1% | 9.0% |
| R1-2208401 | Matched CDRX (solutions from other companies, e.g. rational cycle lengths) | 16.6 (equivalent 17-17-16) | 10 | 4 | H | 8 | 7 | 83.7% | 9.5% | 9.4% |
| R1-2208401 | Two-stage DRX | outer DRX: 16.6; inner DRX: 4 | outer ODT: 10; inner ODT: 2 | 4 | H | 8 | 5 | 71.7% | 14.2% | 14.1% |

Table 3 Results for two-stage DRX, for low load, FR1, Dense Urban scenario, and VR multi-stream traffic: DL video (60 fps, 30 Mbps, ±4 ms jitter, 10 ms PDB), DL audio (10 ms periodicity, 30 ms PDB), UL pose (4 ms periodicity, 10 sm PDB)

| Tdoc # | Power Saving Scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | Load H/L | avg # UEs/Cell | floor (Capacity) | % of satisfied UE | Mean PSG of all UEs | Mean PSG of satisfied UEs |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| R1-2208401 | Always On | - | - | - | L | 2 | 8 | 99.5% | - | - |
| R1-2208401 | R15/16 DRX (Long DRX) | 10 | 8 | 4 | L | 2 | - | 98.3% | 4.6% | 5.0% |
| R1-2208401 | R15/16 DRX (Short DRX) | 4 | 2 | 4 | L | 2 | - | 96.5% | 10.8% | 11.7% |
| R1-2208401 | Matched CDRX (with our solution in Section 2.2.1) | 16.6 | 10 | 4 | L | 2 | - | 97.0% | 10.2% | 10.9% |
| R1-2208401 | Two-stage DRX | outer DRX: 16.6; inner DRX: 4 | outer ODT: 10; inner ODT: 2 | 4 | L | 2 | - | 96.4% | 15.7% | 16.6% |

# Annex C: Simulation assumptions

For all simulation results three flows are assumed: one DL video flow (60 fps, ±4 ms jitter, PDB of 10 ms), one DL audio flow (10 ms periodicity, PDB of 30 ms), and one UL pose flow (4 ms periodicity, PDB of 10 ms). DG is configured for the two DL flows, whereas CG is configured for the UL pose flow, so this flow does not affect the DRX configuration.

The UE power consumption model is adopted from [3], with the additional UL power modelling in [2], namely linear interpolation method in linear scale for UE Tx power values other than 0 dBm and 23 dBm. Other parameters are summarized in Tables A.1 and A.2.

Table A.1: System simulation parameters

| Parameter | Deployment scenarios |
| --- | --- |
|  | Dense Urban (38.913 w/ following parameters) |
| Layout | 9cells with wraparound ISD: 200m |
| Channel model | UMa (38.901) |
| UE Distribution | 80% indoor, 20% outdoor |
| Carrier frequency | 4 GHz |
| Subcarrier spacing | 30 kHz |
| BS height | 25m |
| UE height | For Dense urban and Urban Macro, the UE height for indoor UEs is updated as following based on Table 6-1 in TR 36.873. UE height (hUT) in meters; general equation for UE height; hUT=3(nfl – 1) + 1.5. nfl for outdoor UEs; 1. nfl for indoor UEs; nfl ~ uniform(1,Nfl) where Nfl ~ uniform(4,8). |
| BS noise figure | 5 dB |
| UE noise figure | 9 dB |
| BS receiver | MMSE-IRC |
| UE receiver | MMSE-IRC |
| Channel estimation | Realistic |
| UE speed | 3 km/h |
| MCS | Up to 256QAM |
| BS Antenna Pattern | 3-sector antenna radiation pattern, 8 dBi |
| BS Antenna Configuration | 64 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1;4,8) (dH, dV) = (0.5λ, 0.5λ) |
| UE Antenna Pattern | Omni-directional, 0 dBi |
| UE Antenna Configuration | 2T/4R, (M, N, P, Mg, Ng; Mp, Np) = (1,2,2,1,1;1,2), (dH, dV) = (0.5, N/A)λ |
| Down Tilt | 12 degrees |
| BS Transmit Power | 44 dBm per 20 MHz Note: For system BW larger than above, Tx power scales up accordingly. |
| UE max tx power | 23dBm |
| System Bandwidth | 100 MHz |

Table A.2: Other assumptions

| TDD Configuration | DDDSU |
| --- | --- |
| Scheduler | MU-MIMO PF scheduler |
| PHY processing delay | UE PDSCH processing Capability #1 DL NACK to retransmission delay 1.5ms |
| DMRS overhead | 1 DMRS symbol per PDSCH |
| Power control parameter | alpha: 0.8 |
| Transmission scheme | Reciprocity-based precoding |
| Configured Grant | On |

# Annex C: Proposed modifications to integrate DRX alignment solution in 3GPP specifications

We propose to modify the 3GPP MAC specifications in [6] to capture our DRX alignment solution in Section 2.2.1, as follows. The proposed modifications are marked in red.

1> if the Long DRX cycle is used for a DRX group, and [SFN × 10  + subframe number] modulo (drx-LongCycle) = (drx-StartOffset + n × traffic_time_offset) modulo (drx-LongCycle) (See Annex … for details on how to use the formula.)