3GPP TSG RAN WG2 #119bis-e	R2-2210772

Online, October 10th – 19th, 2022	(is revision of R2-2210656)

Source: 	MediaTek Inc.

Title: 	Considerations on network energy saving techniques

Agenda item:	8.3.2

Document for:	Discussion and Decision

Introduction

In RAN1 #109-e, #110 and RAN Plenary meetings, there agreed basic network evaluation methodology and categorized potential network energy saving techniques of different domain [1][2][3][4]. As per cares and concerns we expressed for some network energy saving techniques in the post RAN2#119-e meeting discussion [5], in this contribution, RAN1 related evaluations [6] are conducted to characterize selected network energy saving schemes in time, frequency, spatial and power domains. From the useful power saving schemes identified, there also reveals the need of efficient adaptation framework(s) to realize the benefit of network energy saving over different network load conditions, by considering UE-group/cell-wise adaptation and practical UE support, for technical direction considerations and decision in RAN2.

Time Domain – Alignment of DRX offsets

- When the traffic load is low, e.g., mainly voice activity, it can be beneficial to aggregate BS transmission time intervals. Since UE expects data activity only within DRX on-durations, such BS transmission time aggregation implies alignment of UE DRX offsets, in a cell-specific or group-specific manner as illustrated in Figure 1. Note that, since DRX cycle, on-duration timer, and inactivity-timer settings are typically optimized per service QoS requirement, it will be more reasonable that the adaptation on UE DRX only changes the DRX offset value.

Figure 1: Illustration of aggregated BS activity by aligning DRX offsets across UEs of a group or a cell

- To justify the benefit based on the agreed network power consumption model, the following evaluation assumptions are considered:

- Scenario: Urban Macro (ISD: 500m) in FR1 of 30kHz; single carrier case

  - Case 1: Random DRX offset per UE (5 ms granularity)

  - Case 2: Aligned DRX offset to 0 for UEs of a group or a cell

- Traffic: VoIP with DRX (cycle, on-duration, inactivity timer) = (40, 4, 10) ms

  - Note: VoIP is considered to simulate low network loading condition while user activities are still frequent

- RU: Low load (0% - 15%)

- In Figure 2, there show the network energy saving gains for Cat 1 and Cat 2 types of BS’s. Accordingly, we also have the following:

Observation 1: For the case of low network load (0% - 15%) while there are still (frequent) user activities (e.g., VoIP), aligning UE DRX offset for aggregated BS activity can achieve good power saving gain, i.e., >28% for Cat 1 BS and >10% for Cat 2 BS.

Proposal 1: Aligning UE DRX offsets in a group-specific or cell-specific manner is recommended for network energy saving.

Ref.

10.6%↓

Ref.

28.5%↓

Figure 2: BS power consumption comparison with aligned UE DRX offsets (VoIP traffic)

Frequency Domain – SSB/SIB1-less SCell

When a cell is used as a capacity booster rather than providing coverage, the cell could be defined as a secondary cell (SCell), and a gNB may turn off SSB or SIB1 transmission on the SCell for network energy savings.

For evaluating the potential benefits, the following settings are considered:

- Scenario: Urban Macro (ISD: 500m) in FR1 of 30kHz with PCell and one SCell; multi-carrier case (inter-band CA)

  - Case 1: SSB, SIB1, and data for SCell

  - Case 2: SSB and data for SCell

  - Case 3: Only data for SCell

- SSB: 20 RBs, 4 symbols, 2 SSBs per slot, and 4 consecutive slots at periodicity of 20 ms

- SIB1: 48 RBs, 8 consecutive slots at periodicity of 80 ms, multiplexing pattern 1 with SSB

- Traffic: Video traffic with DRX (cycle, on-duration, inactivity timer) = (160, 8, 100) ms

  - Note: Video traffic considered since CA is typically for scenarios with higher data activity

- RU: Light load (15% - 30%)

In Figure 3, there show the network energy saving gains for Cat 1 and Cat 2 types of BS’s, and we have

Observation 2: For CA use cases with higher data activity, disabling SSB and/or SIB1 for SCell achieves very limited energy saving gains, i.e., <8% for Cat 1 BS and < 1% for Cat 2 BS.

Proposal 2: Disabling SSB and/or SIB1 for SCell is NOT pursued for network energy saving.

3.0%↓

Ref.

7.4%↓

0.3%↓

0.1%↓

Ref.

Figure 3: BS power consumption comparison with SSB/SIB1-less SCell (video traffic for CA setting)

Furthermore, BWP adaptation framework has been developed in Rel-15 and widely supported by UEs. When the network load is low (0% - 15%), it is possible to adjust BS transmission BW to reduce BS power consumption even though we see no benefit in SSB/SIB1-less SCell proposal here. Since BWP adaptation trigger is UE specific, there will be concern on the signaling overhead and the corresponding BS power consumption that can reduce the benefit of network energy saving. By the above the following proposals are suggested:

Proposal 3: Enhancements to enable UE group-common or cell-specific BWP configuration and /or switching is recommended for network energy saving.

Spatial Domain – Reduced TxRU number

When network load is light (15% - 30%) or medium (30% - 50%), reducing BS active time may not be practical due to the impact to data latency. In this regard, reducing the number of TxRU while keeping data transmission can be utilized to achieve better trade-off in network energy saving and data latency. To characterize the trade-off, the following evaluation settings are considered:

- Scenario: Urban Macro (ISD: 500m) in FR1 of 30kHz; single carrier setting; same PSD per TxRU

  - Case 1: 64 TxRUs of (M, N, P, Mg, Ng; Mp, Np) = (8, 16, 2, 1, 1; 2, 16)

  - Case 2: 32 TxRUs of (M, N, P, Mg, Ng; Mp, Np) = (8, 8, 2, 1, 1; 2, 8)

  - Case 3: 16 TxRUs of (M, N, P, Mg, Ng; Mp, Np) = (4, 8, 2, 1, 1; 1, 8)

  - M: no. vertical antenna elements within a panel, on one polarization; N: no. horizontal antenna elements within a panel, on one polarization; P: no. polarizations; Mg: no. panels in a column; Ng: no. panels in a row; Mp: no. vertical TxRUs within a panel, on one polarization; Np: no. horizontal TxRUs within a panel, on one polarization.

- Traffic: Video traffic with DRX (cycle, on-duration, inactivity timer) = (160, 8, 100) ms

  - Note: Video traffic with more UEs utilized to simulate higher network load conditions

- RU: Light load (15% - 30%) and medium load (30% - 50%)

In Figure 4 and Figure 5, there show the comparison of BS power consumption and data latency with reduced #TxRU for light load and medium load cases, respectively. Accordingly, one can check:

Observation 3: For the NW scenario with light load (15% - 30%), reducing #TxRU from 64 to 32 can bring 15.8% and 16.3% network energy saving gain, respectively, for Cat 1 BS and Cat 2 BS, subject to 4.8% increment in average data packet latency. Further reducing #TxRU to 16 only bring <6% additional energy saving gain while causing >15% data latency increment.

Ref.

21.7%↓

20.3%↑

4.8%↑

Ref.

19.1%↓

Ref.

15.8%↓

16.3%↓

Figure 4: BS power consumption and data latency comparison with reduced #TxRU in light load (15% - 30%) case with video traffic

Observation 4: For the NW scenario with medium load (30% - 50%), reducing #TxRU from 64 to 32 can bring 25.3% and 26.8% network energy saving gain, respectively, for Cat 1 BS and Cat 2 BS, subject to 6.8% increment in average data packet latency. Further reducing #TxRU to 16 only bring <10% additional energy saving gain while causing >70% data latency increment.

Ref.

6.8%↑

31.3%↓

25.3%↓

81.1%↑

36.6%↓

Ref.

Ref.

26.8%↓

Figure 5: BS power consumption and data latency comparison with reduced #TxRU in medium load (30% - 50%) case with video traffic

Given the good trade-off in network energy gain and data latency achieved by reducing #TxRU, this network energy saving scheme is recommended. When #TxRU is changed, the settings of CSI-RS and CSI reporting should also be adapted accordingly. Given these parameters are BWP specific, how to extend BWP framework to accommodate changing #TxRU in a UE-group-specific or cell-specific manner can be further investigated. The following two proposals are then suggested:

Proposal 4: Reducing #TxRU by a limited factor is recommended for network energy saving.

Proposal 5: Further investigate how to extend BWP framework to accommodate changing #TxRU in a UE-group-specific or cell-specific manner.

- At least CSI-RS and CSI reporting related settings should be adapted accordingly

Power Domain – Reduced PDSCH power/PSD-level

Like reducing #TxRU, reducing PDSCH power/PSD-level while keeping data transmission can be utilized to achieve network energy saving while minimizing the data latency impact. To characterize the trade-off, the following evaluation settings are considered:

- Scenario: Urban Macro (ISD: 500m) in FR1 of 30kHz; single carrier setting; #TxRUs is 64

  - Case 1: Reference PDSCH power/PSD-level subject to maximum TX power limit

  - Case 2: PDSCH power/PSD-level is reduced by 3dB w.r.t. Reference/Case 1

  - Case 3: PDSCH power/PSD-level is reduced by 6dB w.r.t. Reference/Case 1

  - Case 4: PDSCH power/PSD-level is reduced by 9dB w.r.t. Reference/Case 1

  - Note: PDSCH power/PSD-level can be adjusted via BWP-specific parameter, powerControlOffset, as illustrated below:

- Traffic: Video traffic with DRX (cycle, on-duration, inactivity timer) = (160, 8, 100) ms

  - Note: Video traffic with more UEs utilized to simulate higher network load conditions

- RU: Light load (15% - 30%) and medium load (30% - 50%)

In Figure 7 and Figure 8, there show the comparison of BS power consumption and data latency with reduced PDSCH power/PSD-level for light load and medium load cases, respectively. Accordingly, one can check the following:

Observation 5: For the NW scenario with light load (15% - 30%), reducing PDSCH power/PSD-level by 6dB can bring 17% netowrk energy saving gain for Cat 1 BS and Cat 2 BS, subject to 6% increment in average data packet latency. On the other hand, further PDSCH power/PSD-level reduction brings ≤1% additional energy saving gain while causing ≥6% data latency increment.

12%↑

6%↑

2%↑

Ref.

11%↓

17%↓

18%↓

Ref.

17%↓

12%↓

Ref.

17%↓

Figure 7: BS power consumption and data latency comparison with reduced PDSCH power/PSD-level in light load (15% - 30%) case with video traffic

Observation 6: For the NW scenario with medium load (30% - 50%), reducing PDSCH power/PSD-level by 6dB can bring ≥26% network energy saving gain for Cat 1 BS and Cat 2 BS, subject to 10% increment in average data packet latency. On the other hand, further PDSCH power/PSD-level reduction brings ≤3% additional energy saving gain while causing ≥14% data latency increment.

24%↑

Ref.

Ref.

10%↑

3%↑

Ref.

30%↓

27%↓

19%↓

26%↓

28%↓

19%↓

Figure 8: BS power consumption and data latency comparison with reduced PDSCH power/PSD-level in medium load (30% - 50%) case with video traffic

Given the good trade-off in network energy gain and data latency achieved by reducing PDSCH power/PSD-level (e.g., by 6dB), this network energy saving scheme is also recommended. Given PDSCH power/PSD-level adjustment can be done via the BWP specific parameter, powerControlOffset, how to extend BWP framework to accommodate changing PDSCH power/PSD-level in a UE-group-specific or cell-specific manner can be further investigated as well. By the above, the following two proposals are suggested:

Proposal 6: Reducing PDSCH power/PSD-level by a limited factor is recommended for network energy saving.

Proposal 7: Further investigate how to extend BWP framework to accommodate changing PDSCH power/PSD-level in a UE-group-specific or cell-specific manner.

Considerations on Adaptation Framework

- From the above evaluations, there identified the following useful network energy saving techniques with justified ES gains:

| - # | - Domain | - Technique | - Network scenarios | - Network energy saving gain | - Impacted UE config. |
| --- | --- | --- | --- | --- | --- |
| - A) | - Time | - Alignment of DRX offsets | - Low load (0% – 15%) | - Cat 1 BS: 28% - Cat 2 BS: 10% | - DRX offset value |
| - B) | - Spatial | - Reduced TxRU number | - Light load (15% - 30%), Medium load (30% - 50%) | - Cat 1 BS: 15% - 25% - Cat 2 BS: 16% - 26% | - CSI-RS and CSI reporting settings |
| - C) | - Power | - Reduced PDSCH power/PSD-level |  | - Cat 1 BS: 17% - 26% - Cat 2 BS: 17% - 27% | - PDSCH power offset, powerControlOffset |

Table 1: Useful network energy saving techniques

To achieve network energy saving with minimum impact to UEs, all UEs in a cell should be aware of the change, and the related configurations, including DRX offset, CSI-RS/CSI-reporting settings, and PDSCH power offset, should be adapted accordingly. Since the configurations are UE specific settings, how to efficiently adapt the parameters in UE-group/cell-wise manner should be further developed; Otherwise, the signaling overhead and power consumption will defeat the benefit of NW energy savings.

Proposal 8: Efficient UE-group/cell-wise signaling and adaptation mechanism should be developed for useful network energy saving techniques; Otherwise, the signaling overhead and power consumption will defeat the energy saving benefits.

To achieve effective UE-group/cell-wise support, wide UE support is essential. From previous characterization, the BWP framework can be utilized for schemes B) and C), regarding the impacted UE settings/parameters are BWP specific. Since the framework is developed since Rel-15, there can ensure comprehensive UE support. Furthermore, a new NES state may not be necessary in consideration of the flexibility of the BWP framework. In this regard, the following proposal is suggested:

Proposal 9: For maximum UE support, extend Rel-15 BWP adaptation framework as the UE-group/cell-wise signaling and adaptation mechanism for network energy saving so that a new NES state is avoidable.

Conclusion

In this contribution, we have the following observations and proposals: (blue-colored ones are directly related to RAN2 scope)

Observation 1: For the case of low network load (0% - 15%) while there are still (frequent) user activities (e.g., VoIP), aligning UE DRX offset for aggregated BS activity can achieve good power saving gain, i.e., >28% for Cat 1 BS and >10% for Cat 2 BS.

Proposal 1: Aligning UE DRX offsets in a group-specific or cell-specific manner is recommended for network energy saving.

Observation 2: For CA use cases with higher data activity, disabling SSB and/or SIB1 for SCell achieves very limited energy saving gains, i.e., <8% for Cat 1 BS and < 1% for Cat 2 BS.

Proposal 2: Disabling SSB and/or SIB1 for SCell is NOT pursued for network energy saving.

Proposal 3: Enhancements to enable UE group-common or cell-specific BWP configuration and /or switching is recommended for network energy saving.

Observation 3: For the NW scenario with light load (15% - 30%), reducing #TxRU from 64 to 32 can bring 15.8% and 16.3% network energy saving gain, respectively, for Cat 1 BS and Cat 2 BS, subject to 4.8% increment in average data packet latency. Further reducing #TxRU to 16 only bring <6% additional energy saving gain while causing >15% data latency increment.

Observation 4: For the NW scenario with medium load (30% - 50%), reducing #TxRU from 64 to 32 can bring 25.3% and 26.8% network energy saving gain, respectively, for Cat 1 BS and Cat 2 BS, subject to 6.8% increment in average data packet latency. Further reducing #TxRU to 16 only bring <10% additional energy saving gain while causing >70% data latency increment.

Proposal 4: Reducing #TxRU by a limited factor is recommended for network energy saving.

Proposal 5: Further investigate how to extend BWP framework to accommodate changing #TxRU in a UE-group-specific or cell-specific manner.

- At least CSI-RS and CSI reporting related settings should be adapted accordingly

Observation 5: For the NW scenario with light load (15% - 30%), reducing PDSCH power/PSD-level by 6dB can bring 17% netowrk energy saving gain for Cat 1 BS and Cat 2 BS, subject to 6% increment in average data packet latency. On the other hand, further PDSCH power/PSD-level reduction brings ≤1% additional energy saving gain while causing ≥6% data latency increment.

Observation 6: For the NW scenario with medium load (30% - 50%), reducing PDSCH power/PSD-level by 6dB can bring ≥26% network energy saving gain for Cat 1 BS and Cat 2 BS, subject to 10% increment in average data packet latency. On the other hand, further PDSCH power/PSD-level reduction brings ≤3% additional energy saving gain while causing ≥14% data latency increment.

Proposal 6: Reducing PDSCH power/PSD-level by a limited factor is recommended for network energy saving.

Proposal 7: Further investigate how to extend BWP framework to accommodate changing PDSCH power/PSD-level in a UE-group-specific or cell-specific manner.

Proposal 8: Efficient UE-group/cell-wise signaling and adaptation mechanism should be developed for useful network energy saving techniques; Otherwise, the signaling overhead and power consumption will defeat the energy saving benefits.

Proposal 9: For maximum UE support, extend Rel-15 BWP adaptation framework as the UE-group/cell-wise signaling and adaptation mechanism for network energy saving so that a new NES state is avoidable.

Reference

RP-213554, 	“New SI: Study on network energy savings for NR”, RAN#94-e, Rapporteur: Huawei

RP-221443,	“SID revision: Study on network energy savings for NR”, RAN#96-e, Rapporteur: Huawei

RP-221442,	“Status report for WI: Study on network energy savings for NR”, RAN#96-e, Rapporteur: Huawei

RP-222546,	“Status report for SI Study on network energy savings for NR”, RAN#97-e, Rapporteur: Huawei

R2-2210417	Report of [POST119-e][313][NES] Details of solutions (Huawei)

R1-2209501	On network energy savings techniques, RAN1#110-bis-e, MediaTek Inc.