3GPP TSG-RAN WG2 Meeting #119bis-e		R2-22xxxxx

- Online, October 10-19, 2022

Title:	LS on L1 measurement, beam indication, RRC, terminology and dynamic cell switching

Response to:

Release:	Rel-18

Work Item:	NR_mob_enh2-Core

Source:	RAN2

To:	RAN1, RAN3, RAN4

Cc:

      - Contact Person:	Li-Chuan TSENG

      - E-mail Address:	li-chuan.tseng@mediatek.com

1. Overall Description:

Regarding L1/L2-based inter-cell mobility (Objective #1) in Rel-18 NR further mobility enhancement, RAN2 made the following agreements and assumptions at RAN2#119bis meeting on L1 measurements, beam indication, terminology and dynamic cell switch:

- Terminology - RAN2 to use “LTM” as term for the L1/L2-triggered mobility. - Use the term “cell switch” for the procedure of triggering change of cells via the LTM feature - Use the term “Subsequent” LTM for the case when cell switch between L1/L2 mobility candidates is done without RRC reconfiguration in between. - L1 measurements and beam indication - RAN2 assumes that RAN1 will drive discussions on L1 measurement enhancements, if any. If RAN1 identifies the need for e.g. event reporting, filtering etc, RAN2 can then be involved if needed. - Inter-freq L1L2 mobility: R2 Confirms that For L1L2 mobility inter-freq scenarios in general should be supported (including mobility to inter-frequency cell that is not a current serving cell), including the support of inter-frequency L1 measurements, if feasible by R4 and R1. - RAN2 assumes that whether to use the unified TCI framework as the baseline for beam indication for L1L2 mobility is up to RAN1 (RAN2 observes that L1/L2 mobility need to support inter-freq cases). - RRC - A L1/L2 inter-cell mobility candidate (target) configuration is received within an RRC message before the L1/L2 dynamic switch is triggered. - For L1L2 mobility, Target Pcell/SCell can be current SCell/PCell, i.e., current SCell/PCell can be configured as candidates. - RAN2 assumes that sequential L1L2 cell change between Candidates without RRC reconfiguration can be supported. - Dynamic cell switching - RAN2 assumes L1/2 mobility trigger information is conveyed in a MAC CE, FFS if the MAC CE or a DCI is used for the actual triggering. - RAN2 assumes the MAC CE for L1/2 mobility trigger contains at least a candidate configuration index. - FFS if it should be possible to perform SCell activation/deactivation (amongst SCells associated with the candidate configuration) simultaneously with L1 L2 mobility trigger MAC CE (if so, FFS how this is determined). - RAN2 assumes that both RACH-based (CFRA, CBRA) and RACH-less procedures for L1 L2 mobility switch may be supported. RACH-less if the UE doesn’t need to acquire TA during the cell switch. RAN2 understands that the feasibility of RACH-less may depend on RAN1, and expect that RAN1 is working on this. - RAN2 assumes RACH resource for CFRA for L1 L2 dynamic switch may be provided in RRC configuration (or potentially by MAC CE FFS). - FFS if the MAC CE can indicate TCI state(s) (or other beam info) to activate for the target Cell(s), dep on RAN1 progress. - R2 assumes that at L1L2 cell switch: Whether the UE performs partial or full MAC reset (FFS what partial reset is, e.g. to avoid data loss), re-establish RLC, perform data recovery with PDCP is explicitly controlled by the network. R2 assumes that this can be configured by RRC. FFS if MAC CE indication(s) is/are needed.

2. Actions

To RAN1, RAN3 and RAN4 groups

ACTION: 	RAN2 kindly asks RAN1, RAN3 and RAN4 to take the above information into consideration in their work on L1/L2-based inter-cell mobility.

3. Date of Next RAN2 Meetings:

TSG RAN WG2 Meeting #120		14 – 18 November 2022		Toulouse FR

TSG RAN WG2 Meeting #121		27 February – 3 March 2023		Athens GR