3GPP TSG-RAN WG2 Meeting #119bis-e                                                 	R2-220xxxx

E-Meeting: October 10-19, 2022

Agenda item:	8.7.2

Source:	Qualcomm Incorporated

Title:	[offline-103] Coverage enhancements

Document for:	Discussion and Decision

# Introduction

This document provides the summary for the following email discussion.

- [AT119bis-e][103][NR NTN Enh] Coverage enhancements (Qualcomm)

Initial scope: Discuss the proposals in the submitted contributions in AI 8.7.2 (apart from those on msg3 repetition enhancements)

Initial intended outcome: Summary of the offline discussion with e.g.:

- List of proposals for agreement (if any)

- List of proposals that require online discussions

- List of proposals that should not be pursued (if any)

Initial deadline (for companies' feedback): Thursday 2022-10-13 1600 UTC

Initial deadline (for rapporteur's summary in R2-2210842): Thursday 2022-10-13 1800 UTC

# Discussion

In RAN#97, the objective of the WID is updated further to clarify the objective as follows.

4.1.1	Coverage enhancement

The Rel-18 NTN objectives are focused on the applicability of the solutions developed by general NR coverage enhancement to NTN, and identifying potential issues and enhancements if necessary, considering the NTN characteristics including large propagation delay and satellite movement. Only NTN-specific characteristics are to be included in this coverage enhancement work, otherwise it should be part of another WI (e.g., UL enhancement of coverage).

The following sentence will be revisited in RAN#99 as part of the DL enhancements discussion:

“The evaluation should also take into account any related regulatory requirements, e.g., ITU limitation of power flux density.” No work on this topic will take place in RAN WGs before the discussion on DL enhancements in RAN#99.

The following reference scenario is considered for the definition of uplink coverage enhancements for NTN: parameter set-1 for LEO-1200 satellite operating at Line of Sight (LOS) and commercial smartphones with -5.5 dBi antenna gain and 3 dB polarisation loss (per antenna port).

- Note: It is understood that the enhancements defined for LEO can also apply to GEO and MEO scenarios as appropriate. No additional work is expected for MEO/GEO.

The targeted services are VoIP using AMR 4.75 kbps and data transmission services with Low data rate of 3 kbps.

The detailed objectives are for NTN:

- To specify PUCCH enhancements for Msg4 HARQ-ACK (e.g. repetition) [RAN1, RAN4]

- To study DMRS bundling for PUSCH taking into account NTN-specifics (e.g. time-frequency pre-compensation) and, if necessary, specify enhancements to the Rel-17 procedures [RAN1]

Have a 1-TU 6-month study phase focusing on the following (to derive clear & limited scope):

- Evaluate the coverage performance and identify the candidate physical radio channels that have coverage issues specific to NTN with following target services taking into account the studies in TR38.830 where appropriate, as well as general coverage enhancement techniques specified in Rel-18 [RAN1,RAN2,RAN4]

  - VoIP and low-data rate services for commercial handset terminals

The following items are shown as examples of areas to consider in the RAN2 study.

- Improved performance of low-rate codecs in link budget limited situation including reducing RAN protocol overhead for VoNR

  - NOTE: Intent is not to introduce a new codec.

RAN to determine by RAN#97 (for RAN1 items) and RAN#98 (for RAN2 items) whether the study phase has identified any need for NTN-specific coverage enhancements in Rel-18. If needed, the set of NTN-specific work item objectives will be further updated.

As highlighted above, RAN has clearly identified the items for RAN2 study. The target service in the highlighted is NTN specific. For TN, we should target a high-quality high codec rate voice service.

It is important to increase the number of repetitions per bit to enhance coverage in NTN due to limited link budget. This can be done several ways and one of them is to reduce the overhead. Basically, two possible solutions have been proposed to achieve this. Note that the two solutions can be combined.

- Packet aggregation and,

- Protocol overhead reduction.

Packet Aggregation

| Tdoc | Packet aggregation |
| --- | --- |
| [1] R2-2209709 | Proposal 1: For coverage enhancement based on application layer frame aggregation, RAN sends the trigger command for the voice frame aggregation (i.e., 2 voice frames per packet) in NGSO. Send LS to SA4 for alignment work to process the command. Proposal 2: For coverage enhancements, study the details on specification change to support PDCP layer frame aggregation. |
| [2] R2-2210645 | Proposal 5: RAN2 to further study the packet aggregation mechanism in UE’s application layer and/or RAN layer(s). |
| [7] R2-2210033 | Proposal 4: Frame aggregation enhancement is not considered in RAN2. |

In [1] and [2], it is described that the voice traffic arrival rate can be 1 frame per packet per 20ms. However, to efficiently utilize the resource and save UE power and still meet the packet delay budget, 40ms CDRX can be used, i.e., voice packets are transmitted every 40ms, but as two separate packets, each incurring full headers. A simple solution is to aggregate two voice frames and reduce overhead by 50%. The 40ms interval enables the PUSCH repetition with number more than 20 to have a further coverage enhancement to fill the coverage gap.

- Do you agree that with scheduling interval of 40ms or more, the PUSCH repetition with number more than 20 is possible to have a further coverage enhancement?

| Company name | Yes/No | Comments |
| --- | --- | --- |

- A voice packet is generated every 20ms. Do you agree that with scheduling interval of 40ms, compared to transmission of two voice packets separately every 40ms (e.g., MAC layer multiplexing), protocol overhead is reduced when two voice packets are aggregated and transmitted?

| Company name | Yes/No | Comments |
| --- | --- | --- |

- If answer to Question 2 is yes, which option do you prefer to prioritize for further study?

- Application layer frame aggregation

- PDCP layer packet aggregation

| Company name | Option | Comments |
| --- | --- | --- |

- If answer to Question 1 or Question 2 is No, please elaborate the reason in comments.

| Company name | Comments |
| --- | --- |

Protocol header reduction

| Tdoc | Protocol overhead reduction |
| --- | --- |
| [4] R2-2209710 | Proposal 1: PDCP compression of ROHC header is used to save 1 byte from the ROHC header and CRC check is ignored at ROHC when PDCP compression of ROHC header is configured. Proposal 2: Introduce 1-byte PDCP header to be configured for voice DRB. Proposal 3: Network can configure not to use RLC header from a PDU associated with the DRB that is configured for voice traffic. Proposal 4 : Consider removing L field from MAC subheader of the PDU associated with the low data rate DRB. |
| [5] R2-2209804 | Proposal 1: RAN2 to discuss whether to consider the new L2 header design customized for voice. Proposal 2: If the new L2 header for the voice service is considered, RAN2 is suggested to discuss the method to distinguish the different L2 header formats for voice service and other service. |
| [6] R2-2209389 | Proposal 1: The mapping function of SDAP is not required for VoNR in NTN network. Proposal 2: VoNR can be configured as TM mode to reduce the protocol overhead. |
| [7] R2-2210033 | Proposal 1: No spec impact is needed for SDAP/PDCP/MAC to reduce RAN overhead for VoNR Proposal 2: RAN2 consider enhancement of RLC layer to reduce RLC header size for VoNR. Proposal 3: RAN2 to discuss whether length field of MAC header can be removed for VoNR. |
| [8] R2-2209508 | Proposal 1: RAN2 does not identify necessary/feasible enhancements to be supported on RAN overhead reduction for VoNR from RAN2 perspective. This could be considered as the RAN2 study phase outcome. |
| [3] R2-2210285 | Proposal 2: Protocol overhead reduction is not pursued for coverage enhancements. |
| [9] R2-2210566 | Proposal 1. RAN2 do not study the L2 header reduction in NTN. |

- Do you agree SDAP header and MAC-I field do not need to be configured for DRB carrying the voice traffic or low data rate traffic to reduce protocol overhead?

| Company name | Yes/No | Comments |
| --- | --- | --- |

The contributions [4], [5], [6] and [7] propose that RLC header can be reduced considering network can avoid segmentation with appropriate TBS allocation for voice traffic and due to long RTT, retransmission of voice packets can also be disabled.

- Do you agree to consider enhancement of RLC layer to reduce RLC header size for VoNR in NTN (for example, configuring RLC TM mode)?

| Company name | Yes/No | Comments |
| --- | --- | --- |

The contributions [4] and [5] propose that PDCP SN length of 12 bit or 18 bit is not necessary and 1-byte PDCP header can be considered since retransmission of voice packet is not expected and reordering window can be smaller.

- Do you agree to consider enhancement for 1-byte PDCP header for VoNR in NTN?

| Company name | Yes/No | Comments |
| --- | --- | --- |

The contributions [4] and [5] propose that the L field from MAC header can also be removed by a new spare 6 bit LCID code point as the VoNR packet size can be known with the given codec rate.

- Do you agree to consider enhancement for 1-byte MAC header for VoNR in NTN?

| Company name | Yes/No | Comments |
| --- | --- | --- |

Contribution [4] proposes that PDCP compression of ROHC header can be used to save 1 byte from the ROHC header and CRC check can be ignored at ROHC when PDCP compression of ROHC header is configured. As 16 bit CRC check in PHY layer is used, it can be clarified that when PDCP compression of ROHC header is used, the CRC check is ignored at ROHC protocol. The SN in ROHC can be associated with the PDCP SN.

- Do you agree to associate the SN in ROHC header with the PDCP SN to reduce 1 byte overhead for VoNR in NTN?

| Company name | Yes/No | Comments |
| --- | --- | --- |

Contributions [8], [3] and [9] propose not to consider protocol overhead reduction.

- Do you agree RAN2 should not consider any of the options in Q5, Q6, Q7, Q8 and Q9 as L2 protocol header reduction for coverage enhancement in NTN?

| Company name | Yes/No | Comments |
| --- | --- | --- |

Coverage enhancement in general

| Tdoc | Coverage enhancement in general |
| --- | --- |
| [3] R2-2210285 | Proposal 1: RAN2 wait until more progress in RAN1 to discuss whether anything needs to be done in RAN to trigger voice frame aggregation. |
| [10] R2-2209406 | Proposal 5: Other coverage enhancement specific for NTN in RAN2 can be studied after more inputs achieved in RAN1. |
| [11] R2-2210685 | Proposal 1: RAN2 to postpone the discussion of high layer coverage enhancements and wait for further input from RAN1. |

- Do you agree RAN2 should wait for more input from RAN1 for guidance in protocol overhead reduction study?

| Company name | Yes/No | Comments |
| --- | --- | --- |

- If your answer to Q11 is yes, do you agree to send LS to RAN1 to ask feedback?

| Company name | Yes/No | Comments |
| --- | --- | --- |

# Conclusion

Following proposals are made [to be updated…].

No table of contents entries found.

# References

[1] R2-2209709	Frame aggregation for coverage enhancement	Qualcomm Incorporated	discussion	Rel-18	NR_NTN_enh

[2] R2-2210645	Discussion on Coverage Enhancements for NR NTN	Nokia, Nokia Shanghai Bell	discussion	Rel-18	NR_NTN_enh

[3] R2-2210285	Consideration on coverage enhancements	ZTE Corporation, Sanechips	discussion	Rel-18

[4] R2-2209710	Protocol overhead reduction for coverage enhancement	Qualcomm Incorporated	discussion	Rel-18	NR_NTN_enh

[5] R2-2209804	Consideration on NTN Coverage Enhancement	Apple	discussion	Rel-18	NR_NTN_enh-Core

[6] R2-2209389	Discussion on coverage enhancement in NR NTN	CAICT	discussion	Rel-18	NR_NTN_enh-Core

[7] R2-2210033	Discussion on coverage enhancement for NR NTN	Xiaomi	discussion	Rel-18

[8] R2-2209508	Discussion on RAN overhead reduction for VoNR support in NTN	vivo	discussion

[9] R2-2210566	Discussion on the L2 header reduction in NTN	LG Electronics Inc.	discussion	NR_NTN_enh-Core

[10] R2-2209406	Discussion on NTN Coverage Enhancement	CATT	discussion	Rel-18	NR_NTN_enh

[11] R2-2210685	Discussion on RAN protocol overhead reduction	Huawei, HiSilicon	discussion	Rel-18	NR_NTN_enh