3GPP TSG-RAN WG2 Meeting #119bis-e                                                 	R2-220xxxx

E-Meeting: October 10-19, 2022

Agenda item:	8.7.2

Source:	Qualcomm Incorporated

Title:	[offline-103] Coverage enhancements

Document for:	Discussion and Decision

# Introduction

This document provides the summary for the following email discussion.

- [AT119bis-e][103][NR NTN Enh] Coverage enhancements (Qualcomm)

Initial scope: Discuss the proposals in the submitted contributions in AI 8.7.2 (apart from those on msg3 repetition enhancements)

Initial intended outcome: Summary of the offline discussion with e.g.:

- List of proposals for agreement (if any)

- List of proposals that require online discussions

- List of proposals that should not be pursued (if any)

Initial deadline (for companies' feedback): Thursday 2022-10-13 1600 UTC

Initial deadline (for rapporteur's summary in R2-2210842): Thursday 2022-10-13 1800 UTC

# Discussion

In RAN#97, the objective of the WID is updated further to clarify the objective as follows.

4.1.1	Coverage enhancement

The Rel-18 NTN objectives are focused on the applicability of the solutions developed by general NR coverage enhancement to NTN, and identifying potential issues and enhancements if necessary, considering the NTN characteristics including large propagation delay and satellite movement. Only NTN-specific characteristics are to be included in this coverage enhancement work, otherwise it should be part of another WI (e.g., UL enhancement of coverage).

The following sentence will be revisited in RAN#99 as part of the DL enhancements discussion:

“The evaluation should also take into account any related regulatory requirements, e.g., ITU limitation of power flux density.” No work on this topic will take place in RAN WGs before the discussion on DL enhancements in RAN#99.

The following reference scenario is considered for the definition of uplink coverage enhancements for NTN: parameter set-1 for LEO-1200 satellite operating at Line of Sight (LOS) and commercial smartphones with -5.5 dBi antenna gain and 3 dB polarisation loss (per antenna port).

- Note: It is understood that the enhancements defined for LEO can also apply to GEO and MEO scenarios as appropriate. No additional work is expected for MEO/GEO.

The targeted services are VoIP using AMR 4.75 kbps and data transmission services with Low data rate of 3 kbps.

The detailed objectives are for NTN:

- To specify PUCCH enhancements for Msg4 HARQ-ACK (e.g. repetition) [RAN1, RAN4]

- To study DMRS bundling for PUSCH taking into account NTN-specifics (e.g. time-frequency pre-compensation) and, if necessary, specify enhancements to the Rel-17 procedures [RAN1]

Have a 1-TU 6-month study phase focusing on the following (to derive clear & limited scope):

- Evaluate the coverage performance and identify the candidate physical radio channels that have coverage issues specific to NTN with following target services taking into account the studies in TR38.830 where appropriate, as well as general coverage enhancement techniques specified in Rel-18 [RAN1,RAN2,RAN4]

  - VoIP and low-data rate services for commercial handset terminals

The following items are shown as examples of areas to consider in the RAN2 study.

- Improved performance of low-rate codecs in link budget limited situation including reducing RAN protocol overhead for VoNR

  - NOTE: Intent is not to introduce a new codec.

RAN to determine by RAN#97 (for RAN1 items) and RAN#98 (for RAN2 items) whether the study phase has identified any need for NTN-specific coverage enhancements in Rel-18. If needed, the set of NTN-specific work item objectives will be further updated.

As highlighted above, RAN has clearly identified the items for RAN2 study. The target service in the highlighted is NTN specific. For TN, we should target a high-quality high codec rate voice service.

It is important to increase the number of repetitions per bit to enhance coverage in NTN due to limited link budget. This can be done several ways and one of them is to reduce the overhead. Basically, two possible solutions have been proposed to achieve this. Note that the two solutions can be combined.

- Packet aggregation and,

- Protocol overhead reduction.

Packet Aggregation

| Tdoc | Packet aggregation |
| --- | --- |
| [1] R2-2209709 | Proposal 1: For coverage enhancement based on application layer frame aggregation, RAN sends the trigger command for the voice frame aggregation (i.e., 2 voice frames per packet) in NGSO. Send LS to SA4 for alignment work to process the command. Proposal 2: For coverage enhancements, study the details on specification change to support PDCP layer frame aggregation. |
| [2] R2-2210645 | Proposal 5: RAN2 to further study the packet aggregation mechanism in UE’s application layer and/or RAN layer(s). |
| [7] R2-2210033 | Proposal 4: Frame aggregation enhancement is not considered in RAN2. |

In [1] and [2], it is described that the voice traffic arrival rate can be 1 frame per packet per 20ms. However, to efficiently utilize the resource and save UE power and still meet the packet delay budget, 40ms CDRX can be used, i.e., voice packets are transmitted every 40ms, but as two separate packets, each incurring full headers. A simple solution is to aggregate two voice frames and reduce overhead by 50%. The 40ms interval enables the PUSCH repetition with number more than 20 to have a further coverage enhancement to fill the coverage gap.

- Do you agree that with scheduling interval of 40ms or more, the PUSCH repetition with number more than 20 is possible to have a further coverage enhancement?

| Company name | Yes/No | Comments |
| --- | --- | --- |
| MediaTek | Yes, but | This could be already done by application layer packet aggregation. No further R2 SPEC change is needed. Note that recommended bit rate is already supported in R15. |
| Intel | Yes, but not RAN2 work | Agree with MTK, it can be done by application layer without RAN2 spec change. |
| Lenovo | Not RAN2 | Agree with above, no RAN2 spec change is needed. |
| Nokia | Yes | In our understanding, the recommended bit rate as mentioned by MediaTek is the codec adaptation (e.g. NW recommend 4.75Kbps or 12.2Kbps via RBR MAC CE based on radio condition or system load status) while the application layer frame aggregation or PDCP layer aggregation is to aggregate two or more voice frames for one transmission which can facilitate NW scheduling UE every 40ms (or aggregation level*20 ms) hence enable >20 PUSCH repetitions for coverage enhancement. |
| Qualcomm | Yes | We share similar view as Nokia. We think mechanism to adjust the application layer frame aggregation would be same as existing “recommended bit rate MAC CE” to adjust codec rate. Only RAN can decide whether 1 frame/packet or 2 frame/packet or 3 frame/packet is suitable based on the use of coverage enhancement for UE. |
| vivo | Not RAN2 | We agree that application aggregation can extend the scheduling interval. But this method will make the TB size larger. Whether it can improve the coverage performance shall be evaluated by RAN1. No RAN2 conclusion/discussion should be made/carried out. We should leave this discussion to RAN1, as this is not RAN2 work. |
| Xiaomi | No, RAN1 to decide | One voice frame has 192bit, including RAN protocol overhead 40bits. With 2 frame aggregation, the protocol overhead reduce to 20bits per voice frame. Then the overhead reduction is 20/192 = 10%. However, if we assume that in CE mode, UE use full power to transmit, the requires RB number for 2 voice frame is doubled. It means the power spectral density is reduced by half. If 20 repetition is required for one voice frame. In frame aggregation mode, more than 40 repetitions may be required. However, the maximum repetition is only 32. Thus, we think that the coverage be be degraded instead of improved. BTW, RAN1 is also discussing this issue. Most companies think that simulation is required to see whether coverage is degraded or not. Thus, we think it is better to wait for RAN1 to conclude on this. |
| CATT | Not RAN2 | We agree that the PUSCH repetition with number more than 20 has been achieved by application layer packet aggregation. Enhancements may be needed only big issues are found for application layer packet aggregation. |
| Huawei, HiSilicon | See comments | We don’t understand the intention of this question. Of course, more PUSCH repetition will have a further coverage enhancement. But if 20 PUSCH repetitions are enough for coverage, why do we need to extend the scheduling interval? Regarding the specific PUSCH repetition number that can be used, we should wait for RAN1’s evaluation. |
| Apple | See comments | Theoretically, for the same packet size, the more repetition number, the stronger the reliability. But for the different packet sizes, it’s better to confirm with RAN1. For the application layer packet aggregation, the aggregation number is decided and performed in APP layer not in AS layer. About the potential AS impact, in our view, some assistance information from AS layer to APP layer may be helpful, but it can be achieved by current UE implementation. |
| LG | Not RAN2 | The scheduling interval is up to application layer implementation. We do not need to discuss it. |
| Google | Yes, but | Agree with majority that no RAN2 spec change is needed. |
| Samsung | See comment | We agree more repetition is allowed by aggregation, but not sure if coverage can be enhanced due to the tradeoff as vivo and xiaomi commented. Leave to RAN1 |
| ZTE | RAN1 shall evaluate first | Share similar views as majorities that RAN1 shall first evaluate the gain of aggregating two voice frames in one TB, there is no work need to be done in RAN2 before they reach conclusions. |
| NEC | Yes | In principle |
| OPPO | Not RAN2 | It is true that with 40ms scheduling interval, RAN protocol overhead can be reduced a bit. However, packet aggregation will generate larger TBS, which requires more RB resources if MCS keeps unchanged, leading to lower power spectral density for limited UL Tx power, this would have a negative impact on coverage enhancement. It is not straightforward for RAN2 to conclude that packet aggregation can improve coverage without RAN1’s evaluation. |

- A voice packet is generated every 20ms. Do you agree that with scheduling interval of 40ms, compared to transmission of two voice packets separately every 40ms (e.g., MAC layer multiplexing), protocol overhead is reduced when two voice packets are aggregated and transmitted?

| Company name | Yes/No | Comments |
| --- | --- | --- |
| MediaTek | Yes, but | Please see our comments in response to Q1. |
| Intel | Yes, but not RAN2 work | Please see our comments in response to Q1. |
| Lenovo | Not RAN2 | Agree with above, no RAN2 spec change is needed. |
| Nokia | Yes |  |
| Qualcomm | Yes | Instead of repeating L1 and L2 protocol overheads in each packet, the packets can be aggregated and only one set of L1 and L2 headers can be used. |
| vivo | Yes, but not RAN2 work | Please see our comments in Q1. |
| Xiaomi | Yes | But the coverage should not be degraded |
| CATT | Yes, but not RAN2 work. | Please see our answers in Q1 |
| Huawei, HiSilicon | See comments | We are not convinced about the necessity of aggregation and whether some gain can be achieved, considering we have to increase the PUSCH repetitions and change the scheduling interval. |
| Apple | Yes |  |
| LG | Not RAN2 | The scheduling interval is up to application layer implementation. We do not need to discuss it. |
| CAICT | Yes |  |
| Google | Yes, but | Same comments as in Q1. |
| Samsung | Yes |  |
| ZTE | Yes, but | The ultimate goal is to improve coverage, thus it’s performance shall be evaluated by RAN1 first. |
| NEC | Yes | In principle. |
| OPPO | Yes, but | See our comments on Q1 |

- If answer to Question 2 is yes, which option do you prefer to prioritize for further study?

- Application layer frame aggregation

- PDCP layer packet aggregation

| Company name | Option | Comments |
| --- | --- | --- |
| MediaTek | 1 | This could be already done by application layer packet aggregation. No further R2 SPEC change is needed. Note that recommended bit rate is already supported in R15. |
| Intel | 1 |  |
| Lenovo | 1 |  |
| Nokia | 1 and 2 | From protocol overhead reduction point of view, both application layer frame aggregation and PDCP layer packet aggregation can reduce the overhead significantly while application layer can additionally save RoHC header(s). Furthermore, both options can enable the PUSCH repetition >20 for coverage enhancement. However, application layer frame aggregation is out of RAN scope which may involve other WG(s) (e.g. SA4). PDCP layer packet aggregation can be further studied by RAN in case the application layer frame aggregation is not supported by network/UE or service providers. |
| Qualcomm | 1 and 2 | We prefer option (1) as UE application just needs a command to know whether it is suitable to generate application layer frame aggregation request. But as Nokia explained we are ok to study PDCP layer packet aggregation, i.e., option 2 as well. |
| Vivo | 1 with comment | Even if option 1 is adopted, we assume no further R2 spec change is needed. |
| Xiaomi | 1 |  |
| CATT | 1 | 2 should be studied cautiously. |
| Huawei, HiSilicon | None | See our reply to Q2. |
| Apple | 1 and 2 |  |
| LG | 1 | The scheduling interval is up to application layer implementation. We do not need to discuss it. |
| CAICT | 1 | Considering the less spec impact, we think option 1 is better |
| Google | 1 | Agree with MTK and vivo that no further R2 spec change is needed. |
| Samsung | 1 |  |
| ZTE | 1 if there is no specs impact. |  |
| NEC | None or 1 | 1 have less impact in our understanding |
| OPPO |  | Too early to discuss this in RAN2. It is true that with 40ms scheduling interval, RAN protocol overhead can be reduced a bit. However, packet aggregation will generate larger TBS, which requires more RB resources if MCS keeps unchanged, leading to lower power spectral density for limited UL Tx power, this would have a negative impact on coverage enhancement. It is not straightforward for RAN2 to conclude that packet aggregation can improve coverage without RAN1’s evaluation. |

- If answer to Question 1 or Question 2 is No, please elaborate the reason in comments.

| Company name | Comments |
| --- | --- |

Protocol header reduction

| Tdoc | Protocol overhead reduction |
| --- | --- |
| [4] R2-2209710 | Proposal 1: PDCP compression of ROHC header is used to save 1 byte from the ROHC header and CRC check is ignored at ROHC when PDCP compression of ROHC header is configured. Proposal 2: Introduce 1-byte PDCP header to be configured for voice DRB. Proposal 3: Network can configure not to use RLC header from a PDU associated with the DRB that is configured for voice traffic. Proposal 4 : Consider removing L field from MAC subheader of the PDU associated with the low data rate DRB. |
| [5] R2-2209804 | Proposal 1: RAN2 to discuss whether to consider the new L2 header design customized for voice. Proposal 2: If the new L2 header for the voice service is considered, RAN2 is suggested to discuss the method to distinguish the different L2 header formats for voice service and other service. |
| [6] R2-2209389 | Proposal 1: The mapping function of SDAP is not required for VoNR in NTN network. Proposal 2: VoNR can be configured as TM mode to reduce the protocol overhead. |
| [7] R2-2210033 | Proposal 1: No spec impact is needed for SDAP/PDCP/MAC to reduce RAN overhead for VoNR Proposal 2: RAN2 consider enhancement of RLC layer to reduce RLC header size for VoNR. Proposal 3: RAN2 to discuss whether length field of MAC header can be removed for VoNR. |
| [8] R2-2209508 | Proposal 1: RAN2 does not identify necessary/feasible enhancements to be supported on RAN overhead reduction for VoNR from RAN2 perspective. This could be considered as the RAN2 study phase outcome. |
| [3] R2-2210285 | Proposal 2: Protocol overhead reduction is not pursued for coverage enhancements. |
| [9] R2-2210566 | Proposal 1. RAN2 do not study the L2 header reduction in NTN. |

- Do you agree SDAP header and MAC-I field do not need to be configured for DRB carrying the voice traffic or low data rate traffic to reduce protocol overhead?

| Company name | Yes/No | Comments |
| --- | --- | --- |
| MediaTek | No | SDAP header is responsible for QoS flow mapping to the DRBs. Hence, without SDAP header it is not sure how the QoS mapping will be done for VoNR packets. |
| Intel | No | Every L2 header has a corresponding function, we see no good reason to remove them. According to RAN1 simulation results, most companies showed that when DMRS bundling is enabled, the coverage can be improved to meet NTN requirements. So we think actually no RAN2 enhancements for coverage is necessary, especially considering protocol overhead reduction can lead to very large spec impact. |
| Lenovo | No | We think this is a large spec change in SDAP with possible further issues (e.g. QoS management) and little benefit. |
| Nokia | No | It is NW configuration/implementation to decide the SDAP header presence and Integrity protection for a DRB. |
| Qualcomm | Yes | Integrity protection for voice is not essential. So, MAC-I field does not need to be used. In DRB configured for VoNR, a single flow of QoS can be used so SDAP is not necessary. Therefore, for such traffic, overhead can be reduced. |
| vivo | No | It is NW implementation to configure the SDAP header and Integrity protection for a DRB. |
| Xiaomi | No | Network implementation to decide whether to configure SDAP header and whether to use integrity protection. |
| CATT | No | SDAP header and integrity protection are configurable. If the SDAP header and integrity are considered not so important, the network may not configure them. |
| Huawei, HiSilicon | No | The UL/DL SDAP header may be absent via the appropriate configuration according to the current specification. But the MAC-I field is needed since integrity protection is needed for security. |
| Apple | No | We can leave it up to NW implementation. |
| LG | No | Since the IP header can be significantly compressed by ROHC, the benefit of overhead reduction would be marginal. Considering that the overhead reduction has an large impact on the specification and the gain is small, we do not need to study the overhead reduction |
| CAICT |  | Mapping function can be ignored to reduce the protocol overhead in NTN system. But it is configurable. It is can up to implementation whether retain the SDAP header. |
| Google | No | The efforts of making such a big change may not match the benefits it brings. |
| Samsung | No | Up to NW configuration |
| ZTE | No | It shall be up to NW to decide whether to configure SDAP header /use integrity protection or not. |
| NEC | No | We think it is up to NW configuration, it is already possible to configure without SDAP header |
| OPPO | No | Whether to have SDAP header/ use integrity protection can be configurable by NW. |

The contributions [4], [5], [6] and [7] propose that RLC header can be reduced considering network can avoid segmentation with appropriate TBS allocation for voice traffic and due to long RTT, retransmission of voice packets can also be disabled.

- Do you agree to consider enhancement of RLC layer to reduce RLC header size for VoNR in NTN (for example, configuring RLC TM mode)?

| Company name | Yes/No | Comments |
| --- | --- | --- |
| MediaTek | No | The design is based on the assumption of fixed voice packet sizes. However, for RoHC, the compressor need to periodically send IR format, so sometimes the voice SDU size can be larger. RLC TM mode should not be used. |
| Intel | No | Please see our comments in response to Q5. |
| Lenovo | No | We think this is a large spec change in RLC with possible sacrifice in robustness and little benefit. |
| Nokia | No | RLC TM will sacrifice the scheduling efficiency/flexibility since no RLC segmentation (e.g. TBS requirement is changing caused by RoHC header change, codec rate change, talk burst vs. SID etc, which need RLC segmentation) |
| Qualcomm | Yes | Whether there is RLC header or not is identified by the MAC subheader. Therefore, we do not see large spec impact. Main point is to avoid RLC segmentation which if used, would be too bad for coverage enhancement. RLC segmentation incurs huge overhead. So we think some enhancement is needed to reduce RLC header. |
| vivo | No | We think there is 1-byte room for RLC protocol header reduction. But it will make the scheduling less flexible and less efficient. Moreover, considering that such reduction can only bring marginal benefit (i.e. around 0.4 dB gain as shown in our contribution) for performance improvement of coverage, we do not think this enhancement is needed. |
| Xiaomi | No | Depends on how much gain there is for the whole protocol reduction. Since RAN1 see that by using DMRS bundling there is no coverage issue, perhaps it is not necessary to do this. |
| CATT | No | The uplink grant may not align with voice packets all the time. If the grant is larger, padding will be introduced. If the uplink grant is smaller, the UE has to skip the grant since segmentation is not supported which causes more waste on resource consumption. |
| Huawei, HiSilicon | No | If RLC TM mode is configured for VoNR, the RLC header can be saved but it is hard for the gNB to allocate suitable UL grants to avoid packet segmentation since both the size of voice frame and the size of PDCP Data PDU are variable. Hence, it may lead to a large waste of resources. If RLC UM mode or AM mode is used, only a few fields in the RLC header can be saved. The potential gain is unclear. |
| Apple | Depend on | We can consider it if RAN1 solution cannot solve the PUSCH coverage issue well. |
| LG | No | Please see our comments in response to Q5. |
| CAICT | Yes | Agree with QC, some enhancement of RLC layer can be considered. |
| Google | No | Same comments as in Q5. |
| Samsung | No | see no much gain |
| ZTE | No | Both robustness and scheduling flexibility will be impacted while the gain is limited. |
| NEC | No | We share the concerns raised by other companies |
| OPPO | No | Share the same view as MediaTek, and it would have an impact on scheduling flexibility. |

The contributions [4] and [5] propose that PDCP SN length of 12 bit or 18 bit is not necessary and 1-byte PDCP header can be considered since retransmission of voice packet is not expected and reordering window can be smaller.

- Do you agree to consider enhancement for 1-byte PDCP header for VoNR in NTN?

| Company name | Yes/No | Comments |
| --- | --- | --- |
| MediaTek | Yes | RAN2 can study and discuss on PDCP header enhancement. |
| Intel | No | Please see our comments in response to Q5. |
| Lenovo | No | We think this is a large spec change in PDCP with possible issues (e.g. integrity protection and Data/Control PDU indication) and little benefit. |
| Nokia | No | Shorter SN means smaller reordering window length which may lead fast to window stalling. Furthermore, too short SN may lead to more HFN desynchronization problem. This may be issues when HARQ feedback is disabled which need more clarifications. |
| Qualcomm | Yes | In LTE, 7 bit SN length can also be used for VoLTE. So it should also be possible in NTN for VoNR. For example, delayed voice packets by 275ms could be just dropped (as is the likely case today in TN) and if we do simple calculation, 7 bit SN should provide sufficient reordering window in NTN scenario considering the large RTT. This will save 1 byte overhead. |
| vivo | No | We think there might be 1-byte room for PDCP protocol header reduction. But such reduction can only bring marginal benefit (as shown in our contribution) for performance improvement of coverage, we do not think this enhancement is needed. |
| Xiaomi | No | Depends on how much gain there is for the whole protocol reduction. Since RAN1 see that by using DMRS bundling there is no coverage issue, perhaps it is not necessary to do this. |
| CATT | No | We agree with Nokia. |
| Huawei, HiSilicon | No | Reducing the size of PDCP SN may have an impact on the performance of VoNR because it reduces the length of the PDCP window. |
| Apple | Yes | We agree with QC that 7-bit SN can work well for both VoLTE and VoNR. If L2 header reduction is needed, we can start from here. |
| LG | No | Please see our comments in response to Q5. |
| CAICT | No strong view |  |
| Google | No | Same comment as in Q5. |
| Samsung | No | see no much gain |
| ZTE | No | Share similar views as Nokia. |
| NEC |  | We are open to discuss of having shorter PDCP SN |
| OPPO | No | Agree with Nokia. |

The contributions [4] and [5] propose that the L field from MAC header can also be removed by a new spare 6 bit LCID code point as the VoNR packet size can be known with the given codec rate.

- Do you agree to consider enhancement for 1-byte MAC header for VoNR in NTN?

| Company name | Yes/No | Comments |
| --- | --- | --- |
| MediaTek | No | The design is based on the assumption of fixed voice packet size, which is not always true. |
| Intel | No | Please see our comments in response to Q5. |
| Lenovo | No | We think this is a large spec change in MAC with possible sacrifice in robustness and little benefit. |
| Nokia | No | TBS requirement is changing caused by RoHC header change, codec rate change, talk burst vs. SID etc, hence L field is needed. |
| Qualcomm | Yes | We agree if UE and gNB negotiate to use it for a specified voice packet length for the configured voice DRB, this works. If the packet length is different than the negotiated one, the UE can fall back to the regular LCID code point with L field for the voice DRB. |
| vivo | No | Considering the variable packet size due to the SID frame, the L field also cannot be removed. Moreover, considering that such reduction can only bring marginal benefit (as shown in our contribution) for performance improvement of coverage, we do not think this enhancement is needed. |
| Xiaomi | No | Depends on how much gain there is for the whole protocol reduction. Since RAN1 see that by using DMRS bundling there is no coverage issue, perhaps it is not necessary to do this. |
| CATT | No | This brings great restriction on scheduling. |
| Huawei, HiSilicon | No | In our understanding, the data in the VoNR includes the silence frames and voice frames, of which the sizes are different. Even if the size of voice frames is fixed, the size of PDCP Data PDU is not fixed because the size of data after the head compression is variable. Therefore, the L field is still needed. |
| Apple | Depends on | The restriction of the MAC header without L field is obvious. And the additional mechanism to distinguish the different MAC headers is also needed. We can consider it if RAN1 solution cannot solve the PUSCH coverage issue well. |
| LG | No | If the packet size is fixed, the L field can be removed in order to reduce the 1-byte header. However, since the ROHC compressor would periodically transmit the IR packet (full header packet), the packet size would be variable. Thus, it is hard to remove the L field. |
| CAICT | No strong view |  |
| Google | No | Same comment as in Q5. |
| Samsung | No | see no much gain |
| ZTE | No | Since voice packet size is not fixed, L field is needed anyway. |
| NEC | No | It would become too restrict if we reduce the MAC header |
| OPPO | No | VoNR packets’ sizes will not be always the same, and it would have an impact on scheduling flexibility. |

Contribution [4] proposes that PDCP compression of ROHC header can be used to save 1 byte from the ROHC header and CRC check can be ignored at ROHC when PDCP compression of ROHC header is configured. As 16 bit CRC check in PHY layer is used, it can be clarified that when PDCP compression of ROHC header is used, the CRC check is ignored at ROHC protocol. The SN in ROHC can be associated with the PDCP SN.

- Do you agree to associate the SN in ROHC header with the PDCP SN to reduce 1 byte overhead for VoNR in NTN?

| Company name | Yes/No | Comments |
| --- | --- | --- |
| MediaTek | No | We have strong concerns on removing CRC header, as it is used to ensure RoHC context sync. We think CRC is needed in ROHC. |
| Intel | No | Please see our comments in response to Q5. |
| Lenovo | No | We think this is a large spec change in ROHC mechanism with possible issues (e.g. multiplexing) and little benefit. |
| Nokia | No | The optimization of RoHC header is not scope of RAN or SA WGs. |
| Qualcomm | Yes | When gNB configures to use this enhancement (say PDCP compression of ROHC header), the SN in ROHC header can be linked with the PDCP SN. We can just clarify that CRC in ROHC is assumed always pass. See how RAN can capture the ROHC behavior below TS 38.323 ### 5.7.4 Header compression using ROHC If ROHC is configured, the ROHC protocol generates two types of output packets: - ROHC compressed packets, each associated with one PDCP SDU; - standalone packets not associated with a PDCP SDU, i.e. interspersed ROHC feedback. A ROHC compressed packet is associated with the same PDCP SN and COUNT value as the related PDCP SDU. The header compression is not applicable to the SDAP header and the SDAP Control PDU if included in the PDCP SDU. For DAPS bearers, the PDCP entity shall perform the header compression for the PDCP SDU using the ROHC protocol either configured for the source cell or configured for the target cell, based on to which cell the PDCP SDU is transmitted. Interspersed ROHC feedback are not associated with a PDCP SDU. They are not associated with a PDCP SN and are not ciphered. NOTE 1: If the MAX_CID number of ROHC contexts are already established for the compressed flows and a new IP flow does not match any established ROHC context, the compressor should associate the new IP flow with one of the ROHC CIDs allocated for the existing compressed flows or send PDCP SDUs belonging to the IP flow as uncompressed packet. NOTE 2: For downlink, the ROHC protocol of the target cell should maintain the IR state if operating in U-mode and O-mode during DAPS handover before release of source cell. |
| vivo | No | ROHC enhancement is out of 3GPP scope. |
| Xiaomi | No | Depends on how much gain there is for the whole protocol reduction. Since RAN1 see that by using DMRS bundling there is no coverage issue, perhaps it is not necessary to do this. |
| CATT | No | We think we should not touch ROHC enhancements in RAN. |
| Huawei, HiSilicon | No | We should be careful of changing the ROHC header. In legacy, we only use the ROHC protocols of RFC defined by other groups than 3GPP. It is not clear whether the change of the ROHC header will be even possible and whether there will be some potential issues expected if we change it. |
| Apple | No | ROHC enhancement is out of 3GPP. |
| LG | No | We same view as MTK. In addition, since the ROHC is not a RAN2 specification, we shall not change any ROHC procedure or ROHC header. |
| CAICT | No strong view |  |
| Google | No | Same view as majority that ROHC enhancement is out of scope. |
| Samsung | No |  |
| ZTE | No | It is not in RAN2 power to decide that CRC for ROHC can be passed. |
| NEC | No |  |
| OPPO | No | ROHC enhancement is out of 3GPP. |

Contributions [8], [3] and [9] propose not to consider protocol overhead reduction.

- Do you agree RAN2 should not consider any of the options in Q5, Q6, Q7, Q8 and Q9 as L2 protocol header reduction for coverage enhancement in NTN?

| Company name | Yes/No | Comments |
| --- | --- | --- |
| MediaTek | Q5, Q6: No Q7: Ok Q8, Q9: No |  |
| Intel | ok to not consider all of them |  |
| Lenovo | Yes |  |
| Nokia | OK to not consider all of them |  |
| Qualcomm | No | There are at least SDAP, PDCP SN length and RLC enhancements that can be done with minimum specific impact. |
| vivo | Yes | To be clear, we don’t think the protocol overhead reduction should enter normative phase. |
| Xiaomi | Yes |  |
| CATT | OK to not consider all of them |  |
| Huawei, HiSilicon | ok to not consider any of them | The gain of L2 protocol header reduction is unclear. Besides, the reduction mechanisms will largely increase the complexity of UE and NG-RAN. |
| Apple |  | We donot need consider Q9. For others, we can wait for RAN1 enhancement first. |
| LG | Yes |  |
| CAICT | No | Some RLC enhancements can be considered. |
| Google | Ok to not consider all of them |  |
| Samsung | Ok to not consider all |  |
| ZTE | OK to not consider all of them |  |
| NEC | Ok |  |
| OPPO | Yes |  |

Coverage enhancement in general

| Tdoc | Coverage enhancement in general |
| --- | --- |
| [3] R2-2210285 | Proposal 1: RAN2 wait until more progress in RAN1 to discuss whether anything needs to be done in RAN to trigger voice frame aggregation. |
| [10] R2-2209406 | Proposal 5: Other coverage enhancement specific for NTN in RAN2 can be studied after more inputs achieved in RAN1. |
| [11] R2-2210685 | Proposal 1: RAN2 to postpone the discussion of high layer coverage enhancements and wait for further input from RAN1. |

- Do you agree RAN2 should wait for more input from RAN1 for guidance in protocol overhead reduction study?

| Company name | Yes/No | Comments |
| --- | --- | --- |
| MediaTek | Yes | We should wait for RAN1 and allow them to make some more progress |
| Intel | Yes |  |
| Lenovo | Yes |  |
| Nokia | No | RAN2 can study packet aggregation which is not dependent on RAN1. |
| Qualcomm | No | It was discussed in the last RAN plenary and WID is updated clearly saying RAN2 to evaluate the solution on protocol overhead. If RAN2 needed to wait, it should have been clarified in the last plenary. The following items are shown as examples of areas to consider in the RAN2 study. - Improved performance of low-rate codecs in link budget limited situation including reducing RAN protocol overhead for VoNR - NOTE: Intent is not to introduce a new codec. |
| vivo | Yes | As to L2 header reduction for a single packet, RAN2 can conclude that there is no need to pursue considering the limited benefit. For packet aggregation, whether it can improve the coverage performance shall be evaluated by RAN1. RAN2 need to wait for RAN1 progress on this topic. |
| Xiaomi | Yes | Since RAN1 see that by using DMRS bundling there is no coverage issue, perhaps it is not necessary to do overhead reduction. |
| CATT | Yes |  |
| Huawei, HiSilicon | Yes | RAN1 is already studying solutions to enhance the NTN coverage for VoIP and low data rate services considering NTN-specific characteristics. We can wait for RAN1 progress to check whether there is still a coverage gap. |
| Apple | Yes | Further RAN2 enhancement depends on whether RAN1 solution can solve the voice PUSCH issue well. |
| LG | No | Since the protocol overhead reduction is a RAN2 scope, we do not need to wait for RAN1 progress. In addition, RAN2 can decide that the overhead reduction is not considered in NTN. |
| CAICT | Yes | We can wait for more RAN1 input, but some RAN2 solution also can be studied as WID saying. |
| Google | Yes |  |
| Samsung | Yes |  |
| ZTE | Yes |  |
| NEC | No | In principle these two enhancement directions are in RAN2 scope, we can make a decision by ourselves. |
| OPPO | Yes |  |

- If your answer to Q11 is yes, do you agree to send LS to RAN1 to ask feedback?

| Company name | Yes/No | Comments |
| --- | --- | --- |
| MediaTek | No | We don’t think sending an LS is needed at this time. |
| Intel | No | RAN1 is already working on it, no need to send an LS. |
| Lenovo | No | No need to send LS. |
| Nokia | No |  |
| Qualcomm |  | RAN1 is not going to study this on their own as WID clearly identifies it as area for RAN2 study. |
| vivo | No | There also has proposal about packet aggregation in RAN1. If there is any progress on this topic, RAN1 will inform us. There is no need for RAN2 to send LS first. |
| Xiaomi | No |  |
| CATT | No | We can wait for more progress in RAN1. |
| Huawei, HiSilicon | No | The LS is not needed at this stage. |
| Apple |  | We are fine to wait for the RAN1 progress. |
| LG | No |  |
| CAICT | No |  |
| Google | No |  |
| Samsung | No |  |
| ZTE | No | The LS provide no additional information for RAN1, they anyway will evaluate all options on the table. |
| NEC | No |  |
| OPPO | No |  |

# Conclusion

Following proposals are made [to be updated…].

No table of contents entries found.

# References

[1] R2-2209709	Frame aggregation for coverage enhancement	Qualcomm Incorporated	discussion	Rel-18	NR_NTN_enh

[2] R2-2210645	Discussion on Coverage Enhancements for NR NTN	Nokia, Nokia Shanghai Bell	discussion	Rel-18	NR_NTN_enh

[3] R2-2210285	Consideration on coverage enhancements	ZTE Corporation, Sanechips	discussion	Rel-18

[4] R2-2209710	Protocol overhead reduction for coverage enhancement	Qualcomm Incorporated	discussion	Rel-18	NR_NTN_enh

[5] R2-2209804	Consideration on NTN Coverage Enhancement	Apple	discussion	Rel-18	NR_NTN_enh-Core

[6] R2-2209389	Discussion on coverage enhancement in NR NTN	CAICT	discussion	Rel-18	NR_NTN_enh-Core

[7] R2-2210033	Discussion on coverage enhancement for NR NTN	Xiaomi	discussion	Rel-18

[8] R2-2209508	Discussion on RAN overhead reduction for VoNR support in NTN	vivo	discussion

[9] R2-2210566	Discussion on the L2 header reduction in NTN	LG Electronics Inc.	discussion	NR_NTN_enh-Core

[10] R2-2209406	Discussion on NTN Coverage Enhancement	CATT	discussion	Rel-18	NR_NTN_enh

[11] R2-2210685	Discussion on RAN protocol overhead reduction	Huawei, HiSilicon	discussion	Rel-18	NR_NTN_enh