3GPP TSG-RAN WG2 Meeting #119bis Electronic                     draft- R2-2210845

- E-Meeting, 10th – 19th Oct., 2022

Agenda item:	7.2.1

Source:	Nokia

Title:	[AT119bis-e][105][IoT NTN] Capability signalling (Nokia)

Document for:	Discussion & Decision

# Introduction

In this document, we capture the company’s views on different options related to TN and NTN capability signaling and storage for IoT-NTN for the below e-mail discussion.

- [AT119bis-e][105][IoT NTN] Capability signalling (Nokia)

Initial scope: Continue the discussion on the different alternatives and attempt to draft a reply LS to SA2 accordingly

Initial intended outcome: offline summary and draft reply LS

Deadline (for companies' feedback):  Thursday 2022-10-13 18:00 UTC

Deadline (for rapporteur's summary in R2-2210845 and draft LS in R2-2210846):  Thursday 2022-10-13 22:00 UTC

# Discussion

The Reply LS from SA2 suggests two options for retrieving and storage of TN and NTN UE capabilities from IoT-NTN devices.

Option 1: Single Container for TN and NTN capability

The preferred solution is for E-UTRAN to provide a single container to the MME for all non-NB-IoT RATs (and network types i.e. TN, NTN) and a single container for NB-IoT including both TN and NTN capabilities.

Option 2: Different Tracking Area for TN and NTN with minimum CN Impacts.

SA2 has also discussed some less optimal solutions and believes that a solution that has minimal core network impact is if the network is configured with different Tracking Areas for TN and NTN, and, the UE complies with the current TS 36.331 and TS 23.401 and performs a TA Update (indicating UE capability update needed) from RRC Idle mode when the UE’s capabilities change at any transition between TN and NTN.

In the following sections, we collect views of companies views on different aspects related to the above options and also the preferred option.

## Specification Impacts

During online discussions, it was pointed out that both options have a specification to either RAN or SA specifications. To arrive at a solution that has overall lesser specification work, we need to understand views in RAN2 for specification efforts for each of the options.

Question 2: Companies are requested to provide their views on the specification changes required for each option for RAN and SA2 specifications. For the impact analysis, the additional efforts required over the minimum changes required at RAN and CN to support IoT-NTN connectivity are to be considered.  The efforts can be stated as high /medium/low along with the reason for the same.

For option 1:

|  | RAN specification Impacts | SA specification impacts | CT1 specification impact |
| --- | --- | --- | --- |
| MediaTek | Large impacts |  |  |
| Qualcomm | Simple change. | No impact | No impact |
| Vodafone | Simple change to correct existing mis-design and incomplete/non-functioning specifications (e.g. there does not seem to be any RAN specification saying how the UE causes the TN (or NTN) capabilities to be retrieved when the UE moves from NTN to TN (or TN to NTN)) Aligns RAN 2 IoT NTN with existing RAN 2 NR NTN design and the overall RAN and SA methodology for 4G and 5G UE radio capability (and UE radio capability for paging) handling, thereby avoiding future design complexity. | No impact except that the handling for UE Radio capability for Paging needs to be discussed -> for BOTH NR-NTN and option 1-IoT-NTN | No impact |
| Ericsson | Large | No impact | No impact |
| Nordic | simple | No impact | No impact |
| Lenovo | simple | No | No |
| OPPO | Simple and aligned with NR NTN | No | No |
| Nokia | Large Impacts for the proposed solution | No | No |
| Xiaomi | Large | No impact | No impact |
| Intel | high impact |  |  |
| Huawei, HiSilicon | Large | No | No |
| ZTE | Large impacts To response QC: we don’t think to introduce a list to include two sets of UE capabilities would be a simple change. That would be a NBC change. To response Vodafone: it’s unfair to say a RAN2 agreement after long discussion is a mis-design. The reason we say large impacts: UE needs to report two sets of UE capabilities even when it camps on one network. Different from NR NTN, IoT UEs are generally with low-cost and low-complexity. The option 1 may place higher requirement on UE storage and cause large air interface signalling overhead that has negative impacts on UE cost and power saving. Moreover, the number of IoT UEs may be huge. Storing and delivering multiple sets of UE capability for these IoT UEs may be also a great burden on the CN. | Low It’s not clear how the CN deliver the set(s) of UE capabilities to the corresponding network (TN or NTN)? The CN may deliver both sets to eNB and let eNB to do selection. That’s not good. For this aspects, the specification enhancement may be needed in CN. | No impact |

For option 2:

|  | RAN specification Impacts | SA specification impacts | CT1 specification impact | CT 4 specification impact |
| --- | --- | --- | --- | --- |
| MediaTek | Less impacts |  |  |  |
| Qualcomm | Some impact to clarify text on UE capability update. | SA2 specifications for rel.17 are also deeply frozen. MME will not delete UE capability container and trigger new UE capability request just because UE changed the tracking area. Impact to clarify text on UE capability update procedure when switching TN and NTN. | CT1 specifications for rel.17 are also deeply frozen. Possible impact to add new trigger on UE capability update for the case of TN and NTN switch. |  |
| Vodafone | Some impact to clarify text on UE capability update, and, for eMTC a mechanism to handle UE capability update in connected mode. RAN (3 ?) specification updates for RACS (for eMTC) for connected mode handover to say that the source RAN shall not send the UE RAC to the target RAN. And then how the target RAN obtains the UE RAC (and either uses or shall ignore any RACS ID in the Handover Request…with some explanation of how the target RAN knows that the source cell was IoT-NTN) | As Qualcomm indicate, plus: - Extra requirements on the MME to NOT allocate a TAI list containing TN and NTN tracking areas. requires the MME’s TAI list handling to be modified to take into account the RAT type sent in the S1 interface signalling ALONG with the RAT type received in the previous TA Update, Attach, (and possibly GUTI reallocation) procedure - For eMTC, high level design work to decide which (or both) of the two RACS capability IDs are transferred at inter MME (and MME-AMF and AMF-AMF) mobility. - (For eMTC) connected mode mobility – we need to specify that the source RAN shall not send the UE RAC to the target RAN. - For connected mode mobility, need to consider which of the two RACS IDs are sent to the target RAN node – and how this is handled at inter MME and AMF to MME handover. |  | (for eMTC) Handling of multiple RACS capability ID at inter MME, MME <-> AMF and inter AMF mobility. |
| Ericsson | No impact | Low | Low |  |
| Nordic | Little to none |  | Agree with Qualcomm |  |
| Qualcomm | simple | Agree with Vodafone | Agree with Qualcomm |  |
| OPPO | little |  | Agree with Qualcomm |  |
| Nokia | No Impact | Low. MME impacts indicated by QC and Vodofone can be addressed if the UE trigger the NAS based capability enquiry using below procedure In UE capability enquiry procedure definition in 36.331. 5.6.3.1 The purpose of this procedure is to transfer UE radio access capability information from the UE to E-UTRAN. If the UE has changed its E-UTRAN radio access capabilities, the UE shall request higher layers to initiate the necessary NAS procedures (see TS 23.401 [41]) that would result in the update of UE radio access capabilities using a new RRC connection. When IoT-NTN UE switches from TN to NTN, UE can trigger the above option along with TAU procedure to enable NW to retrieve the new capability. It may be sufficient to clarify this part for TN and NTN switching if needed. | Low |  |
| Xiaomi | No impact | Low | No impact |  |
| Intel | No impact |  |  |  |
| Huawei, HiSilicon | Little. RAN2 may need to clarify that for TN-NTN mobility, source cell does not provide UE capability to target cell. A clarification in Stage 2 spec is enough. | If Option 2 is adopted, the following description in 23.501 needs to be modified: NOTE 2: To prevent extra signalling load resulting from Mobility Registration Update occurring at every RAT change, it is preferable to avoid generating a RAT-specific TAI list for a UE supporting more than one RAT. |  |  |
| ZTE | No or little impact | Low Disagree with QC’s comment. With reference to TS 24.301, there already is a “UE radio capability information update needed” IE in “Table 8.2.29.1: TRACKING AREA UPDATE REQUEST message content”, so UE capability update is feasible. Moreover, per our knowledge, SA2 has confirmed there is no impacts for eMTC. | No impact |  |

Rapporteur summary :

Based on the company views on RAN and SA impacts following is the summary on specification impacts for the options.

- Both option 1 and option 2 have lesser impacts to SA/CT specifications. Option 2, there are some impacts on MME behaviour changes which does not impact interface specifications. For option 2 it is possible to retrieve capability from new cell during TA update based on UE indication in TAU as suggested by ZTE and Nokia. This can be considered as another option to minimise MME impacts for Option 2 for MME.

- For RAN specification impacts, the impacts are high for option 1 compared to option 2. The proposed solution for RAN specification changes is not complete and companies have some concerns on realisation of this solution. (7/5).

Proposal 1: Based on overall specification impact analysis, RAN2 concludes that specification impacts to implement option 2 is lesser compared to option 1 (7/5).

Following observations from company views are used to make the above proposal.

- Option 1 requires RRC interface changes based on all company views. 7 companies indicates the impact as large and 5 companies thinks that it is simple change. Option 2 does not require RAN specification changes based on the views.

- Interface specification changes are not needed for option 2. The only impact on MME behaviour changes needed for option 2 can also be mitigated with modified UE behaviour. RAN2 to discuss this option to minimise the MME impacts for Rel-17.

## Functionality Impacts of Option 2

Question 3: Please provide views on impacts to idle mode and connected mode mobility scenarios from the RAN perspective for option 2.

|  | Impacts on mobility scenarios |
| --- | --- |
| MediaTek | Mobility management, including cell selection/re-selection and Tracking Area Update (TAU) will be managed differently for TN and NTN systems. Current R-17 standards are already prepared to handle such scenarios. |
| Qualcomm | Connected mode mobility does not work. The UE will have to upload UE capability to the network every time it moves between TN and NTN. Unlike TAU update, UE capability update procedure is large signalling burden. |
| Vodafone | Option 2 requires the UE RAC to be uploaded at every change between TN and NTN (while option 1 only requires a “double size information element” to be uploaded once when the mobile is powered up (and at arrival in a new MME). Option 2 imposes extra signalling load on the NTN (and TN) networks. At connected mode mobility between TN and NTN, the UE’s AS needs to know when to change the capability in use. |
| Ericsson | There is no impact on RAN level. In idle mode, the existing TAU procedure can be used where TN and NTN are configured with different Tas so that UE capabilities can be requested when performing the update. With such configuration for deployment, we understand SA2’s recommendation to have different TAIs for TN and NTN is met. The following text is captured in 23.501, “In order to enable efficient enforcement of Mobility Restrictions, cells of each NR satellite RAT Type (NR(LEO), NR(MEO), NR(GEO) or NR(OTHERSAT)) need to be deployed in Tas different from Tas for other NR satellite RAT Types as well as different from Tas supporting terrestrial access RAT Types”. In connected mode, as SA2 proposed in the LS, there are existing mechanisms for handover to work. The trade-off may be an increase in HO interruption time which we think is not critical in TN-NTN mobility considering the time it would take to acquire GNSS fix. |
| Nokia | No impacts to RAN functionality. Idle mode issues can be resolved by TA configuration suggested by SA2. Connected mode mobility will still work with option 2, with additional time for target node to retrieve the new capability from UE. In our view, optimisation of UE behaviour for TN-NTN mobility is not in Rel-17 scope and there were no discussions on handling this scenario. So the optimisation which is relevant for this scenario is not needed for Rel-17. |
| Xiaomi | No impacts. SA2 already provided the solutions for RRC idle UE and connected UE mobility. And in the previous RAN2 meeting, the mobility was also discussed, and the RRC redirection can be used. |
| Intel | Since the R17 IoT NTN focus on sporadic short transmission in RRC CONNECTED mode, we don’t think connected mode mobility between TN and NTN is a blocking issue. |
| Huawei, HiSilicon | No impacts. For IOT UEs, the UE will not frequently move between TN and NTN, as the operator may want UEs to prioritize TN cells, and only camp on NTN cells when there is no TN coverage. |
| ZTE | No impacts. Similar views as Ericsson and Nokia. Also agree with HW that NB-IoT over NTN is generally with low mobility between TN and NTN, therefore, the issue of UE RAC to be uploaded at every change between TN and NTN is not obvious. |

Rapporteur summary

Proposal 2:  For option 2, no impacts to idle mode mobility.  With option 2 connected mode mobility also work but with the additional interruption which RAN 2 earlier agreed not to optimize in Rel-17. (7/9).

## Specification Impacts and backward compatibility of RAN-based solution proposed for Option 1

Question 4: For option 1, IN [3] solutions are proposed for RAN to support single container with changes in the capability inquiry procedure and also the Radio-paging capability. Please provide views on the specification efforts and backward compatibility aspects of this solution.

|  | Comments on specification impacts and backward compatibility for O1 |
| --- | --- |
| MediaTek | This will have relatively more specification changes in Rel-17 IoT-NTN. |
| Qualcomm | For Emtc, it is rather small change, see R2-2209713, just introduce a new “eutra-ntn” type, no ASN.1 change, no container extension. For NB-IoT, a simple ASN.1 extension. No NBC as UE not supporting change will not include new extension or new “eutra-ntn”. If SA2 guarantees TN and NTN are configured to be in different tracking areas, change in radio paging capability is NOT needed. SA2 specification is also frozen. Either way there will be some inconsistency between different releases. |
| Vodafone | Agree with Qualcomm. Also agree that, even with option 1, the Radio Paging Capability may need some SA2 work to sort out the details. BUT -> that SA2 work would also need to be done to resolve the SAME Radio Paging Capability issues that 3GPP has overlooked with NR-NTN. |
| Ericsson | As the name suggests, the UE-CapabilityRAT-ContainerList is meant to be used for different RAT types, not particular features. The same reasoning could be used for many other features where we would then end up with multiple features been defined “per container” level. This is not sustainable on the long run. Expanding UE-CapabilityRAT-ContainerList would also imply in including it in the filter on UECapabilityEnquiry message. |
| Nordic | Agree with Qualcomm |
| Lenovo | Agree with Qualcomm |
| OPPO | Agree with Qualcomm |
| Nokia | The specification changes indicated in this proposal also impact terrestrial ENB to implement this feature for the TN to NTN mobility to work. Without terrestrial ENB implementing this feature, the benefit is only partial. In our understanding, we prefer the IoT-NTN-specific solution which does not impact existing TN ENB. |
| Xiaomi | Compared with option 2, more specification changes are needed. |
| Intel | The similar concern may arise as NR NTN that an IoT NTN capable UE has double signalling overhead for capability reporting. |
| Huawei, HiSilicon | In our understanding, NTN is not a different RAT, it is within the scope of LTE (IOT NTN) or NR (NR NTN), so it is inappropriate to add a new RAT-Type. Moreover, it doubles the signalling overhead. Besides, the solution adopted for NR NTN is not feasible. In NR NTN, we spend much time going through the existing TN capabilities and identify which capabilities need to differentiate between TN and NTN. There is no time left for IOT NTN to repeat the work. |
| ZTE | Agree with Ericsson and HW’s comments. |

Rapporteur summary

Proposal 3:  For the solution proposed for RRC changes for option 1 following issue needs to be addressed before consideration.(7/12)

- Use of the parameter meant for RAT for Feature differentiation is not appropriate and not sustainable in long run.

- Signaling overload to execute UE capability enquiry procedure two times for TN and NTN in this option.

- The changes will impact the legacy TN ENB implementation to acquire NTN capability from the UE. Otherwise, the solution will not work for TN to NTN mobility scenario.

## Any other Impacts

Question 5: Are any other views /discussion points that need to be considered for the decision?

|  | Additional aspects |
| --- | --- |
| Vodafone | - The existing RAN 2 solution does not seem to work! The UE would only ever upload either TN or NTN capabilities to the MME, and, the MME would never be triggered to delete them, so the MME would supply the wrong capabilities to the target RAN after idle mode mobility to the other access type. - With both options, there seems to be some SA2 (and possibly RAN 3/RAN 2) work needed to solve Radio Paging Capability issues -> but, with option 1, the solution should be the same as we still need to develop for NR-NTN. - RAN 4 work is not complete yet for IoT NTN – so ASN.1 changes are unlikely to delay products, especially as Option 2 has UE NAS impacts (in order to set the TAU message contents correctly). |
| Ericsson | Unlike NR NTN, it has not been possible to identify a complete list of IoT bits for IoT NTN that would allow to send a single container. Moreover, avoiding the duplication of capabilities signalling has been RAN2’s purpose from the beginning of the discussion. SA2 has proposed two feasible options in the LS. SA2 has already discussed the complexity of the solutions before proposing them to RAN2. It should not be up to RAN2 to evaluate the impact in other WG specifications. From RAN2’s perspective, the second option is aligned with RAN2’s previous agreements and does not have impact in RAN level. |
| Nokia | As SA2 has already indicated that solution 2 is feasible eventhough it is not optimum one, considering the spec changes required for option 1 ,for Rel-17 we prefer option-2 which is aligned with RAN2 agreements and no impacts at RAN. Also in our view the impacts to MME can also be managed to minimum level if we consider the possibility of UE triggering the NAS procedure for retrieval on TN to NTN cell change. |

Proposal 4:  Proposal 4:  Option 2 indicated in SA2 is feasible for implementation from SA2 perspective and also aligned with previous RAN2 agreements without RAN specification impacts. (2/3)

## Preferred Solution

Question 6: Please indicate the preferred option based on the analysis provided in the previous questions

|  | The preferred option for Rel-17 |
| --- | --- |
| MediaTek | Option 2 |
| Qualcomm | Option 1 |
| Vodafone | Option 1 |
| Ericsson | Option 2 |
| Nordic | Option 1 |
| Lenovo | Option 1 |
| OPPO | Option 1 |
| Nokia | Option 2 |
| Xiaomi | Option 2 |
| Intel | Option 2 |
| Huawei, HiSilicon | Option 2 |
| ZTE | Option 2 |

Rapporteur Summary :

Based on the majority of views given for the preferred solutions and also the views for specification and functional impacts following is proposed.

Proposal 5: Option 2 is preferred as the overall specification impacts are lower compared to Option 1 and the basic functionality is not impacted for option 1. (7/12)

# Conclusion

12 companies participated in the e-mail discussion. On the specification impacts all the companies agreed that option 1 has slightly more impacts compared to option 2 eventhough there is difference of views on the amount of impacts.  On functional impacts of option 1 , majority views indicates that option to does not affect the basic functionality.  On the final preference for option there is slightly higher support for option 2 than option 1 (7/12).  Below proposals are made considering the views on all the aspects.

Proposals for Agreement

Proposal 1: Based on overall specification impact analysis, RAN2 concludes that specification impacts to implement option 2 are lesser compared to option 1.

The following observations from company views are used to make the above proposal.

- Option 1 requires RRC interface changes based on all company views. 7 companies indicate the impact as large and 5 companies think that it is simple to change. Option 2 does not require RAN specification changes based on the views.

- Interface specification changes are not needed for option 2. The only impact on MME behavior changes needed for option 2 can also be mitigated with modified UE behavior. RAN2 to discuss this option to minimize the MME impacts for Rel-17.

Proposal 2:  From RAN2’s perspective option 2 does not have impacts on idle mode mobility.  With option 2 connected mode mobility works but with the additional interruption which RAN 2 earlier agreed not to optimize in Rel-17. (7/9).

Proposal 4:  Option 2 indicated in SA2 is feasible for implementation from the SA2 perspective and also aligned with previous RAN2 agreements without RAN specification impacts. (2/3)

Proposal 5: Option 2 is preferred as the overall specification impacts are lower compared to Option 1 and the basic functionality is not impacted for option 1. (7/12)

Proposal for Further discussion

Proposal 3:  For the solution proposed for RRC changes for option 1 following issue needs to be addressed for further consideration.(7/12)

- Use of the parameter meant for RAT for Feature differentiation is not appropriate and not sustainable in long run.

- Signaling overload to execute UE capability inquiry procedure two times for TN and NTN in this option.

- The changes will impact the legacy TN ENB implementation to acquire NTN capability from the UE. Otherwise, the solution will not work for TN to NTN mobility scenario.

# References

[1] R2-2209359	Reply to LS on UE capability signalling for IoT-NTN (S2-2207839; contact: Vodafone)	SA2	LS in	Rel-17	LTE_NBIOT_eMTC_NTN	To:RAN2	Cc:CT1, RAN3

[2] R2-2210075	Analysis on the CN impacts for TN and NTN capabilities based on SA2 LS	Nokia, Nokia Shanghai Bell	discussion	Rel-17

[3] R2-2209712	Discussion on SA2 LS reply on UE capability for IoT NTN	Qualcomm Incorporated	discussion	Rel-17	LTE_NBIOT_eMTC_NTN

[4] R2-2210734	UE capability signalling in IoT NTN	Ericsson	discussion	Rel-17	LTE_NBIOT_eMTC_NTN

[5] R2-2210414	UE capability signalling for IoT-NTN	Huawei, HiSilicon	discussion	Rel-17	LTE_NBIOT_eMTC_NTN

[6] R2-2210076	[draft] Reply to LS on SA2	Nokia, Nokia Shanghai Bell	LS out	Rel-17	LTE_NBIOT_eMTC_NTN	To:SA2	Cc:CT1, RAN3

[7] R2-2210528	[DRAFT] Reply LS on RAN feedback for UE capabilities signalling for IoT NTN	ZTE Corporation, Sanechips	LS out	Rel-17	LTE_NBIOT_eMTC_NTN-Core	To:SA2	Cc:RAN3