3GPP TSG-RAN WG2 Meeting #119bis Electronic                     draft- R2-2210845

- E-Meeting, 10th – 19th Oct., 2022

Agenda item:	7.2.1

Source:	Nokia

Title:	[AT119bis-e][105][IoT NTN] Capability signalling (Nokia)

Document for:	Discussion & Decision

# Introduction

In this document, we capture the company’s views on different options related to TN and NTN capability signaling and storage for IoT-NTN for the below e-mail discussion.

- [AT119bis-e][105][IoT NTN] Capability signalling (Nokia)

Initial scope: Continue the discussion on the different alternatives and attempt to draft a reply LS to SA2 accordingly

Initial intended outcome: offline summary and draft reply LS

Deadline (for companies' feedback):  Thursday 2022-10-13 18:00 UTC

Deadline (for rapporteur's summary in R2-2210845 and draft LS in R2-2210846):  Thursday 2022-10-13 22:00 UTC

# Discussion

The Reply LS from SA2 suggests two options for retrieving and storage of TN and NTN UE capabilities from IoT-NTN devices.

Option 1: Single Container for TN and NTN capability

The preferred solution is for E-UTRAN to provide a single container to the MME for all non-NB-IoT RATs (and network types i.e. TN, NTN) and a single container for NB-IoT including both TN and NTN capabilities.

Option 2: Different Tracking Area for TN and NTN with minimum CN Impacts.

SA2 has also discussed some less optimal solutions and believes that a solution that has minimal core network impact is if the network is configured with different Tracking Areas for TN and NTN, and, the UE complies with the current TS 36.331 and TS 23.401 and performs a TA Update (indicating UE capability update needed) from RRC Idle mode when the UE’s capabilities change at any transition between TN and NTN.

In the following sections, we collect views of companies views on different aspects related to the above options and also the preferred option.

## Specification Impacts

During online discussions, it was pointed out that both options have a specification to either RAN or SA specifications. To arrive at a solution that has overall lesser specification work, we need to understand views in RAN2 for specification efforts for each of the options.

Question 2: Companies are requested to provide their views on the specification changes required for each option for RAN and SA2 specifications. For the impact analysis, the additional efforts required over the minimum changes required at RAN and CN to support IoT-NTN connectivity are to be considered.  The efforts can be stated as high /medium/low along with the reason for the same.

For option 1:

|  | RAN specification Impacts | SA specification impacts |
| --- | --- | --- |
| MediaTek | Large impacts |  |

For option 2:

|  | RAN specification Impacts | SA specification impacts |
| --- | --- | --- |
| MediaTek | Less impacts |  |

## Functionality Impacts of Option 2

Question 3: Please provide views on impacts to idle mode and connected mode mobility scenarios from the RAN perspective for option 2.

|  | Impacts on mobility scenarios |
| --- | --- |
| MediaTek | Mobility management, including cell selection/re-selection and Tracking Area Update (TAU) will be managed differently for TN and NTN systems. Current R-17 standards are already prepared to handle such scenarios. |

## Specification Impacts and backward compatibility of RAN-based solution proposed for Option 1

Question 4: For option 1, IN [3] solutions are proposed for RAN to support single container with changes in the capability inquiry procedure and also the Radio-paging capability. Please provide views on the specification efforts and backward compatibility aspects of this solution.

|  | Comments on specification impacts and backward compatibility for O1 |
| --- | --- |
| MediaTek | This will have relatively more specification changes in Rel-17 IoT-NTN. |

## Any other Impacts

Question 5: Are any other views /discussion points that need to be considered for the decision?

|  | Additional aspects |
| --- | --- |

## Preferred Solution

Question 6: Please indicate the preferred option based on the analysis provided in the previous questions

|  | The preferred option for Rel-17 |
| --- | --- |
| MediaTek | Option 2 |

# Conclusion

Based on the response from companies on the two alternatives for TN and NTN capabilities following are the rapporteur proposal for conclusion on this topic.

Proposal X :  To be updated based on company views

# References

[1] R2-2209359	Reply to LS on UE capability signalling for IoT-NTN (S2-2207839; contact: Vodafone)	SA2	LS in	Rel-17	LTE_NBIOT_eMTC_NTN	To:RAN2	Cc:CT1, RAN3

[2] R2-2210075	Analysis on the CN impacts for TN and NTN capabilities based on SA2 LS	Nokia, Nokia Shanghai Bell	discussion	Rel-17

[3] R2-2209712	Discussion on SA2 LS reply on UE capability for IoT NTN	Qualcomm Incorporated	discussion	Rel-17	LTE_NBIOT_eMTC_NTN

[4] R2-2210734	UE capability signalling in IoT NTN	Ericsson	discussion	Rel-17	LTE_NBIOT_eMTC_NTN

[5] R2-2210414	UE capability signalling for IoT-NTN	Huawei, HiSilicon	discussion	Rel-17	LTE_NBIOT_eMTC_NTN

[6] R2-2210076	[draft] Reply to LS on SA2	Nokia, Nokia Shanghai Bell	LS out	Rel-17	LTE_NBIOT_eMTC_NTN	To:SA2	Cc:CT1, RAN3

[7] R2-2210528	[DRAFT] Reply LS on RAN feedback for UE capabilities signalling for IoT NTN	ZTE Corporation, Sanechips	LS out	Rel-17	LTE_NBIOT_eMTC_NTN-Core	To:SA2	Cc:RAN3