3GPP TSG-RAN WG2 Meeting #119bis Electronic                     draft- R2-2210849

- E-Meeting, 10th – 19th Oct., 2022

Agenda item:	7.2.5

Source:	MediaTek

Title:	[AT119bis-e][108][IoT NTN] UE Capabilities (MediaTek)

Document for:	Discussion & Decision

# Introduction

This document summarizes the email discussion below that took place during RAN2#119bis -e meeting:

- [AT119bis-e][108][IoT NTN] UE capabilities (Mediatek)

Initial scope: Discuss proposals in AI 7.2.5 (apart from those on capability signalling for IoT-NTN)

Initial intended outcome: Summary of the offline discussion with e.g.:

- List of proposals for agreement (if any)

- List of proposals that require online discussions

- List of proposals that should not be pursued (if any)

Deadline (for companies' feedback):  Thursday 2022-10-13 18:00 UTC

Deadline (for rapporteur's summary in R2-2210849):  Thursday 2022-10-13 22:00 UTC

Proposals marked "for agreement" in R2-2210849 not challenged until Friday 2022-10-14 10:00 UTC will be declared as agreed via email by the session chair (for the rest the discussion might continue online).

# Reference

The following documents are treated in this email discussion:

7.2.5	UE capabilities corrections

Capability signalling for IoT-NTN

R2-2209439	Add support of reception of SIB32	MediaTek Inc.	CR	Rel-17	36.306	17.2.0	1860	-	F	LTE_NBIOT_eMTC_NTN-Core

- Continue in offline 108

R2-2210078	Corrections for capability for NPRACH segmentated Transmission	Nokia Solutions & Networks (I)	CR	Rel-17	36.306	17.2.0	1861	-	F	LTE_NBIOT_eMTC_NTN

- Continue in offline 108

R2-2210776	Correction in the description of ntn-Connectivity-EPC-r17	Lenovo, Motorola Mobility (rapporteur)	CR	Rel-17	36.306	17.2.0	1862	-	F	LTE_NBIOT_eMTC_NTN-Core

- Continue in offline 108

# Contact information

| Company | Contact Name | Email |
| --- | --- | --- |
| MediaTek | Abhishek Roy | Abhishek.Roy@mediatek.com |

# Discussion

## Add support of reception of SIB32

In the CR in R2-2209439 [1] the following changes to TS 36.306 are proposed:

6.19.2 Discontinuous coverage It is optional for a UE camped on NTN cell to support discontinuous coverage as specified in TS 36.304 [14] and reception of SystemInformationBlockType32(-NB) as specified in TS 36.331 [5]. This feature is only applicable if the UE supports ntn-Connectivity-EPC-r17.

Question 1: Do companies agree on the proposed change in TS 36.306?

| Company | Yes/No | Additional comments |
| --- | --- | --- |

## Corrections for capability for NPRACH segmented Transmission

In the CR R2-2210078 [2] the following changes to TS 36.306 are proposed:

4.3.38 IoT NTN parameters 4.3.38.1 ntn-Connectivity-EPC-r17 This field indicates whether the UE supports NTN access. This field is only applicable if the UE supports ce-ModeA-r13 or any ue-Category-NB. If the UE indicates this capability the UE shall support the following enhancements: - General: - handling of cellBarred-NTN-r17 and trackingAreaList-r17 in SystemInformationBlockType1(-NB) as specified in TS 36.331 [5]; - reception of SystemInformationBlockType31(-NB) as specified in TS 36.331 [5]; - derivation of its position based on its GNSS measurements; - reporting of the remaining GNSS validity duration as specified in TS 36.331 [5]; - PDCP: - if the UE supports ce-ModeA-r13, discardTimerExt-r17 as specified in TS 36.331 [5]; - RLC: - t-ReorderingExt-r17 as specified in TS 36.331 [5]; - MAC: - estimation of UE-gNB RTT as specified in TS 36.321 [4]; - delaying the start of the RA response window as specified in TS 36.321 [4]; - delaying the start of the mac-ContentionResolutionTimer as specified in TS 36.321 [4]; - if the UE supports ce-ModeA-r13 or if the UE supports any ue-Category-NB and supports sr-WithoutHARQ-ACK-r15, sr-ProhibitTimerExt-r17 as specified in TS 36.331 [5]; - Physical layer: - calculation of the UE specific TA in RRC_IDLE and RRC_CONNECTED state based on its GNSS-acquired position and the serving satellite ephemeris as specified in TS 36.211 [17]; - calculation of the common TA in RRC_IDLE and RRC_CONNECTED as specified in TS 36.213 [22]; - frequency pre-compensation using k-Offset-r17 and k-Mac-r17 as specified in TS 36.213 [22]; - timing relationship enhancements using higher layer parameters k-Offset-r17 and k-Mac-r17 as specified in TS 36.213 [22]; - segmented UL transmission using higher layer parameters prach-TxDuration-r17, nprach-TxDurationFmt01-r17, nprach-TxDurationFmt2-r17, pucch-TxDuration-r17 and (n)pusch-TxDuration-r17 as specified in TS 36.331 [5]. except for UEs indicating support of ue-Category-NB and ntn-ScenarioSupport-r17 with value GSO. A UE indicating support of ce-ModeA-r13 and ntn-Connectivity-EPC-r17 shall also indicate support of standaloneGNSS-Location. A UE indicating support for any ue-Category-NB and ntn-Connectivity-EPC-r17 is assumed to have GNSS location capability. A UE indicating support of ce-ModeA-r13 and ntn-Connectivity-EPC-r17 shall also indicate support of standalone GNSS-Location.

Question 2: Do companies agree on the proposed change in TS 36.306?

| Company | Yes/No | Additional comments |
| --- | --- | --- |

## Correction in the description of ntn-Connectivity-EPC-r17

In the CR R2-2210776 [3] the following changes to TS 36.306 are proposed:

#### 4.3.38.1 ntn-Connectivity-EPC-r17 This field indicates whether the UE supports NTN access. This field is only applicable if the UE supports ce-ModeA-r13 or any ue-Category-NB. If the UE indicates this capability the UE shall support the following enhancements: - General: - handling of cellBarred-NTN-r17 and trackingAreaList-r17 in SystemInformationBlockType1(-NB) as specified in TS 36.331 [5]; - reception of SystemInformationBlockType31(-NB) as specified in TS 36.331 [5]; - derivation of its position based on its GNSS measurements; - reporting of the remaining GNSS validity duration as specified in TS 36.331 [5]; - PDCP: - if the UE supports ce-ModeA-r13, discardTimerExt-r17 as specified in TS 36.331 [5]; - RLC: - t-ReorderingExt-r17 as specified in TS 36.331 [5]; - MAC: - estimation of UE-gNB RTT as specified in TS 36.321 [4]; - delaying the start of the RA response window as specified in TS 36.321 [4]; - delaying the start of the mac-ContentionResolutionTimer as specified in TS 36.321 [4]; - if the UE supports ce-ModeA-r13 or if the UE supports any ue-Category-NB and supports sr-WithoutHARQ-ACK-r15, handling of sr-ProhibitTimerOffset-r17 as specified in TS 36.331 [5]; - Physical layer: - calculation of the UE specific TA in RRC_IDLE and RRC_CONNECTED state based on its GNSS-acquired position and the serving satellite ephemeris as specified in TS 36.211 [17]; - calculation of the common TA in RRC_IDLE and RRC_CONNECTED as specified in TS 36.213 [22]; - frequency pre-compensation using k-Offset-r17 and k-Mac-r17 as specified in TS 36.213 [22]; - timing relationship enhancements using higher layer parameters k-Offset-r17 and k-Mac-r17 as specified in TS 36.213 [22]; - segmented UL transmission using higher layer parameters (n)prach-TxDuration-r17, pucch-TxDuration-r17 and (n)pusch-TxDuration-r17 as specified in TS 36.331 [5] except for UEs indicating support of ue-Category-NB and ntn-ScenarioSupport-r17 with value GSO; A UE indicating support of ce-ModeA-r13 and ntn-Connectivity-EPC-r17 shall also indicate support of standaloneGNSS-Location. A UE indicating support for any ue-Category-NB and ntn-Connectivity-EPC-r17 is assumed to have GNSS location capability.

Question 3: Do companies agree on the proposed change in TS 36.306?

| Company | Yes/No | Additional comments |
| --- | --- | --- |

# Conclusion

TBD