3GPP TSG RAN WG2 #119bis-e	R2-2210813

Online, 10th-19th October 2022

Source: 	China Telecom

Title: 	[DRAFT] Summary of [AT119bis-e][204][QoE] Rel-17 leftovers for QoE (China Telecom)

Agenda Item:	8.14.3

Document for:	Discussion and decision

# Introduction

This contribution addresses the following offline discussion:

- [AT119bis-e][204][QoE] Summary of Rel-17 leftovers for QoE (China Telecom)

Scope: Summarize content of Tdocs under AI 8.14.3. Collect companies’ views on the priority of each issue and identify proposals which can be most easily progressed in Rel-18.

Intended outcome: Report in R2-2210813

Initial deadline (for companies’ feedback): Thursday 2022-10-13 0700 UTC

Initial deadline (for rapporteur's summary): Thursday 2022-10-13 1200 UTC

# Contact Information

Respondents to the email discussion are kindly asked to fill in the following table.

| Company | Name | Email Address |
| --- | --- | --- |
| Lenovo | Hyung-Nam Choi | hchoi5@lenovo.com |
| Apple | Ping-Heng Wallace Kuo | pingheng_kuo@apple.com |
| Qualcomm | Jianhua Liu | jianhua@qti.qualcomm.com |
| Huawei, HiSilicon | Jun Chen | jun.chen@huawei.com |
| China Unicom | Shuai Gao | gaos30@chinaunicom.cn |
| CATT | Haocheng Wang | wanghaocheng@catt.cn |
| ZTE | Zhihong Qiu | qiu.zhihong@zte.com.cn |
| Samsung | Seungbeom Jeong | s90.jeong@samsung.com |

# Discussion

## 3.1	Per-slice QoE measurement configuration enhancement

### 3.1.1 Per-slice QoE measurement

On per-slice QoE measurement configuration enhancement, RAN3 agreed to introduce the slice scope information in the configuration container and sent LS to SA4. However, whether to include the slice information outside the QoE configuration and reporting container or inside the QoE configuration container is still unclear. Without the slice information, it may result in erroneous and unnecessary per-slice QoE measurements and reporting by the UE. Since the RAN3 agreement does not impact AS, there seems no impacts to RAN2. In the contributions [2][3][6][7][10], all companies agree to introduce the slice scope information in the configuration container. And the contributions [2][3][6][7] also think there is no further work required for slice-based QoE from RAN2’s perspective.

Q1: Do companies agree that there is no further work in RAN2 for slice-based QoE measurement?

Please provide any view / comments on this topic and the question in bold below:

| Company | Yes/No | Comments |
| --- | --- | --- |
| Lenovo | Yes | At least for the encapsulated QoE measurement configuration. |
| Apple | Yes |  |
| Qualcomm | Yes |  |
| Huawei, HiSilicon | Yes |  |
| CATT | Yes |  |
| ZTE | Yes |  |
| Samsung | Yes |  |

### 3.1.2 Per-slice RAN visible QoE

Another issue raised by [6][9] is whether to introduce the enhancement to per-slice RAN visible QoE. The motivation is to allow UE only report QoE for configured slice. Currently, this issue is still FFS in RAN3. With regard to this issue, companies have different views on future work in RAN2:

- Option 1[6]: no need to introduce any enhancement for per-slice RAN visible QoE as the benefit is limited

- Option 2[9]: to introduce S-NSSAI in RAN visible QoE reports and configuration

- Option 3: RAN2 can wait for RAN3 progress on per-slice RAN visible QoE

Q2: Which of options (i.e. option 1/2/3) do companies prefer?

Please provide any view / comments on this topic and the question in bold below:

| Company | Option1/2/3 | Comments |
| --- | --- | --- |
| Lenovo | Option 3 | We think RAN3 is the group to decide on this. If agreed, RAN2 can work on the AS signalling details. |
| Apple | Option 1/3 | In our views, this is not essential. However, if RAN3 agrees more AS signalling is needed for this, then RAN2 can work on it. |
| Qualcomm | Option 3 | For RVQoE, it depends on whether the slice for RVQoE may be different with container based QoE. Let’s wait for RAN3 progress. |
| Huawei, HiSilicon | Option 3 | RAN3 progress is helpful for RAN2 discussions. |
| CATT | Option 1/Option3 | For RAN Visible QoE, we notice that there are only two RAN visible QoE metrics, while the size of slice configuration is bigger than that, so there is no benefit to introduce enhancement to per-slice RAN visible QoE currently. As this is also discussed in RAN3, we are also fine to wait for RAN3 process. |
| ZTE | Option 3 | Although we think it beneficial to introduce S-NSSAI in RAN visible QoE reports and configuration, but we are fine to wait for RAN3. |
| Samsung | Option 3 | RAN3 is expected to discuss it. |

## 3.2	RAN visible QoE enhancements

### 3.2.1 RAN visible QoE value

In R17 QoE, the metrics in the RAN visible QoE only include the buffer level and playout delay. According to the R18 WID, RAN3 and RAN2 need to discuss the enhancements to RAN visible QoE, e.g. QoE value. Such QoE value report could be useful for RAN to determine whether some radio configuration changes are needed to improve QoE for the user. Most companies think it is up to RAN3 to decide whether or not to introduce this RVQoE value. Therefore, RAN2 needs to wait for the progress of RAN3 before the discussion on the RVQoE value [2][3][5][6][7][10]. One company thinks there is no need to specify RAN visible QoE value, but this may be revisited later in the WI [9].

Q3: Do companies agree that RAN2 needs to wait for the progress of RAN3 and other WGs, e.g. SA4, on RVQoE value?

Please provide any view / comments on this topic and the question in bold below:

| Company | Yes/No | Comments |
| --- | --- | --- |
| Lenovo | Yes | RAN3 discussed this topic for a while (since R17 study phase) and still was not able to reach consensus yet. There is no need for RAN2 to interfere. |
| Apple | Yes | There is no point for RAN2 to discuss before any consensus is reached in RAN3. |
| Qualcomm | Yes | Wait for RAN3 decision |
| Huawei, HiSilicon | Yes |  |
| China Unicom | Yes | We prefer to wait for the progress, or else it’s too early to make decision on whether QoE value is needed. |
| CATT | Yes | It is not clear that how to use RV QoE value in RAN3, so RAN2 can wait for the RAN3’s progress on RV QoE value and then discuss how to support the value transfer. |
| ZTE | Yes | Better to wait for the outcome of RAN3 discussion. |
| Samsung | Yes |  |

### 3.2.2 RAN visible QoE trigger event

The motivation of this enhancement is to introduce RAN visible QoE trigger events which allows the network to collect QoE information from UEs for some deployment related scenarios, such as high-speed scenarios, poor coverage scenarios and high interference scenarios. According to the Rel-18 QoE WID, RAN3 and RAN2 need to discuss whether it would be beneficial to introduce the RAN visible QoE trigger event. With regard to this issue, some companies agree to support the event triggered RVQoE logging and reporting in Rel-18 as it can reduce the size of the QoE measurement report and reporting-related RRC-signalling [4][5][6][9], and relevant proposals are listed in the table below. However, some companies have concerns on the benefit of introducing event-based RVQoE reporting as it may introduce much more complexity for NW and UE[2][3][7][10].

In order to make progress on this topic, moderator also would like to request companies to provide your views on some proposals, e.g. benefits, spec impacts, complexities to UE and NW.

| Tdoc | Proposals related to RAN visible QoE trigger event |
| --- | --- |
| [4], Samsung | Proposal 1. Event-based RAN visible QoE report is supported in Rel-18. Proposal 2. Discuss which layer (AS or application) is responsible to detect event(s) for RVQoE report. |
| [5], Qualcomm | Proposal 2: gNB can configure an event to UE for triggering RVQoE meansurement reporting. Proposal 3: Introduce new Event for BufferLevel reporting: The BufferLevel is below a configured threshold. Proposal 4: Introduce new Event for PlayoutDelay reporting: The PlayoutDelay is above a configured threshold. Proposal 5: RAN2 discusses which layer (application layer or RRC layer) performs RVQoE event evaluation. |
| [6], CATT | Proposal 3: RAN2 study what is the trigger event need to be defined for RV QoE, such as location, time, and some threshold. |
| [9], Ericsson | Proposal 3 Consider event triggered RVQoE logging and reporting in Release 18. Proposal 4 Consider using RVQoE metrics fulfilling certain conditions, as triggers for RVQoE reporting. |

For possible enhancements, here is a summary:

- Option 1: gNB can configure an event to UE for triggering RVQoE meansurement reporting, and it can be:

  - new Event for BufferLevel reporting: The BufferLevel is below a configured threshold. FFS which layer performs RAQoE event evaluation

- Option 2: gNB can configure an event to UE for triggering RVQoE meansurement reporting, and it can be:

  - new Event for PlayoutDelay reporting: The PlayoutDelay is above a configured threshold. FFS which layer performs RAQoE event evaluation

- Option 3: RAN2 study what is the trigger event need to be defined for RV QoE, such as location, time, and some threshold.

- Option 4: Consider using RVQoE metrics fulfilling certain conditions, as triggers for RVQoE reporting.

- Option 5: Others (some companies propose enhancements but there are no details, so more inputs would be needed for understandings)

- Option 6: No enhancements

Q4: Which of options do companies prefer? If you have concerns/comments on one or more options, please also input them in the following table.

| Company | Option 1/2/3/4/5/6 | Comments (e.g. benefits, spec impacts, complexities to UE and NW) |
| --- | --- | --- |
| Lenovo | None | RAN3 is still discussing this topic incl. the clarification of the benefit of such triggers. Therefore, RAN2 should wait for RAN3 conclusion first. |
| Apple | 6 or 5 | Event-triggered QoE reporting increases complexity in both UE implementation and specification. In particular, the UE needs to evaluate from time to time whether a condition is met, which obviously worsens battery consumption. On the other hand, from standardization point of view RAN2 would need to further discuss whether this is conducted by Application layer or AS layer, which can be quite a controversial topic and may require time-consuming discussions. Considering the complexity of this topic, as well as the limited time budget in RAN2 for this WI, we think this issue should be de-prioritized. If reporting overhead is the key motivation for this enhancement, we think RAN2 can consider increase the value range of RVQoE reporting periodicity in the specification. Thus, the gNB can simply configure a large reporting periodicity to minimize the overhead. |
| Qualcomm | 1 and 2 | Proponent. In Rel-17, periodical RVQoE measurement reporting is introduced and with this, lots of the reported data is not useful for gNB and consume much signalling overhead and gNB processing resource. Like today’s radio measurement, event-triggered RVQoE reporting can bring benefit to report only useful measurement to gNB. Such useful measurements refer to those measurements which results are beyond the normal values then gNB can perform real-time radio configuration optimization. Then it is worthy to define an event for UE to report those useful measurements to gNB. |
| Huawei, HiSilicon | FFS | Firstly, we think RAN2 can have some discussions on this topic. Secondly, we think one motivation of the RAN visible QoE trigger event is to reduce the signalling overhead of the reporting, but we have not seen any quantifications of the potential gains. For option 1/2/3/4, we share similar views as Apple about the complexities, which will be a big challenge for RAN2. We think similar goals can be achieved by setting the RVQoE reporting periodicity to longer value and changing it to a more frequent value in case the reported QoE metrics/values fall below a certain level. In general, we are concerned about the workload for RAN2 (and also for other WGs) for the listed options and before we agree to anything, we should clearly how much we can really gain by having this mechanism and whether it is worth the additional complexity. |
| CATT | Option1,2,3 | Although the event-trigger will introduce some complexity, we think it is worth to avoid the unnecessary QoE report and send the useful RV QoE report at the some exact events. For the trigger events, we can mainly discuss the threshold based events and location based events. For the threshold based events, we can introduce the BufferLevel threshold (Option1), PlayoutDelay (Option2), and radio related threshold for poor coverage scenarios, such as RSRP threshold. For location based events, we can introduce the event that when handover occurs, UE should trigger the RV QoE reporting. As the handover procedure may have the impact on RV QoE metric measurement result. At least for high speed scenarios, when UE enters the cell supporting high speed, UE should trigger the RV QoE report for high speed scenarios as the RV QoE measurement result may change a lot. The real-time RV QoE report can help the network to know the UE’s QoE quality in time and make the decision. |
| ZTE | FFS | Similar view as Huawei, it can be discussed case by case with consideration on both the gain and required work. |
| Samsung | Option 1/2/3/4 | As we described in [2], we support event-based RVQoE reporting because - With event-based RVQoE report, gNB can only receive useful QoE reports (e.g., QoE reports indicating poor QoE) by configuring some event(s) it wants. - If RVQoE report is triggered by event(s), this may reduce RVQoE reporting overhead compared to short periodic RVQoE report. - If the event(s) is defined to capture some timing gNB wants to measure, UE can measure/sample the exact QoE data or moment. Most importantly, at this early stage, we would like to focus on whether to adopt event-based RVQoE reporting or not, rather than all detailed options. |

### 3.2.3 RAN visible QoE report

In the last meeting of RAN3, RAN3 has agreed that UE should include QoS flow information in the RVQoE report to RAN. The reason is that the PDU session ID alone may not be sufficient for gNB to know which radio resource configurations may need to be adjusted. In the contributions [3][6][7], it is proposed to introduce the QoS flow information in the RVQoE report from the UE. On the details of QoS flow information e.g., QoS flow ID, DRB ID, PDU session ID, [3][6][7] also agree to use QoS flow ID as QoS flow information in the RAN visible QoE report from the UE.

Q5: Do companies agree to add the QoS flow ID in the RAN visible QoE report from the UE.?

Please provide any view / comments on this topic and the question in bold below:

| Company | Yes/No | Comments |
| --- | --- | --- |
| Lenovo | Wait for RAN3 | RAN3 is further discussing details on QoS flow information e.g., QoS flow ID, DRB ID, PDU session ID. RAN2 should wait for the further details from RAN3. |
| Apple |  | Agree with Lenovo |
| Qualcomm | Yes | RAN3 already agreed to add QoS flow ID, or DRB ID information, RAN2 can confirm they are feasible. |
| Huawei, HiSilicon | Yes | RAN3 has agreed: • UE should include QoS flow information in the RVQoE report to RAN. RAN2 can discuss which of QoS flow information is needed, and QoS flow ID seems the only viable option as application layer has not visibility to DRBs while PDU session ID is not sufficient (which is the main reason to have QoS flow ID). |
| China Unicom | Wait for RAN3 | RAN3 is discussing the same issue. |
| CATT | Yes | Similar view with HW. |
| ZTE | Yes | In our understanding, a PDU session can correspond to one or more QoS flow, and a DRB can correspond to one or more QoS flow. In other words, RAN can know the corresponding DRB through a QoS flow ID. Therefore, we think that it is sufficient to include QoS flow ID in RVQoE report information. |
| Samsung | Wait for RAN3 | Similar view with HW but prefer to wait for RAN3 who is discussing this issue. |

In addition, [7] proposes to send a LS to CT1/SA4 to request CT1/SA4 to provide QoS flow related information to the UE from application layer.

Q6: If it is agreeable to add the QoS flow ID in the RAN visible QoE report from the UE, do companies agree to send a LS to CT1/SA4 to check the possible impacts, e.g. the application layer needs to provide the QoS flow ID to the UE?

Please provide any view / comments on this topic and the question in bold below:

| Company | Yes/No | Comments |
| --- | --- | --- |
| Lenovo | Wait for RAN3 | See answer to Q5 above. There is no need to send a RAN2 LS to CT1/SA4. |
| Apple |  | Agree with Lenovo |
| Qualcomm | Yes |  |
| Huawei, HiSilicon | Yes |  |
| China Unicom | Wait for RAN3 |  |
| CATT | Yes |  |
| ZTE | Yes |  |
| Samsung | Wait for RAN3 |  |

## 3.3	QoE reporting enhancement for overload scenario

In Rel-17 the reporting of encapsulated QoE measurements can be temporarily paused in RAN overload situations. It is left to gNB implementation which encapsulated QoE configurations of a UE to pause for reporting. The RRC specification allows a selective pause, i.e. the gNB can pause reporting of one or multiple or all QoE configurations of a UE. According to the scope of WID in Rel-18, QoE reporting handling enhancement for overload scenario needs to be discussed. In the contributions [6][7][10], companies support to enhance QoE reporting in overload scenario, while two companies [2][8] still have concerns on whether this enhancement is needed and propose to wait for the conclusion or input from RAN3.

Q7: Do companies agree to wait for the progress of RAN3 on QoE reporting enhancement in overload scenario?

Please provide any view / comments on this topic and the question in bold below:

| Company | Yes/No | Comments |
| --- | --- | --- |
| Lenovo | Yes | We wonder why NW has problem with the current Rel-17 flexibility with regards to pausing of encapsulated QoE reporting. But if RAN3 agrees to introduce a new “service priority” attribute which OAM/CN can send to gNB we will not object. |
| Apple | Comments | This depends on what “QoE reporting enhancement” we are considering here. Some enhancements are hinged to RAN3 status while some can be handled by RAN2 alone. In this sense we think RAN2 can first discuss the proposals that only have RAN2 impacts, while waiting for RAN3 on some other issues. |
| Qualcomm | Yes | According to WID discussion, the overload scenario is mainly about QoE priority for reporting pause and resume selection. Wait for RAN3. |
| Huawei, HiSilicon | Ok for discussions, but observe no RAN2 impacts | In our paper, we propose: - OAM should provide priorities for the management based QoE configurations to the gNB to allow the network better control QoE reporting during RAN congestion. We think we could agree this directly in RAN2 as it is related to network control for Uu interface overhead/congestion. However, this would have to be indeed confirmed by RAN3 as the only specifications impact is on network interfaces signalling. |
| China Unicom | No | In the Rel-17, RAN2 is responsible for handling RAN overload issues. So RAN2 can decide whether we need ‘priority’ directly without RAN3’s progress. Besides, In the case that the AS layer buffer is full, the QoE reports will have to be discarded based on UE implementation, which will not satisfy the network requirements of high-priority QoE reports collection. So if gNB can send the priority information to the UE, the UE can discard lower-priority reports when the buffer is full, which will has some benefits for the network. |
| CATT | No | Similar view as CU, RAN2 should discuss this issue. |
| ZTE | Yes | Wait for RAN3’s conclusion. |
| Samsung |  | Agree with Apple. Depends on which option is considered for QoE reporting enhancement in overload scenario. |

If the answer of Q7 is no, moderator would like to invite companies to provide your views on the following issues:

- Whether to send the priority information to UE?

With regard to this issue, there are two alternatives according to the submit contributions in this meeting.

- Alt1: to send priority information to UE[6][10]. In the contribution [10], it is mentioned that the buffer size in UE APP is limited, it is beneficial for UE to get the priority of the certain QoE configuration. Then the UE can buffer the higher priority QoE data at first.

- Alt2: the priority information is only required at the gNB[7]. And it also thinks there is no need to send the priority for signalling based QoE measurements from the CN and proposes to only provide the priority information for management based QoE configurations.

Q8: Which alt do you prefer? Alt1 or Alt2?

Please provide any view / comments on this topic and the question in bold below:

| Company | Alt1/Alt2 | Comments |
| --- | --- | --- |
| Lenovo |  | - On Alt1: Why is the buffer size in UE APP limited? Or is it a typo and it should say “AS layer”? When pause/resume functionality was discussed in Rel-17, some companies preferred to store the QoE reports in APP layer since it provides much larger storage capacity compared to AS layer. In general, we are not convinced of the value of sending the priority information to UE. Does it really matter when QoE reports are sent faster to NW acc. to the priority of the associated QoE configuration? - On Alt2: whether the service priority attribute is used for m-based QoE or both m-based and s-based QoE can be left to RAN3 to decide. |
| Apple | Alt.2 | If any priority information is needed, this is only needed by the gNB to decide which application layer measurement configuration should be paused or resumed. This is a purely gNB implementation and the UE would anyway just follow the indication sent by the gNB, therefore we are not sure what are the benefits of having such information at the UE side. |
| Qualcomm |  | Alt2 should be discussed in RAN3, Alt1 can be further discussed in RAN2 and we see it is beneficial that higher priority QoE data will be buffered when UE buffer is limited. |
| Huawei, HiSilicon | Alt. 2 | Proponent. Agree with Apple’s comments. |
| China Unicom | Alt1 | For Alt1, the ‘APP’ shall be ‘AS’, and the benefits are clear as we answered in Q7. For Alt2, it’s not clear why the service priority cannot be used for s-based QoE but only for m-based QoE. |
| CATT | Alt1 | We think it is beneficial for UE to select QoE measurement tasks. As in current specification, multiple QMC jobs may be inactivated for a UE when NG-RAN overload. When overload is relieved, it is impossible to send all saved QoE report to network at the same time. UE can select the important QoE report to send first based on the priority information. |
| ZTE |  | It seems not need to introduce both NW and UE based solutions. And normally if something can be done by NW without significant burden then we don’t introduce UE based solution since the resource in air interface is more limited. Anyway we will need to wait for RAN3’s conclusion on this topic. |
| Samsung |  | For Alt1, we understand there are 2 different options for UE-side priority based on comments from China Unicom and CATT. Alt1-1) UE discards low priority QoE report first when AS memory is full. Alt1-2) UE sends high-priority QoE report first when resume (i.e., when overload is resolved) We think Alt1-1 can be discussed, but not sure how much its benefit is, at the expense of signalling overhead (e.g., priority from OAM to UE). For Alt1-2, QoE reports do not usually require real-time delivery to gNB or CN. Besides, all logged reports are delayed enough during pause. The order of transmission after resume does not affect QoE report delivery time much. In short, we don’t see clear benefits of Alt 1 (Alt1-1 and Alt 1-2). And Alt 2 can be discussed in RAN3. |

- the granularity of priority

In the contribution [6], it proposes that priority can be configured per QoE reference. If a UE is configured with multiple QoE measurements, each QoE reference is associated with a priority.

Q9: Do companies agree that the priority can be configured per QoE reference?

Please provide any view / comments on this topic and the question in bold below:

| Company | Yes/No | Comments |
| --- | --- | --- |
| Lenovo |  | Can be left to RAN3 to decide. |
| Apple |  | We don’t think this is RAN2 scope |
| Qualcomm |  | Wait for RAN3 decision |
| Huawei, HiSilicon |  | The details can be left to RAN3. |
| China Unicom | Yes |  |
| CATT | Yes | We think it is flexible to configure the priority as per QoE reference. |
| ZTE |  | Shall up to RAN3 if they agree to do so. |
| Samsung |  | If any (UE-side or gNB-side) priority is agreed, RAN3 would discuss it. |

- new UAI message

Contribution [1] proposes a new “UE overload” cases where the UE are not suitable to perform QoE measurement/reporting activities due to their status, e.g. the battery and/or memory of UEs are running low. In order to assist the network to acquire the information about the UE status when deciding if QoE reporting of a UE should be continue, [1] proposes to introduce a new UE Assistance Information (UAI) message to express the UE’s preference on configured QoE reporting (e.g. to pause or release QoE reporting) in Rel-18.

Q10: Do companies agree to introduce a new UAI message to express the UE’s preference on QoE reporting configurations?

Please provide any view / comments on this topic and the question in bold below:

| Company | Yes/No | Comments |
| --- | --- | --- |
| Lenovo |  | Questions for clarifications: - On battery problems: how much does battery consumption account for QoE measurement/reporting compared to the many other things the UE does? And if battery is running low shouldn’t the UE better initiate UAI for power saving purpose, e.g. by indicating its preferred RRC state? - On memory problems: not clear about the memory problems the UE may face. In which scenario may this happen? Are memory problems in AS layer meant? How much does QoE measurement/reporting account in memory usage? |
| Apple (Proponent) | Yes, but we are open for discussion | We think it is beneficial for the network to know UE’s preference about QoE reporting. For example, in M-based QoE the NG-RAN selects UE(s) that meet the required QoE measurement capability, area scope and slice scope. If the NG-RAN further knows the preference of certain UEs, it further helps the gNB to decide which UEs it should choose. Similarly, in RAN overload situations, the gNB may need to pause QoE reporting from some UEs. With UE’s preference information, the gNB can choose the UEs to pause or resume QoE reporting more judiciously. To save UE power, we think the UE cannot always just indicate its preferred RRC state. For instance, the UE may still want to stay in CONNECTED mode in order to continue the operations of the application, even if the battery is running low. In our views, this is fair for UEs to express some preference about whether QoE should be configured, paused, and/or resumed. |
| Qualcomm | No | We don’t think this is in the scope, during Rel-18 WID discussion, this objective is meant for QoE priority. Technically, For battery, agree with Lenovo there is no or less additional power consumption since there is no additional TX/RX for QoE collection. For memory, there is a lot of discussion in Rel-17 about data buffering, and when we agreed to buffer in AS layer with required 64KB size, the UE has to reserve the memory for QoE collection. |
| Huawei, HiSilicon | No | We think this is out of WID scope. Additionally, we also do not see the issue with power consumption. The power consumption due to QoE measurements themselves is probably marginal compared to the power needed for the application running at the UE. In case the UE’s battery life is low, then application can be closed which means also QoE measurements will end. For memory, we also do not see the issue, i.e. compared to the applications for which QoE can be configured, the required memory for measurements only should be marginal. |
| China Unicom | No | It’s unreasonable to forbid QoE measurements collection while UE is still running it’s app. In other words, isn’t there a need to optimize the network experience when the UE is at the low battery level? |
| CATT | No | Similar view with HW. The QoE measurement will not cause serious issues on power consumption and memory. |
| ZTE | No | Share the same view as Huawei and Qualcomm |
| Samsung | No |  |

- QoE reporting via the SN

In the contribution [9], it proposes that in DC scenarios, QoE reporting via the SN will be one possibility to steer the QoE reports to a node which may be less loaded. Since DC may not always be available or may not even be supported by all UEs, for these situations, there is also the possibility to pause QoE reporting.

Q11: Do companies agree to use QoE reporting via the SN to handle overload scenarios in DC scenarios?

Please provide any view / comments on this topic and the question in bold below:

| Company | Yes/No | Comments |
| --- | --- | --- |
| Lenovo |  | Strictly speaking it is mentioned as an observation and not as a proposal. Anyway, should be discussed as part of QoE support in NR-DC. |
| Apple | Yes but | We agree with the intention, but this should be discussed in the agenda item relating to NR-DC |
| Qualcomm |  | Discussion in NR-DC |
| Huawei, HiSilicon | Yes, but… | This is related to NR-DC agenda item and RAN3 has already agreed that the reporting leg can be chosen when QoE configuration is provided in DC scenario. |
| China Unicom |  | It’s not R17 left issues, can be discussed in NR-DC parts. |
| CATT |  | This can be discussed in NR-DC agenda item. |
| ZTE |  | Can be discussed in NR-DC session together with RAN3’s LS. |
| Samsung |  | Can be discussed in NR-DC |

- QoE reporting via unlicensed band

In the contribution [1], it is proposed that QoE reporting via unlicensed band is a sensible approach to deal with potential overload situations during a QoE session. And the unlicensed band resource is simply a complementary data pipe for the UE to provide QoE measurements, which can be entirely independent to operations of the targeted application of QoE. To support SRB4 transmission over unlicensed band, the only required change in specification is the addition of CAPC definition for SRB4. So the foreseeable specification impacts are extremely light. Therefore, it proposes that RAN2 should define CAPC for SRB4 (fixed or configurable) in Rel-18, which enables offloading of QoE reporting to unlicensed band and hence alleviates RAN overloading.

Q12: Do companies agree to define CAPC for SRB4 (fixed or configurable) in Rel-18 to enable offloading of QoE reporting to unlicensed band?

Please provide any view / comments on this topic and the question in bold below:

| Company | Yes/No | Comments |
| --- | --- | --- |
| Lenovo | No | When RAN2 discussed pause/resume functionality in Rel-17, SA4 indicated an average QoE load per application of <100 bits/sec. Even if we consider more advanced services in the future, the average QoE load per application may still be lower than other SRBs or DRBs. Therefore, offloading SRB4 to unlicensed spectrum does not much alleviate RAN overload. Instead, other UL traffic should be better offloaded to unlicensed spectrum. |
| Apple (Proponent) | Yes | When considering any new enhancement for QoE reporting in overload situations, we must first note what offloading mechanisms are already available in 3GPP framework as the baseline. In this case, offloading to unlicensed band is indeed a straightforward approach (with very light specification impacts - no new UE capability or new RRC parameter is needed) as NR-U feature is anyway already available since Rel-16. Therefore, regardless of what is the average QoE load per application, 3GPP specification should not intentionally forbid transmission of SRB4 on unlicensed band without technical arguments. When unlicensed band resource is available (e.g. can be configured as a Scell), both gNB and UE should have the freedom to use it for SRB4 (or any other radio bearer) if the gNB/UE wants to do this for whatever reasons – such resource allocation is an implementation issue, and we do not see why 3GPP specification should artificially restrict the scheduling flexibility. We think CAPC for SRB4 should be defined, just like any other SRBs and DRBs. This also provides more specification consistency. If companies see any serious technical problem of sending QoE reports on unlicensed band, we are also open for discussion. |
| Qualcomm |  | Open for discussion if there is no enhancement expected. |
| Huawei, HiSilicon |  | QoE+NR-U is an new requirement in our opinion, and we are not sure whether it is in the WID scope. |
| China Unicom | No | Out of the scope. |
| CATT |  | Probably not in the WID scope |
| ZTE |  | Out of WID scope. |
| Samsung | No | Out of the scope. |

# Conclusion

# Reference

- R2-2209784	Views on QoE Reporting for Overload Scenarios	Apple	discussion	Rel-18	NR_QoE_enh-Core

- R2-2209830	Discussion on Rel-17 leftover features for QoE	Lenovo	discussion	Rel-18	NR_QoE_enh-Core

- R2-2209833	Discussion on Rel-17 leftover issues for QoE	ZTE Corporation, Sanechips	discussion	Rel-18	NR_QoE_enh-Core

- R2-2209837	Event-based RAN visible QoE report	Samsung	discussion	Rel-18

- R2-2209845	Discussion on RAN visible QoE trigger event	Qualcomm Incorporated	discussion	NR_QoE_enh-Core

- R2-2210015	Discussion on Rel-17 leftover issues for QoE	CATT	discussion	Rel-18	NR_QoE_enh-Core

- R2-2210204	Support of R17 left-over features	Huawei, HiSilicon	discussion	Rel-18	NR_QoE_enh-Core

- R2-2210275	QMC enhancements for RAN overload	Nokia, Nokia Shanghai Bell	discussion	Rel-18	NR_QoE_enh-Core

- R2-2210306	Discussion on rel-17 leftovers	Ericsson	discussion	Rel-18	NR_QoE_enh-Core

- R2-2210573	Discussion on QoE Rel-17 leftover issues	China Telecom Corporation Ltd.	discussion