3GPP TSG-RAN WG2#119bis-e	Draft R2-2210811

Online, 10 - 19 October 2022

Source: 	Huawei (rapporteur)

Title: 	[AT119-e][209][DCCA] UE measurements for CPC (Huawei)

Agenda Item:	6.2.2

Document for:	Discussion and decision

# 1	Introduction

This document is the summary of the following email discussion:

[AT119-e][209][DCCA] UE measurements for CPC (Huawei)

Scope: Discuss R2-2210718, R2-2210719 and R2-2210720.

Intended outcome: Report

Deadline: Deadline 2

Companies are invited to indicate the person providing input in the table below:

| Company | Name / Email |
| --- | --- |
| Huawei, HiSilicon | David Lecompte / david.lecompte at huawei.com |
| MediaTek | Felix Tsai / Chun-fan.tsai at mediatek.com |
| CATT | Bufang Zhang / zhangbufang@catt.cn |
| Intel | Tangxun / xun.tang@intel.com |
| Ericsson | Cecilia Eklöf / cecilia.eklof@ericsson.com |
| NEC | Hisashi Futaki/ hisashi.futaki @ nec.com |
| Nokia | jedrzej.stanczak@ nokia.com |
| Lenovo | pmallick@lenovo.com |
| Qualcomm | Punyaslok Purkayastha / punyaslo@qti.qualcomm.com |
| OPPO | Shukun Wang / wangshukun@oppo.com |
| ZTE | Mengjie Zhang / zhang.mengjie@zte.com.cn |
| vivo | Chenli / Chenli5g@vivo.com |
| DENSO | Daiki Maemoto / daiki.maemoto.j7w@jp.denso.com |
| LGE | Hongsuk Kim / hassium.kim at lge.com |
| ITRI | Nai-Lun Huang / NellenHuang at itri.org.tw |
| Samsung | June Hwang / june77.hwang@samsung.com |

# 2	Discussion

## 2.1	UE measurement requirements for conditional events in TS 38.331 (from R2-2210719)

Current requirements are:

### 5.5.3 Performing measurements #### 5.5.3.1 General [...] The UE shall: 1> for each measId included in the measIdList within VarMeasConfig: [...] 2> if the reportType for the associated reportConfig is periodical, eventTriggered; or 2> if the reportType for the associated reportConfig is condTriggerConfig and the measId is indicated in the condExecutionCond or in the condExecutionCondSCG associated to a condReconfigId in VarConditionalReconfig:

In the highlighted sentence, there is no distinction between MCG and SCG measId, and also not between MCG and SCG VarConditionalReconfig, so it seems the CondExecutionCond or the CondExecutionCondSCG in any VarConditionalReconfig (MCG or SCG) can indicate any measId.

For example, at CPC preparation, if the source SN have provided the configuration of an SCG measId that has the value "12" but the candidate target SN did not prepare the corresponding target cell and the MN still sent the configuration to the UE before informing the source SN, the UE will be configured with an SCG measId that has the value "12".

According to the above, it seems that if any entry in the MCG VarConditionalReconfig has an execution condition in CondExecutionCond including the value "12", the UE is required to perform measurements for the SCG measId with value "12". However, that entry is actually for CHO, CPA or MN-initiated CPC.

Therefore, it is proposed to clarify which field in which VarConditionalReconfiguration (MCG or SCG) can indicate which type of measId (MCG or SCG) as follows:

### 5.5.3 Performing measurements #### 5.5.3.1 General [...] The UE shall: 1> for each measId included in the measIdList within VarMeasConfig: [...] 2> if the reportType for the associated reportConfig is periodical, eventTriggered; or 2> if the reportType for the associated reportConfig is condTriggerConfig, the measId is within the MCG VarMeasConfig and is indicated in the condExecutionCond associated to a condReconfigId in the MCG VarConditionalReconfig; or 2> if the reportType for the associated reportConfig is condTriggerConfig, the measId is within the SCG VarMeasConfig and is indicated in the condExecutionCond associated to a condReconfigId in the SCG VarConditionalReconfig; or 2> if the reportType for the associated reportConfig is condTriggerConfig, the measId is within the SCG VarMeasConfig and is indicated in the condExecutionCondSCG associated to a condReconfigId in the MCG VarConditionalReconfig:

Question 1: Do companies agree that the statement on when the UE is required to perform measurements for conditional events in TS 38.331 should be improved to indicate how to match which measId - MCG or SCG - with which field - CondExecutionCond or CondExecutionCondSCG - in entries of which VarConditionalReconfig - MCG or SCG - (see TP above)?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Huawei | Yes | Proponent |
| MediaTek | Yes with comment | Change is okay but in the later CR coversheet, we would suggest to indicate which scenario is covered by the 3 different condTriggerConfig clauses For example First level 2> bullet is for CHO, CPA, and MN-initiated inter-SN CPC Second level 2> bullet is for intra-SN CPC without MN involvement Third level 2> bullet is for MN-initiated inter-SN CPC. Hope that our understanding is correct. |
| CATT | See comment | In general, we agree the principle “clarify which measId within which measConfig is associated with which conditional reconfiguration. However, how to capture it into the spec depends on the scenarios that the restriction of “skipped measIds” applies. Since there is not any common understanding of the scenarios that the restriction of “skipped measIds” applies (it was postponed in online meeting), so we do not think the TP/statement can be agreed now. |
| Intel | Yes | sympathy to MTK’s suggestion |
| Ericsson |  | We are fine with capturing the clarifications, even though we don’t see a big need for it. |
| NEC | Yes |  |
| Nokia | Yes | In previous release it was not possible to have multiple different conditional reconfigurations, so the problem of linking the condition to wrong configuration was not present. In Rel-17 this is possible, so clarification is OK to have. In addition, we suggest to add descriptive text to each subclause, to differentiate the cases clearly. I.e. that first subclause is for CHO and MN-initiated inter-SN CPC, second for Intra-SN CPC and third for: SN-initiated inter-SN CPC. |
| Lenovo | Yes |  |
| Qualcomm | Yes | The clarifications are fine. |
| OPPO | Yes |  |
| ZTE | Yes | We are also fine to capture the descriptive texts to clearly distinguish different cases, as Nokia suggested. |
| vivo | Yes | Although it’s not a critical issue in previous text, we are fine to capture some clarifications. |
| DENSO | Yes | Agree with Nokia. |
| LGE | See comment | We think this looks clearer if CHO/MN-initiated/inter or intra SN-initiated scenarios require this measurement requirement. However, since RAN2 needs some more discussion to decide whether only SN-initiated scenario requires this measurement requirement, we can’t accept this change proposal for now. |
| ITRI | Yes | We are fine with the change and support MediaTek and Nokia’s suggestions. |
| Samsung | Yes |  |

Proposal 1: Clarify the condition for the UE to perform measurements for conditional events, i.e. how to match which measId - MCG or SCG - with which field - CondExecutionCond or CondExecutionCondSCG - in entries of which VarConditionalReconfig - MCG or SCG, using R2-2210719 as baseline.

For inter-SN SN-initiated CPC in EN-DC, the measId of the execution condition is defined in the NR (SCG) VarMeasConfig so the applicable clause is 5.5.3.1 in TS 38.331. However, the corresponding conditional reconfiguration is VarConditionalReconfiguration as specified in TS 36.331.

Since current text in TS 38.331 for conditional events does not refer to this, the UE is not required to perform any conditional measurements for inter-SN CPC in EN-DC.

To correct this, the highlighted text is proposed to be added, in addition to the previous proposal.

### 5.5.3 Performing measurements #### 5.5.3.1 General [...] The UE shall: 1> for each measId included in the measIdList within VarMeasConfig: [...] 2> if the reportType for the associated reportConfig is periodical, eventTriggered; or 2> if the reportType for the associated reportConfig is condTriggerConfig, the measId is within the MCG VarMeasConfig and is indicated in the condExecutionCond associated to a condReconfigId in the MCG VarConditionalReconfig; or 2> if the reportType for the associated reportConfig is condTriggerConfig, the measId is within the SCG VarMeasConfig and is indicated in the condExecutionCond associated to a condReconfigId in the SCG VarConditionalReconfig; or 2> if the reportType for the associated reportConfig is condTriggerConfig, the measId is within the SCG VarMeasConfig and is indicated in the condExecutionCondSCG associated to a condReconfigId in the MCG VarConditionalReconfig; or 2> if the reportType for the associated reportConfig is condTriggerConfig, the measId is within the SCG VarMeasConfig and is indicated in the triggerConditionSN associated to a condReconfigurationId in VarConditionalReconfiguration as specified in TS 36.331 [10]:

Question 2: Do companies agree that, in the statement on when the UE is required to perform measurements for conditional events in TS 38.331, it is necessary to add a condition referring to VarConditionalReconfiguration in TS 36.331 as suggested above?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Huawei | Yes | Proponent |
| MediaTek | Yes |  |
| CATT | see comment | Same as Q1, before RAN2 concludes which scenario the restriction of “skipped measIds” applies, it is hard to decide whether the TP/statement can be agreed. |
| Intel | Yes |  |
| Ericsson |  | We are fine with the addition, even though we don’t see a big need for it. |
| NEC | Yes |  |
| Nokia | Yes | If we list the cases separately, then EN-DC shall be added. Also here, we suggest the same clarification as we have described for Q1. |
| Lenovo | Yes |  |
| Qualcomm | Yes | The clarifications are fine. |
| OPPO | Yes |  |
| ZTE | Yes |  |
| vivo | Yes |  |
| DENSO | Yes |  |
| LGE |  | Same as our answer of Q1 |
| ITRI | Yes |  |
| Samsung | Yes |  |

Proposal 2: Add the missing case of condition referring to VarConditionalReconfiguration in TS 36.331 (for SN-initiated inter-SN CPC) using R2-2210719 as baseline.

## 2.2	UE measurement requirements for conditional events in TS 36.331 (from R2-2210720)

In RAN2#119-e, the following was agreed:

[223] 6	Add similar restriction in LTE as in NR that UE ignores measId(s) that were not indicated in the triggerCondition/triggerConditionSN in the 5.5.3.1 of LTE RRC specification.

However, there was finally no addition to TS 36.331.

R2-2210720 explains that since the motivation is only for SN-initiated inter-SN CPC and the corresponding measurements are in 38.331, there is no reason to modify TS 36.331.

Question 3: Do companies agree that there is no need to modify anything for UE requirements with respect to conditional events in TS 36.331?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Huawei | Yes | Proponent |
| MediaTek | Yes |  |
| CATT | No | See comments on Q2. |
| Intel | Yes |  |
| Ericsson | Yes |  |
| NEC | Yes | Although we do not remember the background of the agreement above exactly, there seems to be no need for LTE spec. |
| Nokia | Not sure | This is probably changing the UE behavior which was defined in Rel-16. So now the UE ignores, previously it was just measuring, without checking if there was a corresponding configuration provided. Maybe no serious inter-operability issue, but it is also not a desirable approach to suddenly change the Rel-16 behavior. [Rapporteur] The proposal is "no change". How can "no change" change the UE behaviour? |
| Lenovo | Yes | We think this will not create any new issue. |
| Qualcomm |  | Similar concern as Nokia. [Rapporteur] The proposal is "no change". How can "no change" change the UE behaviour? |
| OPPO | No strong view |  |
| ZTE | Yes |  |
| vivo | Yes |  |
| DENSO | Yes |  |
| LGE |  | Same as our answer of Q1 |
| ITRI | Yes |  |
| Samsung | Yes |  |

Proposal 3: No change is needed to TS 36.331 for CPC.

## 2.3	UE measurement capability (from R2-2210718)

In Rel-16, the UE is required to perform L3 measurements following the contents of VarMeasConfig according to the following statement:

### 5.5.3 Performing measurements #### 5.5.3.1 General [...] The UE shall: [...] 1> for each measId included in the measIdList within VarMeasConfig: [...] 2> if the reportType for the associated reportConfig is periodical, eventTriggered or condTriggerConfig: 3> if a measurement gap configuration is setup, or 3> if the UE does not require measurement gaps to perform the concerned measurements: 4> if s-MeasureConfig is not configured, or 4> if s-MeasureConfig is set to ssb-RSRP and the NR SpCell RSRP based on SS/PBCH block, after layer 3 filtering, is lower than ssb-RSRP, or 4> if s-MeasureConfig is set to csi-RSRP and the NR SpCell RSRP based on CSI-RS, after layer 3 filtering, is lower than csi-RSRP: 5> if the measObject is associated to NR and the rsType is set to csi-rs: 6> if reportQuantityRS-Indexes and maxNrofRS-IndexesToReport for the associated reportConfig are configured: 7> derive layer 3 filtered beam measurements only based on CSI-RS for each measurement quantity indicated in reportQuantityRS-Indexes, as described in 5.5.3.3a; 6> derive cell measurement results based on CSI-RS for the trigger quantity and each measurement quantity indicated in reportQuantityCell using parameters from the associated measObject, as described in 5.5.3.3; 5> if the measObject is associated to NR and the rsType is set to ssb: 6> if reportQuantityRS-Indexes and maxNrofRS-IndexesToReport for the associated reportConfig are configured: 7> derive layer 3 beam measurements only based on SS/PBCH block for each measurement quantity indicated in reportQuantityRS-Indexes, as described in 5.5.3.3a; 6> derive cell measurement results based on SS/PBCH block for the trigger quantity and each measurement quantity indicated in reportQuantityCell using parameters from the associated measObject, as described in 5.5.3.3;

In Rel-17, it is:

### 5.5.3 Performing measurements #### 5.5.3.1 General [...] The UE shall: 1> for each measId included in the measIdList within VarMeasConfig: [...] 2> if the reportType for the associated reportConfig is periodical, eventTriggered; or 2> if the reportType for the associated reportConfig is condTriggerConfig and the measId is indicated in the condExecutionCond or in the condExecutionCondSCG associated to a condReconfigId in VarConditionalReconfig: 3> if a measurement gap configuration is setup, or 3> if the UE does not require measurement gaps to perform the concerned measurements: 4> if s-MeasureConfig is not configured, or 4> if s-MeasureConfig is set to ssb-RSRP and the NR SpCell RSRP based on SS/PBCH block, after layer 3 filtering, is lower than ssb-RSRP, or 4> if s-MeasureConfig is set to csi-RSRP and the NR SpCell RSRP based on CSI-RS, after layer 3 filtering, is lower than csi-RSRP: 5> if the measObject is associated to NR and the rsType is set to csi-rs: 6> if reportQuantityRS-Indexes and maxNrofRS-IndexesToReport for the associated reportConfig are configured: 7> derive layer 3 filtered beam measurements only based on CSI-RS for each measurement quantity indicated in reportQuantityRS-Indexes, as described in 5.5.3.3a; 6> derive cell measurement results based on CSI-RS for the trigger quantity and each measurement quantity indicated in reportQuantityCell using parameters from the associated measObject, as described in 5.5.3.3; 5> if the measObject is associated to NR and the rsType is set to ssb: 6> if reportQuantityRS-Indexes and maxNrofRS-IndexesToReport for the associated reportConfig are configured: 7> derive layer 3 beam measurements only based on SS/PBCH block for each measurement quantity indicated in reportQuantityRS-Indexes, as described in 5.5.3.3a; 6> derive cell measurement results based on SS/PBCH block for the trigger quantity and each measurement quantity indicated in reportQuantityCell using parameters from the associated measObject, as described in 5.5.3.3;

This change also applies for CHO and intra-SN CPC and if a network implementation wants, at some point, to configure or keep conditional measurements configured for CHO or for intra-SN CPC while there is no corresponding conditional reconfiguration, the network can configure the UE so and the Rel-17 UE is not required to perform the measurements while the Rel-16 is required to perform measurements.

However, the network is implemented according to a single version of the specification so it is not aware that the Rel-16 UE is required to perform the measurements. When considering UE measurements capabilities, as specified in TS 38.133, if the network considers that these conditional measurements are not performed, it may exceed the Rel-16 UE capability.

To avoid any issue, it is proposed that the network ensures that the total number of configured L3 measurement events and frequencies, including all configured conditional measurements, regardless whether there is an associated conditional reconfiguration or not, does not exceed the applicable UE capabilities specified in TS 38.133.

This could be captured as a note in TS 38.331:

NOTE:	The UE capabilities for measurements and event triggering specified in TS 38.133 [14] include measurements and events according to all configured meaIds regardless whether the UE is actually required to perform measurement on the associated NR measObject.

Question 4: Do companies agree that the network ensures that the total number of configured L3 measurement events and frequencies, including all configured conditional measurements, regardless whether there is an associated conditional reconfiguration or not, does not exceed the applicable UE capabilities specified in TS 38.133? Do companies agree with the proposed note?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Huawei | Yes | Proponent |
| MediaTek | Yes |  |
| CATT | See comment | In general, we agree the principle “the network ensures that the total number of configured L3 measurement events and frequencies, including all configured conditional measurements, regardless whether there is an associated conditional reconfiguration or not, does not exceed the applicable UE capabilities specified in TS 38.133”. But for the TP, I am not sure. Since it is not clear whether the TS36.331 is also affected, and that depends on how RAN2 conclude on the scenarios that the restriction of “skipped measId” applies. |
| Intel | Yes |  |
| Ericsson | Yes to first, No to second | Agree with the intention, but this can be left to network implementation, no need to capture anything in specifications. |
| NEC | Yes | Agree with Rapporteur observations. |
| Nokia | Yes to the part on NW behavior, No to the NOTE | We do not think the Note is needed. But obviously the UE capabilities related to the number of measurement events/frequencies shall not be exceeded. |
| Lenovo | Yes |  |
| Qualcomm | Same answer and comment as Ericsson |  |
| OPPO | Yes |  |
| ZTE | Yes to first, No to second | Share the same view with Ericsson. We think it should be a common understanding that the UE capabilities shall not be exceeded. No need to capture anything in the specs. |
| vivo | See comments | Agree with the observation and understanding. However, since the common understanding is that the UE capability for measurement is related to configured measurements, we think there is no need to capture anything. |
| DENSO | Yes |  |
| LGE | See comment | If our understanding is correct, we wonder if this limitation decrease the UE performance because configurable report configuration for the legacy measurement can decrease due to conditional reconfiguration. For example, the network may want to configure other reporting configuration for the same measurement object with the conditional reconfiguration to receive some of measurement reporting before performing the conditional mobility. In this case, the network could not configure some other report configuration unlike the legacy measurement situation due to this limitation. [Rapporteur] CHO, CPA and CPC configuration include reconfiguration of source configuration, so it is straightforward to add conditional measurements only when a corresponding conditional reconfiguration is provided. This does not reduce UE performance at all. If the network wants the UE to measure on an MO and configure CPC for it later, it can include the conditional measurements later, when it configures CPC. Therefore, this creates no real restriction. |
| ITRI | Yes to first, No to second | We agree the network behavior, but share the same view with Ericsson. |
| Samsung | Yes to the first, No to the second | We have the same view with proponent for the intention. However, we understand that explicit restriction proposed in the CR would consume the UE capability on the measurement unnecessarily when the measurement configured but not indicated for conditional reconfiguration, thus ignored. Network will know all the information necessary, it seems better to let the network configure based its own decision. [Rapporteur] The current text makes it possible to do that towards a Rel-16 UE supporting CHO and that UE is required to perform the measurements, regardless whether conditional measurements are configured or not. So in this case, it does consume UE capability and, from that perspective, Rel-17 specification is not compatible with Rel-16 CHO UEs. This is why some restriction is needed. However, this restriction does cause any issue: the network can simple release the useless conditional events when it wants to add additional measurements. The restriction only forces the network to "clean the configuration". |

The rapporteur is not sure whether the problem was really understood: for CHO, according to Rel-16 38.331, UEs are required to perform conditional measurements regardless whether there is an associated conditional reconfiguration but according to Rel-17 38.331, they are not.

Therefore, if there is a Rel-16 UE in a Rel-17 network, the network could configure conditional events "in advance" and keep them because it thinks the UE is not doing them, but the UE is doing them, because it is implemented according to Rel-16 specification.

If the network does not take that into account and configures a number of them "in advance", it will exceed the UE capability and the UE may simply trigger RRC re-establishment, and the network will not understand why.

An alternative to the proposal could be a note like: "Note: according to Rel-16 TS 38.331, the UE is required to perform measurements for all conditional events, regardless whether there is an associated conditional reconfiguration".

Proposal 4: Continue to discuss how to solve the situation that, unlike Rel-17 UEs, Rel-16 UEs are required to perform conditional measurements regardless whether there is an associated conditional reconfiguration, and the Rel-17 network is not aware of this.

# 3	Conclusion

Based on the input from all companies, the rapporteur makes the following proposals::

Proposal 1: Clarify the condition for the UE to perform measurements for conditional events, i.e. how to match which measId - MCG or SCG - with which field - CondExecutionCond or CondExecutionCondSCG - in entries of which VarConditionalReconfig - MCG or SCG, using R2-2210719 as baseline.

Proposal 2: Add the missing case of condition referring to VarConditionalReconfiguration in TS 36.331 (for SN-initiated inter-SN CPC) using R2-2210719 as baseline.

Proposal 3: No change is needed to TS 36.331 for CPC.

Proposal 4: Continue to discuss how to solve the situation that, unlike Rel-17 UEs, Rel-16 UEs are required to perform conditional measurements regardless whether there is an associated conditional reconfiguration, and the Rel-17 network is not aware of this.

# 4 References

- R2-2210718	UE measurement capability handling for conditional measurements without a corresponding conditional reconfiguration	Huawei, HiSilicon

- R2-2210719	UE measurement requirements for conditional events in TS 38.331	Huawei, HiSilicon

- R2-2210720	UE measurement requirements for conditional events in TS 36.331	Huawei, HiSilicon