3GPP TSG-RAN WG2 #119bis-e	R2-22xxxxx

Online, 10 – 19 October, 2022

Agenda Item:	8.9.1

Source: 	LG Electronics Inc.

Title: 	[AT119bis-e][415][Relay] LS on authorization for UE-to-UE relay (LG)

Document for:	Discussion and decision

# Introduction

This is the trigger of the following email discussion:

- [AT119bis-e][415][Relay] LS on authorization for UE-to-UE relay (LG)

Scope: Discuss the LS in R2-2209357 and attempt to converge on a reply.

Intended outcome: Approvable LS

Deadline: Friday 2022-10-14 1000 UTC

Companies are requested to provide their views on the issues and questions listed in this document.

# Discussion

### Issue 1: “5G ProSe authorised” information for UE-to-UE Relay operation

SA2 is progressing Rel-18 FS_5G_ProSe_Ph2 in TR 23.700-33.

During the study, SA2 discussed the aspect on ProSe Authorization information related to UE-to-UE Relay operation sent from AMF to NG-RAN. This aspect is related to one of objectives for UE-to-UE Relay listed in Rel-18 NR_SL_relay_enh WID, i.e. "ii. Signalling support for Relay and remote UE authorization if SA2 concludes it is needed [RAN3]".

The NR_SL_relay_enh WID reads that this aspect is up to SA2 and SA2 thinks that it is technically feasible to deliver the authorization information related to UE-to-UE Relay operation from AMF to NG-RAN as part of the NGAP message. However, SA2 has not concluded on how such information can be used for U2U Relay operation in NG-RAN.

Because how NG-RAN operation is performed to support UE-to-UE Relay operation, e.g. applying the network scheduled operation mode is within RAN2 remit and NGAP/XnAP is within RAN3 remit, SA2 believes that coordination with RAN WGs is needed to make a decision on this aspect.

SA2 question 1-1: Whether the "5G ProSe authorised" information needs to be enhanced to include the authorization information for UE-to-UE Relay operation?

| Company | Y/N | Comment |
| --- | --- | --- |
| LG Electronics | N | None |
| OPPO | See comment. | We see this is connected to the discussion on how to handle the RRC_CONNECTED UEs for U2U relay. The principle should be if there is UE-specific configuration, the authorization would be applicable. Yet we understand we need to have a view on the RRC_CONNECTED UEs handling before answering this Q. |

SA2 question 1-2: If the answer to Q1 is yes, which bullet(s) need to be included?

Regarding UE-to-UE Relay operation, it can be considered that "5G ProSe authorised" information sent by the AMF to NG-RAN may include one or more of the following:

1)	whether the UE is authorized to act as a 5G ProSe Layer-2 UE-to-UE Relay;

2)	whether the UE is authorized to act as a 5G ProSe Layer-3 UE-to-UE Relay;

3)	whether the UE is authorized to act as a 5G ProSe Layer-2 U2U UE;

4)	whether the UE is authorized to act as a 5G ProSe Layer-3 U2U UE.

| Company | Bullet number (e.g., 1/2/3/4) | Comment |
| --- | --- | --- |
| LG Electronics | None |  |
| OPPO |  | See our reply to Q1 above. |

# Conclusion and recommendation

In conclusion, Rapporteur recommends agreeing the following proposals: