3GPP TSG-RAN WG2 #119bis-e	R2-22xxxxx

Online, 10 – 19 October, 2022

Agenda Item:	8.9.1

Source: 	LG Electronics Inc.

Title: 	[AT119bis-e][415][Relay] LS on authorization for UE-to-UE relay (LG)

Document for:	Discussion and decision

# Introduction

This is the trigger of the following email discussion:

- [AT119bis-e][415][Relay] LS on authorization for UE-to-UE relay (LG)

Scope: Discuss the LS in R2-2209357 and attempt to converge on a reply.

Intended outcome: Approvable LS

Deadline: Friday 2022-10-14 1000 UTC

Companies are requested to provide their views on the issues and questions listed in this document.

# Discussion

### Issue 1: “5G ProSe authorised” information for UE-to-UE Relay operation

SA2 is progressing Rel-18 FS_5G_ProSe_Ph2 in TR 23.700-33.

During the study, SA2 discussed the aspect on ProSe Authorization information related to UE-to-UE Relay operation sent from AMF to NG-RAN. This aspect is related to one of objectives for UE-to-UE Relay listed in Rel-18 NR_SL_relay_enh WID, i.e. "ii. Signalling support for Relay and remote UE authorization if SA2 concludes it is needed [RAN3]".

The NR_SL_relay_enh WID reads that this aspect is up to SA2 and SA2 thinks that it is technically feasible to deliver the authorization information related to UE-to-UE Relay operation from AMF to NG-RAN as part of the NGAP message. However, SA2 has not concluded on how such information can be used for U2U Relay operation in NG-RAN.

Because how NG-RAN operation is performed to support UE-to-UE Relay operation, e.g. applying the network scheduled operation mode is within RAN2 remit and NGAP/XnAP is within RAN3 remit, SA2 believes that coordination with RAN WGs is needed to make a decision on this aspect.

SA2 question 1-1: Whether the "5G ProSe authorised" information needs to be enhanced to include the authorization information for UE-to-UE Relay operation?

| Company | Y/N | Comment |
| --- | --- | --- |
| LG Electronics | N | None |
| OPPO | See comment. | We see this is connected to the discussion on how to handle the RRC_CONNECTED UEs for U2U relay. The principle should be if there is UE-specific configuration, the authorization would be applicable. Yet we understand we need to have a view on the RRC_CONNECTED UEs handling before answering this Q. |
| MediaTek | Y | We see some benefits to enable the authorization information for UE-to-UE Relay operation. The UE-to-UE Relay operation should be governed by the network. In addition, from resource usage perspective, the UE-to-UE Relay operation should follow the (pre)configured resources. If the UE-to-UE Relay operation is fully UE implementation, the radio resource utilization may lead to problems within a given area. |
| Xiaomi | N | We understand legacy resource allocation mechanism can be reused in U2U relay. Therefore, gNB is not aware of the sidelink communication is destined to U2U relay or non-U2U relay. So, authorization at gNB is not needed. |
| Ericsson | N | Share the same view as xiaomi, no need to involve the gNB. Since U2U traffic doesn’t go thorough the gNB. |
| Nokia | N | We think that it is better to keep U2U relay transparent from the RAN perspective |
| Samsung | See comment | Since RAN2 have not yet discussed U2U relay operation fully we think that it is a bit early stage to conclude this issue. We share the view that this depends the handling of RRC_CONNECTE UEs. |
| vivo | Yes with comments | First, we agree with companies that it depends whether/how the network can schedule the UEs for U2U relay operation. In our understanding, L3 U2U UEs may be simply regarded as normal sidelink UEs but for L2, not sure whether there should be U2U specific handling. We slightly prefer to have the authorization information for U2U relay operation, and are also ok to postpone the decision after we finalize the U2U discussions. |
| China telecom | See comment | We think that whether authorization information is needed depends on the handling of RRC_CONNECTED UEs. In our view, we may determine this issue according to the process of RAN2’s discussion. |
| Apple | No | We think the authorization of “ProSe direct communication” for R18 UE shall automatically be considered an authorization of “UE to UE relay”. We think the gNB has no way to control the D2D communication is one-hop or two-hop. There is no need for new U2U relay authorization |
| Sharp | N | Share the same view with Xiaomi. |
| CATT | See comment | We share the same view as Samsung. |
| Lenovo | See comments | Whether gNB should be aware of the authorization depends on whether there are U2U-UE specific configuration/resource. Up to now, no U2U UE specific configuration/resource is needed. Additionally, we think that the U2U relaying is most important for Out of Coverage cases and there’s no serving gNB there to specifically check for U2U Relay/ Remote UE’s authorization. So, we believe that SA3 anyway needs to ensure that no security holes exist while using U2U relaying. The same technique might be applicable in-coverage (including for RRC Connected UEs). Currently, we don’t see the need of enhancement from RAN2 perspective. |
| ZTE | Yes | For L2 U2U relay/remote UE in RRC_CONNECTED state, gNB needs to know and verify it is a relay/remote UE. Then gNB may allocate SL resources and provide U2U relay related SLRB/PC5 RLC channel configuration to them. Therefore, authorization information for L2 U2U relay UE and L2 U2U remote UE (source UE or destination UE) are needed. However, for L3 U2U relay communication, it is almost equivalent to normal SL direct communication hop by hop and the procedures are the same as ProSe direct communication. Therefore, for L3 U2U relay/remote UE to perform relay communication, they only need to be subject to the authorization of ProSe direct communication. The authorization information for L3 U2U relay/remote UE is not needed. |
| Qualcomm | See comments | Share view with Samsung, we can postpone this issue but can reply current RAN2 situation to SA2 this meeting and inform SA2 that RAN2 will notify the progress if any. We believe this is not urgent issue for SA2. |

SA2 question 1-2: If the answer to Q1 is yes, which bullet(s) need to be included?

Regarding UE-to-UE Relay operation, it can be considered that "5G ProSe authorised" information sent by the AMF to NG-RAN may include one or more of the following:

1)	whether the UE is authorized to act as a 5G ProSe Layer-2 UE-to-UE Relay;

2)	whether the UE is authorized to act as a 5G ProSe Layer-3 UE-to-UE Relay;

3)	whether the UE is authorized to act as a 5G ProSe Layer-2 U2U UE;

4)	whether the UE is authorized to act as a 5G ProSe Layer-3 U2U UE.

| Company | Bullet number (e.g., 1/2/3/4) | Comment |
| --- | --- | --- |
| LG Electronics | None |  |
| OPPO |  | See our reply to Q1 above. |
| MediaTek | See comments | We see the need to enable UE authorization. But we do not think there is a need to distinguish L2 and L3 operation for U2U. The UE authorization can be common to both L2 and L3. There may be a need to authorize U2U Relay UE and U2U Remote UE separately. |
| Xiaomi | None |  |
| Ericsson | none |  |
| Nokia | None |  |
| Samsung |  | See our comment to Q1 as above |
| vivo | See comments | If we would like to make a progress then we can exclude L3 if that is acceptable to companies, as we explained in Q1. Or it is also ok to postpone the decision. |
| China telecom |  | See our comment to Q1 above |
| Sharp | None |  |
| Lenovo |  | See comments for Q1. |
| ZTE | 1/3 | See comment in question 1-1. |
| Qualcomm |  | See our comment to Q1 as above |

** Summary

7 of 15 companies answered that authorized information for UE-to-UE relay is not necessary because gNB will not aware of UE-to-UE relay operation.

6 of 15 companies have concerns that it depends on whether gNB will do any special control for RRC_CONNECTED remote or relay UE in UE-to-UE relay. They think it is related to whether/how gNB schedule resource allocation for UE-to-UE remote or relay UE. So, they want to be postponed the decision until RAN2 makes a decision about whether gNB will control UE-to-UE relay operation.

However, 2 of 15 companies answered that authorized information for UE-to-UE relay is necessary. They think UE-to-UE relay should be governed by the connected gNB.

The companies (7/15) answered that authorized information enhancement for UE-to-UE relay is not necessary is a little slightly more than the other companies (6/15) answered having concerns. But it looks like quite a lot of companies still want to postpone making decisions until it is enough discussed in RAN2. So, we can make the following proposals.

Proposal 1: RAN2 will postpone the discussion on whether authorized information needs to be enhanced for U2U relay until RAN2 has more progress on how to control the RRC_CONNECTED UEs for U2U relay.

Proposal 2: RAN2 is requested to further discuss whether to prioritize RAN2 discussion on how to control the RRC_CONNRECTED UEs for U2U relay for reply LS in RAN2#120. Or RAN2 could further discuss whether to inform SA2 that RAN2 will respond to the SA2 questions after more progress.

# Conclusion and recommendation

In conclusion, Rapporteur recommends agreeing the following proposals:

Proposal 1: RAN2 will postpone the discussion on whether authorized information needs to be enhanced for U2U relay until RAN2 has more progress on how to control the RRC_CONNECTED UEs for U2U relay.

Proposal 2: RAN2 is requested to further discuss whether to prioritize RAN2 discussion on how to control the RRC_CONNRECTED UEs for U2U relay for reply LS in RAN2#120. Or RAN2 could further discuss whether to inform SA2 that RAN2 will respond to the SA2 questions after more progress.