3GPP TSG-RAN2 #119bis	R2-22xxxxxx

Online, 10th– 19th Oct, 2022

Agenda Item:	8.2.1

Source: 	Xiaomi

Title:  	[Draft] Report of [AT119bis-e][423][POS] LS to SA2 on SL positioning terminology (Xiaomi)

Document for:	Discussion and Decision

# Introduction

This document is intended to collect views on the incoming LS from SA2 [1] on terminology alignment and provide reply LS to SA2 as per the following email discussion guidelines:

- [AT119bis-e][423][POS] LS to SA2 on SL positioning terminology (Xiaomi)

Scope: Discuss the LS in document R2-2209351 and develop a response.

Intended outcome: Report and agreeable LS

Deadline: Friday 2022-10-14 1000 UTC (for comments)

Companies are invited to provide views on the issues listed in this document.

# Contact information

| Company | Name | Email Address |
| --- | --- | --- |
| Xiaomi | xiaowei jiang | jiangxiaowei@xiaomi.com |
| Ericsson | Ritesh Shreevastav | Ritesh.shreevastav@ericsson.com |
| MediaTek | Nathan Tenny | nathan.tenny@mediatek.com |
| ZTE | Yu Pan | pan.yu24@zte.com.cn |
| CATT | Jianxiang Li | lijianxiang@catt.cn |

# Discussion

SA2 has defined the following terminologies on ranging/SL positioning in S2-2207129/R2-2209188 [1]:

Ranging: refers to the determination of the distance between two UEs or more UEs and/or the direction of one UE (i.e. Target UE) from another UE (i.e. Reference UE) via PC5 interface. SL Reference UE: A UE, supporting positioning of target UE, e.g., by transmitting and/or receiving reference signals for positioning, providing positioning-related information, etc., using sidelink. NOTE 1: SL Reference UE is understood as "Anchor UE" in RAN1 TR 38.859 [21]. NOTE 2: “Reference UE” mentioned in KIs and Solutions of this TR refers to “SL Reference UE”. Target UE: A UE whose distance, direction and/or position is measured with the support from one or multiple SL Reference UEs using Sidelink in the Ranging based service and Sidelink positioning. Assistant UE: A UE supporting Ranging/Sidelink Positioning between a SL Reference UE and a Target UE over PC5, when the direct Ranging/Sidelink positioning between the SL Reference UE and Target UE cannot be supported. The measurement/result of Ranging/Sidelink Positioning between the Assistant UE and the SL Reference UE and that between the Assistant UE and the Target UE are determined and used to derive the Ranging/Sidelink Positioning result between Target UE and SL Reference UE. Located UE: A SL Reference UE of which the location is known or is able to be known using Uu based positioning. A Located UE can be used to determine the location of a Target UE using Sidelink Positioning. SL Positioning Server UE: A UE offering location calculation, for Sidelink Positioning and Ranging based service. It interacts with other UEsover PC5 as necessary in order to calculate the location of the Target UE. Target UE or SL Reference UE can act as SL Positioning Server UE if location calculation is supported. SL Positioning Client UE: A third-party UE, other than SL Reference UE and Target UE, which initiates Ranging/Sidelink positioning service request on behalf of the application residing on it. NOTE 3: The SL Positioning Client UE does not have to support Ranging/Sidelink positioning capability, but a communication between the SL Positioning Client UE and SL Reference UE/Target UE has to be established, either via PC5 or via 5GC, for the transmission of the service request and the result. Sidelink Positioning: Positioning UE using PC5 to obtain absolute position, relative position, or ranging information. Positioning: A functionality, which detects a geographical location and optionally, velocity (of e.g. a mobile terminal). Networkassisted Operation: Operation of Ranging/Sidelink Positioning with the involvement of 5GC NFs for the service request handling and result calculation. UE-only Operation: Operation of Ranging/Sidelink Positioning in which the service request handling and result calculation are performed by UE. NOTE 4: For UE-only Operation, the communication among UEs are over PC5. Relative position: An estimate of the UE position relative to other network elements or relative to other UEs.

In the following sections, rapporteur suggest to discuss them one by one.

## Ranging

SA2 definition:

Ranging: refers to the determination of the distance between two UEs or more UEs and/or the direction of one UE (i.e. Target UE) from another UE (i.e. Reference UE) via PC5 interface.

RAN1 definition:

Ranging: determination of the distance and/or the direction between a UE and another entity, e.g., anchor UE.

The only difference between RAN1 and SA2 is whether distance is between two UEs or two or more UEs. In rapporteur perspective, RAN1’s definition doesn’t preclude that ranging cannot be performed in a 1-to-M manner. It is more from the basic measurement required point of view, and is solution agnostic. Besides, RAN1 definition is aligned with the ranging definition in TS22.261:

Ranging: refers to the determination of the distance between two UEs and/or the direction of one UE from the other one via direct device connection.

Question 1)	What is your preferred option regarding the definition of ranging?

- Option 1: RAN1 definition on Ranging

- Option 2: SA2 definition on Ranging

| Company | Preferred Option(s) | Additional comments |
| --- | --- | --- |
| Ericsson | Option 1 | SA2 definition is a bit over-whelming with too many terms and too many UEs. RAN1 is simple and easy to understand. |
| MediaTek | Option 1 | From AS perspective, we understand that ranging among multiple UEs just represents several different operations of ranging between two UEs, so we don’t see a functional difference between the RAN1 and SA2 definitions. |
| ZTE | Option 1 | There is no functional difference. From RAN2 perspective option 1 is more simple. |
| Xiaomi | Option 1 |  |
| CATT | Option 1 | From RAN’s perspective, option 1 is clear. |

## SL Reference UE & Located UE

SA2 definition:

SL Reference UE: A UE, supporting positioning of target UE, e.g., by transmitting and/or receiving reference signals for positioning, providing positioning-related information, etc., using sidelink. NOTE 1: SL Reference UE is understood as "Anchor UE" in RAN1 TR 38.859 [21]. NOTE 2: “Reference UE” mentioned in KIs and Solutions of this TR refers to “SL Reference UE”.

RAN2/RAN1 definition:

Anchor UE: UE supporting positioning of target UE, e.g., by transmitting and/or receiving reference signals for positioning, providing positioning-related information, etc., over the SL interface. FFS: clarification of the knowledge of the anchor UE.

SA2 has aligned its “SL reference UE” definition with RAN1/RAN2 but removed the FFS part. Instead, SA2 defined another kind of SL reference UE with known location, i.e. located UE:

Located UE: A SL Reference UE of which the location is known or is able to be known using Uu based positioning. A Located UE can be used to determine the location of a Target UE using Sidelink Positioning.

Clearly, in SA2’s definition, SL reference UE can either has the knowledge of its location or not. If it has  its location, it is considered as located UE.

To align with SA2’s definition, there are following options:

-	Option 1: remove the FFS part.

-	Option 2: replace the FFS part with the following[4]:

- The location of the anchor UE is known or is able to be known using Uu based positioning. One or more Anchor UEs may be used to determine the location of a Target UE using Sidelink Positioning.

- Anchor UE may assume an Assistant UE role as per the SA2 definition.”

- Anchor UE may assume a SL Positioning Server UE role as per the SA2 definition.

-	Option 3: Ask SA2 to use the term of “anchor UE” instead of “SL reference UE” amd adopt the definition exactly adopted by RAN1/RAN2

-	Option 4: Wait for RAN1 conclusion on the FFS part and adopt whatever agreed by RAN1.

For option 2, [4] clarifies that an anchor UE has to have a known location by nature of the definition, as a non-anchor UE may be converted to an anchor-UE via Uu or other location determination procedures. For option 3, rapporteur’s understanding is that it is SA2’s freedom to use any term as long as the definition is aligned with RAN.

Question 2.1)	What is your preferred option regarding the definition of anchor UE?

-	Option 1: remove the FFS part.

-	Option 2: replace the FFS part with the following[4]:

- The location of the anchor UE is known or is able to be known using Uu based positioning. One or more Anchor UEs may be used to determine the location of a Target UE using Sidelink Positioning.

- Anchor UE may assume an Assistant UE role as per the SA2 definition.”

- Anchor UE may assume a SL Positioning Server UE role as per the SA2 definition.

-	Option 3: Ask SA2 to use the term of “anchor UE” instead of “SL reference UE” and use the definition of anchor UE exactly adopted by RAN1/RAN2.

-	Option 4: Wait for RAN1 conclusion on the FFS part and adopt whatever agreed by RAN1.

| Company | Preferred Option(s) | Additional comments |
| --- | --- | --- |
| Ericsson | Option 4 |  |
| MediaTek | Options 1/4 (see comment) | Option 1 would be acceptable; we understand that the FFS part is not part of the actual definition (it also contains a confusing typo, because the phrase “the location of” was omitted in the original minutes). We understand this can be combined with option 4: We drop the FFS part from our definition, with the understanding that we will ultimately align with RAN1 terminology for the TR. We are not sure about the argument in [4] for knowledge of the anchor UE’s location. It seems that some operations, e.g., ranging, could be performed on a relative basis only, without any entity needing to know the anchor UE’s location, and it seems certainly not true that Uu positioning can always be used to find an anchor UE’s location (what if it is OOC?). So we think option 2 is not quite right as stated. We don’t have a strong preference between the terms “reference” and “anchor”, although we agree it would be good for all groups to align by the time we start normative work. SA2 are evidently already aware of the difference and can decide if they want to align terms, so we don’t think option 3 is necessary. |
| ZTE | Option 1/4 | For option 2, based on different capability and scenarios/use cases, we think anchor UE can include SL positioning server UE, assistance UE, reference UE or located UE. We do not think the terms between RAN and SA2 have to be completely the same as long as the two WGs have clear definition on the function of the terms(basic function should be aligned). SA2 introduce terms useful for their further discussion, and RAN introduce ours. However we suggest RAN group to keep align, i.e., RAN1&2&3&4 to have unified terms for further discussion. |
| Xiaomi | Option 1/4 |  |
| CATT | Option 3/4 | SA2 is asking the feedback from RAN, so RAN2s can try to align the terms with SA2 in our reply LS. So option 3 makes sense. ACTION: RAN WGs evaluates if the terminologies defined by SA2 are aligned and if any further updates are needed, and gives a feedback to SA2. |

Regarding SA2’s definition on located UE, [5] suggests to adopt the definition of located UE in RAN2, while [2] suggests to ask SA2 to not use the term of “located UE” but integrate it into SL reference UE.

Question 2.2)	Under the assumption that anchor UE can be either with or without the knowledge of its location, what is your preferred option regarding the definition of located UE?

- Option 1: RAN2 to adopt the definition of located UE

- Option 2: Ask SA2 to integrate located UE into SL reference UE

- Option 3: RAN2 to not adopt the definition of located UE

- Option 4: Others

| Company | Preferred Option(s) | Additional comments |
| --- | --- | --- |
| Ericsson | Option 4, Option 3 | TS 38.305 already has definition of PRU. Isn’t PRU same as located UE? SA2 seems to be overall defining too many new term and complicating the architecture. |
| MediaTek | Option 3 | We don’t really see a need for the “located UE” term in RAN2. If we find a need for it in drafting specifications in normative work, we can introduce it then. |
| ZTE | Option 3 | Anchor UE can include located UE when the location of itself is known |
| Xiaomi | Option 3 |  |
| CATT | Option 3 | Located UE is an anchor UE with knowledge of location. No need to introduce the term of located UE in RAN2. |

Besides, in [3], it explains that, according to SA2 definition, it seems that SL reference UE corresponds to the anchor UE in relative positioning; while located UE corresponds to the anchor UE in absolute positioning. Thus, it suggests to send a LS to SA2 to confirm that:

- SL Reference UE corresponds to the role of Anchor UE in relative positioning that the location of the anchor UE can be unknown.

- Located UE corresponds to the role of Anchor UE in absolute positioning that the location of the anchor UE should be known.

However, rapporteur thinks that, from the definition of SL reference UE and located UE, SL reference UE includes located UE and UE without location known, it can have a role in both relative positioning and absolute positioning. For located UE, it is quite clear that the location of the anchor UE is used for absolute positioning. There is no ambiguity on this two definition.

Question 2.3)	Do you agree that there is no need to send a LS to SA2 to confirm the following:

- SL Reference UE corresponds to the role of Anchor UE in relative positioning that the location of the anchor UE can be unknown.

- Located UE corresponds to the role of Anchor UE in absolute positioning that the location of the anchor UE should be known.

| Company | Agree/Disagree | Additional comments |
| --- | --- | --- |
| Ericsson | Too early to send LS | We should 1st see if Located UE is needed or not and also if we will adopt SA2 SL Reference UE or simply RAN1 Anchor UE. |
| MediaTek | No LS needed | We agree with the rapporteur’s understanding and we don’t see a need for an LS. |
| ZTE | Yes(no need) |  |
| Xiaomi | Yes | No need to send the LS on this |
| CATT | No need |  |

## Target UE

SA2 definition:

Target UE: A UE whose distance, direction and/or position is measured with the support from one or multiple SL Reference UEs using Sidelink in the Ranging based service and Sidelink positioning.

RAN2 definition:

Target UE: UE to be positioned.

SA2’s definition mentioned with the support of SL reference UE and using SL. Although RAN2’s definition doesn't have it, rapporteur thinks that SL positioning always involves anchor UE and SL. Thus, the definition of target UE is actually aligned with RAN2.

Question 3)	Do you agree that the definition of target UE in SA2 and RAN2 is aligned?

| Company | Agree/Disagree | Additional comments |
| --- | --- | --- |
| Ericsson | Disagree, SA2 definition is confusing | We should simply adopt RAN2 definition which is simple and easy to follow. |
| MediaTek | Agree | The SA2 definition does seem wordy from the RAN2 perspective, but we assume SA2 are using the verbiage they need. However, we don’t see a need to change the RAN2 definition. |
| ZTE | Yes |  |
| Xiaomi | Agree | The two term are generally aligned. There is no need to ask SA2 to strictly follow RAN2 definition. |
| CATT | Disagree | RAN1’s definition is brief and clear. |

## Assistant UE

SA2 definition:

Assistant UE: A UE supporting Ranging/Sidelink Positioning between a SL Reference UE and a Target UE over PC5, when the direct Ranging/Sidelink positioning between the SL Reference UE and Target UE cannot be supported. The measurement/result of Ranging/Sidelink Positioning between the Assistant UE and the SL Reference UE and that between the Assistant UE and the Target UE are determined and used to derive the Ranging/Sidelink Positioning result between Target UE and SL Reference UE.

There is no corresponding definition of assistant UE in RAN1/RAN2. In rapporteur’s understanding, assistant UE relates to the scenario that there is no direct link between target UE and anchor UE, so a third UE (i.e. assistant UE) will assist the SL positioning between target UE and anchor UE. Only one hop is considered in SA2, and the assistant UE does not serve as a repeater to forward the ranging/SL positioning signals from/to anchor UE/Target UE. RAN2 has not discussed this scenario. In rapporteur’s view, RAN2 need first discuss whether RAN2 will consider this scenario.

Question 4.1)	Whether the role of assistant UE is supported in RAN2 if SA2 concludes to support it?

| Company | Yes/No | Additional comments |
| --- | --- | --- |
| Ericsson | No | We need to see why UWB is successful. It is simple ranging between 2 UEs. We need to strive to provide a simple solution; not make it complicated or paper feature with complicated scenarios/multiple considerations. |
| MediaTek | See comment | If SA2 conclude to support an “assistant UE” concept with AS implications, we should of course support it in RAN2. However, so far we don’t see a need to capture this in RAN2, and we think the scenario still needs some clarification in SA2. [Xiaomi] Not sure if SA2 is able to tell us that there is RAN impact. Perhaps RAN2 can also evaluate this to see if there is RAN impact. |
| ZTE | Not for now | We agree with Ericsson, and think that at this stage the basic function of SL positioning has not been set up completely. Suggest to deprioritize this scenario for now |
| Xiaomi | Yes | If SA2 concludes to support assistant UE, RAN2 should support it. RAN2 can further evaluate if there is RAN2 impact or not. The priority can decided later. It is too early to prioritize one over the other, similar to the discussion about partial coverage. |
| CATT | No | The scenario of relative positioning between two UEs without direct SL connection is out of scope of RAN WID. The operation of assistant UE and reference/target UE can be considered as combination of two separate SL positioning operations. How to use multiple the SL positioning results to acquire positioning between UEs without direct SL connection is up to SA2. From RAN’s perspective, assistant UE is target UE or anchor UE. |

Question 4.2)	If the answer to Q4.1 is yes, whether RAN2 adopts the definition of assistant UE in SA2?

| Company | Yes/No | Additional comments |
| --- | --- | --- |
| MediaTek | Not now | We should do this if SA2 take decisions that require the assistant UE to be visible in RAN2. |
| Xiaomi | No | Too early to define the terminology, RAN2 should first focus on the scenarios and RAN2 impacts. We are not sure SA2 can make decision on whether there is RAN impact or not. Suggest RAN2 to evaluate on this. |

## Sidelink Positioning

SA2 definition:

Sidelink Positioning: Positioning UE using PC5 to obtain absolute position, relative position, or ranging information.

RAN1 definition:

Sidelink positioning: Positioning UE using reference signals transmitted over SL, i.e., PC5 interface, to obtain absolute position, relative position, or ranging information.

For sidelink positioning definition, the only difference between RAN1 and SA2 definition is whether using reference signals transmitted over SL. From RAN2 point of view, the key issue is whether SL positioning always involves reference signals transmission. To be more specific, [7] suggests to support two categories of SL positioning:

Cat 1: RAT-independent SL positioning method, e.g. SL A-GNSS, SL WLAN, SL bluetooth, SL sensor;

Cat 2: RAT-dependent SL positioning method, e.g. SL RTT, SL TDOA, SL AOA/AOD, SL carrier phase.

For cat 1, [1] further explains that it may be applicable for the case of out of coverage scenario, where assistant information relating to GNSS/WLAN/Sensor/TBS can be transfered from the assistant UE to the target UE. For example, if assistant UE is out of coverage, but it can acquire GNSS information (e.g. Ephemeris, reference time), it can provide these assistant GNSS information to the target UE to accelerate the GNSS positioning of target UE.

If RAT-independent SL positioning method is to be agreed by RAN2, it doesn’t involves reference signals transmission. Then there would be no need to have the the limitation of “using reference signals transmitted over SL” in SL positioning definition. As RAT-independent positioning is invisible to RAN1, RAN1 doesn’t need to consider it.

Question 5.1)	Whether RAT-independent SL positioning method, e.g. SL A-GNSS is supported for SL positioning?

| Company | Yes/No | Additional comments |
| --- | --- | --- |
| Ericsson | No | We should not complicate. We should only consider SL Reference signal-based positioning at least for Rel-18. |
| MediaTek | No | Agree with Ericsson. We understand that “sidelink positioning” was originally intended to mean “positioning with measurements from the PC5 interface”. We can see where there could be use cases for providing assistance data from UE to UE over PC5 for RAT-independent positioning, but it’s a different problem with a different solution and we shouldn’t try to solve it now. |
| ZTE | No | Agree RAN2 to keep align with RAN1 and to study the SL positioning with reference signal transferred via PC5 |
| Xiaomi | Yes | We don’t see much standard effort to support RAT independent positioning, Uu RAT independent positioning assistant information can be reused as much as possible. And It is too early to exclude this in the study phase. |
| CATT | No | We stick to the SID as below. The SI scope covers combination of SL positioning measurements with other RAT dependent positioning measurements (e.g. Uu based measurements), but there is no RAT-independent SL positioning method mentioned. ### 4.1 Objective of SI or Core part WI or Testing part WI - Study solutions for sidelink positioning considering the following: [RAN1, RAN2] - Scenario/requirements - Coverage scenarios to cover: in-coverage, partial-coverage and out-of-coverage - Requirements: Based on requirements identified in TR38.845 and TS22.261 and TS22.104 - Use cases: V2X (TR38.845), public safety (TR38.845), commercial (TS22.261), IIOT (TS22.104) - Spectrum: ITS, licensed - Identify specific target performance requirements to be considered for the evaluation based on existing 3GPP work and inputs from industry forums [RAN1] - Define evaluation methodology with which to evaluate SL positioning for the uses cases and coverage scenarios, reusing existing methodologies from sidelink communication and from positioning as much as possible [RAN1]. - Study and evaluate performance and feasibility of potential solutions for SL positioning, considering relative positioning, ranging and absolute positioning: [RAN1, RAN2] - Evaluate bandwidth requirement needed to meet the identified accuracy requirements [RAN1] - Study of positioning methods (e.g. TDOA, RTT, AOA/D, etc) including combination of SL positioning measurements with other RAT dependent positioning measurements (e.g. Uu based measurements) [RAN1] - Study of sidelink reference signals for positioning purposes from physical layer perspective, including signal design, resource allocation, measurements, associated procedures, etc, reusing existing reference signals, procedures, etc from sidelink communication and from positioning as much as possible [RAN1] - Study of positioning architecture and signalling procedures (e.g. configuration, measurement reporting, etc) to enable sidelink positioning covering both UE based and network based positioning [RAN2, including coordination and alignment with RAN3 and SA2 as required] Note: When the bandwidth requirements have been determined and the study of sidelink communication in unlicensed spectrum has progressed, it can be reviewed whether unlicensed spectrum can be considered in further work. Checkpoint at RAN#97 to see if sufficient information is available for this review. |

Question 5.2)	What is your preferred option regarding the definition of SL positioning in RAN2?

- Option 1: RAN1 definition on SL positioning

- Option 2: SA2 definition on SL positioning

| Company | Preferred Option(s) | Additional comments |
| --- | --- | --- |
| Ericsson | Option 1 |  |
| MediaTek | Option 1 | It’s probably good for the definition to be specific. |
| ZTE | Option 1 |  |
| Xiaomi | Option 2 |  |
| CATT | Option 1 | The Study on expanded and improved NR positioning is led by RAN1 so we RAN2 will follow the RAN1 definition on SL positioning. |

## SL Positioning Server UE

SA2 definition:

SL Positioning Server UE: A UE offering location calculation, for Sidelink Positioning and Ranging based service. It interacts with other UEsover PC5 as necessary in order to calculate the location of the Target UE. Target UE or SL Reference UE can act as SL Positioning Server UE if location calculation is supported.

Regarding definition of SL Positioning Server UE, RAN2 agreed that:

=> RAN2 follow SA2 on the architecture, including the possibility of a UE as a location server. FFS from RAN2 perspective if there are cases without a UE in the location server role.

One company suggest in [6] to ask SA2 to remove SL Positioning Server UE from terminologies and consider that Target UE or SL Reference UE have the capability of location calculation on behalf of UE’s which are not capable of computing the location themselves. However, from rapporteur point of view, RAN2 has already decided to follow SA2 decision on the introduction of SL positioning server UE. There is no need to reopen the discussion in RAN2.

According to the definition of SL positioning server UE, only location calculation functionality is mentioned. From rapporteur poinit of view, SL positioning server UE is more like LMF. It should be able to perform more functionalities of LMF. According to TS 23.273, LMF possesses the following functionalities:

-	Manages the overall co-ordination and scheduling of resources required for the location of a UE that is 	registered with or accessing 5GCN.

-	Calculates or verifies a final location and any velocity estimate and may estimate the achieved accuracy.

-	Support a request for a single location received from a serving AMF for a target UE.

-	Support a request for periodic or triggered location received from a serving AMF for a target UE.

-	Determine type and number of position methods and procedures based on UE and PLMN capabilities, 	QoS, UE connectivity state per access type, LCS Client type, co-ordinate type and optionally service 	type.

-	Report UE location estimates directly to a GMLC for periodic or triggered location of a target UE.

-	Support cancelation of periodic or triggered location for a target UE.

-	Support the provision of broadcast assistance data to UEs via NG-RAN in ciphered or unciphered form 	and forward any ciphering keys to subscribed UEs via the AMF.

-	Support change of a serving LMF for periodic or triggered location reporting for a target UE.

-	Support of receiving stored UE Positioning Capability from AMF and support of providing updated UE 	Positioning Capability to AMF.

-	Map the UE location to a country or an international area based on the request from AMF.

-	Support determination of a UE location at a scheduled location time.

It is rapporteur’s understanding that at least the functionality of managing the overall co-ordination and scheduling of resources, determining type and number of position methods and procedures are essential for out of coverage scenario. Besides, [8] also suggests SL positioning server UE to determine how many and which UEs act as anchor UEs. Rapporteur thinks it is also a valid suggestion for positioning methods e.g. SL TDOA, SL-AOA.

Question 6.1)	From RAN2 perspective, which of the following functionalities aside from location calculation should also be considered as a functionality of SL positioning server UE?

- a): managing the overall co-ordination and scheduling of resources

- b): determining type and number of position methods

- c): determine how many and which UEs act as anchor UEs

- d): Other

| Company | Preferred Option(s) | Additional comments |
| --- | --- | --- |
| Ericsson | D | SA2 has not fully concluded on SL Positioning Server. Even if SA2 considers, we need to wait for SA3. Pls note that the reason to not allow gNB to host the location server (local-LMF; as discussed in Rel-16 Positioning Study) was that gNB was considered vulnerable in terms of security as compared to Core Network. So, it sounds a bit alarming that UE can host the location server. Pls note that for resource coordination we will use Mode1, mode2 based resource allocation and Inter-UE coordination to remove collision etc. based upon SL communication framework and we do not need to introduce yet another a new mechanism based upon server. Ranging should be simple between two UEs where both take part, grab the resources, exchange their capabilities, decide upon method, and calculate the ranging. This can be also extended to a group without as such requiring a server. |
| MediaTek | At least C, but see comment | RAN2 agreed to follow SA2 on this, and we think we should stick to that agreement. So our basic answer is “follow SA2”. However, we understand that SA2’s current definition of the server UE is out of sync with the SA2 solutions that motivated it, and we understand that this issue is already being discussed in SA2. In our understanding, the solutions require additional functionality from the server UE, especially for OOC cases. For positioning with a group of anchor UEs, how does the target UE determine which of the (possibly very many) nearby UEs are suitable anchor UEs whose locations can be known to the position calculation function (note that they may not be known to the target UE itself)? Note that the anchor UEs will not be the same for all scenarios (consider V2X use cases: Ranging must be possible between any two vehicular UEs for collision avoidance, but absolute positioning of the target UE depends on UEs with stable known locations, most likely RSUs), so it is not as simple as looking for UEs that advertise “I can be an anchor UE”. We think it is obviously necessary to have some entity that can coordinate the UEs for positioning operations—this doesn’t mean it has to have all the functions of an LMF, but more than just the position calculation function is needed. From RAN2 perspective, we see a need for the server (UE or LMF) to identify anchor UEs, provide corresponding assistance data (which may or may not include the anchor UE locations—cf. the DL-TDOA assistance data on Uu), receive location information (measurements or location estimate) from the anchor/target UE(s), and in some cases compute the location estimate from the measurements. These operations require the server to know the capabilities/configurations of the target and anchor UEs. In other words, we see that the server needs to support at least the functionalities corresponding to the LPP procedures: - Configuration of the target and/or anchor UEs to perform a positioning operation - Provision of assistance data (implying selection of the anchor UEs—equivalent to the LMF selecting which TRPs to involve) - Collection of location information - Computation of location estimate There may be additional upper-layer requirements such as support of LCS procedures, but those are not visible to RAN2. |
| ZTE |  | Suggest RAN2 to not make decisions on which specific functions that have to be supported at this stage; suggest to wait for SA2’s study on SL positioning server UE |
| Xiaomi | A, B ,C | For A, in multi-anchor and out of coverage scenario, e.g. SL-TDOA, anchors UE need to coordinate with each other to avoid resource conflict. For B, there should be only one entity to decide which method is used for the positioning. The one deciding this should be the server UE, target UE can request for a specific method or multiple methods. For C, we agree with mediatek that anchor UEs in this case are infrastructures e.g. RSU which are managed by operators, they should have means to control and coordinate which anchor UEs participates a specific positioning session to efficiently utilizing anchor UE resources. |
| CATT | a,b,c | If server UE is taken as LMF role, maybe a, b and c are supposed as well. We may further discuss it after serving UE is clear in SA2. |

Question 6.2)	Do you agree to send LS to SA2 to ask them to take the addition functionalities of SL positioning server UE identified in Q6.2 into account, if any?

| Company | Yes/No | Additional comments |
| --- | --- | --- |
| Ericsson |  | We can inform SA2 that SL UE location server is not needed. |
| MediaTek | Yes | Since RAN2 are expected to design the protocol between the server and target, we can indicate to SA2 what functionalities we need from the location server. SA2 need to resolve this issue and would probably benefit from the RAN2 perspective. |
| ZTE | Yes |  |
| Xiaomi | Yes |  |
| CATT | Yes |  |

## SL Positioning Client UE

SA2 definition:

SL Positioning Client UE: A third-party UE, other than SL Reference UE and Target UE, which initiates Ranging/Sidelink positioning service request on behalf of the application residing on it. NOTE 3: The SL Positioning Client UE does not have to support Ranging/Sidelink positioning capability, but a communication between the SL Positioning Client UE and SL Reference UE/Target UE has to be established, either via PC5 or via 5GC, for the transmission of the service request and the result.

Rapporteur think that SL Positioning Client UE is invisible to RAN2, there is no need for RAN2 to define it.

Question 7)	Do you agree that there is no need for RAN2 to define SL Positioning Client UE?

| Company | Yes/No | Additional comments |
| --- | --- | --- |
| Ericsson | Yes | In OOC environment when we don’t have network node, we don’t need to replicate all NW element from UEs. We need to keep it simple else it will become only paper work. |
| MediaTek | Yes | In the end we may need to mention the client UE in stage 2, as we mention the LCS client in a few places today, but we don’t foresee any substantial impact of this concept on RAN2 and we don’t think it needs to be involved in the SI phase. |
| ZTE | Yes |  |
| Xiaomi | Yes |  |
| CATT | Yes | SL Positioning Client UE is not a SL positioning participant. It is invisible to RAN. |

## Relative Positioning

SA2 definition:

Relative position: An estimate of the UE position relative to other network elements or relative to other UEs.

Regarding the term relative positioning, both RAN1 and RAN2 have not defined it. From RAN2 point of view, rapporteur thinks we can adopt SA2 definition, which actually refers to the relative positioning definition in TS22.261.

Question 8)	Do you agree that RAN2 adopt SA2’s definition on relative positioning?

| Company | Yes/No | Additional comments |
| --- | --- | --- |
| Ericsson | No | Let RAN1 decide it. |
| MediaTek | If needed | So far RAN2 have not encountered a need to capture this term, but if we find we need it, the SA2 definition seems adequate. |
| ZTE | Not for now | Either wait for RAN1’s decision or RAN2 introduces the term when needed |
| Xiaomi | Yes | Ranging, relative positioning and abosolute positioning are three main cases RAN will handle, and are used by a lot of contributions, it would be good to define it for people to have the same understanding. |
| CATT | Not for now | There is no such requirement for RAN2 to adopt the definition so far. We will wait for RAN1 agreement if there is. |

## Network assisted Operation

SA2 definition:

Networkassisted Operation: Operation of Ranging/Sidelink Positioning with the involvement of 5GC NFs for the service request handling and result calculation.

Regarding the term Network assisted Operation, it is not aligned with RAN2 using of ‘assisted’. According to TS38.305, suffix "-assisted" refers to node that provides measurements rather than making positioning calculation. However, SA2 Network assisted Operation involves network node calculating results.

TS38.305: As used in this document, the suffixes "-based" and "-assisted" refer respectively to the node that is responsible for making the positioning calculation (and which may also provide measurements) and a node that provides measurements (but which does not make the positioning calculation). Thus, an operation in which measurements are provided by the UE to the LMF to be used in the computation of a position estimate is described as "UE-assisted" (and could also be called "LMF-based"), while one in which the UE computes its own position is described as "UE-based".

Question 9)	Do you agree to ask SA2 to align the use of the suffixes "-based" and "-assisted" with TS38.305?

| Company | Yes/No | Additional comments |
| --- | --- | --- |
| Ericsson | No | There is no UE-Based or UE-Assisted in SL. Else we will confuse the current terminology as well. These are defined for in coverage scenario or partial coverage. But when NW node is not involved, we do not need to associate UE-Based or UE-Assisted |
| MediaTek | Yes; see comment | We don’t understand Ericsson’s comment; the point of the “network-assisted” case in SA2 seems to be that network functions are involved, so we can sensibly talk about the NFs offering “assistance” to the UE. So this is not about pure UE-to-UE operation (which has its own confusion with the “-based” and “-assisted” terms, but that is a separate issue). We agree that the term “network-assisted” is not ideal; we should keep the “-assisted” suffix with its current meaning in terms of providing location information to a position calculation function located elsewhere. Perhaps “network-involved operation” would be a better term. In any case we can indicate to SA2 that their current term is confusing in light of the existing “-based” and “-assisted” terms with a long history in RAN2. We aren’t sure that the new term is needed in RAN2, so maybe SA2 can be informed of the issue and left to determine what they prefer to do. |
| ZTE | No | We do not think SA2’s definition should be changed according to RAN’s rule. The “network-involved operation” proposed by MediaTek looks good to us. |
| Xiaomi | Yes | We can tell SA2 about the confusion of using “network-assisted”, as it is not aligned with RAN2 use of “assisted”. We agree with MediaTek about using “network-invlovled operation”. RAN2 can recommend it to SA2, and SA2 can decide whether or not to adopt it. From RAN2 point of view, there is no need to adopt this definition at this stage. We can adopt it when needed. |
| CATT | No | The suffixes "-based" and "-assisted" within TS38.305 are only used for RAN. SA2’s terms definition with the suffixes are not restricted by it. Functionalities in SA2 are different from RAN. Therefore, it is not needed to ask SA2 to align the use of the suffixes "-based" and "-assisted" within TS38.305. |

# Conclusions

TBD

# References

- R2-2209188 LS on Terminology Alignment for Ranging/Sidelink Positioning, SA2.

- R2-2209400	Discussion on SL Positioning	CATT	discussion	Rel-18	FS_NR_pos_enh2.

- R2-2209425	Discussion on sidelink positioning	Huawei, HiSilicon	discussion	Rel-18

- R2-2210003	On SL Positioning Protocol and Architecture Aspects	Lenovo	discussion	Rel-18

- R2-2210040	Discussion on Terminology alignment with SA2	Xiaomi	discussion	Rel-18

- R2-2210316	SL positioning Terminology and Protocol Aspects	Ericsson	discussion	Rel-18

- R2-2210042	Discussion on SL positioning	Xiaomi	discussion	Rel-18

- R2-2209560	Discussion on sidelink positioning	vivo	discussion	Rel-18	FS_NR_pos_enh2