3GPP TSG-RAN WG2 #119bis-e	R2-22xxxxx

Online, 10 – 19 October, 2022

Agenda Item:	8.9.4

Source: 	LG Electronics Inc.

Title: 	Report of [AT119bis-e][425][Relay] Adaptation layer for scenario 2 (LG)

Document for:	Discussion and decision

# Introduction

This is the trigger of the following email discussion:

- [AT119bis-e][425][Relay] Adaptation layer for scenario 2 (LG)

Scope:

- Discuss the potential for an adaptation layer on the Uu and UE-to-UE links in scenario 2, considering the possibility of making the adaptation layer configurable/optional on either link, and focussing on whether the following aspects can/should be supported in Scenario 2 without an adaptation layer:

-	Possibility of restriction to the relay UE serving only one remote UE

-	Possibility of restriction to 1:1 bearer mapping only over non-3GPP UE-to-UE link and 3GPP Uu link.

-	Mapping a PDCP entity of the remote UE to a RLC entity of the relay UE to ensure that a PDCP PDU is delivered to an intended PDCP entity or RLC entity for support of more than one RB

-	Possibility to support interoperability between two UEs from different vendors

-	Ensuring identification of data own by the relay UE and data relayed from/to the remote UE over the Uu link.

- Consider whether SRAP is a suitable baseline for a scenario 2 adaptation layer, considering both Uu and the ideal link (potentially different conclusions for the two links)

Intended outcome: Report to CB session

Deadline: Monday 2022-10-17 1700 UTC

As captured in the report of [Post119-e][408][Relay] Path operations in multi-path relaying in R2-2210027, a majority of companies see the benefit of using the adaptation layer over Uu link. But, there is no majority’s view on the adaptation layer over non-3GPP link.

In R2-2210027, RAN2 is suggested to study need of an adaptation layer on the UE-to-UE link and the Uu link between relay UE and the gNB for Scenario 2, considering whether the following aspects can/should be supported in Scenario 2 without an adaptation layer:

- Possibility of restriction to the relay UE serving only one remote UE

- Possibility of restriction to 1:1 bearer mapping only over non-3GPP UE-to-UE link and 3GPP Uu link.

- Mapping a PDCP entity of the remote UE to a RLC entity of the relay UE to ensure that a PDCP PDU is delivered to an intended PDCP entity or RLC entity for support of more than one RB

- Possibility to support interoperability between two UEs from different vendors

- Ensuring identification of data own by the relay UE and data relayed from/to the remote UE over the Uu link .

For continuation of the study on an adaptation layer on the UE-to-UE link and the Uu link between relay UE and the gNB for Scenario 2, companies are requested to provide their views on the following questions listed in this document.

# Discussion

### Feasibility of support of scenario 2 with/without an adaptation layer:

Some companies think that the relay UE can be restricted to serve only one remote UE for scenario 2.

Question 1A: Do you think that the relay UE can be restricted to serve only one remote UE?

| Company | Yes/No | Comment |
| --- | --- | --- |
| LG Electronics | Yes | We can simplify Scenario 2. |
| Apple | Yes with comment | From the use case perspective, we think there could be a static relationship between remote UE/relay UE. In this case, the relay UE will always move along with a unique remote UE, thereby unlikely serving another remote UE effectively at the same time. But we also believe the use case of “UE aggregation” may evolves to include other topologies. 3GPP design need to make some room for future extensions. |
| Huawei, HiSilicon | Yes |  |
| ASUSTeK | No | From application point of view, serving only one remote UE is too restrictive. |
| CMCC | Yes | To address the restricted UL bitrates and fulfils the 5G UL wide bandwidth service requirement, and in most cases, the UE wherein is non handheld UE, e.g. equipped in the assembling line of factory or vehicle, with live 8K video mobile control room, where multiple UEs are expected to be equipped to take over the UL big datarate service delivery. Or else, it is tricky to realize it due to the limited UL power transmission. Consequently, such linking topology in scenario 1 is not feasible, whereas the appropriate linking topology we expect is anchor UE (refers to remote UE) connects to more than one aggregated UE (refers to relay UE). |
| vivo | Yes | It is the most typical case for scenario 2 that the relay UE only serves one remote UE since there should be static and ideal backhaul between two UEs. Furthermore, for future application scenarios, the case of multiple relay UEs serving one remote UE will be more promising than the case of one relay UE serving multiple remote UEs. |
| Intel | Yes | We think that the baseline scenario is where the Remote UE and the Relay UE have a semi-static relationship as we already agreed. Therefore, it is possible that there is only one such pair in scenario 2 i.e. to allow one remote UE per Relay UE. |
| Spreadtrum | Yes |  |
| ZTE | Yes | We think the most possible case for UE aggregation is that relay UE is associated with one remote UE. |
| CATT | Yes |  |
| Lenovo | Yes | It is a typical scenario that only one remote UE is served by a aggregated(relay) UE. But, the relay UE is allowed to be changed. |

Some companies think that if the relay UE can be restricted to serve only one remote UE for scenario 2, UE identification is not needed.

Question 1B: Do you think that UE identification is not needed over non-3GPP link, regardless of whether or not the adaptation layer is supported?

| Company | Needed or not | Comment |
| --- | --- | --- |
| LG Electronics | Not needed | If the relay UE is restricted to serve only one remote UE, the relay UE does not need to differentiate different remote UEs. Thus, UE ID field is not needed. |
| Apple | Not needed* | On the condition “if only serving one remote UE” |
| Huawei, HiSilicon | Not needed | In general, we understand UE implementation can handle any differentiation on non-3GPP interface for remote UE id or E2E bearer id. Adaptation layer could be one way of UE implementation, but other ways are also possible. |
| ASUSTeK | See comments | This could be left to UE implementation. |
| CMCC | it can be left to UE implementation |  |
| vivo | Not needed | Since the relay UE only serves one remote UE. |
| Intel | See comment | It is not needed specifically over the non-3GPP link as it is up to UE implementation, but it may be needed in general to inform the gNB. |
| Spreadtrum | Not needed | If Q1A is agreed, i.e. the relay UE can be restricted to serve only one remote UE, then UE identification is not needed over non-3GPP link. |
| ZTE | Not needed | If only one remote UE is connected with the relay UE, it is not necessary to identify the remote UE over non-3GPP link. |
| CATT | Not needed | The relay UE is restricted to serve only one remote UE for scenario 2, hence the UE identification is not needed. |
| Lenovo | Not needed | It is not needed if only one remote UE is served. |

Question 1C: Do you think that UE identification is not needed over Uu link, regardless of whether or not the adaptation layer is supported?

| Company | Needed or not | Comment |
| --- | --- | --- |
| LG Electronics | See comments | If LCID space on Uu link are separated for data from/to remote UE and relay UE, UE identification seems not needed over Uu link. Otherwise, UE identification is needed over Uu link for differentiation of data from/to remote UE and relay UE. |
| Apple | Needed | Similar view as LG. But we think the number of LCHs in Uu cannot fully support both local traffic and forwarded traffic, So, some kind of differentiation needs to distinguish the source or destination of the Uu traffic. In overall, we think the UE identifier is still needed. |
| Huawei, HiSilicon | Yes (See comments.) | If the assumption in Q1A can be agreed (i.e. one relay services one remote always) then UE identification is not needed over Uu interface. How to differentiate remote UE and relay UE’s data at Uu hop is another topic, and in Rel-17, RAN2 agreed to use different Uu RLC channels for that purpose. |
| ASUSTeK | Needed | UE identification is needed over Uu link to support multiple remote UEs. |
| CMCC | Yes | In scenario 2, as explained in Q1A, the relay UE services one remote UE. Network can identify the packet without the UE identification. |
| vivo | Yes | In Uu link, the bearer mapping between relay UE RLC channel and remote UE E2E RB will be always 1-to-1. Hence, UE identification is not needed. |
| Intel | Yes | If different Uu RLC channels and LCID are used to differentiate Remote UE traffic (such that Relay UE is restricted to serve one Remote UE), it seems not needed. |
| Spreadtrum | Yes | If Q1A is agreed, UE identification is also not needed over Uu link. |
| ZTE | Not needed | Suppose the relay UE serves only one remote UE and 1:1 bearer mapping between remote UE’s RB and Uu RLC channel is assumed, it is not necessary to identify the remote UE over Uu link any more. |
| CATT | Not Needed | Same view as HW. |
| Lenovo | No needed | Agree with LG that if LCID space on Uu link are separated for relay data and relayed data, UE identification is not needed over Uu link. gNB is aware of the pair when the end-to-end RRC establishment. |

Some companies propose to focus on 1:1 bearer mapping only over non-3GPP UE-to-UE link and 3GPP Uu link for less impact on adaptation functionality.

Question 2A: Do you think that RAN2 should restrict to 1:1 bearer mapping only over non-3GPP UE-to-UE link and 3GPP Uu link?

| Company | Yes/No | Comment |
| --- | --- | --- |
| LG Electronics | No strong view |  |
| Apple | No | 1:1 bearer mapping seems assume that relay UE does not have its own Uu traffic, so all remote UE E2E bRBs can one-to-one map to Uu RB with a unique LCID. We think this assumption is too restrictive. We shall not build a design based on such a limited foundation. |
| Huawe, HiSilicon | Yes (See comments) | For non-3GPP interface, we still think this can be left to UE implementation, given that there may be no concept of bearer at all on the non-3GPP interface. For Uu interface, if Q1A can be agreed, 1:1 mapping is simpler considering the existing RLC bearer number should be sufficient for the total number of relay’s own and only one remote UE’s E2E bearer. |
| ASUSTeK | No | Suggest to support m:1 bearer mapping so as to align with U2N relay in R17. |
| CMCC | it can be left to UE implementation |  |
| vivo | Yes | This is the most typical and simplest case. |
| Intel | Yes | For the baseline version i.e. in this first release of UE aggregation, we can consider restriction to 1:1 mapping. In any case, for non-3GPP link, we cannot do any optimization and it has to be left to UE implementation. |
| Spreadtrum | See comments | it can be left to UE implementation |
| ZTE | Yes | 1:1 bearer mapping works. Actually it can be up to implementation without specification. |
| CATT | Yes |  |
| Lenovo | Yes | We can simplify the mechanism. |

Some companies think that with restriction to 1:1 bearer mapping only over non-3GPP UE-to-UE link and 3GPP Uu link, the adaptation layer is not needed for Scenario 2.

Question 2B: Do you think that bearer identification is not needed over UE-to-UE link, with restriction to 1:1 bearer mapping only over non-3GPP UE-to-UE link and 3GPP Uu link, regardless of whether or not the adaptation layer is supported?

| Company | Needed or not | Comment |
| --- | --- | --- |
| LG Electronics | Needed | Without bearer identification, it is unclear how UEs can ensure mapping a PDCP entity of the remote UE to a RLC entity of the relay UE to deliver a PDCP PDU to an intended PDCP entity or RLC entity for support of more than one RB. |
| Apple | Needed | Same view as LG |
| Huawei, HiSilicon | Not needed | Similar answer to Q1B, in general, we understand UE implementation can handle any differentiation on non-3GPP interface for remote UE id or E2E bearer id. Adaptation layer could be one way of UE implementation, but other ways are also possible. for instance, if the non-3GPP is over fiber over IP, the bearer id can be carried in ip header like DSCP, and if the non-3GPP connection is over wifi the LWAAP can be used by UE implementation. To be brief, in RAN2 we do not need to discuss/specify bearer id for non-3GPP interface. |
| ASUSTeK | See comments | Bearer identification over UE-to-UE link could be left to UE implementation. |
| CMCC | we prefer to left it to the UE implementation |  |
| vivo | Not needed | It can be completely left to UE implementation for UE-to-UE operation. |
| Intel | Not needed | We think that it can be left to UE implementation to handle the bearer mapping. The gNB will provide configuration to the Remote UE and Relay UE to map the Remote UE’s Uu bearer to a Uu RLC channel at the Relay UE. Then it is up to the Relay UE’s implementation to handle the mapping. We do not think we should spend special effort to support this for scenario 2 in this release. |
| Spreadtrum | Not needed | it can be left to UE implementation |
| ZTE | Not needed | If we can assume that 1:1 bearer mapping is used and the UE aggregation support is restricted to the UEs from same vendor, it is not necessary to support the adapt layer. |
| CATT | Not needed |  |
| Lenovo | Not needed | The relay UE can identify the mapping between ingress and egress based on the gNB configuration in the case of 1:1 bearer mapping. |

Question 2C: Do you think that bearer identification is not needed over Uu link (except LCID in MAC header), with restriction to 1:1 bearer mapping only over non-3GPP UE-to-UE link and 3GPP Uu link, regardless of whether or not the adaptation layer is supported?

| Company | Needed or not | Comment |
| --- | --- | --- |
| LG Electronics | Maybe, not needed | LCID in MAC header could work for bearer identification. |
| LG | Needed | LCID space will not be big enough. |
| Huawei, HiSilicon | Not needed | We do not expect there would be a lot of E2E bearer needing relay UE’s help, so the existing maximum number of LCID should be enough assuming relay only helps one remote UE. |
| ASUSTeK | Needed |  |
| CMCC | Not needed | For link between relay UE and gNB in scenario 2, we understand, it may be a work assumption that 1:1 bearer mapping between remote UE’s RB and Uu RLC channel of relay UE are configured. That is, the granularity of the aggregation is the DRB, which will be configured one common PDCP for involved UEs and RLC respectively, rather than the UE. During discussion, it seems some companies misunderstood that multiple aggregated DRBs in one UE will be configured only one common PDCP for involved UEs. That is different from scenario 1. The adaption layer is not needed to distinguish RB or remote UE for scenario 2. |
| vivo | Not needed | As the above mentioned, the bearer mapping between relay UE RLC channel and remote UE E2E RB will be always 1-to-1. Hence, bearer identification is not needed. |
| Intel | Not needed | If 1:1 bearer mapping is supported, then, we have the same view as Huawei that existing maximum number is sufficient considering the semi-static relation between Remote UE and Relay UE for UE aggregation. We can handle the mapping related configuration in RRC layer. |
| Spreadtrum | Not needed |  |
| ZTE | Not needed | If 1:1 bearer mapping is assumed, bearer identification is not needed. |
| CATT | Note needed |  |
| Lenovo | Not needed |  |

Some companies think that we need to ensure that a PDCP PDU is delivered to an intended PDCP entity or RLC entity for support of more than one RB based on proper mapping a PDCP entity of the remote UE to a RLC entity of the relay UE e.g. by bearer identification in the adaptation layer.

Question 3A: Do you think that a PDCP PDU can be delivered to an intended PDCP entity or RLC entity for support of more than one RB without the adaptation layer over non-3GPP link.

| Company | Yes/No | Comment |
| --- | --- | --- |
| LG Electronics | No | Regardless of restriction to 1:1 bearer mapping, the adaptation layer is needed to add bearer ID field to each PDU. But, RAN2 should still need to clarify whether the adaptation layer is based on 3GPP standard or UE vendor specific implementation. |
| Apple | No | The bearer ID field is needed. It could be similar to the one used in SRAP |
| Huawei, HiSilicon | Not relevant to adaptation layer | We are not quite sure about the question. For UL at remote UE, it just needs to pass the data received from relay to the PDCP associated with an E2E bearer, as we discussed in Q2B, differentiation among E2E bearers is not an issue which can be left to UE implementation. For DL at remote UE, if needed, the RLC entity can be configured to be Uu interface or non-3GPP interface, it does not matter to SRAP is absent or present. |
| ASUSTeK | Yes | This could be ensured by UE implementation. |
| CMCC | it can be left to UE implementation |  |
| vivo | Yes | RB differentiation over non-3GPP link may be non-specified and up to UE implementation. |
| Intel | Yes | It will be up to gNB configuration to aid with the end-to-end bearer mapping and up to UE implementation to handle the UE-to-UE link appropriately. |
| Spreadtrum | Yes | it can be left to UE implementation |
| ZTE | Yes | For the PDCP PDU delivery to RLC entity at remote UE, it depends on whether direct, indirect or split bearer is configured. For the split bearer, it should be associated with two RLC entity and which RLC entity is used depends on the data split/duplication rules. From the relay UE perspective, suppose the UEs participating the aggregation are from the same vendor, how to deliver the received PDCP PDU to the Uu RLC channel at relay UE can be up to implementation. |
| CATT | Yes |  |
| Lenovo | See comment | Not clear about the question if the more than one bearer is associated with Uu bearer or U2U bearer. If multiple end-to-end bearers can be mapping to one U2U RLC bearer, bearer ID could be needed. But, we prefer to 1:1 mapping. |

Question 3B: Do you think that a PDCP PDU can be delivered to an intended PDCP entity or RLC entity for support of more than one RB without the adaptation layer over Uu link.

| Company | Yes/No | Comment |
| --- | --- | --- |
| LG Electronics | Maybe, yes | Yes, with restriction to 1:1 bearer mapping and separate LCID spaces for data from/to remote UE and relay UE. |
| Apple | No | We think there is no enough LCID space to separately support both local traffic and E2E Uu SRB/DRBs. |
| Huawei, HiSilicon | Not relevant to adaptation layer | We are not quite sure about the question. From relay perspective, it just forwards the data received from remote UE to corresponding Uu the RLC channel on UL. From network perspective, it’s NW implementation which path to use on DL for data split over split bearer, and both path will be used for data duplication. |
| ASUSTeK | No |  |
| CMCC | Yes | For link between relay UE and gNB in scenario 2, we understand, it may be a work assumption that 1:1 bearer mapping between remote UE’s RB and Uu RLC channel of relay UE are configured. That is, the granularity of the aggregation is the DRB, which will be configured one common PDCP for involved UEs and RLC respectively, rather than the UE. During discussion, it seems some companies misunderstood that multiple aggregated DRBs in one UE will be configured only one common PDCP for involved Ues. That is different from scenario 1. The adaption layer is not needed to distinguish RB or remote UE for scenario 2. |
| Vivo | Yes | NW configures 1-to-1 mapping between relay UE Uu RLC channel and remote UE E2E RB to the relay UE. Via relay UE’s LCID in MAC PDU, relay UE can map each packet to the proper remote UE’s PDCP entity |
| Intel | Potentially Yes | We think that it is achievable based on network configuration, and this can be handled in RRC. |
| Spreadtrum | Yes |  |
| ZTE | Yes | From the relay UE perspective, suppose the Ues participating the aggregation are from the same vendor, how to deliver the received PDCP PDU to the Uu RLC channel at relay UE can be up to implementation. Then assume 1:1 mapping is configured at the Uu link, the gNB should be aware of the association between remote UE’s RB and Uu RLC channel and then deliver the received PDCP PDU to the corresponding PDCP entity. |
| CATT | Yes | With proper configuration, there is no issues foreseen. |
| Lenovo | See comments | See above. |

Some companies think that interoperability between two UEs from different vendors needs to be supported.

Question 4A: Do you think that interoperability between two UEs from different vendors needs to be supported?

| Company | Yes/No | Comment |
| --- | --- | --- |
| LG Electronics | Yes | Interoperability between two UEs is considered high-level 3GPP requirement. Any technology without interoperability seems not subject to 3GPP standardization. |
| Apple | Yes | Same view as LG |
| Huawei, HiSilicon | No | We are confused with this interoperability cross different vendors. what is the non-3GPP interface is not specified, if the remote UE and relay UE are not aware of the physical form of the interface, how to avoid the effort of UE implementation even with adaptation layer. |
| ASUSTeK | No | Since the UE-UE inter-connection is non-standardized, it doesn’t seem necessary to support interoperability between two UEs from different vendors. In our opinion, this seems to be out of 3GPP standardization scope. |
| CMCC | At least not support in this release |  |
| vivo | Comments | In the WID, it is clearly declared that inter-UE operation for the ideal-backhaul case should not be specified. |
| Intel | No | Since we are not sure of the non-3GPP interface, we cannot guarantee support based on this interface. |
| Spreadtrum | No |  |
| ZTE | See comments | Actually it depends on the operator’s requirements. If operator only have interest with the UE aggregation support of UEs from the same vendor, the interoperability is not necessary to be considered. |
| CATT | No | The interface is non-3GPP interface, hence it is hard to consider interoperability issue. |
| Lenovo | No |  |

Question 4B: Do you think that interoperability between two UEs from different vendors can be supported over non-3GPP link without 3GPP standard work on adaptation layer?

| Company | Yes/No | Comment |
| --- | --- | --- |
| LG Electronics | No | Two UEs from different vendors need to understand different bearer IDs between two UEs based on the adaptation layer specified by 3GPP. |
| Apple | No | If nothing is defined on top of non-3GPP link by 3GPP, 3GPP cannot claim inter-vendor inter-operability. |
| Huawei, HiSilicon | See comments | Our question is even with the adaptation layer, how to support this interoperability cross different vendors as the remote UE and relay UE are not aware of the physical form of the non-3GPP interface. |
| ASUSTeK | See comments | Even if adaptation layer is supported over non-3GPP link, the lower layers below the adaptation layer is still non-standardized. In this situation, interoperability between two UEs from different vendors still cannot be ensured. |
| CMCC | See comments | At least not support in this release |
| vivo | Maybe yes | Up to implementation, if needed. |
| Intel | See comment | Same comment as above + we cannot do optimization to support interoperability over non-3GPP link as we do not know what this link will be. |
| Spreadtrum | See comments | At least not support in this release |
| ZTE | No |  |
| CATT | No | The interface is non-3GPP interface, hence it is hard to consider interoperability issue. |
| Lenovo | No |  |

Question 4C: Do you think that interoperability between two UEs from different vendors can be supported over Uu link without 3GPP standard work on adaptation layer?

| Company | Yes/No | Comment |
| --- | --- | --- |
| LG Electronics | Maybe, yes | Yes, with restriction to 1:1 bearer mapping and separate LCID spaces for data from/to remote UE and relay UE. Separate LCID spaces need to be configured by 3GPP spec. |
| Apple | See comment | I think the interoperability issue is only about non-3GPP link. Uu link is always fully defined by 3GPP. |
| Huawei, HiSilicon | See comments | Same view as Apple. |
| ASUSTeK | See comment | Share the same view as Apple. |
| CMCC | See comments | At least not support in this release. |
| vivo | Yes | Based on 1-to-1 bearer mapping configuration. |
| Intel | Yes | For the Uu link it will be supported as defined by 3GPP |
| Spreadtrum | See comments | At least not support in this release. |
| ZTE | Yes | Interoperability only concerns with the UE-UE interface. It is not relevant with Uu link. |
| CATT | See comments | Same view as Apple. |

Some companies think that the adaptation layer over Uu link needs to be supported for identification of data own by the relay UE and data relayed from/to the remote UE over the Uu link.

Question 5A: Do you think that RAN2 should support identification of data own by the relay UE and data relayed from/to the remote UE over the Uu link?

| Company | Yes/No | Comment |
| --- | --- | --- |
| LG Electronics | Yes |  |
| Apple | Yes |  |
| Huawei, HiSilicon | See comments | How to differentiate remote UE and relay UE’s data at Uu hop is not a new issue. In Rel-17, RAN2 agreed to use different Uu RLC channels for that purpose. |
| ASUSTeK | Yes |  |
| CMCC | With comments | Same solution in R17 SL relay can be reused. It not necessary that issue here. |
| vivo | Yes | Based on NW configuration, different LCIDs are allocated to relay UE’ own data and remote UE’ data. |
| Intel | Yes | Same comment as Huawei, HiSilicon. |
| Spreadtrum | Yes | Different LCIDs can used to identify the data own by the relay UE and data relayed from/to the remote UE over the Uu link |
| ZTE | Yes | This can be supported via legacy mechanism, i.e. the data from remote UE and relay UE are delivered via different RLC channels. |
| CATT | See comment | Same concern as HW. |
| Lenovo | Yes |  |

Question 5B: Do you think that adaptation layer over Uu link needs to be supported for identification of data own by the relay UE and data relayed from/to the remote UE over the Uu link?

| Company | Yes/No | Comment |
| --- | --- | --- |
| LG Electronics | Maybe, no | No, with restriction to 1:1 bearer mapping and separate LCID spaces for data from/to remote UE and relay UE. Separate LCID spaces need to be configured by 3GPP spec. |
| Apple | Yes | As explained in Q3B, 1:1 mapping is not a sufficient solution. |
| Huawei, HiSilicon | No | As answered to Q5A, Furthermore the relay UE’s own data is terminated between relay and NW, it should not go through adaptation layer. |
| ASUSTeK | No | In R17 U2N relay, remote UE traffic and relay UE own traffic are separated in different Uu RLC bearers in Uu hop. We think the same concept can be reused and thus the adaptation layer does not need to take care of this. |
| CMCC | No | Just reuse the solution in R17. |
| vivo | No | Current LCID in MAC header is enough. |
| Intel | No | With 1:1 mapping to different Uu RLC channels, we think that it is not needed. |
| Spreadtrum | No |  |
| ZTE | No |  |
| CATT | No |  |
| Lenovo | No |  |

### Feasibility of support of SRAP in scenario 2

As captured in the report of [Post119-e][408][Relay] Path operations in multi-path relaying in R2-2210027, if adaptation layer is supported for Scenario 2, the majority of companies think that SRAP is considered as baseline for design of the adaptation layer for scenario 2.

However, if adaptation layer is supported for Scenario 2, regardless of whether SRAP is considered as baseline for design of the adaptation layer, it is still unclear whether the existing SRAP specification can be used for specification of the adaptation layer in Scenario 2 or new specification needs to be created for the adaptation layer.

Question 6A: If adaptation layer is supported for Scenario 2, which option do you prefer for specification of the adaptation layer in Scenario 2 over UE-to-UE link, regardless of whether SRAP is considered as baseline for design of the adaptation layer?

- Option 1: The existing SRAP specification for UE-to-UE link

- Option 2: New adaptation layer specification for UE-to-UE link

| Company | Option | Comment |
| --- | --- | --- |
| LG Electronics | 2 | We think that option 2 will look simpler than option 1 in a spec. New spec can be simply based on some SARP specification texts, if not specific to PC5. The adaptation layer does not need to differentiate SL data and non-3GPP data. Any SL specific procedural text in 38.351 cannot be applied to non-3GPP data. Assuming that UE ID is not needed, new data PDU format needs to be considered. |
| Apple | 1 | We can use R17 SRAP as baseline and consider some change/enhancements. There is no need to create a new spec other than 38.351, |
| OPPO | 1 | Same view as Apple |
| Huawei, HiSilicon | See comments | The issue is the existing SRAP is not fully compatible with scenario 2. The UE id may not be needed, the function of determine egree PC5 relay channel is not needed. The length of bearer ID is not enough. So we still think no adaptation layer could be the easy way for this non-3GPP interface. |
| ASUSTeK | 1 |  |
| CMCC | we prefer to left it to the UE implementation |  |
| vivo | None | Prefer None, acceptable for Option 2 |
| Intel | None | We don’t think that existing SRAP can be supported over the non-3GPP as it can be any interface (for e.g. it is not clear how it can be configured as PC5 RLC channel). |
| Spreadtrum | None |  |
| ZTE | 1 | Agree with OPPO. |
| CATT | See comments | We prefer do not involve introducing SRAP is better. |
| Lenovo | None |  |

Question 6B: If adaptation layer is supported for Scenario 2, which option do you prefer for specification of the adaptation layer in Scenario 2 over Uu link, regardless of whether SRAP is considered as baseline for design of the adaptation layer?

- Option 1: The existing SRAP specification for Uu link

- Option 2: New adaptation layer specification for Uu link

| Company | Option | Comment |
| --- | --- | --- |
| LG Electronics | 2 | We think that option 2 will look simpler than option 1 in a spec. New spec can be simply based on some SARP specification texts, if not specific to PC5. The adaptation layer does not need to differentiate SL data and non-3GPP data. Any SL specific procedural text in 38.351 cannot be applied to non-3GPP data. Assuming that UE ID is not needed, new data PDU format needs to be considered. |
| Apple | 1 | Same reason as answered in Q6A |
| OPPO | 1 | Same view as Apple |
| Huawei, HiSilicon | 1 |  |
| ASUSTeK | 1 |  |
| CMCC | NULL | We think adaption layer is not need for Uu link. |
| vivo | None | Prefer None, acceptable for Option 2 |
| Intel | None | If 1:1 mapping is adopted, we don’t think there is any need of adaptation layer support for scenario 2. If majority of the companies prefer, we can go with option 1 for Uu link. |
| Spreadtrum | Null | Adaption layer is not need for Uu link |
| ZTE | 1 |  |
| CATT | See comments | We prefer do not involve introducing SRAP is better. |
| Lenovo | None | Option1 is also fine. |

### Potential compromise for adaptation layer in scenario 2

For compromise, RAN2 discussed configurable adaptation layer and/or optional support of adaptation layer.

Question 7A: If adaptation layer is supported, do you think that the adaptation layer over UE-to-UE link can be configurable by the network?

| Company | Configurable or not | Comment |
| --- | --- | --- |
| LG Electronics | Not configurable, but OK with configurable layer. | We think that support of configurable adaptation layer requires more effort for standardization. But, if a majority of companies support configurable adaptation layer, we are fine to support it for compromise. |
| Apple | OK with configurable |  |
| OPPO | OK with configurable |  |
| Huawei, HiSilicon | No | It is quite strange have a configurable 3GPP layer on a “non-3GPP” interface. |
| ASUSTeK | No | Share the same view as Huawei. |
| CMCC | we prefer to left it to the UE implementation |  |
| vivo | NOT configurable |  |
| Intel | No | We don’t think we can agree/converge on adaptation layer over non-3GPP link. |
| Spreadtrum | No |  |
| ZTE | OK with configurable |  |
| CATT | No | Same view as HW. |
| Lenovo | No |  |

Question 7B: If adaptation layer is supported, do you think that the adaptation layer over Uu link can be configurable by the network?

| Company | Configurable or not | Comment |
| --- | --- | --- |
| LG Electronics | Not configurable. | We think that support of configurable adaptation layer requires more effort for standardization. Thus, if adaptation layer is supported, it should not be configurable. |
| Apple | Not configurable | It shall be always used, as similar to R17 SL relay. |
| OPPO | Can follow majority view |  |
| Huawei, HiSilicon | Not configurable | If configurable means optional UE capability, we are not keen about defining many UE types, which means double specification effort and network implementation effort to accommodate all the UE types, if not necessary. In Rel-17, configurable PC5 adaptation layer was avoided due to such considerations. |
| ASUSTeK | No |  |
| CMCC | configurable can be accepted as compromise |  |
| vivo | Configurable |  |
| Intel | Not configurable | If it is to be supported, we can follow R17 as baseline. |
| Spreadtrum | Configurable |  |
| ZTE | OK with configurable |  |
| CATT | No | Same view as HW. |
| Lenovo | No | ‘configurable’ makes system complicated. |

Some companies think that if configurable adaptation layer is supported, UE needs to report its capability of the adaptation layer support to gNB. Namely, support of the adaptation layer can be optional feature for Scenario 2. Moreover, there is a view in that adaptation layer can be optional feature for Scenario 2, regardless of whether adaptation layer is configurable or not.

Question 8A: If adaptation layer is supported, do you think that the adaptation layer over UE-to-UE link can be optional feature in UE capability for UE supporting Scenario 2?

| Company | Optional or not | Comment |
| --- | --- | --- |
| LG Electronics | Not optional for UE supporting Scenario 2 | We think that optionality of adaptation layer requires more effort for standardization. But, if a majority of companies support configurable adaptation layer, we are fine to support optionality of adaptation layer for compromise. |
| Apple |  | Too early to decide. Depends on which kind of Non-3GPP link the UE has. |
| OPPO | If configurability is confirmed, UE capability is needed anyway | If we allow configurability, we understand it means 3GPP confirms there are two UE implementations Implementation-1: the bearer ID identification is by UE implementation, w/o a need a SRAP Implementation-2: the bearer ID identification is by the support of SRAP These two implementation are for the same reason, so we see no reason to mandate the UE to support both. Thus UE capability is needed to indicate whether UE support A/ implementation-1 only B/ implementation-2 only C/ Both |
| Huawei, HiSilicon | No | Same view as LG that optionality of adaptation layer requires more effort for standardization as well as for NW implementation. |
| ASUSTeK | No | Share the same view as Huawei. |
| CMCC | left it to the UE implementation |  |
| vivo | No | Since in Q7A, we think the adaptation layer over UE-to-UE link is NOT configurable, the corresponding UE capability is not needed. |
| Intel | No | Over UE-to-UE link, it can be left to UE implementation. |
| Spreadtrum | No | It can be left to UE implementation |
| ZTE | Optional | If the adapt layer is configurable, it is natural that the adapt layer can be optional capability. |
| CATT | No | Same view as LG. |
| Lenovo | No |  |

Question 8B: If adaptation layer is supported, do you think that the adaptation layer over Uu link can be optional feature in UE capability for UE supporting Scenario 2?

| Company | Optional or not | Comment |
| --- | --- | --- |
| LG Electronics | Not optional for UE supporting Scenario 2 | We think that optionality of adaptation layer requires more effort for standardization. Thus, if adaptation layer is supported, it should not be optional for Uu link. |
| Apple | Not optional |  |
| OPPO | If configurability is confirmed, UE capability is needed anyway | Similar to our answer to 8A above. |
| Huawei, HiSilicon | No | Same view as LG that optionality of adaptation layer requires more effort for standardization as well as for NW implementation. |
| ASUSTeK | No |  |
| CMCC | configurable can be accepted as compromise |  |
| vivo | Optional feature in UE capability |  |
| Intel | Not optional | Same view as LG. |
| Spreadtrum | configurable can be accepted as compromise |  |
| ZTE | optional | If the adapt layer is configurable, it is natural that the adapt layer can be optional capability. |
| CATT | No | Same view as LG. |

Question 9A: Which option is your preferred one regarding the support of the adaptation layer over UE-to-UE link in scenario 2 (Do not change the below options. If you want to change, you can add a new option below the existing options)?

Option 1: Specify adaptation layer in the protocol stack for scenario 2 in RAN2 (but not configurable);

Option 2: Do not specify adaptation layer from the protocol stack for scenario 2 in RAN2 (whether/how to use adaptation layer is up to UE implementation);

Option 3: Adaptation layer in scenario 2 is specified and configurable, along with or without a UE capability for scenario 2;

Option 4: Any other option?

| Company | Yes/No | Comment |
| --- | --- | --- |
| Huawei, HiSilicon | For non-3GPP interface: option2 | For Uu inferface, it relates to the conclusion on configurability and UE capability. Our preference is if the layer is configurable, the UE supports adaptation layer is configured or not, and then option1 can be adopted. |
| LG Electronics | Option 1 | Option 3 is also fine based on a majority’s view. |
| ASUSTeK | Option 2 |  |
| CMCC | Option 2 |  |
| vivo | Option 2 |  |
| Apple | Option 3 (also fine with Option 1) |  |
| OPPO | See comment | Our understanding is option-3 is a way-out based on the situation here. |
| Intel | Option 2 |  |
| Spreadtrum | Option 2 |  |
| Huawei, HiSilicon | Option 2 |  |
| TCL | Option 2 |  |
| ZTE | Option 2 |  |
| CATT | Option 2 |  |
| Lenovo | Option 2 |  |

Question 9B: Which option is your preferred one regarding the support of the adaptation layer over Uu link in scenario 2 (Do not change the below options. If you want to change, you can add a new option below the existing options)?

Option 1: Specify adaptation layer in the protocol stack for scenario 2 in RAN2 (but not configurable);

Option 2: Do not specify adaptation layer from the protocol stack for scenario 2 in RAN2 (whether/how to use adaptation layer is up to UE implementation);

Option 3: Adaptation layer in scenario 2 is specified and configurable, along with or without a UE capability for scenario 2;

Option 4: Any other option?

Option 5: Do not specify adaptation layer in the protocol stack for scenario 2 in RAN2.

| Company | Yes/No | Comment |
| --- | --- | --- |
| LG Electronics | No strong view |  |
| ASUSTeK | Option 1 | To align with U2N relay in R17 as possible so as to reduce standardization efforts. |
| CMCC | Option 5; but can accept option 3 as compromise |  |
| vivo | Option 2 | Can consider option 3 as compromise |
| Apple | Option 1 | We share the same view as ASUSTek |
| OPPO | See comment | Our understanding is option-3 is a way-out here. |
| Intel | Option 5, with comment | We think it is easier to not specify any adaptation layer for scenario 2 in this release especially if we intend to have semi-static relationship between Remote UE and Relay UE and with 1:1 bearer mapping, gNB configuration can handle the delivery of data appropriately. If 1:1 bearer mapping cannot agreed, then we prefer option 1. We are not sure of option 2 wording about whether/how to use adaptation layer is up to UE implementation. |
| Spreadtrum | Option 5 |  |
| Huawei, HiSilicon | Option 5 | Same view as Intel. |
| TCL | Option 5 | We agree with intel. |
| ZTE | Option 5 |  |
| CATT | Option 5, with comment | Same view as Intel. |
| Lenovo | Option 5 | We can follow other option if majority view is not option 5. |

# Conclusion and recommendation

In conclusion, Rapporteur recommends agreeing the following proposals: