3GPP TSG-RAN WG2 #119bis-e	R2-22xxxxx

Online, 10 – 19 October, 2022

Agenda Item:	8.9.4

Source: 	LG Electronics Inc.

Title: 	Report of [AT119bis-e][425][Relay] Adaptation layer for scenario 2 (LG)

Document for:	Discussion and decision

# Introduction

This is the trigger of the following email discussion:

- [AT119bis-e][425][Relay] Adaptation layer for scenario 2 (LG)

Scope:

- Discuss the potential for an adaptation layer on the Uu and UE-to-UE links in scenario 2, considering the possibility of making the adaptation layer configurable/optional on either link, and focussing on whether the following aspects can/should be supported in Scenario 2 without an adaptation layer:

-	Possibility of restriction to the relay UE serving only one remote UE

-	Possibility of restriction to 1:1 bearer mapping only over non-3GPP UE-to-UE link and 3GPP Uu link.

-	Mapping a PDCP entity of the remote UE to a RLC entity of the relay UE to ensure that a PDCP PDU is delivered to an intended PDCP entity or RLC entity for support of more than one RB

-	Possibility to support interoperability between two UEs from different vendors

-	Ensuring identification of data own by the relay UE and data relayed from/to the remote UE over the Uu link.

- Consider whether SRAP is a suitable baseline for a scenario 2 adaptation layer, considering both Uu and the ideal link (potentially different conclusions for the two links)

Intended outcome: Report to CB session

Deadline: Monday 2022-10-17 1700 UTC

As captured in the report of [Post119-e][408][Relay] Path operations in multi-path relaying in R2-2210027, a majority of companies see the benefit of using the adaptation layer over Uu link. But, there is no majority’s view on the adaptation layer over non-3GPP link.

In R2-2210027, RAN2 is suggested to study need of an adaptation layer on the UE-to-UE link and the Uu link between relay UE and the gNB for Scenario 2, considering whether the following aspects can/should be supported in Scenario 2 without an adaptation layer:

- Possibility of restriction to the relay UE serving only one remote UE

- Possibility of restriction to 1:1 bearer mapping only over non-3GPP UE-to-UE link and 3GPP Uu link.

- Mapping a PDCP entity of the remote UE to a RLC entity of the relay UE to ensure that a PDCP PDU is delivered to an intended PDCP entity or RLC entity for support of more than one RB

- Possibility to support interoperability between two UEs from different vendors

- Ensuring identification of data own by the relay UE and data relayed from/to the remote UE over the Uu link .

For continuation of the study on an adaptation layer on the UE-to-UE link and the Uu link between relay UE and the gNB for Scenario 2, companies are requested to provide their views on the following questions listed in this document.

# Discussion

### Feasibility of support of scenario 2 with/without an adaptation layer:

Some companies think that the relay UE can be restricted to serve only one remote UE for scenario 2.

Question 1A: Do you think that the relay UE can be restricted to serve only one remote UE?

| Company | Yes/No | Comment |
| --- | --- | --- |
| LG Electronics | Yes | We can simplify Scenario 2. |
| Apple | Yes with comment | From the use case perspective, we think there could be a static relationship between remote UE/relay UE. In this case, the relay UE will always move along with a unique remote UE, thereby unlikely serving another remote UE effectively at the same time. But we also believe the use case of “UE aggregation” may evolves to include other topologies. 3GPP design need to make some room for future extensions. |
| Huawei, HiSilicon | Yes |  |
| ASUSTeK | No | From application point of view, serving only one remote UE is too restrictive. |
| CMCC | Yes | To address the restricted UL bitrates and fulfils the 5G UL wide bandwidth service requirement, and in most cases, the UE wherein is non handheld UE, e.g. equipped in the assembling line of factory or vehicle, with live 8K video mobile control room, where multiple UEs are expected to be equipped to take over the UL big datarate service delivery. Or else, it is tricky to realize it due to the limited UL power transmission. Consequently, such linking topology in scenario 1 is not feasible, whereas the appropriate linking topology we expect is anchor UE (refers to remote UE) connects to more than one aggregated UE (refers to relay UE). |
| vivo | Yes | It is the most typical case for scenario 2 that the relay UE only serves one remote UE since there should be static and ideal backhaul between two UEs. Furthermore, for future application scenarios, the case of multiple relay UEs serving one remote UE will be more promising than the case of one relay UE serving multiple remote UEs. |
| Intel | Yes | We think that the baseline scenario is where the Remote UE and the Relay UE have a semi-static relationship as we already agreed. Therefore, it is possible that there is only one such pair in scenario 2 i.e. to allow one remote UE per Relay UE. |
| Spreadtrum | Yes |  |
| ZTE | Yes | We think the most possible case for UE aggregation is that relay UE is associated with one remote UE. |
| CATT | Yes |  |
| Lenovo | Yes | It is a typical scenario that only one remote UE is served by a aggregated(relay) UE. But, the relay UE is allowed to be changed. |
| Ericsson | Yes | This can be a reasonable assumption. |
| Sharp | Yes |  |
| MediaTek | Yes with comment | It is one particular use case with simplification of general use cases. |
| Samsung | No | Presumably the whole purpose of the UE aggregation case is not to have this restriction. If it makes things simpler for this Release, we can discuss having this restriction. But it would change the use-case (the way we understand it) significantly. |
| Nokia | Yes |  |
| Xiaomi | Yes | It’s up to use case. Seems current use case only requires one-to-one mapping. |
| China Telecom | Yes |  |

Summary and proposal

All companies except 2 companies think that the relay UE can be restricted to serve only one remote UE. Note that the remote UE is already restricted to connect to at most one relay UE based on the WID for both MP scenarios.

Based on the majority’s view:

Proposal 1A: The relay UE is restricted to serve only one remote UE in Scenario 2.

Some companies think that if the relay UE can be restricted to serve only one remote UE for scenario 2, UE identification is not needed.

Question 1B: Do you think that UE identification is not needed over non-3GPP link, regardless of whether or not the adaptation layer is supported?

| Company | Needed or not | Comment |
| --- | --- | --- |
| LG Electronics | Not needed | If the relay UE is restricted to serve only one remote UE, the relay UE does not need to differentiate different remote UEs. Thus, UE ID field is not needed. |
| Apple | Not needed* | On the condition “if only serving one remote UE” |
| Huawei, HiSilicon | Not needed | In general, we understand UE implementation can handle any differentiation on non-3GPP interface for remote UE id or E2E bearer id. Adaptation layer could be one way of UE implementation, but other ways are also possible. |
| ASUSTeK | See comments | This could be left to UE implementation. |
| CMCC | it can be left to UE implementation |  |
| vivo | Not needed | Since the relay UE only serves one remote UE. |
| Intel | See comment | It is not needed specifically over the non-3GPP link as it is up to UE implementation, but it may be needed in general to inform the gNB. |
| Spreadtrum | Not needed | If Q1A is agreed, i.e. the relay UE can be restricted to serve only one remote UE, then UE identification is not needed over non-3GPP link. |
| ZTE | Not needed | If only one remote UE is connected with the relay UE, it is not necessary to identify the remote UE over non-3GPP link. |
| CATT | Not needed | The relay UE is restricted to serve only one remote UE for scenario 2, hence the UE identification is not needed. |
| Lenovo | Not needed | It is not needed if only one remote UE is served. |
| Ericsson | Not | For non 3GPP interface, RAN2 shall avoid spend efforts. As Huawei commented, it can be fully left for UE implementation, including SRAP like and/or LCID in lower layers. In addition, the transmission medium in this interface could be any physical form, therefore, it is unnecessary/challenging to have a layer to cover all physical forms. |
| Sharp | Not needed | Same view as Apple |
| MediaTek | Not needed | If only one remote UE is served. |
| Samsung | Not needed / can be left to UE implementation | If the Relay UE only serves one Remote UE (this restriction is still under discussion, please see Q1A), then the UE identification is not needed. Even if we allow multiple Remote UEs per Relay UE, this identification (while clearly needed in this case) can be left to implementation. |
| Nokia | Not needed | If some identification is needed, then it is out of scope of 3GPP |
| Xiaomi | Not needed |  |
| China Telecom | Not needed |  |

Summary and proposal

Most companies think that UE identification is not needed over non-3GPP link for scenario 2. Some companies pointed out that this could be left to UE implementation.

Based on the majority’s view:

Proposal 1B: UE identification is not needed in L2 PDU over non-3GPP link in Scenario 2.

Question 1C: Do you think that UE identification is not needed over Uu link, regardless of whether or not the adaptation layer is supported?

| Company | Needed or not | Comment |
| --- | --- | --- |
| LG Electronics | See comments | If LCID space on Uu link are separated for data from/to remote UE and relay UE, UE identification seems not needed over Uu link. Otherwise, UE identification is needed over Uu link for differentiation of data from/to remote UE and relay UE. |
| Apple | Needed | Similar view as LG. But we think the number of LCHs in Uu cannot fully support both local traffic and forwarded traffic, So, some kind of differentiation needs to distinguish the source or destination of the Uu traffic. In overall, we think the UE identifier is still needed. |
| Huawei, HiSilicon | Yes (See comments.) | If the assumption in Q1A can be agreed (i.e. one relay services one remote always) then UE identification is not needed over Uu interface. How to differentiate remote UE and relay UE’s data at Uu hop is another topic, and in Rel-17, RAN2 agreed to use different Uu RLC channels for that purpose. |
| ASUSTeK | Needed | UE identification is needed over Uu link to support multiple remote UEs. |
| CMCC | Yes | In scenario 2, as explained in Q1A, the relay UE services one remote UE. Network can identify the packet without the UE identification. |
| vivo | Yes | In Uu link, the bearer mapping between relay UE RLC channel and remote UE E2E RB will be always 1-to-1. Hence, UE identification is not needed. |
| Intel | Yes | If different Uu RLC channels and LCID are used to differentiate Remote UE traffic (such that Relay UE is restricted to serve one Remote UE), it seems not needed. |
| Spreadtrum | Yes | If Q1A is agreed, UE identification is also not needed over Uu link. |
| ZTE | Not needed | Suppose the relay UE serves only one remote UE and 1:1 bearer mapping between remote UE’s RB and Uu RLC channel is assumed, it is not necessary to identify the remote UE over Uu link any more. |
| CATT | Not Needed | Same view as HW. |
| Lenovo | No needed | Agree with LG that if LCID space on Uu link are separated for relay data and relayed data, UE identification is not needed over Uu link. gNB is aware of the pair when the end-to-end RRC establishment. |
| Ericsson | No strong view/open for discussion | Support UE identification may have some benefits for future extension, e.g., multiple remote UEs are served by the same relay UE. Meanwhile, if only limited one remote UE in this release, the need of UE identification will be weak. |
| Sharp | See comments | If we decide it “regardless of whether or not the adaptation layer is supported,” it may not be needed. But if adaptation layer is supported, Remote UE ID is needed. |
| MediaTek | Yes (See comments.) | Same view as HW |
| Samsung | See comments | This depends on answer to Q1A. But even if we only have one Remote UE per Relay UE, given LCID space limitations on the Uu link, some form of identification may be needed. |
| Nokia | Maybe | Rel-17 solution based on RLC channels to differentiate traffic of remote UE and relay UE should be re-used if possible |
| Xiaomi | Not needed | It’s technically feasible to differentiate data to relay and remote UE without UE identity.. |
| China Telecom | Not needed |  |

Summary and proposal

Most companies think that UE identification is not needed over Uu link, mostly because relay UE serves only one remote UE and different Uu RLC channels can be assumed for the remote UE and the relay UE. Two companies think that UE identification is needed over Uu link.

Based on the majority’s view:

Proposal 1C: UE identification is not needed over Uu link in Scenario 2, if relay UE serves only one remote UE (as in Proposal 1A) and/or different Uu RLC channels can be assumed for the remote UE and the relay UE (as in Proposal 5A).

Some companies propose to focus on 1:1 bearer mapping only over non-3GPP UE-to-UE link and 3GPP Uu link for less impact on adaptation functionality.

Question 2A: Do you think that RAN2 should restrict to 1:1 bearer mapping only over non-3GPP UE-to-UE link and 3GPP Uu link?

| Company | Yes/No | Comment |
| --- | --- | --- |
| LG Electronics | No strong view |  |
| Apple | No | 1:1 bearer mapping seems assume that relay UE does not have its own Uu traffic, so all remote UE E2E bRBs can one-to-one map to Uu RB with a unique LCID. We think this assumption is too restrictive. We shall not build a design based on such a limited foundation. |
| Huawe, HiSilicon | Yes (See comments) | For non-3GPP interface, we still think this can be left to UE implementation, given that there may be no concept of bearer at all on the non-3GPP interface. For Uu interface, if Q1A can be agreed, 1:1 mapping is simpler considering the existing RLC bearer number should be sufficient for the total number of relay’s own and only one remote UE’s E2E bearer. |
| ASUSTeK | No | Suggest to support m:1 bearer mapping so as to align with U2N relay in R17. |
| CMCC | it can be left to UE implementation |  |
| vivo | Yes | This is the most typical and simplest case. |
| Intel | Yes | For the baseline version i.e. in this first release of UE aggregation, we can consider restriction to 1:1 mapping. In any case, for non-3GPP link, we cannot do any optimization and it has to be left to UE implementation. |
| Spreadtrum | See comments | it can be left to UE implementation |
| ZTE | Yes | 1:1 bearer mapping works. Actually it can be up to implementation without specification. |
| CATT | Yes |  |
| Lenovo | Yes | We can simplify the mechanism. |
| Ericsson | comments | As other companies indicated, this can be just left to UE implementation. |
| Sharp | No | It is left to UE implementation. |
| MediaTek | No strong view |  |
| Samsung | No | Seems too restrictive for the Uu interface. For the non-3GPP link, this can be left to implementation. |
| Nokia | See comments | RAN2 cannot assume any bearer model over the non-3GPP link. The only thing that we may assume is a 1:1 mapping between Uu RLC channels of Relay UE and Remote UE E2E bearers. |
| Xiaomi | Yes | It’s up to use case. Seems current use case only requires one-to-one mapping. |
| China Telecom | No strong view |  |

Summary and proposal

8 companies think that RAN2 should restrict to 1:1 bearer mapping over Uu link, while 4 companies think that this assumption is too restrictive. Meanwhile, among 15 companies, 8 companies think that it can be left to UE implementation over non-3GPP link.

Proposal 2A: RAN2 is requested to discuss whether to restrict to 1:1 bearer mapping especially over Uu link for data from/to remote UE in scenario 2.

Some companies think that with restriction to 1:1 bearer mapping only over non-3GPP UE-to-UE link and 3GPP Uu link, the adaptation layer is not needed for Scenario 2.

Question 2B: Do you think that bearer identification is not needed over UE-to-UE link, with restriction to 1:1 bearer mapping only over non-3GPP UE-to-UE link and 3GPP Uu link, regardless of whether or not the adaptation layer is supported?

| Company | Needed or not | Comment |
| --- | --- | --- |
| LG Electronics | Needed | Without bearer identification, it is unclear how UEs can ensure mapping a PDCP entity of the remote UE to a RLC entity of the relay UE to deliver a PDCP PDU to an intended PDCP entity or RLC entity for support of more than one RB. |
| Apple | Needed | Same view as LG |
| Huawei, HiSilicon | Not needed | Similar answer to Q1B, in general, we understand UE implementation can handle any differentiation on non-3GPP interface for remote UE id or E2E bearer id. Adaptation layer could be one way of UE implementation, but other ways are also possible. for instance, if the non-3GPP is over fiber over IP, the bearer id can be carried in ip header like DSCP, and if the non-3GPP connection is over wifi the LWAAP can be used by UE implementation. To be brief, in RAN2 we do not need to discuss/specify bearer id for non-3GPP interface. |
| ASUSTeK | See comments | Bearer identification over UE-to-UE link could be left to UE implementation. |
| CMCC | we prefer to left it to the UE implementation |  |
| vivo | Not needed | It can be completely left to UE implementation for UE-to-UE operation. |
| Intel | Not needed | We think that it can be left to UE implementation to handle the bearer mapping. The gNB will provide configuration to the Remote UE and Relay UE to map the Remote UE’s Uu bearer to a Uu RLC channel at the Relay UE. Then it is up to the Relay UE’s implementation to handle the mapping. We do not think we should spend special effort to support this for scenario 2 in this release. |
| Spreadtrum | Not needed | it can be left to UE implementation |
| ZTE | Not needed | If we can assume that 1:1 bearer mapping is used and the UE aggregation support is restricted to the UEs from same vendor, it is not necessary to support the adapt layer. |
| CATT | Not needed |  |
| Lenovo | Not needed | The relay UE can identify the mapping between ingress and egress based on the gNB configuration in the case of 1:1 bearer mapping. |
| Ericsson | No | For UE to UE link, It can be fully left for UE implementation. |
| Sharp | Needed | Same view as LG |
| MediaTek | Left it to the UE implementation | RAN2 don't discuss/specify bearer id for non-3GPP interface, left it to the UE implementation |
| Samsung | See comments | Same view as CMCC and vivo for the UE-to-UE link. Also – this seems to repeat previous questions… |
| Nokia | See comment | Bearer identification over the non-3GPP interface can be left to UE implementation |
| Xiaomi | Not needed |  |
| China Telecom | Left it to the UE implementation |  |

Summary and proposal

8 companies think that bearer identification is not needed over UE-to-UE link while 3 companies think that bearer identification is needed over UE-to-UE link. Meanwhile, among 15 companies, 10 companies think that it can be left to UE implementation over non-3GPP link.

Proposal 2B: RAN2 is requested to discuss whether to support bearer identification over UE-to-UE link.

Question 2C: Do you think that bearer identification is not needed over Uu link (except LCID in MAC header), with restriction to 1:1 bearer mapping only over non-3GPP UE-to-UE link and 3GPP Uu link, regardless of whether or not the adaptation layer is supported?

| Company | Needed or not | Comment |
| --- | --- | --- |
| LG Electronics | Maybe, not needed | LCID in MAC header could work for bearer identification. |
| LG | Needed | LCID space will not be big enough. |
| Huawei, HiSilicon | Not needed | We do not expect there would be a lot of E2E bearer needing relay UE’s help, so the existing maximum number of LCID should be enough assuming relay only helps one remote UE. |
| ASUSTeK | Needed |  |
| CMCC | Not needed | For link between relay UE and gNB in scenario 2, we understand, it may be a work assumption that 1:1 bearer mapping between remote UE’s RB and Uu RLC channel of relay UE are configured. That is, the granularity of the aggregation is the DRB, which will be configured one common PDCP for involved UEs and RLC respectively, rather than the UE. During discussion, it seems some companies misunderstood that multiple aggregated DRBs in one UE will be configured only one common PDCP for involved UEs. That is different from scenario 1. The adaption layer is not needed to distinguish RB or remote UE for scenario 2. |
| vivo | Not needed | As the above mentioned, the bearer mapping between relay UE RLC channel and remote UE E2E RB will be always 1-to-1. Hence, bearer identification is not needed. |
| Intel | Not needed | If 1:1 bearer mapping is supported, then, we have the same view as Huawei that existing maximum number is sufficient considering the semi-static relation between Remote UE and Relay UE for UE aggregation. We can handle the mapping related configuration in RRC layer. |
| Spreadtrum | Not needed |  |
| ZTE | Not needed | If 1:1 bearer mapping is assumed, bearer identification is not needed. |
| CATT | Note needed |  |
| Lenovo | Not needed |  |
| Ericsson | comments | For the Uu interface, RAN2 only needs to answer the question whether to have an adaptation layer, as we said we have no strong view. But if it is decided to have the adaptation layer, it is sufficient to fully reuse the SRAP in R17, no needs to enhance it by excluding the RB ID. The RB ID may be still useful for future extension. |
| Sharp | See comments | If we decide it “regardless of whether or not the adaptation layer is supported,” it may not be needed. But if adaptation layer is supported, bearer ID is needed. |
| MediaTek | Not needed | If 1:1 bearer mapping is assumed. |
| Samsung | Needed | Please see response to Q1C. |
| Nokia | Not needed |  |
| Xiaomi | Not needed | If one Uu RLC carries either remote UE’s bearer or relay UE’s bearer without mix, bearer ID may not needed. |
| China Telecom | Not needed |  |

Summary and proposal

13 companies think that bearer identification is not needed over Uu link while 3 companies think that bearer identification is needed over Uu link. Based on the comments, bearer identification seems not needed over Uu link, especially when 1:1 bearer mapping is supported over Uu link for the indirect path of the remote UE.

Based on majority’s view:

Proposal 2C: Bearer identification except LCID is not needed in L2 PDU over Uu link in Scenario 2. 1:1 bearer mapping is supported over Uu link for the indirect path.

Some companies think that we need to ensure that a PDCP PDU is delivered to an intended PDCP entity or RLC entity for support of more than one RB based on proper mapping a PDCP entity of the remote UE to a RLC entity of the relay UE e.g. by bearer identification in the adaptation layer.

Question 3A: Do you think that a PDCP PDU can be delivered to an intended PDCP entity or RLC entity for support of more than one RB without the adaptation layer over non-3GPP link.

| Company | Yes/No | Comment |
| --- | --- | --- |
| LG Electronics | No | Regardless of restriction to 1:1 bearer mapping, the adaptation layer is needed to add bearer ID field to each PDU. But, RAN2 should still need to clarify whether the adaptation layer is based on 3GPP standard or UE vendor specific implementation. |
| Apple | No | The bearer ID field is needed. It could be similar to the one used in SRAP |
| Huawei, HiSilicon | Not relevant to adaptation layer | We are not quite sure about the question. For UL at remote UE, it just needs to pass the data received from relay to the PDCP associated with an E2E bearer, as we discussed in Q2B, differentiation among E2E bearers is not an issue which can be left to UE implementation. For DL at remote UE, if needed, the RLC entity can be configured to be Uu interface or non-3GPP interface, it does not matter to SRAP is absent or present. |
| ASUSTeK | Yes | This could be ensured by UE implementation. |
| CMCC | it can be left to UE implementation |  |
| vivo | Yes | RB differentiation over non-3GPP link may be non-specified and up to UE implementation. |
| Intel | Yes | It will be up to gNB configuration to aid with the end-to-end bearer mapping and up to UE implementation to handle the UE-to-UE link appropriately. |
| Spreadtrum | Yes | it can be left to UE implementation |
| ZTE | Yes | For the PDCP PDU delivery to RLC entity at remote UE, it depends on whether direct, indirect or split bearer is configured. For the split bearer, it should be associated with two RLC entity and which RLC entity is used depends on the data split/duplication rules. From the relay UE perspective, suppose the UEs participating the aggregation are from the same vendor, how to deliver the received PDCP PDU to the Uu RLC channel at relay UE can be up to implementation. |
| CATT | Yes |  |
| Lenovo | See comment | Not clear about the question if the more than one bearer is associated with Uu bearer or U2U bearer. If multiple end-to-end bearers can be mapping to one U2U RLC bearer, bearer ID could be needed. But, we prefer to 1:1 mapping. |
| Ericsson | yes | Same as Huawei. We are also confused with the question. This interface can be fully up to UE implementation including SRAP like |
| Sharp | it can be left to UE implementation | SRAP header like information is needed. But it can be left to UE implementation. |
| MediaTek | No | Bearer ID is needed |
| Samsung | Yes | Same view as ASUSTeK. |
| Nokia | Yes | The bearer differentiation over the non-3GPP link should be left to UE implementation or a non-3GPP specification. |
| Xiaomi | Yes |  |

Summary and proposal

Most companies except 3 companies think PDCP PDU can be delivered to an intended PDCP entity or RLC entity for support of more than one RB over UE-to-UE link, mostly based on UE implementation.

Proposal 3A: RAN2 assumes that without the adaptation layer over non-3GPP link, a PDCP PDU can be delivered to an intended PDCP entity or RLC entity for support of more than one RB over UE-to-UE link based on UE implementation.

Question 3B: Do you think that a PDCP PDU can be delivered to an intended PDCP entity or RLC entity for support of more than one RB without the adaptation layer over Uu link.

| Company | Yes/No | Comment |
| --- | --- | --- |
| LG Electronics | Maybe, yes | Yes, with restriction to 1:1 bearer mapping and separate LCID spaces for data from/to remote UE and relay UE. |
| Apple | No | We think there is no enough LCID space to separately support both local traffic and E2E Uu SRB/DRBs. |
| Huawei, HiSilicon | Not relevant to adaptation layer | We are not quite sure about the question. From relay perspective, it just forwards the data received from remote UE to corresponding Uu the RLC channel on UL. From network perspective, it’s NW implementation which path to use on DL for data split over split bearer, and both path will be used for data duplication. |
| ASUSTeK | No |  |
| CMCC | Yes | For link between relay UE and gNB in scenario 2, we understand, it may be a work assumption that 1:1 bearer mapping between remote UE’s RB and Uu RLC channel of relay UE are configured. That is, the granularity of the aggregation is the DRB, which will be configured one common PDCP for involved UEs and RLC respectively, rather than the UE. During discussion, it seems some companies misunderstood that multiple aggregated DRBs in one UE will be configured only one common PDCP for involved Ues. That is different from scenario 1. The adaption layer is not needed to distinguish RB or remote UE for scenario 2. |
| Vivo | Yes | NW configures 1-to-1 mapping between relay UE Uu RLC channel and remote UE E2E RB to the relay UE. Via relay UE’s LCID in MAC PDU, relay UE can map each packet to the proper remote UE’s PDCP entity |
| Intel | Potentially Yes | We think that it is achievable based on network configuration, and this can be handled in RRC. |
| Spreadtrum | Yes |  |
| ZTE | Yes | From the relay UE perspective, suppose the Ues participating the aggregation are from the same vendor, how to deliver the received PDCP PDU to the Uu RLC channel at relay UE can be up to implementation. Then assume 1:1 mapping is configured at the Uu link, the gNB should be aware of the association between remote UE’s RB and Uu RLC channel and then deliver the received PDCP PDU to the corresponding PDCP entity. |
| CATT | Yes | With proper configuration, there is no issues foreseen. |
| Lenovo | See comments | See above. |
| Ericsson | Yes | If we follow R17 rule, relay UE local traffic and relay traffic are mapped to different Uu RLC channel, the answer is yes. |
| Sharp | No | From gNB’s point of view, architecture of scenario 1 and 2 should be same as much as possible. |
| MediaTek | Not relevant to adaptation layer | Agree with Huawei |
| Samsung | No | Please see our previous response. In short – we agree with Apple on this one. |
| Nokia | Yes, but | We think that this is possible with 1: 1 bearer mapping with RLC channels but using SRAP over Uu may make easier to re-use Rel-17, and scenario 1 solution. |
| Xiaomi | Yes | In case one-to-one bearer mapping and without mix of remote and relay UE’s bearer in one RLC entity, it’s feasible. |
| China Telecom | Yes |  |

Summary and proposal

11 companies think that a PDCP PDU can be delivered to an intended PDCP entity or RLC entity for support of more than one RB without the adaptation layer over Uu link while 4 companies do not think so e.g. due to LCID space limitation. 2 companies think that this question is not relevant to adaptation layer. One company think that using SRAP over Uu is an easier solution.

Some companies think that based on gNB can properly configure 1:1 bearer mapping and different Uu RLC channels for relay UE local traffic and relay traffic for PDU delivery. The rapporteur observed that a majority of companies would think that gNB already has means to ensure proper delivery of PDUs.

Proposal 3B: Without the adaptation layer over Uu link, a PDCP PDU can be delivered to an intended PDCP entity or RLC entity for support of more than one RB over Uu link e.g. by configuring 1:1 bearer mapping and different Uu RLC channels for relay UE local traffic and relay traffic for PDU delivery.

Some companies think that interoperability between two UEs from different vendors needs to be supported.

Question 4A: Do you think that interoperability between two UEs from different vendors needs to be supported?

| Company | Yes/No | Comment |
| --- | --- | --- |
| LG Electronics | Yes | Interoperability between two UEs is considered high-level 3GPP requirement. Any technology without interoperability seems not subject to 3GPP standardization. |
| Apple | Yes | Same view as LG |
| Huawei, HiSilicon | No | We are confused with this interoperability cross different vendors. what is the non-3GPP interface is not specified, if the remote UE and relay UE are not aware of the physical form of the interface, how to avoid the effort of UE implementation even with adaptation layer. |
| ASUSTeK | No | Since the UE-UE inter-connection is non-standardized, it doesn’t seem necessary to support interoperability between two UEs from different vendors. In our opinion, this seems to be out of 3GPP standardization scope. |
| CMCC | At least not support in this release |  |
| vivo | Comments | In the WID, it is clearly declared that inter-UE operation for the ideal-backhaul case should not be specified. |
| Intel | No | Since we are not sure of the non-3GPP interface, we cannot guarantee support based on this interface. |
| Spreadtrum | No |  |
| ZTE | See comments | Actually it depends on the operator’s requirements. If operator only have interest with the UE aggregation support of UEs from the same vendor, the interoperability is not necessary to be considered. |
| CATT | No | The interface is non-3GPP interface, hence it is hard to consider interoperability issue. |
| Lenovo | No |  |
| Ericsson | No | Agree with CMCC and CATT, we don’t see the need to consider interoperability issue in this release. |
| Sharp |  | Same view as vivo |
| MediaTek | No |  |
| Samsung | Please see comments | The question is unclear to us. The UE-to-UE connection is beyond 3GPP scope. |
| Nokia | No | The interoperability over a non-3GPP link is out of scope of 3GPP |
| Xiaomi | No |  |
| China Telecom | No |  |

Summary and proposal

Most companies except two companies think that interoperability between two UEs from different vendors does not need to be supported.

Proposal 4A: Interoperability between two UEs from different vendors is not supported for scenario 2 in this release.

Question 4B: Do you think that interoperability between two UEs from different vendors can be supported over non-3GPP link without 3GPP standard work on adaptation layer?

| Company | Yes/No | Comment |
| --- | --- | --- |
| LG Electronics | No | Two UEs from different vendors need to understand different bearer IDs between two UEs based on the adaptation layer specified by 3GPP. |
| Apple | No | If nothing is defined on top of non-3GPP link by 3GPP, 3GPP cannot claim inter-vendor inter-operability. |
| Huawei, HiSilicon | See comments | Our question is even with the adaptation layer, how to support this interoperability cross different vendors as the remote UE and relay UE are not aware of the physical form of the non-3GPP interface. |
| ASUSTeK | See comments | Even if adaptation layer is supported over non-3GPP link, the lower layers below the adaptation layer is still non-standardized. In this situation, interoperability between two UEs from different vendors still cannot be ensured. |
| CMCC | See comments | At least not support in this release |
| vivo | Maybe yes | Up to implementation, if needed. |
| Intel | See comment | Same comment as above + we cannot do optimization to support interoperability over non-3GPP link as we do not know what this link will be. |
| Spreadtrum | See comments | At least not support in this release |
| ZTE | No |  |
| CATT | No | The interface is non-3GPP interface, hence it is hard to consider interoperability issue. |
| Lenovo | No |  |
| Ericsson | No | Agree with CMCC, in the first release to introduce UE aggregation, no need to consider interoperability to simplify the design. |
| Sharp | Maybe yes but | If interoperability can be supported, it should be achieved by two UEs with outside of 3GPP standardization work. |
| MediaTek | No |  |
| Samsung | See comments | Same view as ASUSTeK. |
| Nokia | See comments | Other SDO may create specification for this link, but the interoperability over a non-3GPP link is out of scope of 3GPP |
| Xiaomi | No |  |
| China Telecom | No |  |

9 companies think that interoperability between two UEs from different vendors cannot be supported over non-3GPP link without 3GPP standard work on adaptation layer. However, considering proposal 4A, we do not need to make any conclusion on question 4B.

Observation 4B: No proposal is needed for question 4B if proposal 4A is agreed.

Question 4C: Do you think that interoperability between two UEs from different vendors can be supported over Uu link without 3GPP standard work on adaptation layer?

| Company | Yes/No | Comment |
| --- | --- | --- |
| LG Electronics | Maybe, yes | Yes, with restriction to 1:1 bearer mapping and separate LCID spaces for data from/to remote UE and relay UE. Separate LCID spaces need to be configured by 3GPP spec. |
| Apple | See comment | I think the interoperability issue is only about non-3GPP link. Uu link is always fully defined by 3GPP. |
| Huawei, HiSilicon | See comments | Same view as Apple. |
| ASUSTeK | See comment | Share the same view as Apple. |
| CMCC | See comments | At least not support in this release. |
| vivo | Yes | Based on 1-to-1 bearer mapping configuration. |
| Intel | Yes | For the Uu link it will be supported as defined by 3GPP |
| Spreadtrum | See comments | At least not support in this release. |
| ZTE | Yes | Interoperability only concerns with the UE-UE interface. It is not relevant with Uu link. |
| CATT | See comments | Same view as Apple. |
| Ericsson | comments | Agree with Apple and CMCC. |
| Sharp | comments | Same view as Apple. |
| MediaTek | See comments | Same view as Apple |
| Samsung | See comment | Same view as Apple and others. |
| Nokia | Comments | We do not understand the question. As commented above Uu is fully defined by 3GPP. |
| Xiaomi | Yes | Interoperability doesn’t depend on adaptation layer on Uu. |

Summary and proposal

Most companies think that interoperability issue is only about non-3GPP link.

Observation 4C: Interoperability issue is only about non-3GPP link.

Some companies think that the adaptation layer over Uu link needs to be supported for identification of data own by the relay UE and data relayed from/to the remote UE over the Uu link.

Question 5A: Do you think that RAN2 should support identification of data own by the relay UE and data relayed from/to the remote UE over the Uu link?

| Company | Yes/No | Comment |
| --- | --- | --- |
| LG Electronics | Yes |  |
| Apple | Yes |  |
| Huawei, HiSilicon | See comments | How to differentiate remote UE and relay UE’s data at Uu hop is not a new issue. In Rel-17, RAN2 agreed to use different Uu RLC channels for that purpose. |
| ASUSTeK | Yes |  |
| CMCC | With comments | Same solution in R17 SL relay can be reused. It not necessary that issue here. |
| vivo | Yes | Based on NW configuration, different LCIDs are allocated to relay UE’ own data and remote UE’ data. |
| Intel | Yes | Same comment as Huawei, HiSilicon. |
| Spreadtrum | Yes | Different LCIDs can used to identify the data own by the relay UE and data relayed from/to the remote UE over the Uu link |
| ZTE | Yes | This can be supported via legacy mechanism, i.e. the data from remote UE and relay UE are delivered via different RLC channels. |
| CATT | See comment | Same concern as HW. |
| Lenovo | Yes |  |
| Ericsson | Yes | Agree with Huawei comments. This is already supported to use different Uu RLC channels for that purpose. |
| Sharp | Yes |  |
| MediaTek | Yes |  |
| Samsung | See comments | Same view as Huawei. |
| Nokia | Yes, Comments | If the Rel-17 solution is followed, then Uu RLC channels are used to differentiate the data of relay and remote UEs. |
| Xiaomi | No | In case one-to-one bearer mapping and without mix of remote and relay UE’s bearer in one RLC entity, additional ID may not be needed. |
| China Telecom | Yes | Agree with HW. |

Summary and proposal

Most companies think that RAN2 should support identification of data own by the relay UE and data relayed from/to the remote UE over the Uu link. In addition, some companies think that as in Rel-17, different Uu RLC channels can be configured for identification of data own by the relay UE and data relayed from/to the remote UE over the Uu link. The rapporteur thinks that this Rel-17 rule can be applied to both MP scenarios.

Proposal 5A: For Scenario 2 (as well as Scenario 1), different Uu RLC channels are configured for identification of data own by the relay UE and data relayed from/to the remote UE over the Uu link of the indirect path, as in Rel-17.

Question 5B: Do you think that adaptation layer over Uu link needs to be supported for identification of data own by the relay UE and data relayed from/to the remote UE over the Uu link?

| Company | Yes/No | Comment |
| --- | --- | --- |
| LG Electronics | Maybe, no | No, with restriction to 1:1 bearer mapping and separate LCID spaces for data from/to remote UE and relay UE. Separate LCID spaces need to be configured by 3GPP spec. |
| Apple | Yes | As explained in Q3B, 1:1 mapping is not a sufficient solution. |
| Huawei, HiSilicon | No | As answered to Q5A, Furthermore the relay UE’s own data is terminated between relay and NW, it should not go through adaptation layer. |
| ASUSTeK | No | In R17 U2N relay, remote UE traffic and relay UE own traffic are separated in different Uu RLC bearers in Uu hop. We think the same concept can be reused and thus the adaptation layer does not need to take care of this. |
| CMCC | No | Just reuse the solution in R17. |
| vivo | No | Current LCID in MAC header is enough. |
| Intel | No | With 1:1 mapping to different Uu RLC channels, we think that it is not needed. |
| Spreadtrum | No |  |
| ZTE | No |  |
| CATT | No |  |
| Lenovo | No |  |
| Ericsson | No |  |
| Sharp | No | Not for identification of data. Solution of R17 SL relay can be reused. |
| MediaTek | No |  |
| Samsung | No | Not for identification of data. |
| Nokia | No, but | We think that this is possible but using SRAP over Uu may make easier to re-use Rel-17, and scenario 1 solution |
| Xiaomi | No |  |
| China Telecom | No |  |

Summary and proposal

Most companies except one company think that adaptation layer over Uu link is not needed for identification of data own by the relay UE and data relayed from/to the remote UE over the Uu link.

Proposal 5B: Adaptation layer over Uu link is not needed for identification of data own by the relay UE and data relayed from/to the remote UE over the Uu link.

### Feasibility of support of SRAP in scenario 2

As captured in the report of [Post119-e][408][Relay] Path operations in multi-path relaying in R2-2210027, if adaptation layer is supported for Scenario 2, the majority of companies think that SRAP is considered as baseline for design of the adaptation layer for scenario 2.

However, if adaptation layer is supported for Scenario 2, regardless of whether SRAP is considered as baseline for design of the adaptation layer, it is still unclear whether the existing SRAP specification can be used for specification of the adaptation layer in Scenario 2 or new specification needs to be created for the adaptation layer.

Question 6A: If adaptation layer is supported for Scenario 2, which option do you prefer for specification of the adaptation layer in Scenario 2 over UE-to-UE link, regardless of whether SRAP is considered as baseline for design of the adaptation layer?

- Option 1: The existing SRAP specification for UE-to-UE link

- Option 2: New adaptation layer specification for UE-to-UE link

| Company | Option | Comment |
| --- | --- | --- |
| LG Electronics | 2 | We think that option 2 will look simpler than option 1 in a spec. New spec can be simply based on some SARP specification texts, if not specific to PC5. The adaptation layer does not need to differentiate SL data and non-3GPP data. Any SL specific procedural text in 38.351 cannot be applied to non-3GPP data. Assuming that UE ID is not needed, new data PDU format needs to be considered. |
| Apple | 1 | We can use R17 SRAP as baseline and consider some change/enhancements. There is no need to create a new spec other than 38.351, |
| OPPO | 1 | Same view as Apple |
| Huawei, HiSilicon | See comments | The issue is the existing SRAP is not fully compatible with scenario 2. The UE id may not be needed, the function of determine egree PC5 relay channel is not needed. The length of bearer ID is not enough. So we still think no adaptation layer could be the easy way for this non-3GPP interface. |
| ASUSTeK | 1 |  |
| CMCC | we prefer to left it to the UE implementation |  |
| vivo | None | Prefer None, acceptable for Option 2 |
| Intel | None | We don’t think that existing SRAP can be supported over the non-3GPP as it can be any interface (for e.g. it is not clear how it can be configured as PC5 RLC channel). |
| Spreadtrum | None |  |
| ZTE | 1 | Agree with OPPO. |
| CATT | See comments | We prefer do not involve introducing SRAP is better. |
| Lenovo | None |  |
| Ericsson | none | It is challenging/unnecessary to have the adaption layer over the non-3GPP interface to cover all possible physical forms. We just fully left for UE implementation. |
| Sharp | comment | If adaptation layer is exist, option 1 can be reused and RAN2 can reduce unnecessary signalling from it. |
| MediaTek | 1 |  |
| Samsung | Neither | Same view as CMCC. |
| Nokia | Comments | We do not think that an adaptation can be specified over the non-3GPP interface, e.g., RAN2 cannot know if the implementation of an adaptation layer is feasible over the non-3GPP link. |
| Qualcomm | See comments | If the restrictions from Q1-Q4 can be confirmed, then it is possible to leave to UE implementation. |
| China Unicom | See comments | UE implementation solutions that do not involve SRAP have some benefits in terms of reduced specification impact and complexity. |
| China Telecom | Left it to the UE implementation |  |

Summary and proposal

6 companies think that if needed, the existing SRAP specification can be reused for UE-to-UE link. 5 companies think that if needed, the existing SRAP is not fully compatible with scenario 2. Meanwhile, 10 companies may think that adaptation layer is not needed for Scenario 2.

Observation 6A: There is no consensus on how to specify adaptation layer over non-3GPP link, if adaptation layer over non-3GPP link is needed.

Question 6B: If adaptation layer is supported for Scenario 2, which option do you prefer for specification of the adaptation layer in Scenario 2 over Uu link, regardless of whether SRAP is considered as baseline for design of the adaptation layer?

- Option 1: The existing SRAP specification for Uu link

- Option 2: New adaptation layer specification for Uu link

| Company | Option | Comment |
| --- | --- | --- |
| LG Electronics | 2 | We think that option 2 will look simpler than option 1 in a spec. New spec can be simply based on some SARP specification texts, if not specific to PC5. The adaptation layer does not need to differentiate SL data and non-3GPP data. Any SL specific procedural text in 38.351 cannot be applied to non-3GPP data. Assuming that UE ID is not needed, new data PDU format needs to be considered. |
| Apple | 1 | Same reason as answered in Q6A |
| OPPO | 1 | Same view as Apple |
| Huawei, HiSilicon | 1 |  |
| ASUSTeK | 1 |  |
| CMCC | NULL | We think adaption layer is not need for Uu link. |
| vivo | None | Prefer None, acceptable for Option 2 |
| Intel | None | If 1:1 mapping is adopted, we don’t think there is any need of adaptation layer support for scenario 2. If majority of the companies prefer, we can go with option 1 for Uu link. |
| Spreadtrum | Null | Adaption layer is not need for Uu link |
| ZTE | 1 |  |
| CATT | See comments | We prefer do not involve introducing SRAP is better. |
| Lenovo | None | Option1 is also fine. |
| Ericsson | 1 | As we mentioned, we are open to discuss the need for the SRAP in the Uu link. Any enhancement should be avoided given limited time in R18. If RAN2 agrees to support the adaption layer in the Uu interface, the SRAP protocol in Rel17 should be sufficient. |
| Sharp | 1 |  |
| MediaTek | 1 |  |
| Samsung | 1 |  |
| Nokia | 1, comments | We should try to re-use Rel-17, and the solution for scenario 1 |
| Qualcomm | See comments | Whether to existing or new SRAP on Uu link depends on the restrictions from Q1 to Q4, suggest to postpone this. |
| China Unicom | None | Prefer None. |
| China Telecom | None |  |

Summary and proposal

10 companies think that if needed, the existing SRAP specification can be reused for Uu link. 8 companies think that adaption layer is not need for Uu link. One company propose to postpone this discussion.

Observation 6B: There is no consensus on how to specify adaptation layer over Uu link, if adaptation layer over Uu link is needed.

### Potential compromise for adaptation layer in scenario 2

For compromise, RAN2 discussed configurable adaptation layer and/or optional support of adaptation layer.

Question 7A: If adaptation layer is supported, do you think that the adaptation layer over UE-to-UE link can be configurable by the network?

| Company | Configurable or not | Comment |
| --- | --- | --- |
| LG Electronics | Not configurable, but OK with configurable layer. | We think that support of configurable adaptation layer requires more effort for standardization. But, if a majority of companies support configurable adaptation layer, we are fine to support it for compromise. |
| Apple | OK with configurable |  |
| OPPO | OK with configurable |  |
| Huawei, HiSilicon | No | It is quite strange have a configurable 3GPP layer on a “non-3GPP” interface. |
| ASUSTeK | No | Share the same view as Huawei. |
| CMCC | we prefer to left it to the UE implementation |  |
| vivo | NOT configurable |  |
| Intel | No | We don’t think we can agree/converge on adaptation layer over non-3GPP link. |
| Spreadtrum | No |  |
| ZTE | OK with configurable |  |
| CATT | No | Same view as HW. |
| Lenovo | No |  |
| Ericsson | No | Agree with Huawei and CMCC |
| Sharp | we prefer to left it to the UE implementation | NW can provide configuration but the implementation is up to UE. |
| MediaTek | OK with configurable |  |
| Samsung | See comments | Same view as CMCC. |
| Nokia | No | Impacting the network due to a non-3GPP interface is not acceptable |
| Xiaomi | Not configurable | How can NW decide adaptation is needed or not on non-3GPP interface? |
| Qualcomm | See comments | If the restrictions from Q1-Q4 can be confirmed, then it is possible to leave to UE implementation. |
| China Unicom | No | It can based on UE implementation. |
| China Telecom | Left it to the UE implementation |  |

Summary and proposal

4 companies think that the adaptation layer over UE-to-UE link can be configurable while 12 companies think that the adaptation layer over UE-to-UE link cannot be configurable. Meanwhile, 6 companies think that the adaptation layer over UE-to-UE link can be based on UE implementation.

Proposal 7A: If the adaptation layer over UE-to-UE link is needed, the adaptation layer over UE-to-UE link is not configurable.

Question 7B: If adaptation layer is supported, do you think that the adaptation layer over Uu link can be configurable by the network?

| Company | Configurable or not | Comment |
| --- | --- | --- |
| LG Electronics | Not configurable. | We think that support of configurable adaptation layer requires more effort for standardization. Thus, if adaptation layer is supported, it should not be configurable. |
| Apple | Not configurable | It shall be always used, as similar to R17 SL relay. |
| OPPO | Can follow majority view |  |
| Huawei, HiSilicon | Not configurable | If configurable means optional UE capability, we are not keen about defining many UE types, which means double specification effort and network implementation effort to accommodate all the UE types, if not necessary. In Rel-17, configurable PC5 adaptation layer was avoided due to such considerations. |
| ASUSTeK | No |  |
| CMCC | configurable can be accepted as compromise |  |
| vivo | Configurable |  |
| Intel | Not configurable | If it is to be supported, we can follow R17 as baseline. |
| Spreadtrum | Configurable |  |
| ZTE | OK with configurable |  |
| CATT | No | Same view as HW. |
| Lenovo | No | ‘configurable’ makes system complicated. |
| Ericsson | comments | Although we have the same concern as Huawei, but we can accept configurability as a compromise way if there is majority view |
| Sharp | Not configurable | Same view as Apple. |
| MediaTek | Can follow majority view |  |
| Samsung | Not configurable | Same view as Apple and LG. |
| Nokia | No | Unnecessary complexity |
| Xiaomi | Not configurable |  |
| China Uniocm | Not configurable | Same view as HW. |
| China Telecom | Not configurable |  |

Summary and proposal

2 companies think that the adaptation layer over Uu link can be configurable while 12 companies think that the adaptation layer over Uu link is not configurable. 2 companies think that the configurable adaptation layer over Uu link is acceptable for compromise.

Proposal 7B: If the adaptation layer over Uu link is needed, the adaptation layer over Uu link is not configurable.

Some companies think that if configurable adaptation layer is supported, UE needs to report its capability of the adaptation layer support to gNB. Namely, support of the adaptation layer can be optional feature for Scenario 2. Moreover, there is a view in that adaptation layer can be optional feature for Scenario 2, regardless of whether adaptation layer is configurable or not.

Question 8A: If adaptation layer is supported, do you think that the adaptation layer over UE-to-UE link can be optional feature in UE capability for UE supporting Scenario 2?

| Company | Optional or not | Comment |
| --- | --- | --- |
| LG Electronics | Not optional for UE supporting Scenario 2 | We think that optionality of adaptation layer requires more effort for standardization. But, if a majority of companies support configurable adaptation layer, we are fine to support optionality of adaptation layer for compromise. |
| Apple |  | Too early to decide. Depends on which kind of Non-3GPP link the UE has. |
| OPPO | If configurability is confirmed, UE capability is needed anyway | If we allow configurability, we understand it means 3GPP confirms there are two UE implementations Implementation-1: the bearer ID identification is by UE implementation, w/o a need a SRAP Implementation-2: the bearer ID identification is by the support of SRAP These two implementation are for the same reason, so we see no reason to mandate the UE to support both. Thus UE capability is needed to indicate whether UE support A/ implementation-1 only B/ implementation-2 only C/ Both |
| Huawei, HiSilicon | No | Same view as LG that optionality of adaptation layer requires more effort for standardization as well as for NW implementation. |
| ASUSTeK | No | Share the same view as Huawei. |
| CMCC | left it to the UE implementation |  |
| vivo | No | Since in Q7A, we think the adaptation layer over UE-to-UE link is NOT configurable, the corresponding UE capability is not needed. |
| Intel | No | Over UE-to-UE link, it can be left to UE implementation. |
| Spreadtrum | No | It can be left to UE implementation |
| ZTE | Optional | If the adapt layer is configurable, it is natural that the adapt layer can be optional capability. |
| CATT | No | Same view as LG. |
| Lenovo | No |  |
| Ericsson | No | Agree with LG and Huawei |
| Sharp | left it to the UE implementation |  |
| MediaTek | If configurability is confirmed, UE capability is needed anyway | Agree with OPPO. |
| Samsung | See comments | Same view as CMCC |
| Nokia | No | Impacting the network due to the non-3GPP interface is not acceptable |
| Xiaomi | No |  |
| China Unicom | No | Share the same view with LG. From operator’s view, it’s not prefered to add more optionality to the NW. |
| China Telecom | Left it to the UE implementation |  |

Summary and proposal

12 companies think that the adaptation layer over UE-to-UE link cannot be optional while 3 companies think that the adaptation layer over UE-to-UE link can be optional, e.g. if configurability is confirmed. Meanwhile, 4 companies think that the adaptation layer over UE-to-UE link can be based on UE implementation.

Proposal 8A: If the adaptation layer over UE-to-UE link is needed, the adaptation layer over UE-to-UE link is not optional.

Question 8B: If adaptation layer is supported, do you think that the adaptation layer over Uu link can be optional feature in UE capability for UE supporting Scenario 2?

| Company | Optional or not | Comment |
| --- | --- | --- |
| LG Electronics | Not optional for UE supporting Scenario 2 | We think that optionality of adaptation layer requires more effort for standardization. Thus, if adaptation layer is supported, it should not be optional for Uu link. |
| Apple | Not optional |  |
| OPPO | If configurability is confirmed, UE capability is needed anyway | Similar to our answer to 8A above. |
| Huawei, HiSilicon | No | Same view as LG that optionality of adaptation layer requires more effort for standardization as well as for NW implementation. |
| ASUSTeK | No |  |
| CMCC | configurable can be accepted as compromise |  |
| vivo | Optional feature in UE capability |  |
| Intel | Not optional | Same view as LG. |
| Spreadtrum | configurable can be accepted as compromise |  |
| ZTE | optional | If the adapt layer is configurable, it is natural that the adapt layer can be optional capability. |
| CATT | No | Same view as LG. |
| Ericsson | comments | Although we concern the design complexity, but we are ok to accept the configurability as a compromised way if there is majority view. |
| Sharp | NOT optional |  |
| MediaTek | If configurability is confirmed, UE capability is needed anyway | Agree with OPPO |
| Samsung | Not optional |  |
| Nokia | No | Unnecessary complexity |
| Xiaomi | No |  |
| China Unicom | No | Share the same view with LG. |
| China Telecom | No |  |

Summary and proposal

12 companies think that the adaptation layer over Uu link cannot be optional while 3 companies think that the adaptation layer over Uu link can be optional, e.g. if configurability is confirmed. Additional 3 companies think that the adaptation layer over Uu link can be optional for compromise.

Proposal 8B: If the adaptation layer over Uu link is needed, the adaptation layer over UE-to-UE link is not optional.

Question 9A: Which option is your preferred one regarding the support of the adaptation layer over UE-to-UE link in scenario 2 (Do not change the below options. If you want to change, you can add a new option below the existing options)?

Option 1: Specify adaptation layer in the protocol stack for scenario 2 in RAN2 (but not configurable);

Option 2: Do not specify adaptation layer from the protocol stack for scenario 2 in RAN2 (whether/how to use adaptation layer is up to UE implementation);

Option 3: Adaptation layer in scenario 2 is specified and configurable, along with or without a UE capability for scenario 2;

Option 4: Any other option?

| Company | Yes/No | Comment |
| --- | --- | --- |
| Huawei, HiSilicon | For non-3GPP interface: option2 | For Uu inferface, it relates to the conclusion on configurability and UE capability. Our preference is if the layer is configurable, the UE supports adaptation layer is configured or not, and then option1 can be adopted. |
| LG Electronics | Option 1 | Option 3 is also fine based on a majority’s view. |
| ASUSTeK | Option 2 |  |
| CMCC | Option 2 |  |
| vivo | Option 2 |  |
| Apple | Option 3 (also fine with Option 1) |  |
| OPPO | See comment | Our understanding is option-3 is a way-out based on the situation here. |
| Intel | Option 2 |  |
| Spreadtrum | Option 2 |  |
| Huawei, HiSilicon | Option 2 |  |
| TCL | Option 2 |  |
| ZTE | Option 2 |  |
| CATT | Option 2 |  |
| Lenovo | Option 2 |  |
| Ericsson | Option 2 |  |
| Sharp | Option 2 |  |
| MediaTek | Option 3 |  |
| Samsung | Option 2 (for non-3GPP interface) |  |
| Nokia | Option 2 |  |
| Xiaomi | Option 2 |  |
| Qualcomm | See comments | If the restrictions from Q1-Q4 can be confirmed, then option 2 is feasible. |
| China Unicom | Option 2 |  |
| China Telecom | Option 2 |  |

Summary and proposal

18 (+1) companies prefer Option 2 while only a few companies prefer Option 1 or 3.

Based on a majority’s view:

Proposal 9A: Do not specify adaptation layer over UE-to-UE link for scenario 2 in RAN2 (whether/how to use adaptation layer is up to UE implementation).

Question 9B: Which option is your preferred one regarding the support of the adaptation layer over Uu link in scenario 2 (Do not change the below options. If you want to change, you can add a new option below the existing options)?

Option 1: Specify adaptation layer in the protocol stack for scenario 2 in RAN2 (but not configurable);

Option 2: Do not specify adaptation layer from the protocol stack for scenario 2 in RAN2 (whether/how to use adaptation layer is up to UE implementation);

Option 3: Adaptation layer in scenario 2 is specified and configurable, along with or without a UE capability for scenario 2;

Option 4: Any other option?

Option 5: Do not specify adaptation layer in the protocol stack for scenario 2 in RAN2.

| Company | Yes/No | Comment |
| --- | --- | --- |
| LG Electronics | No strong view |  |
| ASUSTeK | Option 1 | To align with U2N relay in R17 as possible so as to reduce standardization efforts. |
| CMCC | Option 5; but can accept option 3 as compromise |  |
| vivo | Option 2 | Can consider option 3 as compromise |
| Apple | Option 1 | We share the same view as ASUSTek |
| OPPO | See comment | Our understanding is option-3 is a way-out here. |
| Intel | Option 5, with comment | We think it is easier to not specify any adaptation layer for scenario 2 in this release especially if we intend to have semi-static relationship between Remote UE and Relay UE and with 1:1 bearer mapping, gNB configuration can handle the delivery of data appropriately. If 1:1 bearer mapping cannot agreed, then we prefer option 1. We are not sure of option 2 wording about whether/how to use adaptation layer is up to UE implementation. |
| Spreadtrum | Option 5 |  |
| Huawei, HiSilicon | Option 5 | Same view as Intel. |
| TCL | Option 5 | We agree with intel. |
| ZTE | Option 5 |  |
| CATT | Option 5, with comment | Same view as Intel. |
| Lenovo | Option 5 | We can follow other option if majority view is not option 5. |
| Ericsson | Option 5 or Option 1 | Option 3 is also acceptable, if there is majority view. |
| Sharp | Option 1 | Same view as ASUSTek |
| MediaTek | Option 3 |  |
| Samsung | Option 1 |  |
| Nokia | Option 1 (preferred) or Option 2/5 | We prefer to have solution for scenario 2 close to the solution for scenario 1 and having the adaptation layer may be more future proof if the support of multiple Remote UEs is introduced. However, we can accept that there is no SRAP over UU in this release. It is not clear for us what is difference between option 2 and 5, as our understanding is that in both options there is no SRAP over Uu. Option 3 is the worst option, as it is the most complex. |
| Xiaomi | Option 5 |  |
| Qualcomm | See comments | If the restrictions from Q1-Q4 can be confirmed, then option 5 is feasible. With or without or configurable SRAP can leave to normative work. |
| China Unicom | Option 5 |  |
| China Telecom | Option 5 |  |

Summary and proposal

12 companies prefer Option 5 and one company prefer Option 2 which seems similar to Option 5. Two companies can accept Option 5.

5 companies prefer Option 1 and one company is also fine with Option 1.

3 companies can accept Option 3 and one company prefer Option 3.

Based on a majority’s view:

Proposal 9B: Do not specify adaptation layer over Uu link for scenario 2 in RAN2.

# Conclusion and recommendation

In conclusion, Rapporteur propose to agree the following proposals:

Proposal 1A: The relay UE is restricted to serve only one remote UE in Scenario 2.

Proposal 5A: For Scenario 2 (as well as Scenario 1), different Uu RLC channels are configured for identification of data own by the relay UE and data relayed from/to the remote UE over the Uu link of the indirect path, as in Rel-17.

Proposal 9A: Do not specify adaptation layer over UE-to-UE link for scenario 2 in RAN2 (whether/how to use adaptation layer is up to UE implementation).

In addition to proposal 9A:

Proposal 3A: RAN2 assumes that without the adaptation layer over non-3GPP link, a PDCP PDU can be delivered to an intended PDCP entity or RLC entity for support of more than one RB over UE-to-UE link based on UE implementation.

Proposal 4A: Interoperability between two UEs from different vendors is not supported for scenario 2 in this release.

Proposal 1B: UE identification is not needed in L2 PDU over non-3GPP link in Scenario 2.

Proposal 9B: Do not specify adaptation layer over Uu link for scenario 2 in RAN2.

In addition to proposal 9B.

Proposal 1C: UE identification is not needed over Uu link in Scenario 2, if relay UE serves only one remote UE (as in Proposal 1A) and/or different Uu RLC channels can be assumed for the remote UE and the relay UE (as in Proposal 5A).

Proposal 2C: Bearer identification except LCID is not needed in L2 PDU over Uu link in Scenario 2. 1:1 bearer mapping is supported over Uu link for the indirect path.

Proposal 3B: Without the adaptation layer over Uu link, a PDCP PDU can be delivered to an intended PDCP entity or RLC entity for support of more than one RB over Uu link e.g. by configuring 1:1 bearer mapping and different Uu RLC channels for relay UE local traffic and relay traffic for PDU delivery.

If some of the above proposals are not agreed, RAN2 is requested to discuss the following proposals:

Proposal 2A: RAN2 is requested to discuss whether to restrict to 1:1 bearer mapping especially over Uu link for data from/to remote UE in scenario 2.

Proposal 2B: RAN2 is requested to discuss whether to support bearer identification over UE-to-UE link.

If support of the adaptation layer is considered over UE-to-UE link and/or Uu link, RAN2 is requested to agree the following proposals:

Proposal 5B: Adaptation layer over Uu link is not needed for identification of data own by the relay UE and data relayed from/to the remote UE over the Uu link.

Proposal 7A: If the adaptation layer over UE-to-UE link is needed, the adaptation layer over UE-to-UE link is not configurable.

Proposal 7B: If the adaptation layer over Uu link is needed, the adaptation layer over Uu link is not configurable.

Proposal 8A: If the adaptation layer over UE-to-UE link is needed, the adaptation layer over UE-to-UE link is not optional.

Proposal 8B: If the adaptation layer over Uu link is needed, the adaptation layer over UE-to-UE link is not optional.