3GPP TSG-RAN WG2 #119bis-e	R2-221xxxx

E-meeting, October 2022

Agenda Item:	8.15.2

Source:	OPPO

Title:	[Draft]Summary of [AT119bis-e][503][V2X/SL] CAPC (OPPO)

Document for:	Discussion, Decision

# Introduction

This document is a report on the following email discussion:

- [AT119bis-e][503][V2X/SL] CAPC (OPPO)

Scope: Discuss SL CAPC:

Q1: SL CAPC determination based on PQI or SL priority or any other?

Q2: For SL DRBs, is SL CAPC (pre)configurable or fixed?

Q3: For SL SRBs and SL MAC CEs, is SL CAPC (pre)configurable or fixed?

Q4: If SL CAPC is determined based on PQI, e.g. what should be criterion to make a mapping table? what is companies’ thinking on mapping table between CPAC and PQI?

Q5: If SL CAPC is determined based on PQI, how to handle non-standardized PQI?

Q6: How to SL CAPC when different SL LCHs, SL MAC CEs and SL SRBs are multiplexed?

Intended outcome: Discussion summary in R2-2210934.

Deadline: 10/13 10:00 (UTC).

- According to the scope of this offline discussion, the following issues will be discussed:

Q1: SL CAPC determination based on PQI or SL priority or any other?

Q2: For SL DRBs, is SL CAPC (pre)configurable or fixed?

Q3: For SL SRBs and SL MAC CEs, is SL CAPC (pre)configurable or fixed?

Q4: If SL CAPC is determined based on PQI, e.g. what should be criterion to make a mapping table? what is companies’ thinking on mapping table between CPAC and PQI?

Q5: If SL CAPC is determined based on PQI, how to handle non-standardized PQI?

Q6: How to SL CAPC when different SL LCHs, SL MAC CEs and SL SRBs are multiplexed?

# Contact Information

| Company | Name | Email Address |
| --- | --- | --- |
| OPPO | Bingxue Leng | lengbingxue@oppo.com |
| Apple | Peng Cheng | pcheng24@apple.com |
| MediaTek | Xuelong Wang | xuelong.wang@mediatek.com |
| Xiaomi | Li Zhao | zhaoli6@xiaomi.com |
| ZTE | Wei LUO | luo.wei11@zte.com.cn |
| Lenovo | Joachim Löhr | jlohr@lenovo.com |
| vivo | Jing Liang | liangjing@vivo.com |
| Qualcomm | Qing Li | qin.li@qti.qualcomm.com |
| Sharp | Chongming Zhang | Chongming.zhang@cn.sharp-world.com |
| Huawei, HiSilicon | Xiangyu Li | lixiangyu14@huawei.com |
| LG | Giwon Park | giwon.park@lge.com |
| NEC | Satoaki Hayashi | Satoaki-hayashi@nec.com |
| Ericsson | Min Wang | min.w.wang@ericsson.com |
| CATT | Jie Shi | shijie@catt.cn |
| Fraunhofer | Gustavo Costa | gustavo.wagner.oliveira.da.costa@iis.Fraunhofer.de |
| Nokia | Jakob Buthler | Jakob.buthler@nokia.com |
| InterDigital | Martino Freda | martino.freda@interdigital.com |

# Discussion

## Q1: SL CAPC determination based on PQI or SL priority or any other?

The following contributions have discussed the CAPC mapping determination, i.e., whether the mapping should be based on PQI or SL priority or any other (only restricted to RAN2 aspects):

| R2-2209385 | Proposal 2 R2 discusses mapping PQI 90/91/92/93/21/22/23/55/56/57/58 to CAPC priority class 1. Proposal 3 R2 discusses mapping PQI 59/61 to CAPC priority class 3. Proposal 4 R2 discusses mapping PQI 25 to CAPC priority class 2. Proposal 5 R2 discusses mapping PQI 24/26/60 to CAPC priority class 1 or CAPC priority class 2. | OPPO |
| --- | --- | --- |
| R2-2209521 | Proposal-1: PDB based CAPC split is used for SL-U for standardized PQI. | MediaTek Inc. |
| R2-2209737 | Proposal 2a: RAN2 is proposed to agree that for SL-U, a CAPC mapping to PQI shall be defined based on the standardized PQI to PC5 QoS characteristics mapping, similar to the existing mapping between CAPC and 5QI. | Intel Corporation |
| R2-2209761 | Proposal 3: For the support of CAPC for SL-U, RAN2 discuss the following 2 alternatives: - Alt-1: Introduce a new mapping table from PQI to CAPC, similar to Uu Table of mapping from 5QI to CAPC. FFS details of the table - Alt-2: Introduce a mapping from L1 priority to CAPC. FFS whether the mapping is fixed or configurable | Apple |
| R2-2209891 | Proposal 4: If CAPC mapping table is introduced, using PQI for CAPC definition. FFS for detail mapping between CAPC and PQI. | Lenovo |
| R2-2209973 | Proposal 5: RAN2 study and define the mapping table between CAPC and PQI in SL-U. | Spreadtrum Communications |
| R2-2210250 | Proposal 3 A mapping table between CAPC values and standardized PQI values need to be defined by RAN2. | Ericsson |
| R2-2210256 | Proposal 1: CAPC is determined at least from the priority of the logical channel(s) associated with the data transmission. FFS on how to handle MCR. Proposal 2: RAN2 further discusses whether SL factors other than QoS can affect the CAPC associated with data transmissions | InterDigital |
| R2-2210379 | Proposal 1: In SL-U, a table for mapping between PQI and CAPC, similar to Table 5.6.2-1 in 3GPP TS 38.300, shall be specified. FFS the detailed mapping between PQI and CAPC. | Xiaomi |
| R2-2210552 | [Proposal 1]: SL priority based SL CAPC determination is proposed. [Proposal 2]: Configurable mapping between SL CAPC and SL priority is proposed. | Samsung Research America |
| R2-2209465 | As in NR-U, a mapping table between CAPC and PQI can be specified at a stage-2 level as a guideline to gNB-configuration/pre-configuration. RAN2 to decide the specific mapping relationship after RAN1 concludes the specific channel access parameters per CAPC for SL. | vivo |
| R2-2209598 | Proposal 1: In SL-U, the table of the mapping between 5QI and CAPC in NR-U can be reused by replacing 5QI with PQI. | Huawei, HiSilicon |
| R2-2209678 | Proposal 1：RAN2 is suggested to discuss how to decide sidelink CAPC value. | ZTE Corporation, Sanechips |
| R2-2209742 | Proposal 2：For SL-U, the mapping between PQI and CAPC can be defined in specification. | CATT |
| R2-2210280 | Proposal 2. Support that SL CAPC is mapped with L1 priority values. | Qualcomm India Pvt Ltd |
| R2-2209612 | Proposal 1. UL Channel Access Priority Class (CAPC) table can be a baseline for SL-CAPC. Proposal 2. Similar to NR-U, SL-CAPCs can be mapped to PQIs. | LG Electronics France |

Among the above 16 contributions, 12 contributions are proposing the PQI based CAPC mapping, while 4 contributions are proposing the priority based CAPC mapping (including 2 proposing SL-LCH priority and 2 proposing L1 priority).

For the PQI based CAPC mapping, the logic is similar to NR-U design, i.e, it is network to use the mapping to configure per-LCH CAPC, which is independent of per-LCH priority. At UE side, if there is no CAPC indicated in DCI, the per-LCH CAPC can be used to derive a per-MAC-PDU CAPC (further details on Q2/3/6).

For the CAPC mapped from per-LCH priority, rapp understand the design is to configure a LCH-agnostic mapping between per-LCH priority and CAPC, and then if there is no CAPC indicated in DCI, a per-LCH CAPC can be derived (instead of being configured) from per-LCH priority, and then a per-MAC-PDU CAPC can be derived from the per-LCH CAPC. It is encouraged that proponent of this approach to clarify if there is any misunderstanding here.

For the CAPC mapped from L1 priority, rapp understand the design is to configure a LCH-agnostic mapping between L1 priority and CAPC, and then if there is no CAPC indicated in DCI, UE firstly decide on L1 priority (based per-LCH priority as in legacy), and derive the CAPC based on the derived L1 priority. It is encouraged that proponent of this approach to clarify if there is any misunderstanding here.

The following Q is to check companies’ view on the CAPC mapping:

Question 1: What’s your view on the following options for CAPC mapping?

Option-1: based on PQI, similar to NR-U where 5QI is used to derive the CAPC;

Option-2: based on the priority of SL-LCHs;

Option-3: based on L1 priority;

Option-4: any other (only restricted to RAN2 aspect)

| Company | Option | Comments |
| --- | --- | --- |
| OPPO | Option-1 | This is the way adopted in NR-U. |
| Apple | Keep open on Option-1 and Option-3 (more analysis is required) | Generally, we prefer to reuse the existing mechanism (i.e. NR-U solution based on 5QI). However, we identify below 3 issues if reusing NR-U CAPC framework to SL-U (i.e. option 1): - How to (pre)configure CAPC per logical channel of SL-DRB for Mode 2 UE in IDLE/INACTIVE state and OOC UE? Different from Uu DRB only applied to CONNECTED UE, SL-DRB is also applied to Mode 2 UE in IDLE / INACTIVE/OOC based on static SLRB mapping specified in NR Rel-16 V2X. Then, it is possible that more than 1 PC5 QoS flows are (pre)configured to map to SL-DRB associated with one logical channel, but only 1 PC5 QoS flow is actually mapped to this SL-DRB for on-going transmission. In this case, CAPC (pre)configured per LCG based on multiple QoS flows assumption may be overestimated. 2) How to avoid conflicting between CAPC and L1 priority in SCI which both serve intention of resource grabbing? It is worth mentioning that It is still not clear how LBT works with resource (re)selection for mode 2. As discussed in RAN1, there are multiple companies proposing mode 2 resource (re)selection enhancement with consideration of LBT outcome. If such solutions are adopted (i.e. LBT will be coupled with resource selection), this issue will become more critical. 3) In case of non-standardized PQI NOT reported to NW, how to handle Mode 2 UE in IDLE/INACTIVE and OOC UE? Please note that CAPC is configured only by gNB in NR-U (i.e. not by UE). Then, if IDLE/INACTIVE/OOC UE has a traffic with non-standardized PQI NOT reported to NW yet, it has no way to find CAPC value (pre)configured by gNB per LCH. We think option 3 (i.e. based on L1 priority) can resolve above 3 issues. However, it is a different framework of NR-U. Meanwhile, we don't think companies supporting option 1 can well resolve these issues in this meeting. Thus, we prefer to keep option-1 and option-3 open for further analysis / down-selection. |
| MediaTek | Option-1 | We prefer to reuse the existing mechanism (i.e., NR-U solution) while based on PQI for standardized QoS flows. More discussions may be taken for non-standardized QoS flows as shown at Q5. |
| Xiaomi | Option-1 | We also prefer to follow NR-U solution. The intention to define this table is to assist the NW to (pre-)configure proper CAPC value for a DRB taking the QoS of all the flows associated with the same DRB into account. From this perspective, we think PQI which can reflect the QoS characteristics is more aligned with the intention. |
| ZTE | Option-1 | As we know, during NR Uu, the specification neither explicit indicate the relationship between CAPC and priority value nor the relationship between CAPC and PDB, so it is hard to say whether only priority or PDB is considered when deciding the CAPC for each QoS profile. It is likely that both priority value and PDB requirement are taken into account when deciding CAPC such that a mapping table between each 5QI and CAPC are provided in NR-U. So we think it is better to follow the same way as NR-U, that is giving a mapping table between PQI and CAPC as a guidline. |
| Lenovo | Option-1 with comment | Preference would be to reuse the NR-U solution. However, we should also discuss the linkage between priority signaled within SCI and the selected CAPC, e.g. whether there is a linkage. |
| vivo | Option-1 | We prefer the NR-U solution to be reused. From the UE side, PQI based CAPC mapping is simpler for both IC/OOC UE to determine CAPC. However, as mentioned in Q5, we need to properly address the issue of handling non-standardized PQI which has no mapping. |
| Qualcomm | Option-1 or Option-3 with comment | PQI based CAPC base lined from NR-U is generally OK at high level, e.g., based on traffic characteristics instead of PDB only. However, sidleink communications are more diverse comparing with NR-U traffic (i.e., DL or UL traffic at RRC Connected state fully under gNB’s control). For example, UE is at OOC or RRC Idle or Inactive state. For another example, standard PQI tables or non-standard PQIs may be associated different sidelink application or services. Additionally, current sidelink procedures are mostly based on priority, e.g., resource selection, logical channel multiplexing and MAC PDU assembly, transmission dropping or pre-emption, etc. PQI based CAPC may cause possible conflict with priority-based process as well as more spec impact. For example, PQI = 56 for V2X service, with PDB = 20 ms, priority = 6, and packet error rate 10-1, CAPC priority is highest (if mapped with PDB) while the priority and error rate are the lowest in the PQI table. For another example, PQI = 92 and 93 for ProSe with PDB = 5 and 10 ms, priority = 5 and 6, and packet error rate 10-4 and 10-4 respectively. For priority-based transmission dropping or pre-emption, these traffics mapped with highest CAPC priority (if based on PDB) may be dropped or pre-empted. At the first meeting on SL-U, down selection is too early to conclude. |
| Intel | Option-1 | We do not see any issue with adopting the NR-U way. Regarding the L1 priority, RAN1 is also discussing this aspect and whether any relationship between CAPC and SCI priority can be defined. However, as ZTE has mentioned, the mapping to PQI implies that both PDB and priority value shall be considered, so Option-1 seems preferrable to us. |
| Sharp | Option-1 | Option 1 is aligned with NR-U |
| Samsung | Option-2 or option-3 | We prefer SL priority based SL CAPC with the following considerations: - SL priority is associated with priority and transmission delay budget. - Resource allocation and logical channel prioritization are performed based on SL priority. - SL priority is already defined for SL SRBs and MAC CE. No need of separate discussion for them. - No need of separate analysis and discussion for all defined PQIs. - No need of further analysis and discussion for new PQIs to be introduced later. - No need of separate discussion regarding how to handle non-standardized PQI. |
| Huawei, HiSilicon | Option-1 | Prefer to reuse the legacy mechanism in NR-U. |
| LG | Option-1 |  |
| NEC | Option-1 | Prefer to follow NR-U solution based on 5QI. |
| Ericsson | Option 1 | We don’t see reasons to not reuse NR-U solution. For the three issues that Apple has spotted. For issue 1, if multiple QoS flows are mapped to the same DRB, meaning they are similar in terms of QoS, therefore, either only one flow or multiple flows are being actually carried by the RB is not the issue, since they are similar in terms of QoS. For issue 2, one thing to clarify is that, CAPC is not used for resource grabbing, it is only used for channel access purpose, while, L1 priority is used for resource grabbing. So, CAPC is mainly defined to ensure a good balance between QoS consideration and fairness of channel access. From this perspective, there is no difference between NR-U and SL-U. For issue 3, we don’t really understand, in NR-U, there is also non-standardized 5QI. In that case, the gNB or UE chooses the standardized 5QI values which best matches the QoS of the non standardized values. There is no difference between NR-U and SL-U either. |
| CATT | Option 1 | To reuse the rule in NR-U which is based on the mapping of QoS(PQI) and CAPC. For option-2 and option-3, both are associated to LCH-priority, which may be configured by network, they may not directly reflect the requirement to CAPC as the QOS does. |
| Fraunhofer | Option 1 |  |
| Nokia, NSB | Option-1 or Option-3 | We agree with Apple that further analysis is needed on the use of either Option-1 or Option-3, before down selection is performed. |
| InterDigital | Option 2 or 3 | We agree with Samsung and Apple that CAPC-based cannot resolve the issues of applying CAPC-based directly to SL, including that a bearer can have multiple QoS flows mapped to it, but that some QoS flows may not be active. Furthermore, it should be noted that in NR-U, the 5QI to CAPC mapping is at the gNB, but here, in this question, we are not making that distinction. Is the intention that also the UE will be configured with this same mapping, or is a per bearer mapping of CAPC intended with option 1? |

Rapporteur summary:

Among 19 companies that replied to the question, the view for each option is:

Option 1: 17

Option 2: 2

Option 3: 5

According to above companies reply, there is a clear majority view (17/19) on Option-1, i.e., use the PQI based CAPC mapping. On the other hand there are also some views on further analysis of the priority, considering this is the first meeting for the SL-U discussion, Rapporteur would like to suggest we take the majority view on PQI-based CAPC mapping as baseline and also leave the consideration of priority for further discussion.

Proposal 1: (17/19) PQI is used to determine the CAPC mapping as in NR-U, FFS whether priority can be considered in addition.

## Q2: For SL DRBs, is SL CAPC (pre)configurable or fixed?

The following contributions are discussing the CAPC for SL DRBs:

| R2-2209385 | Proposal 1 For CAPC configuration in SL-U, following NR-U principle, adopt fixed CAPC value for SRB and MAC CE, but configurable CAPC value for DRB. | OPPO |
| --- | --- | --- |
| R2-2209521 | Proposal-4: Discuss the following proposals for the CAPC of SL-U: - Fixed to the highest priority for SL-SRB0, SL-SRB1, SL-SRB4 and SL MAC CEs; - Configurable for SL-SRB2/SL-SRB3 and SL-DRB. | MediaTek Inc. |
| R2-2209973 | Proposal 2: the CAPC of sidelink DRBs are (pre)configured. | Spreadtrum Communications |
| R2-2210250 | Proposal 2 CAPC for a SL DRB can be configured by the gNB via RRC signalling. | Ericsson |
| R2-2210379 | Proposal 4: In SL-U, channel access priority for SL DRBs are (pre)configured by the gNB.. | Xiaomi |
| R2-2209465 | As NR-U design, the CAPC value is fixed to SL MAC CEs and SL SRB0/1/2/3/4, and configurable to SL DRBs. | vivo |
| R2-2209598 | Proposal 2: Similar to NR-U, a LCH for SL DRB has a configurable CAPC. | Huawei, HiSilicon |
| R2-2209679 | The CAPC is configurable for SL-DRB and SL IUC related MAC CE, and the CAPC is fixed for other SL MAC CEs and SL-SRBs. | ZTE Corporation, Sanechips |
| R2-2209742 | Proposal 1：For SL-U, the CAPC of SL-SRBs, SL MAC CEs and SL-DRB can be determined based on the following rule: - For SL-SRB0/ SL-SRB1/ SL-SRB2/ SL-SRB3/ SL-SRB4 and SL MAC CEs, fixed to the highest priority; - For SL-DRBs, If UE is in RRC Connected mode, the CAPC is configured by gNB; If UE is in RRC Inactive or Idle mode, it can acquire the mapping between QoS profile and CAPC by SIB; If UE is OOC, it can acquire the mapping between QoS profile and CAPC by pre-configuration. | CATT |

All the contributions above propose to use configurable CAPC for DRB.

The following Q is to check companies’ view on the configurable CAPC for DRB:

Question 2: Do you agree with for DRB, the CAPC value is (pre)configured as in NR-U?

| Company | Yes/No | Comments |
| --- | --- | --- |
| OPPO | Yes | This is the way adopted in NR-U. |
| Apple | Yes, but ... | No matter option 1 or option 3 in Q1 is agreed, we think CAPC should be able to be (pre)configured but also a fixed table for reference (i.e. similar to principle of NR-U) should be specified. The (pre)configurable CAPC is for system flexibility consideration, and fixed table is for fair coexistence with WiFi. We emphasize that the later one (i.e. fair coexistence) is important during LAA/NR-U discussion due to the strong concern from WiFi alliance. Therefore, we suggest to change the proposal to: for DRB, the CAPC value is (pre)configured based on a fixed mapping table in spec as reference as in NR-U [Rapp]: Thanks for the suggestion, intentionally we are aligned and we understand the “as in NR-U” can express the configuration is determined by NW considering the CAPC table. |
| MediaTek | Yes | We see no reason to deviate from the mechanism as adopted by NR-U |
| Xiaomi | Yes | Reuse NR-U solution. |
| ZTE | Yes | It is better to follow the same way as NR-U. |
| Lenovo | Yes | We should re-use the NR-U framework. |
| vivo | Yes | Reusing NR-U solution to have more flexibility. |
| Qualcomm | Yes w. comment | (Pre-)configured is generally OK, but how to avoid unfair treatment for the traffics with low CAPC priorities? |
| Intel | Yes | We can reuse the NR-U principle |
| Sharp | Yes |  |
| Samsung | Yes |  |
| Huawei, HiSilicon | Yes | Prefer to reuse the legacy mechanism in NR-U. |
| LG | Yes |  |
| NEC | Yes |  |
| Ericsson | Yes but | It is good to mention that, the gNB can configure but, the gNB may choose to not configure, anyway, it is up to the gNB implementation. So, the question needs to be corrected as The CAPC can be configured by the gNB/preconfigured Regarding the comments raised by Apple, we don’t think it is needed. We can not regulate how gNB should behave. In the 38.300, there was a sentence When choosing the CAPC of a DRB, the gNB takes into account the 5QIs of all the QoS flows multiplexed in that DRB while considering fairness between different traffic types and transmissions. So, we can consider to reuse similar texts as in the above for SL-U in the stage 2 spec. |
| CATT | Yes |  |
| Fraunhofer | Yes |  |
| Nokia, NSB | Yes | We should reuse NR-U solution. |
| InterDigital | Yes, with comments. | We should clarify that the configuration is applied when the gNB does not indicate the CAPC. Also, the configuration is based on a fixed mapping table as indicated by Apple. [Rapp]: thanks for the comments, for the first comment, our understanding is this is an issue for Q6? And for the second comment, please see our reply to Apple’s suggestion. |

Rapporteur summary:

Among 19 companies that replied to the question, all companies agree with the configurable CAPC for DRB. And one company proposes to state in the agreement that the configuration should be based on the mapping table to achieve fairness, while there is also a view think there is no need to restrict NW behavior in the agreement and by reusing the similar text in 38.300 can achieve fairness. And from Rapporteur’s understanding, saying “as in NR-U” in the agreement can express the configuration of the CAPC for DRB from gNB is determined based on the CAPC mapping table which is similar to NR-U.

According to the above companies’ reply, Rapporteur would like to suggest the following proposal.

Proposal 2: (19/19) For DRB, the CAPC value is (pre)configured per-DRB as in NR-U.

## Q3: For SL SRBs and SL MAC CEs, is SL CAPC (pre)configurable or fixed?

The following contributions are discussing the CAPC for SL SRBs and MAC CEs:

| R2-2209385 | Proposal 1 For CAPC configuration in SL-U, following NR-U principle, adopt fixed CAPC value for SRB and MAC CE, but configurable CAPC value for DRB. | OPPO |
| --- | --- | --- |
| R2-2209521 | Proposal-4: Discuss the following proposals for the CAPC of SL-U: - Fixed to the highest priority for SL-SRB0, SL-SRB1, SL-SRB4 and SL MAC CEs; - Configurable for SL-SRB2/SL-SRB3 and SL-DRB. | MediaTek Inc. |
| R2-2209737 | Proposal 1: For SL-U, a fixed CAPC is used for sidelink MAC CEs, fixed to the highest priority (same as in the case of Uu design). | Intel Corporation |
| R2-2209891 | Proposal 5: The logical channels for SL-SRBs are mapped to the highest CAPC priority (lowest CAPC number) Proposal 6: Sidelink MAC CEs are mapped to the highest CAPC priority (lowest CAPC number) | Lenovo |
| R2-2209973 | Proposal 1: the CAPC of SL-SRB0, SL-SRB1, SL-SRB2, SL-SRB3 and SL-SRB4 are all fixed to the highest priority CAPC. Proposal 3: the CAPC of Sidelink CSI reporting MAC CE and Sidelink DRX MAC CE are fixed to the highest priority CAPC. Proposal 4: the CAPC of Inter-UE Coordination Information MAC CE and Inter-UE Coordination Request MAC CE are (pre)configured. | Spreadtrum Communications |
| R2-2210250 | Proposal 1 CAPC is fixed to the highest priority for SL SRBs and MAC CEs. | Ericsson |
| R2-2210256 | Proposal 3: Fixed CAPC is used for SL MAC CEs. FFS on which MAC CEs have highest/lowest priority. Proposal 4: Fixed (highest priority) CAPC is used for SL SRB3. Proposal 5: (Pre)configured CAPC is used for SL SRB0, SL SRB1, SL SRB2 and SL SRB4. | InterDigital |
| R2-2210379 | Proposal 3: In SL-U, channel access priority for all the SL MAC CEs are fixed to the highest channel access priority (lowest CAPC value). Proposal 5: In SL-U, channel access priority for all the SL SRBs are fixed to the highest channel access priority (lowest CAPC value). | Xiaomi |
| R2-2209465 | As NR-U design, the CAPC value is fixed to SL MAC CEs and SL SRB0/1/2/3/4, and configurable to SL DRBs. RAN2 to decide whether to set the highest priority class (i.e. lowest CAPC value) for all the SL SRBs and all the SL MAC CEs. | vivo |
| R2-2209598 | Proposal 3: The CAPC of all SL SRBs are fixed to the highest priority in SL-U. Proposal 4: The CAPC of all SL MAC CEs are fixed to the highest priority in SL-U. | Huawei, HiSilicon |
| R2-2209679 | The CAPC is configurable for SL-DRB and SL IUC related MAC CE, and the CAPC is fixed for other SL MAC CEs and SL-SRBs. | ZTE Corporation, Sanechips |
| R2-2209742 | Proposal 1：For SL-U, the CAPC of SL-SRBs, SL MAC CEs and SL-DRB can be determined based on the following rule: - For SL-SRB0/ SL-SRB1/ SL-SRB2/ SL-SRB3/ SL-SRB4 and SL MAC CEs, fixed to the highest priority; - For SL-DRBs, If UE is in RRC Connected mode, the CAPC is configured by gNB; If UE is in RRC Inactive or Idle mode, it can acquire the mapping between QoS profile and CAPC by SIB; If UE is OOC, it can acquire the mapping between QoS profile and CAPC by pre-configuration. | CATT |

In the above 12 contributions, for SL-SRBs, 2 contributions propose to use configurable CAPC for some of the SRBs (e.g., SRB2/3 or SRB 0/1/2/4) while the others propose to use fixed CAPC for SL-SRB.

The following Q is to check companies’ view on the CAPC for SL-SRB.

Question 3-1: What is your view on the CAPC of SL-SRB?

Option-1: Fixed CAPC for all SL-SRBs;

Option-2: Configurable CAPC for some of the SRBs (please list the SRBs need configurable value), others are fixed;

Option-3: Configurable CAPC for all SRBs

| Company | Option | Comments |
| --- | --- | --- |
| OPPO | Option-1 | Since all the SL-SRBs are specified configurations, i.e., it is natural to rely on specified configuration for CAPC as well. |
| Apple | Option 1 | We tend to agree with OPPO that all SL-SRB should have fixed (or lowest) CAPC value, no matter if Option 1 of Q1 (i.e. PQI based mapping) or Option 3 of Q1 (i.e. L1 priority based mapping) is agreed. |
| MediaTek | Option-1 | Same mechanism as for NR-U |
| Xiaomi | Option 1 | Reuse NR-U solution. |
| ZTE | Option-1 | Agree with Rapp. |
| Lenovo | Option 1 | Reuse NR-U framework. |
| vivo | Option-1 | Similar to NR-U, the CAPC of all the SL-SRBs should be fixed (to the highest priority). |
| Qualcomm | Option-1 | Similar to NR-U |
| Intel | Option-1 | We can reuse the NR-U principle |
| Sharp | Option1 | Reuse NR-U solution |
| Samsung | See comment | No issue if we go to SL priority based CAPC (since we already have corresponding SL priority). If we go to PQI-based CAPC, option-1. |
| Huawei, HiSilicon | Option-1 | Agree with OPPO. |
| LG | Option-1 | Same mechanism as for NR-U |
| NEC | Option-1 | No clear motivation of Option-2 and Option-3. |
| Ericsson | Option 1 |  |
| Fraunhofer | Option 1 |  |
| Nokia, NSB | Option-1 | We should reuse NR-U solution. |
| InterDigital | Option 1 or 2, with comments | We are fine to go with fixed priority if it is the majority, however, we would like to point out that in NR-U, SRB2 is in fact configurable CAPC. |

Rapporteur summary:

Among 18 companies that replied to the question, all companies agree with the fixed CAPC for SRB.

According to the above companies’ reply, Rapporteur would like to suggest the following proposal.

Proposal 3-1: (18/18) For all SL-SRBs, fixed CAPC value is adopted.

In the above 12 contributions, for MAC CEs, 2 contributions propose to use configurable CAPC for the IUC related MAC CEs, while the others propose to use fixed CAPC for all MAC CEs.

The following Q is to check companies’ view on the CAPC for SL MAC CE.

Question 3-2: What is your view on the CAPC of SL MAC CE?

Option-1: Fixed CAPC for all SL MAC CE as in NR-U;

Option-2: Configurable CAPC for some MAC CEs(please list the MAC CEs need configurable value), others are fixed;

Option-3: Configurable CAPC for all MAC CEs

| Company | Option | Comments |
| --- | --- | --- |
| OPPO | Option-1 | This is the way adopted in NR-U. |
| Apple | Yes if PQI based mapping is agreed No if L1 priority based mapping is agreed | If PQI based mapping framework is agreed, we think fixed CAPC for SL MAC-CE is the only feasible way. However, if L1 priority based mapping is agreed, we think it can be further discussed because we may introduce an unified mapping framework from L1 priority to CAPC (including MAC PDU with only data, MAC PDU with only MAC-CE, and MAC PDU with both data and MAC-CE). |
| MediaTek | Option-1 | No need to distinguish the different MAC-CEs for CAPC |
| Xiaomi | Option-1 | Reuse NR-U solution. |
| ZTE | Option-2 | According to our understanding and RAN1’s agreement, the CAPC somewhat depends on priority. As we know, during current specification, all the Uu MAC CE has fixed priority so it is natural that all the Uu MAC CE has fixed CAPC. But for sidelink, the priority of SL IUC MAC CE and SL IUC REQ MAC CE are configurable for PHY layer when performing sensing, considering the CAPC is also used to dertermine LBT sensing parameters for PHY layer, so we think the CAPC of SL IUC MAC CE and SL IUC REQ MAC CE shall be configurable,too. For other MAC CE, the CAPC can be fixed. |
| Lenovo | Option-1 | NR-U framework should be reused. |
| vivo | Option-1 | Although the priority of IUC MAC CEs for sensing/resource selection are configurable in Rel-17, we think there’s no need to deprioritize the CAPC priority for IUC MAC CE for channel access purpose, otherwise the things would go complicated. |
| Qualcomm | Option-1 or Option-2 comment | Option 1: PQI based Option 2: Priority based |
| Intel | Option-1 | We do not see need for different CAPC for different MAC CEs. A single value should be sufficient |
| Sharp | Option 1 | NR-U framework could be reused. |
| Samsung | See comment | No issue if we go to SL priority based CAPC (since we already have corresponding SL priority). If we go to PQI-based CAPC, option-1. |
| Huawei, HiSilicon | Option-1 | Prefer to reuse the legacy mechanism in NR-U. |
| LG | Option-1, Option-2 | CAPC of SL IUC MAC CE and SL IUC REQ MAC CE can be configurable |
| NEC | Option-1 | Follow NR-U solution. |
| Ericsson | Option 1 |  |
| CATT | Option 1 | Reuse NR-U solution. |
| Fraunhofer | Option 1 |  |
| Nokia, NSB | Option-1 | Currently there is no clear justification to have the CAPC of the SL MAC CEs to be configurable. |
| InterDigital | Option 1 or 3 | Same view as Apple – this depends on PQI based or priority based solution. |

Rapporteur summary:

Among 19 companies that replied to the question, the views as follows:

Option-1: 18

Option-2: 3

Option-3: 1

According to the above companies’ replies, the clear majority companies are fine with fixed CAPC for all the SL MAC CEs. So Rapporteur would like to suggest we agree on the fixed CAPC for all MAC CEs.

Proposal 3-2: (18/19) For all SL MAC CEs, fixed CAPC value is adopted.

## Q4: If SL CAPC is determined based on PQI, e.g., what should be criterion to make a mapping table? what is companies’ thinking on mapping table between CPAC and PQI?

The following contributions are discussing the detailed CAPC mapping rules:

| R2-2209385 | Proposal 2 R2 discusses mapping PQI 90/91/92/93/21/22/23/55/56/57/58 to CAPC priority class 1. Proposal 3 R2 discusses mapping PQI 59/61 to CAPC priority class 3. Proposal 4 R2 discusses mapping PQI 25 to CAPC priority class 2. Proposal 5 R2 discusses mapping PQI 24/26/60 to CAPC priority class 1 or CAPC priority class 2. (PDB based mapping rule) | OPPO |
| --- | --- | --- |
| R2-2209521 | Proposal-3: Discuss the PDB range of PQI for different CAPCs (e.g., less than 50ms, 50-100ms and above 100ms are used). | MediaTek Inc. |
| R2-2209737 | Proposal 2b: RAN2 is proposed to discuss and downselect between Table 1 (PDB based) and Table 2 (classified based on Default Priority level) above to for mapping between CAPC and PQI. | Intel Corporation |

In the above contributions, all contributions propose to use the PDB as the criterion for determining the mapping between CAPC and PQI as in NR-U.

The following Q is to check companies’ view on this issue:

Question 4-1: If SL CAPC is determined based on PQI, do you agree with using the PDB as the criterion for determining the mapping between CAPC and PQI as in NR-U? (if disagree, please clarify what is the preferred criterion)

| Company | Agree/Disagree | Comments |
| --- | --- | --- |
| OPPO | Agree with comments | We agree with using the PDB as the general criterion to determine the mapping between CAPC and PQI as in NR-U. Besides that, we understand there are some exceptional cases where the PDB is not the only reason to be considered when category a 5QI, i.e., the Mission-critical service (5QI-70), so we wonder whether the same consideration is needed in SL-U for PQI 24/26/60. |
| Apple | See comments | We think it is premature to discuss the specific mapping. Of course, both PDB and default priority level can be considered. But we are not sure whether other criteria can also be considered as well. It seems not hurry to start this discussion. What's more, SA2 should be consulted if we go this way. |
| MediaTek | Option-1 | We propose to at least agree PDB as the baseline for CAPC mapping. Other consideration FFS |
| Xiaomi | See comments | Agree with Apple. |
| ZTE | comments | We think at least priority and PDB need to be considered when determing the mapping between CAPC and PQI. The simplest way is to reuse the table of mapping between 5QI and CAPC in NR-U by replacing 5QI with PQI having simialry QoS characteristics. |
| Lenovo | Comments | PDB should be considered for the mapping to a CAPC value. However, there might be other factors which needs to be also considered. For now, we should only agree, that PDB is considered for the CAPC mapping. |
| vivo | See comments | PDB-based criteria for CAPC-PQI mapping can be reused. However, as the PDB requirement for SL services are usually less than 100ms, we can evaluate whether proper adjustment to the PDB range for different CAPCs if necessary. Moreover, we suggest to further conclude CAPC-PQI mapping after RAN1 determining the channel access parameters (e.g. max/min contention window, COT related value, etc.) for each CAPC. |
| Qualcomm | Comment | It’s still too early to conclude the detailed mapping since we don’t know the CAPC table yet (still under RAN1’s discussion). Additionally, at least PDB and priority need to be considered for mapping CAPC. For example, PQI = 56 for V2X service, with PDB = 20 ms, priority = 6, and packet error rate 10-1, CAPC priority is highest (if mapped with PDB) while the priority and error rate are the lowest in the PQI table. For another example, PQI = 92 and 93 for ProSe with PDB = 5 and 10 ms, priority = 5 and 6, and packet error rate 10-4 and 10-4 respectively. For priority-based transmission dropping or pre-emption, these traffics mapped with highest CAPC priority (if based on PDB) may be dropped or pre-empted. |
| Sharp | Agree | PDB could be the criterion for determining the mapping between CAPC and PQI. Other criterions could be FFS. |
| Samsung | Agree with comment | PDB + priority level. |
| Huawei, HiSilicon | See comments | PDB should be considered. FFS other QoS parameters. |
| LG | comment | Agree with apple. Both PDB and default priority level can be considered. But we are not sure whether other criteria can also be considered as well. It seems not hurry to start this discussion. |
| NEC | Agree in principle | PDB can be considered as general criterion while other aspects would not be excluded at this stage. |
| Ericsson | Agree with comments | As MediaTek said, we can agree on at least PDB can be considered. Then FFS on other criteria. In NR-U, it was RAN2 to do the job, it is sufficient that RAN2 carefully check the QoS table in the SA2 spec, no need to bother SA2. Since SA2 may not understand CAPC. |
| Nokia, NSB | See comments | Agree with Apple. |
| InterDigital | See comments | We think other factors beyond priority and PDB should be considered and to some extent, this may also be dependant on RAN1 discussion. For example, RAN1 is deciding whether to use UL or DL CAPC or both, so the PQI to CAPC mapping may further depend on the outcome of this discussion. |

Rapporteur summary:

Among 16 companies that replied to the question, all companies think at least PDB can be considered and would like to further discuss the other possible criterion. So Rapporteur would like to suggest we firstly agree on the PDB as a criterion and leave the other possible criterions as FFS.

Proposal 4-1: (16/16) If PQI-based CAPC mapping is agreed, at least PDB can be used as the criterion to determine the CAPC mapping. FFS if any other additional criterions needed.

And for the detailed CAPC value, in LTE-LAA and NR-U, only the mapping between the first 3 CAPC levels (CAPC 1/2/3) and standardized 5QI have been defined and CAPC 4 is left to other usages (e.g., left to the operator to map or the MAC CE for Recommended bit rate query).

Table: Mapping between Channel Access Priority Classes and 5QI

| CAPC | 5QI |
| --- | --- |
| 1 | 1, 3, 5, 65, 66, 67, 69, 70, 79, 80, 82, 83, 84, 85 |
| 2 | 2, 7, 71 |
| 3 | 4, 6, 8, 9, 72, 73, 74, 76 |
| 4 | - |
| NOTE: lower CAPC value means higher priority - |  |

The following contribution proposes RAN2 to discuss whether to reuse the NR-U principle, i.e., define 3 CAPC levels or define all of the 4 CAPC levels in the Mapping between Channel Access Priority Classes and standardized PQI:

| R2-2209521 | Proposal-2: Discuss if 4 CAPCs or 3 CAPCs are used for the Mapping between Channel Access Priority Classes and PQI. | MediaTek Inc. |
| --- | --- | --- |

The following Q is to check companies’ view on this issue

Question 4-2: If SL CAPC is determined based on PQI, do you agree with defining the 3 CAPC levels in the mapping between CAPC and standardized PQI as in NR-U?

| Company | Agree/Disagree | Comments |
| --- | --- | --- |
| OPPO | Agree | This is the way adopted in NR-U. |
| Apple | Premature to discuss this issue | We think it is premature to discuss the specific mapping. Of course, both PDB and default priority level can be considered. But we are not sure whether other criteria can also be considered as well. It seems not hurry to start this discussion. What’s more, SA2 should be consulted if we go this way. |
| MediaTek | Comments | Not sure if there is RAN1 impact if we decide 3 or 4 levels CAPC. We may need to understand if there is any problem to adopt 4 levels CAPC. If we look at the wide range of PDB for different PQIs, comparing with 5QI. It seems more than 3 levels CAPC may present better distribution of the PDB |
| Xiaomi | See comments | Agree with Apple. |
| ZTE | Agree | The same way used in NR-U can be followed. |
| Lenovo | Comments | Agree with Apple, that this is too premature to decide now. Need to better understand the ranges of the PQIs. |
| Vivo | See comments | Agree with Apple that it is premature to discuss this, which is related to the channel access parameters pending RAN1 discussion and the PDB range division for different CAPCs as mentioned in Q4-1. |
| Qualcomm | Comment | Similar to previous comment, it’s premature for this discussion. |
| Intel | Agree with Comment | We agree that we can follow NR-U principle, but on the other hand, this can be discussed once the basic principle of CAPC mapped to PQI is agreed. |
| Sharp | Agree | 3 CAPCs could be the start point. |
| Samsung | Agree |  |
| Huawei, HiSilicon | See comments | This is related to Q 4-1. It can be FFS whether 3 levels CAPC or 4 levels CAPC are used. |
| LG | Premature to discuss this issue |  |
| NEC | Comment | No strong view at this stage. Can check it later. |
| Ericsson | disagree | It is too early to discuss the exact mapping table as we commented during the online meeting. |
| CATT | Agree | PDB is considered as the criterion as in NR-U, others could also be considered. |
| Fraunhofer | Premature to discuss this issue |  |
| Nokia, NSB | See comments | Agree with Apple. |
| InterDigital | Disagree | It is too early to discuss the actual levels at this stage. |

Among 19 companies that replied to the question, 11 companies think it is premature to discuss this. So Rapporteur would like to suggest leaving this issue for companies’ further consideration, and thus no proposal is provided.

## Q5: If SL CAPC is determined based on PQI, how to handle non-standardized PQI?

The following contributions are discussing the handling of non-standardized PQI:

| R2-2209737 | Proposal 3: For the case of non-standardized PQI, RAN2 shall follow the same principle as that for non-standardized 5QI i.e to use the CAPC of the standardized PQI which best matches the QoS characteristics of the non-standardized PQI. | Intel Corporation |
| --- | --- | --- |
| R2-2210250 | Proposal 4 A QoS flow corresponding to a non-standardized PQI (i.e. operator specific PQI) should use the CAPC of the standardized PQI which best matches the QoS characteristics of the non-standardized PQI. | Ericsson |
| R2-2210379 | Proposal 2: In SL-U, a QoS flow corresponding to a non-standardized PQI should use the CAPC of the standardized PQI which best matches the QoS characteristics of the non-standardized PQI. | Xiaomi |
| R2-2209465 | For the SL-DRB configuration via dedicated 18ignalling, the gNB should reuse the same rule in NR-U to treat the CAPC value for the PC5 QoS flow with non-standardized PQI, i.e. a PC5 QoS flow corresponding to the non-Standardized PQI should use the CAPC of the standardized PQI which best matches the QoS characteristics of the non-standardized PQI. RAN2 to discuss how to treat the non-standardized PQI value that is not covered by the SL-DRB configurations in the SIB/pre-configuration. | Vivo |

All the contributions above propose to reuse NR-U principle, i.e., to use the CAPC of the standardized PQI which best matches the QoS characteristics of the non-standardized PQI.

The following Q is to check companies’ view on the handling of non-standardized PQI

Question 5: If SL CAPC is determined based on PQI, do you agree, as in NR-U, to use the CAPC of the standardized PQI which best matches the QoS characteristics of the non-standardized PQI?

| Company | Yes/No | Comments |
| --- | --- | --- |
| OPPO | Yes |  |
| Apple | Yes for some case (See comments) | First, please note that in NR-U, this principle is for gNB rather than UE. For SL-U, we can discuss two cases: - For CONNECTED UE in mode 1, we think this principle can be agreed because the UE will report QoS profile of the non-standardized PQI to NW. Then, NR-U framework can be reused (i.e. gNB will configure CAPC either in DCI or LCH). - For IDLE/INACTIVE/OOC UE, as we indicated in Q1, we have one SL-U specific issue: if IDLE/INACTIVE/OOC UE has a traffic with non-standardized PQI NOT reported to NW yet, it has no way to find CAPC value (pre)configured by gNB per LCH. In this case, We are not sure whether each company can have consensus that the UE can use this principle to determine the CAPC without notifying gNB. |
| MediaTek | Yes |  |
| Xiaomi | Yes | Reuse NR-U solution. |
| ZTE | Yes | The same way used in NR-U can be followed. |
| Lenovo | Yes |  |
| vivo | Yes for gNB, FFS for UE | We agree with Apple that this principle ‘use the CAPC of the standardized PQI which best matches the QoS characteristics of the non-standardized PQI’ is for gNB rather than UE and should be further evaluated in SL-U to handle IDLE/INACTIVE/OOC UE. Specifically, the QoS flow with QoS profile of non-standardized PQI can be mapped to default SL-DRB, so it is not clear whether there is associated ‘default CPAC’ configuration. Also, if we make the UE to use the same principle in NR-U, it is also not clear for the same non-standardized PQI, whether the peer Ues can have different implementation of the principle, leading to one UE always adopting higher priority CAPC value while the other UE always adopting a lower priority CAPC value. We suggest RAN2 to further discuss this issue. |
| Qualcomm | Yes comment | Yes for Mode1 under gNB’s management. Still need discussion for other cases, e.g., OOC or RRC Idle and Inactive. |
| Intel | Yes with comment | Regarding Apple’s comment on IDLE/INACTIVE/OOC UE, we assume that we have to rely on UE implementation to perform this mapping under the same principle as the CONNECTED mode. If there is any other option, we are fine to discuss it |
| Sharp | Yes |  |
| Samsung | Yes |  |
| Huawei, HiSilicon | Yes | It can be up to NW implementation to configure CAPC for the SL DRB associated with non-standardized PQI (i.e. default SL DRB or non-default SL DRB), with considering such restriction. |
| LG | Yes |  |
| NEC | Yes |  |
| Ericsson | Yes |  |
| CATT | Yes | Reuse the NR-U rule. |
| Fraunhofer | Yes |  |
| Nokia, NSB | Yes |  |
| InterDigital | See comment | We agree with Apple that this is only possible for a UE in CONNECTED. For IDLE/INACTIVE, UE implementation may create some problems with WiFi coexistence, so a specified rule is preferrable. |

Rapporteur summary:

Among 19 companies that replied to the question, all companies think the NR-U rule can be reused from NW perspective, and some companies have concerns for the case when NW is not aware of the non-standardized PQI. So according to the above companies’ reply, Rapporteur would like to suggest we agree to reuse the NR-U principle as baseline and leave the further optimization to the concerned cases (i.e., IDLE/INACTIVE/OOC) as FFS.

Proposal 5: (19/19) As in NR-U, if SL CAPC is determined based on PQI, as a baseline, for non-standardized PQI, to use the CAPC of the standardized PQI which best matches the QoS characteristics of the non-standardized PQI. FFS if any specific work needed for RRC_INACTIVE/RRC_IDLE/OOC UEs.

## Q6: How to SL CAPC when different SL LCHs, SL MAC CEs and SL SRBs are multiplexed?

The following contributions are discussing the CAPC determination when different SL LCHs, SL MAC CEs and SL SRBs are multiplexed:

| R2-2209385 | Proposal 6 RAN2 to discuss how to decide the CAPC of a MAC PDU in SL-U, e.g., always following the lowest priority CAPC, or allowing exceptional cases where the highest priority CAPC is followed as in NR-U. | OPPO |
| --- | --- | --- |
| R2-2209891 | Proposal 7: Suggest RAN2 to consider above rules for UE to select CAPC of sidelink TB When performing LBT for the transmission of a sidelink TB the UE selects the CAPC as: - If only SL MAC CE(s) are included in the SL TB, the highest priority CAPC of those SL MAC CE(s) is used; or - If SBCCH SDU(s) are included in the TB, the highest priority CAPC is used; - or If Sidelink Control Channel (SCCH) SDU(s) are included in the TB, the highest priority CAPC is used; or - The highest priority CAPC of the SL logical channel(s), e.g. STCH, with MAC SDU multiplexed in the TB is used otherwise. | Lenovo |
| R2-2209973 | Proposal 6: the UE select the CAPC as follows: 1) if only MAC CE(s) are included in the TB, the highest priority CAPC of those MAC CE(s) is used; or 2) if SCCH SDU(s) are included in the TB, the highest priority CAPC is used; or 3) The lowest priority CAPC of the logical channel(s) with MAC SDU multiplexed in the TB is used otherwise. | Spreadtrum Communications |
| R2-2210250 | Proposal 5 When performing Type 1 LBT for the transmission using a configured SL grant or a SL grant obtained via Mode 2 RA, the UE shall select the CAPC as follows: a. If only MAC CE(s) are included in the TB, the highest priority CAPC is used; or b. If SDU(s) of a SRB are included in the TB, the highest priority CAPC is used; or c. The lowest priority CAPC of the logical channel(s) with MAC SDU multiplexed in the TB is used otherwise. | Ericsson |
| R2-2210357 | Proposal 1: For selection of CAPC for PSCCH/PSSCH, reuse similar approach as used for NR-U UL CG PUSCH, i.e., based on priority of LCH data multiplexed in the TB. FFS potential exceptions, e.g. SL SRB. | Nokia, Nokia Shanghai Bell |
| R2-2210379 | Proposal 7: In SL-U, CAPC is selected as the highest priority CAPC if only SL MAC CEs are included in the TB for SL transmissions on configure SL grant and selected SL grant. Proposal 8: In SL-U, CAPC is selected as the highest priority CAPC if only SCCH SDUs are included in the TB for SL transmissions on configure SL grant and selected SL grant. Proposal 9: In SL-U, RAN2 to further discuss how to select the CAPC and whether to introduce any restriction on data multiplexing for SL transmissions on configure SL grant and selected SL grant. | Xiaomi |
| R2-2209891 | Proposal 2: for sidelink UE in mode 2 and for configured SL grant transmissions (mode 1), sidelink Tx UE selects the CAPC value for SL TB transmission according to some predefined rules. | Lenovo |
| R2-2209465 | RAN2 to discuss whether the following rules for per TB transmission can be used for SL-U, in case CAPC is not indicated in the DCI for a Mode-1 UE and in case the UE is configured with Mode-2: If only MAC CE(s) are included in the TB, the highest priority CAPC of those SL MAC CE(s) is used; or If SCCH SDU(s) are included in the TB, the highest priority CAPC of the SCCH(s) is used; or The lowest priority CAPC of the logical channel(s) with MAC SDU multiplexed in the TB is used, otherwise. | vivo |
| R2-2209598 | Proposal 5: The CAPC determination of a TB in SL-U could follow existing NR-U mechanism with adaption for sidelink: If SL MAC CE(s) are included in the TB, the highest priority CAPC is used; or If SCCH SDU(s) are included in the TB, the highest priority CAPC is used; or The lowest priority CAPC of the logical channel(s) multiplexed with SL MAC SDU in the TB is used otherwise. | Huawei, HiSilicon |
| R2-2209679 | When selecting the CAPC for performing Type 1 LBT for the transmission of a sidelink TB, the similar principle used for uplink shall be followed. | ZTE Corporation, Sanechips |
| R2-2209742 | Proposal 3：For resource allocation mode 1, if CAPC is indicated by network, UE follows the indicated CAPC. Proposal 4: For resource allocation mode 2 or resource allocation mode 1 but CAPC is not indicated by network, UE determines the CAPC based on the contents of the SL TB as follows: - If only SL MAC CE(s) are included in the SL TB, the highest priority SL CAPC of those SL MAC CE(s) is used; or - If SCCH SDU(s) are included in the SL TB, the highest priority SL CAPC is used; or - The lowest priority SL CAPC of the SL logical channel(s) with MAC SDU multiplexed in the SL TB is used otherwise. | CATT |

Almost all the contributions above propose to reuse NR-U principle, i.e., if CAPC is not indicated by network (via DCI), UE determines the CAPC based on the contents of the SL TB as follows:

- If only SL MAC CE(s) are included in the SL TB, the highest priority SL CAPC of those SL MAC CE(s) is used; or

- If SCCH SDU(s) are included in the SL TB, the highest priority SL CAPC is used; or

- The lowest priority SL CAPC of the SL logical channel(s) with MAC SDU multiplexed in the SL TB is used otherwise.

The following Q is to check companies’ view on the determination of CAPC of the TB when the CAPC value is not indicated in SL grant (DCI).

Question 5: Do you agree to reuse NR-U rules as follows for determining the CAPC of the SL TB when the CAPC is not indicated in the DCI?

- If only SL MAC CE(s) are included in the SL TB, the highest priority SL CAPC of those SL MAC CE(s) is used; or

- If SCCH SDU(s) are included in the SL TB, the highest priority SL CAPC is used; or

- The lowest priority SL CAPC of the SL logical channel(s) with MAC SDU multiplexed in the SL TB is used otherwise.

| Company | Yes/No | Comments (if No, please clarify what is the difference compared to NR-U scheme) |
| --- | --- | --- |
| OPPO | Yes |  |
| Apple | Yes if PQI based mapping is agreed No if L1 priority based mapping is agreed | If PQI based mapping framework is agreed, we tend to reuse same mechanism of NR-U. However, if L1 priority based mapping is agreed, we think it can be further discussed because we may introduce an unified mapping framework from L1 priority to CAPC (including MAC PDU with only data, MAC PDU with only MAC-CE, and MAC PDU with both data and MAC-CE). This is much simpler than NR-U framework. |
| MediaTek | Yes |  |
| Xiaomi | Yes | Reuse NR-U solution. |
| ZTE | Yes |  |
| Lenovo | No | We should study this further carefully before drawing some premature conclusion. Simply reusing the NR-U principle will lead to some issue for the Mode 2, i.e. resource selection. Applying the same behaviour for Sidelink transmissions, e.g. when Tx UE selects the CAPC value, would potentially lead to a UE selecting a high CAPC index (low priority value), e.g. when low priority data with large PDB is contained in the TB, even though sidelink resources are selected (mode 2) according to the PDB of the TB which is determined based on the LCH with the most stringed PDB requirements. Hence, there may be a mismatch between the CAPC value selected for a SL TB and the PDB of the SL TB. For example, the LBT procedure including the contention window determined as a function of the CAPC may not reach the condition for allowing a transmission before the PDB expires. We think therefore that CAPC selection for mode 2 resource selection (also for SL CG grants) needs to be further studied since such resource allocation mode is not supported in NR-U. Just to note that there was already in NR-U a lengthy discussion on whether to select the highest CAPC value (lowest priority) of LCHs multiplexed in a TB. Already for Rel-16 NR companies had a concern that the selection of the lowest priority CAPC for a MAC PDU which multiplexes different LCHs is not optimal since high priority data will be subject to delay when lower priority data is multiplexed in the same MAC PDU. The reason why it was though finally decided to go for the lowest priority CAPC value was the fact that for NR-U CG transmission, the traffic type is was assumed to be predictable, and the network could also make sure by proper configuration that LCHs with similar CAPC are multiplexed into the same MAC PDU. However, for SL mode 2 transmission we don’t think that same assumptions cannot be made anymore. Therefore, reusing simple the NR-U principle, i.e., always selecting the highest CAPC value (lowest priority) of LCHs multiplexed in a TB, may lead to some problems. |
| vivo | Yes with comments | We prefer to reuse NR-U solution. For the problem raised by Lenovo, we are not sure the CPAC selection has impact on mode-2 resource selection, it may be FFS. |
| Qualcomm | Yes w. comment | PQI based CAPC: yes, baselined with NR-U. Priority based: the current priority based scheme can be applied. |
| Intel | Yes |  |
| Sharp | Yes |  |
| Samsung | Yes |  |
| Huawei, HiSilicon | See comments | If RAN2 agree to fix CAPC with highest priority for all SL MAC CEs, the first condition can be changed into “If only SL MAC CE(s) are included in the SL TB, the highest priority SL CAPC of those SL MAC CE(s) is used”. Since there is no need to distinguish the difference for SL SRBs and SL MAC CEs, if we agree to use fixed CAPC with highest priority for them. [Rapp]: Thanks for the comment, since whether “fix CAPC with highest priority for all SL MAC CEs” is still open, and also considering there may be lower priority MAC CE in the future, it is safer to include the “only”. |
| LG | Yes |  |
| NEC | Yes |  |
| Ericsson | Yes |  |
| CATT | Yes |  |
| Fraunhofer | Yes, but | The points raised by Lenovo should be further investigated. |
| Nokia, NSB | No | This will depend if PQI or L1 based mapping for the CAPC is used. |
| InterDigital | No | We agree with the issue raised by Lenovo for Mode 2. We think this agreement can be at least made for mode 1 for the time being, and mode 2 can be investigated further. |

Rapporteur summary:

Among 19 companies that replied to the question, 16 companies are fine with reusing NR-U principle at least in case the PQI-based CAPC mapping is agreed, so Rapporteur would suggest to go for the clear majority view

Proposal 6: (16/19) As in NR-U, to determine the CAPC of the SL TB when the CAPC is not indicated in the DCI:

- If only SL MAC CE(s) are included in the SL TB, the highest priority SL CAPC of those SL MAC CE(s) is used;

- If SCCH SDU(s) are included in the SL TB, the highest priority SL CAPC is used;

- The lowest priority SL CAPC of the SL logical channel(s) with MAC SDU multiplexed in the SL TB is used otherwise.

# Conclusion

We have the following proposals:

Proposal 1: (17/19) PQI is used to determine the CAPC mapping as in NR-U, FFS whether priority can be considered in addition.

Proposal 2: (19/19) For DRB, the CAPC value is (pre)configured per-DRB as in NR-U.

Proposal 3-1: (18/18) For all SL-SRBs, fixed CAPC value is adopted.

Proposal 3-2: (18/19) For all SL MAC CEs, fixed CAPC value is adopted.

Proposal 4-1: (16/16) If PQI-based CAPC mapping is agreed, at least PDB can be used as the criterion to determine the CAPC mapping. FFS if any other additional criterions needed.

Proposal 5: (19/19) As in NR-U, if SL CAPC is determined based on PQI, as a baseline, for non-standardized PQI, to use the CAPC of the standardized PQI which best matches the QoS characteristics of the non-standardized PQI. FFS if any specific work needed for RRC_INACTIVE/RRC_IDLE/OOC UEs.

Proposal 6: (16/19) As in NR-U, to determine the CAPC of the SL TB when the CAPC is not indicated in the DCI:

- If only SL MAC CE(s) are included in the SL TB, the highest priority SL CAPC of those SL MAC CE(s) is used;

- If SCCH SDU(s) are included in the SL TB, the highest priority SL CAPC is used;

- The lowest priority SL CAPC of the SL logical channel(s) with MAC SDU multiplexed in the SL TB is used otherwise.

# Reference

- R2-2209385	Discussion on CAPC definition in SL-U	OPPO	discussion	Rel-18	NR_SL_enh2

- R2-2209465	On CAPC in SL-U	vivo	discussion

- R2-2209521	Channel Access Priority Classes for SL-U	MediaTek Inc.	discussion	NR_SL_enh2

- R2-2209598	Discussion on CAPC for SL-U	Huawei, HiSilicon	discussion	Rel-18	NR_SL_enh2

- R2-2209612	Discussion on RAN2 aspects in SL-U	LG Electronics France	discussion	Rel-18	NR_SL_enh2

- R2-2209678	Discussion on RAN2 scope of SL-U	ZTE Corporation, Sanechips	discussion	Rel-18	NR_SL_enh2

- R2-2209679	Discussion on CAPC definition and consistent sidelink LBT failure handling	ZTE Corporation, Sanechips	discussion	Rel-18	NR_SL_enh2

- R2-2209737	On CAPC for SL-U	Intel Corporation	discussion	Rel-18	NR_SL_enh2

- R2-2209742	Consideration on CAPC for SL-U	CATT	discussion	Rel-18	NR_SL_enh2

- R2-2209761	Control plane aspects of sidelink on unlicensed spectrum (SL-U)	Apple	discussion	Rel-18	NR_SL_enh2

- R2-2209891	Discussion on channel access priority for NR SL-U	Lenovo	discussion	Rel-18	NR_SL_enh2-Core

- R2-2209973	Consideration on channel access priority in SL-U	Spreadtrum Communications	discussion	Rel-18

- R2-2210250	CAPC table and MAC multiplex rules	Ericsson	discussion	Rel-18	NR_SL_enh2

- R2-2210256	CAPC and COT sharing for SL Unlicensed	InterDigital	discussion	Rel-18	NR_SL_enh2

- R2-2210280	Discussion on sidelink CAPC	Qualcomm India Pvt Ltd	discussion

- R2-2210357	On channel access priority class and HARQ feedback	Nokia, Nokia Shanghai Bell	discussion	NR_SL_enh2

- R2-2210379	Discussion on channel access for sidelink operation on unlicensed spectrum	Xiaomi	discussion	NR_SL_enh2

- R2-2210552	SL CAPC	Samsung Research America	discussion	Rel-18	NR_SL_enh2