3GPP TSG-RAN WG2 #119bis-e	R2-221xxxx

E-meeting, October 2022

Agenda Item:	8.15.2

Source:	OPPO

Title:	[Draft]Summary of [AT119bis-e][503][V2X/SL] CAPC (OPPO)

Document for:	Discussion, Decision

# Introduction

This document is a report on the following email discussion:

- [AT119bis-e][503][V2X/SL] CAPC (OPPO)

Scope: Discuss SL CAPC:

Q1: SL CAPC determination based on PQI or SL priority or any other?

Q2: For SL DRBs, is SL CAPC (pre)configurable or fixed?

Q3: For SL SRBs and SL MAC CEs, is SL CAPC (pre)configurable or fixed?

Q4: If SL CAPC is determined based on PQI, e.g. what should be criterion to make a mapping table? what is companies’ thinking on mapping table between CPAC and PQI?

Q5: If SL CAPC is determined based on PQI, how to handle non-standardized PQI?

Q6: How to SL CAPC when different SL LCHs, SL MAC CEs and SL SRBs are multiplexed?

Intended outcome: Discussion summary in R2-2210934.

Deadline: 10/13 10:00 (UTC).

- According to the scope of this offline discussion, the following issues will be discussed:

Q1: SL CAPC determination based on PQI or SL priority or any other?

Q2: For SL DRBs, is SL CAPC (pre)configurable or fixed?

Q3: For SL SRBs and SL MAC CEs, is SL CAPC (pre)configurable or fixed?

Q4: If SL CAPC is determined based on PQI, e.g. what should be criterion to make a mapping table? what is companies’ thinking on mapping table between CPAC and PQI?

Q5: If SL CAPC is determined based on PQI, how to handle non-standardized PQI?

Q6: How to SL CAPC when different SL LCHs, SL MAC CEs and SL SRBs are multiplexed?

# Contact Information

| Company | Name | Email Address |
| --- | --- | --- |
| OPPO | Bingxue Leng | lengbingxue@oppo.com |
| Apple | Peng Cheng | pcheng24@apple.com |

# Discussion

## Q1: SL CAPC determination based on PQI or SL priority or any other?

The following contributions have discussed the CAPC mapping determination, i.e., whether the mapping should be based on PQI or SL priority or any other (only restricted to RAN2 aspects):

| R2-2209385 | Proposal 2 R2 discusses mapping PQI 90/91/92/93/21/22/23/55/56/57/58 to CAPC priority class 1. Proposal 3 R2 discusses mapping PQI 59/61 to CAPC priority class 3. Proposal 4 R2 discusses mapping PQI 25 to CAPC priority class 2. Proposal 5 R2 discusses mapping PQI 24/26/60 to CAPC priority class 1 or CAPC priority class 2. | OPPO |
| --- | --- | --- |
| R2-2209521 | Proposal-1: PDB based CAPC split is used for SL-U for standardized PQI. | MediaTek Inc. |
| R2-2209737 | Proposal 2a: RAN2 is proposed to agree that for SL-U, a CAPC mapping to PQI shall be defined based on the standardized PQI to PC5 QoS characteristics mapping, similar to the existing mapping between CAPC and 5QI. | Intel Corporation |
| R2-2209761 | Proposal 3: For the support of CAPC for SL-U, RAN2 discuss the following 2 alternatives: - Alt-1: Introduce a new mapping table from PQI to CAPC, similar to Uu Table of mapping from 5QI to CAPC. FFS details of the table - Alt-2: Introduce a mapping from L1 priority to CAPC. FFS whether the mapping is fixed or configurable | Apple |
| R2-2209891 | Proposal 4: If CAPC mapping table is introduced, using PQI for CAPC definition. FFS for detail mapping between CAPC and PQI. | Lenovo |
| R2-2209973 | Proposal 5: RAN2 study and define the mapping table between CAPC and PQI in SL-U. | Spreadtrum Communications |
| R2-2210250 | Proposal 3 A mapping table between CAPC values and standardized PQI values need to be defined by RAN2. | Ericsson |
| R2-2210256 | Proposal 1: CAPC is determined at least from the priority of the logical channel(s) associated with the data transmission. FFS on how to handle MCR. Proposal 2: RAN2 further discusses whether SL factors other than QoS can affect the CAPC associated with data transmissions | InterDigital |
| R2-2210379 | Proposal 1: In SL-U, a table for mapping between PQI and CAPC, similar to Table 5.6.2-1 in 3GPP TS 38.300, shall be specified. FFS the detailed mapping between PQI and CAPC. | Xiaomi |
| R2-2210552 | [Proposal 1]: SL priority based SL CAPC determination is proposed. [Proposal 2]: Configurable mapping between SL CAPC and SL priority is proposed. | Samsung Research America |
| R2-2209465 | As in NR-U, a mapping table between CAPC and PQI can be specified at a stage-2 level as a guideline to gNB-configuration/pre-configuration. RAN2 to decide the specific mapping relationship after RAN1 concludes the specific channel access parameters per CAPC for SL. | vivo |
| R2-2209598 | Proposal 1: In SL-U, the table of the mapping between 5QI and CAPC in NR-U can be reused by replacing 5QI with PQI. | Huawei, HiSilicon |
| R2-2209678 | Proposal 1：RAN2 is suggested to discuss how to decide sidelink CAPC value. | ZTE Corporation, Sanechips |
| R2-2209742 | Proposal 2：For SL-U, the mapping between PQI and CAPC can be defined in specification. | CATT |
| R2-2210280 | Proposal 2. Support that SL CAPC is mapped with L1 priority values. | Qualcomm India Pvt Ltd |
| R2-2209612 | Proposal 1. UL Channel Access Priority Class (CAPC) table can be a baseline for SL-CAPC. Proposal 2. Similar to NR-U, SL-CAPCs can be mapped to PQIs. | LG Electronics France |

Among the above 16 contributions, 12 contributions are proposing the PQI based CAPC mapping, while 4 contributions are proposing the priority based CAPC mapping (including 2 proposing SL-LCH priority and 2 proposing L1 priority).

For the PQI based CAPC mapping, the logic is similar to NR-U design, i..e, it is network to use the mapping to configure per-LCH CAPC, which is independent of per-LCH priority. At UE side, if there is no CAPC indicated in DCI, the per-LCH CAPC can be used to derive a per-MAC-PDU CAPC (further details on Q2/3/6).

For the CAPC mapped from per-LCH priority, rapp understand the design is to configure a LCH-agnostic mapping between per-LCH priority and CAPC, and then if there is no CAPC indicated in DCI, a per-LCH CAPC can be derived (instead of being configured) from per-LCH priority, and then a per-MAC-PDU CAPC can be derived from the per-LCH CAPC. It is encouraged that proponent of this approach to clarify if there is any misunderstanding here.

For the CAPC mapped from L1 priority, rapp understand the design is to configure a LCH-agnostic mapping between L1 priority and CAPC, and then if there is no CAPC indicated in DCI, UE firstly decide on L1 priority (based per-LCH priority as in legacy), and derive the CAPC based on the derived L1 priority. It is encouraged that proponent of this approach to clarify if there is any misunderstanding here.

The following Q is to check companies’ view on the CAPC mapping:

Question 1: What’s your view on the following options for CAPC mapping?

Option-1: based on PQI, similar to NR-U where 5QI is used to derive the CAPC;

Option-2: based on the priority of SL-LCHs;

Option-3: based on L1 priority;

Option-4: any other (only restricted to RAN2 aspect)

| Company | Option | Comments |
| --- | --- | --- |
| OPPO | Option-1 | This is the way adopted in NR-U. |
| Apple | Keep open on Option-1 and Option-3 (more analysis is required) | Generally, we prefer to reuse the existing mechanism (i.e. NR-U solution based on 5QI). However, we identify below 3 issues if reusing NR-U CAPC framework to SL-U (i.e. option 1): - How to (pre)configure CAPC per logical channel of SL-DRB for Mode 2 UE in IDLE/INACTIVE state and OOC UE? Different from Uu DRB only applied to CONNECTED UE, SL-DRB is also applied to Mode 2 UE in IDLE / INACTIVE/OOC based on static SLRB mapping specified in NR Rel-16 V2X. Then, it is possible that more than 1 PC5 QoS flows are (pre)configured to map to SL-DRB associated with one logical channel, but only 1 PC5 QoS flow is actually mapped to this SL-DRB for on-going transmission. In this case, CAPC (pre)configured per LCG based on multiple QoS flows assumption may be overestimated. 2) How to avoid conflicting between CAPC and L1 priority in SCI which both serve intention of resource grabbing? It is worth mentioning that It is still not clear how LBT works with resource (re)selection for mode 2. As discussed in RAN1, there are multiple companies proposing mode 2 resource (re)selection enhancement with consideration of LBT outcome. If such solutions are adopted (i.e. LBT will be coupled with resource selection), this issue will become more critical. 3) In case of non-standardized PQI NOT reported to NW, how to handle Mode 2 UE in IDLE/INACTIVE and OOC UE? Please note that CAPC is configured only by gNB in NR-U (i.e. not by UE). Then, if IDLE/INACTIVE/OOC UE has a traffic with non-standardized PQI NOT reported to NW yet, it has no way to find CAPC value (pre)configured by gNB per LCH. We think option 3 (i.e. based on L1 priority) can resolve above 3 issues. However, it is a different framework of NR-U. Meanwhile, we don't think companies supporting option 1 can well resolve these issues in this meeting. Thus, we prefer to keep option-1 and option-3 open for further analysis / down-selection. |

## Q2: For SL DRBs, is SL CAPC (pre)configurable or fixed?

The following contributions are discussing the CAPC for SL DRBs:

| R2-2209385 | Proposal 1 For CAPC configuration in SL-U, following NR-U principle, adopt fixed CAPC value for SRB and MAC CE, but configurable CAPC value for DRB. | OPPO |
| --- | --- | --- |
| R2-2209521 | Proposal-4: Discuss the following proposals for the CAPC of SL-U: - Fixed to the highest priority for SL-SRB0, SL-SRB1, SL-SRB4 and SL MAC CEs; - Configurable for SL-SRB2/SL-SRB3 and SL-DRB. | MediaTek Inc. |
| R2-2209973 | Proposal 2: the CAPC of sidelink DRBs are (pre)configured. | Spreadtrum Communications |
| R2-2210250 | Proposal 2 CAPC for a SL DRB can be configured by the gNB via RRC signalling. | Ericsson |
| R2-2210379 | Proposal 4: In SL-U, channel access priority for SL DRBs are (pre)configured by the gNB.. | Xiaomi |
| R2-2209465 | As NR-U design, the CAPC value is fixed to SL MAC CEs and SL SRB0/1/2/3/4, and configurable to SL DRBs. | vivo |
| R2-2209598 | Proposal 2: Similar to NR-U, a LCH for SL DRB has a configurable CAPC. | Huawei, HiSilicon |
| R2-2209679 | The CAPC is configurable for SL-DRB and SL IUC related MAC CE, and the CAPC is fixed for other SL MAC CEs and SL-SRBs. | ZTE Corporation, Sanechips |
| R2-2209742 | Proposal 1：For SL-U, the CAPC of SL-SRBs, SL MAC CEs and SL-DRB can be determined based on the following rule: - For SL-SRB0/ SL-SRB1/ SL-SRB2/ SL-SRB3/ SL-SRB4 and SL MAC CEs, fixed to the highest priority; - For SL-DRBs, If UE is in RRC Connected mode, the CAPC is configured by gNB; If UE is in RRC Inactive or Idle mode, it can acquire the mapping between QoS profile and CAPC by SIB; If UE is OOC, it can acquire the mapping between QoS profile and CAPC by pre-configuration. | CATT |

All the contributions above propose to use configurable CAPC for DRB.

The following Q is to check companies’ view on the configurable CAPC for DRB:

Question 2: Do you agree with for DRB, the CAPC value is (pre)configured as in NR-U?

| Company | Yes/No | Comments |
| --- | --- | --- |
| OPPO | Yes | This is the way adopted in NR-U. |
| Apple | Yes, but ... | No matter option 1 or option 3 in Q1 is agreed, we think CAPC should be able to be (pre)configured but also a fixed table for reference (i.e. similar to principle of NR-U) should be specified. The (pre)configurable CAPC is for system flexibility consideration, and fixed table is for fair coexistence with WiFi. We emphasize that the later one (i.e. fair coexistence) is important during LAA/NR-U discussion due to the strong concern from WiFi alliance. Therefore, we suggest to change the proposal to: for DRB, the CAPC value is (pre)configured based on a fixed mapping table in spec as reference as in NR-U |

## Q3: For SL SRBs and SL MAC CEs, is SL CAPC (pre)configurable or fixed?

The following contributions are discussing the CAPC for SL SRBs and MAC CEs:

| R2-2209385 | Proposal 1 For CAPC configuration in SL-U, following NR-U principle, adopt fixed CAPC value for SRB and MAC CE, but configurable CAPC value for DRB. | OPPO |
| --- | --- | --- |
| R2-2209521 | Proposal-4: Discuss the following proposals for the CAPC of SL-U: - Fixed to the highest priority for SL-SRB0, SL-SRB1, SL-SRB4 and SL MAC CEs; - Configurable for SL-SRB2/SL-SRB3 and SL-DRB. | MediaTek Inc. |
| R2-2209737 | Proposal 1: For SL-U, a fixed CAPC is used for sidelink MAC CEs, fixed to the highest priority (same as in the case of Uu design). | Intel Corporation |
| R2-2209891 | Proposal 5: The logical channels for SL-SRBs are mapped to the highest CAPC priority (lowest CAPC number) Proposal 6: Sidelink MAC CEs are mapped to the highest CAPC priority (lowest CAPC number) | Lenovo |
| R2-2209973 | Proposal 1: the CAPC of SL-SRB0, SL-SRB1, SL-SRB2, SL-SRB3 and SL-SRB4 are all fixed to the highest priority CAPC. Proposal 3: the CAPC of Sidelink CSI reporting MAC CE and Sidelink DRX MAC CE are fixed to the highest priority CAPC. Proposal 4: the CAPC of Inter-UE Coordination Information MAC CE and Inter-UE Coordination Request MAC CE are (pre)configured. | Spreadtrum Communications |
| R2-2210250 | Proposal 1 CAPC is fixed to the highest priority for SL SRBs and MAC CEs. | Ericsson |
| R2-2210256 | Proposal 3: Fixed CAPC is used for SL MAC CEs. FFS on which MAC CEs have highest/lowest priority. Proposal 4: Fixed (highest priority) CAPC is used for SL SRB3. Proposal 5: (Pre)configured CAPC is used for SL SRB0, SL SRB1, SL SRB2 and SL SRB4. | InterDigital |
| R2-2210379 | Proposal 3: In SL-U, channel access priority for all the SL MAC CEs are fixed to the highest channel access priority (lowest CAPC value). Proposal 5: In SL-U, channel access priority for all the SL SRBs are fixed to the highest channel access priority (lowest CAPC value). | Xiaomi |
| R2-2209465 | As NR-U design, the CAPC value is fixed to SL MAC CEs and SL SRB0/1/2/3/4, and configurable to SL DRBs. RAN2 to decide whether to set the highest priority class (i.e. lowest CAPC value) for all the SL SRBs and all the SL MAC CEs. | vivo |
| R2-2209598 | Proposal 3: The CAPC of all SL SRBs are fixed to the highest priority in SL-U. Proposal 4: The CAPC of all SL MAC CEs are fixed to the highest priority in SL-U. | Huawei, HiSilicon |
| R2-2209679 | The CAPC is configurable for SL-DRB and SL IUC related MAC CE, and the CAPC is fixed for other SL MAC CEs and SL-SRBs. | ZTE Corporation, Sanechips |
| R2-2209742 | Proposal 1：For SL-U, the CAPC of SL-SRBs, SL MAC CEs and SL-DRB can be determined based on the following rule: - For SL-SRB0/ SL-SRB1/ SL-SRB2/ SL-SRB3/ SL-SRB4 and SL MAC CEs, fixed to the highest priority; - For SL-DRBs, If UE is in RRC Connected mode, the CAPC is configured by gNB; If UE is in RRC Inactive or Idle mode, it can acquire the mapping between QoS profile and CAPC by SIB; If UE is OOC, it can acquire the mapping between QoS profile and CAPC by pre-configuration. | CATT |

In the above 12 contributions, for SL-SRBs, 2 contributions propose to use configurable CAPC for some of the SRBs (e.g., SRB2/3 or SRB 0/1/2/4) while the others propose to use fixed CAPC for SL-SRB.

The following Q is to check companies’ view on the CAPC for SL-SRB.

Question 3-1: What is your view on the CAPC of SL-SRB?

Option-1: Fixed CAPC for all SL-SRBs;

Option-2: Configurable CAPC for some of the SRBs (please list the SRBs need configurable value), others are fixed;

Option-3: Configurable CAPC for all SRBs

| Company | Option | Comments |
| --- | --- | --- |
| OPPO | Option-1 | Since all the SL-SRBs are specified configurations, i.e., it is natural to rely on specified configuration for CAPC as well. |
| Apple | Option 1 | We tend to agree with OPPO that all SL-SRB should have fixed (or lowest) CAPC value, no matter if Option 1 of Q1 (i.e. PQI based mapping) or Option 3 of Q1 (i.e. L1 priority based mapping) is agreed. |

In the above 12 contributions, for MAC CEs, 2 contributions propose to use configurable CAPC for the IUC related MAC CEs, while the others propose to use fixed CAPC for all MAC CEs.

The following Q is to check companies’ view on the CAPC for SL MAC CE.

Question 3-2: What is your view on the CAPC of SL MAC CE?

Option-1: Fixed CAPC for all SL MAC CE as in NR-U;

Option-2: Configurable CAPC for some MAC CEs(please list the MAC CEs need configurable value), others are fixed;

Option-3: Configurable CAPC for all MAC CEs

| Company | Option | Comments |
| --- | --- | --- |
| OPPO | Option-1 | This is the way adopted in NR-U. |
| Apple | Yes if PQI based mapping is agreed No if L1 priority based mapping is agreed | If PQI based mapping framework is agreed, we think fixed CAPC for SL MAC-CE is the only feasible way. However, if L1 priority based mapping is agreed, we think it can be further discussed because we may introduce an unified mapping framework from L1 priority to CAPC (including MAC PDU with only data, MAC PDU with only MAC-CE, and MAC PDU with both data and MAC-CE). |

## Q4: If SL CAPC is determined based on PQI, e.g., what should be criterion to make a mapping table? what is companies’ thinking on mapping table between CPAC and PQI?

The following contributions are discussing the detailed CAPC mapping rules:

| R2-2209385 | Proposal 2 R2 discusses mapping PQI 90/91/92/93/21/22/23/55/56/57/58 to CAPC priority class 1. Proposal 3 R2 discusses mapping PQI 59/61 to CAPC priority class 3. Proposal 4 R2 discusses mapping PQI 25 to CAPC priority class 2. Proposal 5 R2 discusses mapping PQI 24/26/60 to CAPC priority class 1 or CAPC priority class 2. (PDB based mapping rule) | OPPO |
| --- | --- | --- |
| R2-2209521 | Proposal-3: Discuss the PDB range of PQI for different CAPCs (e.g., less than 50ms, 50-100ms and above 100ms are used). | MediaTek Inc. |
| R2-2209737 | Proposal 2b: RAN2 is proposed to discuss and downselect between Table 1 (PDB based) and Table 2 (classified based on Default Priority level) above to for mapping between CAPC and PQI. | Intel Corporation |

In the above contributions, all contributions propose to use the PDB as the criterion for determining the mapping between CAPC and PQI as in NR-U.

The following Q is to check companies’ view on this issue:

Question 4-1: If SL CAPC is determined based on PQI, do you agree with using the PDB as the criterion for determining the mapping between CAPC and PQI as in NR-U? (if disagree, please clarify what is the preferred criterion)

| Company | Agree/Disagree | Comments |
| --- | --- | --- |
| OPPO | Agree with comments | We agree with using the PDB as the general criterion to determine the mapping between CAPC and PQI as in NR-U. Besides that, we understand there are some exceptional cases where the PDB is not the only reason to be considered when category a 5QI, i.e., the Mission-critical service (5QI-70), so we wonder whether the same consideration is needed in SL-U for PQI 24/26/60. |
| Apple | See comments | We think it is premature to discuss the specific mapping. Of course, both PDB and default priority level can be considered. But we are not sure whether other criteria can also be considered as well. It seems not hurry to start this discussion. What's more, SA2 should be consulted if we go this way. |

And for the detailed CAPC value, in LTE-LAA and NR-U, only the mapping between the first 3 CAPC levels (CAPC 1/2/3) and standardized 5QI have been defined and CAPC 4 is left to other usages (e.g., left to the operator to map or the MAC CE for Recommended bit rate query).

Table: Mapping between Channel Access Priority Classes and 5QI

| CAPC | 5QI |
| --- | --- |
| 1 | 1, 3, 5, 65, 66, 67, 69, 70, 79, 80, 82, 83, 84, 85 |
| 2 | 2, 7, 71 |
| 3 | 4, 6, 8, 9, 72, 73, 74, 76 |
| 4 | - |
| NOTE: lower CAPC value means higher priority - |  |

The following contribution proposes RAN2 to discuss whether to reuse the NR-U principle, i.e., define 3 CAPC levels or define all of the 4 CAPC levels in the Mapping between Channel Access Priority Classes and standardized PQI:

| R2-2209521 | Proposal-2: Discuss if 4 CAPCs or 3 CAPCs are used for the Mapping between Channel Access Priority Classes and PQI. | MediaTek Inc. |
| --- | --- | --- |

The following Q is to check companies’ view on this issue

Question 4-2: If SL CAPC is determined based on PQI, do you agree with defining the 3 CAPC levels in the mapping between CAPC and standardized PQI as in NR-U?

| Company | Agree/Disagree | Comments |
| --- | --- | --- |
| OPPO | Agree | This is the way adopted in NR-U. |
| Apple | Premature to discuss this issue | We think it is premature to discuss the specific mapping. Of course, both PDB and default priority level can be considered. But we are not sure whether other criteria can also be considered as well. It seems not hurry to start this discussion. What's more, SA2 should be consulted if we go this way. |

## Q5: If SL CAPC is determined based on PQI, how to handle non-standardized PQI?

The following contributions are discussing the handling of non-standardized PQI:

| R2-2209737 | Proposal 3: For the case of non-standardized PQI, RAN2 shall follow the same principle as that for non-standardized 5QI i.e to use the CAPC of the standardized PQI which best matches the QoS characteristics of the non-standardized PQI. | Intel Corporation |
| --- | --- | --- |
| R2-2210250 | Proposal 4 A QoS flow corresponding to a non-standardized PQI (i.e. operator specific PQI) should use the CAPC of the standardized PQI which best matches the QoS characteristics of the non-standardized PQI. | Ericsson |
| R2-2210379 | Proposal 2: In SL-U, a QoS flow corresponding to a non-standardized PQI should use the CAPC of the standardized PQI which best matches the QoS characteristics of the non-standardized PQI. | Xiaomi |
| R2-2209465 | For the SL-DRB configuration via dedicated signaling, the gNB should reuse the same rule in NR-U to treat the CAPC value for the PC5 QoS flow with non-standardized PQI, i.e. a PC5 QoS flow corresponding to the non-Standardized PQI should use the CAPC of the standardized PQI which best matches the QoS characteristics of the non-standardized PQI. RAN2 to discuss how to treat the non-standardized PQI value that is not covered by the SL-DRB configurations in the SIB/pre-configuration. | vivo |

All the contributions above propose to reuse NR-U principle, i.e., to use the CAPC of the standardized PQI which best matches the QoS characteristics of the non-standardized PQI.

The following Q is to check companies’ view on the handling of non-standardized PQI

Question 5: If SL CAPC is determined based on PQI, do you agree, as in NR-U, to use the CAPC of the standardized PQI which best matches the QoS characteristics of the non-standardized PQI?

| Company | Yes/No | Comments |
| --- | --- | --- |
| OPPO | Yes |  |
| Apple | Yes for some case (See comments) | First, please note that in NR-U, this principle is for gNB rather than UE. For SL-U, we can discuss two cases: - For CONNECTED UE in mode 1, we think this principle can be agreed because the UE will report QoS profile of the non-standardized PQI to NW. Then, NR-U framework can be reused (i.e. gNB will configure CAPC either in DCI or LCH). - For IDLE/INACTIVE/OOC UE, as we indicated in Q1, we have one SL-U specific issue: if IDLE/INACTIVE/OOC UE has a traffic with non-standardized PQI NOT reported to NW yet, it has no way to find CAPC value (pre)configured by gNB per LCH. In this case, We are not sure whether each company can have consensus that the UE can use this principle to determine the CAPC without notifying gNB. |

## Q6: How to SL CAPC when different SL LCHs, SL MAC CEs and SL SRBs are multiplexed?

The following contributions are discussing the CAPC determination when different SL LCHs, SL MAC CEs and SL SRBs are multiplexed:

| R2-2209385 | Proposal 6 RAN2 to discuss how to decide the CAPC of a MAC PDU in SL-U, e.g., always following the lowest priority CAPC, or allowing exceptional cases where the highest priority CAPC is followed as in NR-U. | OPPO |
| --- | --- | --- |
| R2-2209891 | Proposal 7: Suggest RAN2 to consider above rules for UE to select CAPC of sidelink TB When performing LBT for the transmission of a sidelink TB the UE selects the CAPC as: - If only SL MAC CE(s) are included in the SL TB, the highest priority CAPC of those SL MAC CE(s) is used; or - If SBCCH SDU(s) are included in the TB, the highest priority CAPC is used; - or If Sidelink Control Channel (SCCH) SDU(s) are included in the TB, the highest priority CAPC is used; or - The highest priority CAPC of the SL logical channel(s), e.g. STCH, with MAC SDU multiplexed in the TB is used otherwise. | Lenovo |
| R2-2209973 | Proposal 6: the UE select the CAPC as follows: 1) if only MAC CE(s) are included in the TB, the highest priority CAPC of those MAC CE(s) is used; or 2) if SCCH SDU(s) are included in the TB, the highest priority CAPC is used; or 3) The lowest priority CAPC of the logical channel(s) with MAC SDU multiplexed in the TB is used otherwise. | Spreadtrum Communications |
| R2-2210250 | Proposal 5 When performing Type 1 LBT for the transmission using a configured SL grant or a SL grant obtained via Mode 2 RA, the UE shall select the CAPC as follows: a. If only MAC CE(s) are included in the TB, the highest priority CAPC is used; or b. If SDU(s) of a SRB are included in the TB, the highest priority CAPC is used; or c. The lowest priority CAPC of the logical channel(s) with MAC SDU multiplexed in the TB is used otherwise. | Ericsson |
| R2-2210357 | Proposal 1: For selection of CAPC for PSCCH/PSSCH, reuse similar approach as used for NR-U UL CG PUSCH, i.e., based on priority of LCH data multiplexed in the TB. FFS potential exceptions, e.g. SL SRB. | Nokia, Nokia Shanghai Bell |
| R2-2210379 | Proposal 7: In SL-U, CAPC is selected as the highest priority CAPC if only SL MAC CEs are included in the TB for SL transmissions on configure SL grant and selected SL grant. Proposal 8: In SL-U, CAPC is selected as the highest priority CAPC if only SCCH SDUs are included in the TB for SL transmissions on configure SL grant and selected SL grant. Proposal 9: In SL-U, RAN2 to further discuss how to select the CAPC and whether to introduce any restriction on data multiplexing for SL transmissions on configure SL grant and selected SL grant. | Xiaomi |
| R2-2209891 | Proposal 2: for sidelink UE in mode 2 and for configured SL grant transmissions (mode 1), sidelink Tx UE selects the CAPC value for SL TB transmission according to some predefined rules. | Lenovo |
| R2-2209465 | RAN2 to discuss whether the following rules for per TB transmission can be used for SL-U, in case CAPC is not indicated in the DCI for a Mode-1 UE and in case the UE is configured with Mode-2: If only MAC CE(s) are included in the TB, the highest priority CAPC of those SL MAC CE(s) is used; or If SCCH SDU(s) are included in the TB, the highest priority CAPC of the SCCH(s) is used; or The lowest priority CAPC of the logical channel(s) with MAC SDU multiplexed in the TB is used, otherwise. | vivo |
| R2-2209598 | Proposal 5: The CAPC determination of a TB in SL-U could follow existing NR-U mechanism with adaption for sidelink: If SL MAC CE(s) are included in the TB, the highest priority CAPC is used; or If SCCH SDU(s) are included in the TB, the highest priority CAPC is used; or The lowest priority CAPC of the logical channel(s) multiplexed with SL MAC SDU in the TB is used otherwise. | Huawei, HiSilicon |
| R2-2209679 | When selecting the CAPC for performing Type 1 LBT for the transmission of a sidelink TB, the similar principle used for uplink shall be followed. | ZTE Corporation, Sanechips |
| R2-2209742 | Proposal 3：For resource allocation mode 1, if CAPC is indicated by network, UE follows the indicated CAPC. Proposal 4: For resource allocation mode 2 or resource allocation mode 1 but CAPC is not indicated by network, UE determines the CAPC based on the contents of the SL TB as follows: - If only SL MAC CE(s) are included in the SL TB, the highest priority SL CAPC of those SL MAC CE(s) is used; or - If SCCH SDU(s) are included in the SL TB, the highest priority SL CAPC is used; or - The lowest priority SL CAPC of the SL logical channel(s) with MAC SDU multiplexed in the SL TB is used otherwise. | CATT |

Almost all the contributions above propose to reuse NR-U principle, i.e., if CAPC is not indicated by network (via DCI), UE determines the CAPC based on the contents of the SL TB as follows:

- If only SL MAC CE(s) are included in the SL TB, the highest priority SL CAPC of those SL MAC CE(s) is used; or

- If SCCH SDU(s) are included in the SL TB, the highest priority SL CAPC is used; or

- The lowest priority SL CAPC of the SL logical channel(s) with MAC SDU multiplexed in the SL TB is used otherwise.

The following Q is to check companies’ view on the determination of CAPC of the TB when the CAPC value is not indicated in SL grant (DCI).

Question 5: Do you agree to reuse NR-U rules as follows for determining the CAPC of the SL TB when the CAPC is not indicated in the DCI?

- If only SL MAC CE(s) are included in the SL TB, the highest priority SL CAPC of those SL MAC CE(s) is used; or

- If SCCH SDU(s) are included in the SL TB, the highest priority SL CAPC is used; or

- The lowest priority SL CAPC of the SL logical channel(s) with MAC SDU multiplexed in the SL TB is used otherwise.

| Company | Yes/No | Comments (if No, please clarify what is the difference compared to NR-U scheme) |
| --- | --- | --- |
| OPPO | Yes |  |
| Apple | Yes if PQI based mapping is agreed No if L1 priority based mapping is agreed | If PQI based mapping framework is agreed, we tend to reuse same mechanism of NR-U. However, if L1 priority based mapping is agreed, we think it can be further discussed because we may introduce an unified mapping framework from L1 priority to CAPC (including MAC PDU with only data, MAC PDU with only MAC-CE, and MAC PDU with both data and MAC-CE). This is much simpler than NR-U framework. |

# Conclusion

We have the following proposals:

Proposal 1	xxx.

# Reference

- R2-2209385	Discussion on CAPC definition in SL-U	OPPO	discussion	Rel-18	NR_SL_enh2

- R2-2209465	On CAPC in SL-U	vivo	discussion

- R2-2209521	Channel Access Priority Classes for SL-U	MediaTek Inc.	discussion	NR_SL_enh2

- R2-2209598	Discussion on CAPC for SL-U	Huawei, HiSilicon	discussion	Rel-18	NR_SL_enh2

- R2-2209612	Discussion on RAN2 aspects in SL-U	LG Electronics France	discussion	Rel-18	NR_SL_enh2

- R2-2209678	Discussion on RAN2 scope of SL-U	ZTE Corporation, Sanechips	discussion	Rel-18	NR_SL_enh2

- R2-2209679	Discussion on CAPC definition and consistent sidelink LBT failure handling	ZTE Corporation, Sanechips	discussion	Rel-18	NR_SL_enh2

- R2-2209737	On CAPC for SL-U	Intel Corporation	discussion	Rel-18	NR_SL_enh2

- R2-2209742	Consideration on CAPC for SL-U	CATT	discussion	Rel-18	NR_SL_enh2

- R2-2209761	Control plane aspects of sidelink on unlicensed spectrum (SL-U)	Apple	discussion	Rel-18	NR_SL_enh2

- R2-2209891	Discussion on channel access priority for NR SL-U	Lenovo	discussion	Rel-18	NR_SL_enh2-Core

- R2-2209973	Consideration on channel access priority in SL-U	Spreadtrum Communications	discussion	Rel-18

- R2-2210250	CAPC table and MAC multiplex rules	Ericsson	discussion	Rel-18	NR_SL_enh2

- R2-2210256	CAPC and COT sharing for SL Unlicensed	InterDigital	discussion	Rel-18	NR_SL_enh2

- R2-2210280	Discussion on sidelink CAPC	Qualcomm India Pvt Ltd	discussion

- R2-2210357	On channel access priority class and HARQ feedback	Nokia, Nokia Shanghai Bell	discussion	NR_SL_enh2

- R2-2210379	Discussion on channel access for sidelink operation on unlicensed spectrum	Xiaomi	discussion	NR_SL_enh2

- R2-2210552	SL CAPC	Samsung Research America	discussion	Rel-18	NR_SL_enh2