3GPP TSG RAN WG2#119bis	R2-2210791, revision of R2-2209494

e-Meeting, Oct., 2022

Title:	Reply LS on active TCI state list for UL TCI

Response to:	R4-2214972

Release:	Release 17

Work Item:	NR_feMIMO-Core

Source:	OPPO(To be RAN2)

To:	RAN4

Cc:	RAN1

Contact Person:

Name:	Zonda Du

E-mail Address:	duzhongda@oppo.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

- Overall Description:

RAN2 discussed the pathloss reference signal issue in R4-2214972 and come to the conclusion that it is up to network’s implementation that UE need maintain only up to 4 pathloss reference signals. More specifically RAN2 provide answers to the 3 questions as below:

Question 1: What is the relationship between  qd and active UL (or joint) TCI state list? How does network configure the RS resource indexes qd ?

Answer 1:  Pathloss reference signal with index qd is assigned via DCI along with UL(or joint) TCI state indirectly since there is one pathloss reference signal is configured per UL(or joint) TCI state. The codepoints in DCI refer to activated UL(or joint) TCI states by MAC CE.

Question 2: Is UE expected to maintain the pathloss RSs of all the pathloss reference RS in the active UL (or joint) TCI list?

Answer 2:  Yes

Question 3: What is the UE behaviour when number of active UL (or joint) TCI states is larger than 4 if corresponding pathloss RSs need to be maintained?

Answer 3: No extra UE behaviour need be defined for such case as some of the assigned reference signals should be identical by network’s implementation.

2. Actions:

To RAN4:

RAN2 respectfully asks RAN4 to take above answers into account in further work.

3. Date of Next TSG-RAN WG2 Meetings:

3GPP RAN2#120	Nov.14~Nov.18 2022	Toulouse, France

3GPP RAN2#121	Feb.27~March.03 2023	Athens, Greece