3GPP TSG-RAN WG2 Meeting #120	R2-2212382

Toulouse, France, 14 – 18 November 2022

Agenda item:	8.18

Source:	Nokia, Nokia Shanghai Bell

Title:	MT-SDT procedure

WID/SID:	NR_MT_SDT-Core - Release 18

Document for:	Discussion and Decision

# 1	Introduction

RAN has agreed WI [1] for Mobile Terminated-Small Data Transmission (MT-SDT) with the following objectives:

Specify the support for paging-triggered SDT (MT-SDT) [RAN2, RAN3] - MT-SDT triggering mechanism for UEs in RRC_INACTIVE, supporting RA-SDT and CG-SDT as the UL response; - MT-SDT procedure for initial DL data reception and subsequent UL/DL data transmissions in RRC_INACTIVE. Note: Data transmission in DL within paging message is not in scope of this WI.

In this contribution we discuss the objectives further.

# 2	Discussion

It is expected that MT SDT procedure will reduce signalling overhead and UE power consumption by not transitioning the UE to RRC_CONNECTED and reducing latency by allowing fast transmission of (small and infrequent) packets. MT EDT procedure has been specified for LTE. MT-EDT is intended for a single downlink data transmission during the random access procedure. The same procedure is not suitable for MT-SDT since subsequent UL/DL transmissions are supported in REL-17 SDT. Also, transmission using PUR is specified in LTE which allows one uplink transmission from RRC_IDLE using a preconfigured uplink resource without performing the random access procedure. However, paging initiated PUR procedure is not supported which is required for MT-SDT.  According to WI [1] both RA-SDT and CG-SDT should be supported as the UL response. In REL-17 in MO-SDT the UE can be configured for RA-SDT or CG-SDT or both and SDT is configured in dedicated manner with RRCRelease message. We assume that the same applies for MT-SDT. In addition. according to the WI [1] objectives paging-triggered MT SDT procedure shall be specified. Based on that we propose:

Proposal 1: The UE can be configured for RA-SDT or CG-SDT or both for MT-SDT.

Proposal 2: MT-RA-SDT and MT-CG-SDT is configured with RRC Release message

Proposal 3: Multiple UL/DL data transmissions are supported in MT-SDT.

Proposal 4: Paging triggers the UE to initiate RA-SDT or CG-SDT MT-SDT procedure.

In REL-17 MO-SDT is enabled on a radio bearer basis and is initiated by the UE only if less than a configured amount of UL data awaits transmission across all radio bearers for which SDT is enabled, the DL RSRP is above a configured threshold, and a valid SDT resource is available. We assume that some conditions are needed also for MT-SDT, but it needs to be further discussed whether the conditions are the same than for MO-SDT or different.

Proposal 5: MT-SDT procedure is initiated in case condition(s) for MT-SDT are fulfilled

Proposal 6: RAN2 to discuss which conditions are needed for MT-SDT initiation e.g. RSRP threshold, UL data amount threshold, TA condition for CG-SDT etc.

# 3	Conclusion

The following is proposed:

Proposal 1: The UE can be configured for RA-SDT or CG-SDT or both for MT-SDT.

Proposal 2: MT-RA-SDT and MT-CG-SDT is configured with RRC Release message

Proposal 3: Multiple UL/DL data transmissions are supported in MT-SDT.

Proposal 4: Paging triggers the UE to initiate RA-SDT or CG-SDT MT-SDT procedure.

Proposal 5: MT-SDT procedure is initiated in case condition(s) for MT-SDT are fulfilled

Proposal 6: RAN2 to discuss which conditions are needed for MT-SDT initiation e.g. RSRP threshold, UL data amount threshold, TA condition for CG-SDT etc.

# Reference

[1] RP-213583, New WI: Mobile Terminated-Small Data Transmission (MT-SDT) for NR, ZTE Corporation