3GPP TSG RAN WG2#120	R2-2211160

Toulouse, 14th - 18th November, 2022

3GPP TSG-RAN WG3 Meeting #117-bis-e	R3-226003

Online, Oct 10th – Oct 18th 2022

Title:	LS on inter-RAT SHR and SPCR

Release:	Rel-18

Work Item:	NR_ENDC_SON_MDT_enh2-Core

Source:	RAN3

To:	RAN2

Cc:	-

Contact Person:

Name: 		Shankar Krishnan

E-mail Address:	shakrish@qti.qualcomm.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall description:

RAN3 discussed inter-RAT Successful Handover Report (SHR) and Successful PSCell Change Report (SPCR) and have the following questions:

- Inter-RAT SHR

RAN3 agreed to support T310 and T312 related triggers for inter-RAT SHR from NR to LTE. But there was no consensus on whether to also support T304 trigger for inter-RAT SHR from NR to LTE. Considering this might impact RAN2 specifications (including LTE specifications), RAN3 has the following questions:

- Q1. Is RAN2 planning to impact LTE specifications to support inter-RAT SHR?

- Q2. Whether T304 trigger for inter-RAT SHR from NR to LTE is to be supported?

- Q3. If yes to Q2, whether the inter-RAT SHR is always encoded in source RAT format or can be encoded based on the RAT format which generates the inter-RAT SHR trigger condition (e.g., inter-RAT SHR encoded in NR format for T310/T312 triggers and in LTE format for T304 triggers for inter-RAT HO from NR to LTE)?

Further RAN3 is not sure on the retrieval and reporting of inter-RAT SHR and would like to ask RAN2 the following question:

- Q4. If yes to Q2, and if inter-RAT SHR is collected due to T304 triggers (configured by target LTE node), what is RAN2’s preference on the following two options?

- Option 1: It is sufficient for UE to report the inter-RAT SHR once UE is back to NR

- Option 2: The LTE node should have the capability to retrieve the inter-RAT SHR

- RAN3 further discussed on the potential contents for inter-RAT successful handover and agreed on the following:

- Source NR cell information

- Target LTE cell information

- Measurement results for source, target and neighbours

- Cause to indicate which inter-RAT SHR triggering condition was met

- UE location Information

Considering there might be parallel discussion in RAN2, RAN3 would like to check the following:

- Q5: Can RAN2 confirm the support for above parameters for inter-RAT SHR from NR to LTE? Whether the existing IEs defined in Rel-17 for intra-NR SHR can be reused is up to RAN2 decision.

- Successful PSCell Change Report (SPCR)

- RAN3 also discussed different aspects related to SPCR and have the following questions to RAN2:

- Q6. Whether the SPCR can be stored at the UE and sent later to the gNB or is sent immediately after the successful PSCell change or addition?

- Q7. Which node (MN or SN) retrieves the SPCR from the UE?

- Q8. Which node (MN or SN) sends the Successful PSCell Change/Addition configuration to the UE?

- 2. Actions:

RAN3 respectfully asks RAN2 to provide responses to Q1-Q8 and provide necessary specification support.

3. Date of next TSG RAN WG3 meetings:

RAN3#118-e	14th - 18th Nov 2022	Toulouse, France