3GPP TSG-RAN WG2 Meeting #121	R2-2301901

Athens, Greece, Feb 27th- Mar 3th, 2023

Agenda item:	9.1

Source: 	Vice Chairman (ZTE Corporation)

Title:	Report from Break-out session on NR-NTN, IoT-NTN and RedCap

Document for:	Approval

General

Recording of voice or video at meetings is not used in 3GPP. This applies also to this e-Meeting. At this e-Meeting, no specific actions are taken to prevent the recording of web conferences. Companies that have concerns related to recordings, if any, may express those by email in the main meeting organizational thread [AT121][000]

Organizational

- All organization emails and notes will be shared over the following email discussion throughout the meeting:

[AT121][100] Organizational – NR-NTN, IoT-NTN and RedCap session (RAN2 VC)

Scope:

    - Share plans for the meeting and list of ongoing email discussions for the sessions related to NR-NTN, IoT-NTN and RedCap

    - Share meetings notes and agreements for review and endorsement

Schedule/Plan

|  | Main room | Brk 1 room | Brk 2 room | Brk 3 room |
| --- | --- | --- | --- | --- |
| Monday |  |  |  |  |
| 09:00 – 10:30 | [1], [2], [3] 10-15 min NR1516 CP (Johan) - 5.1.1, 5.1.3 (except URLLC CP CRs) NR17 (Johan) - 6.1.1, 6.13 -- General -- 71GHz -- ePowSav | Breakout to start after NR common items in the main room: NR151617 UP (Diana) - 5.1.2, 6.1.2 - 6.4 SDT, 6.12 RACH - 6.3 URLLC IIOT - other early item, if any | Breakout to start after formal opening of meeting in main room: NRLTE1516 (Kyeongin) - 4.2, 5.2 NR17 (Kyeongin). - 6.10 |  |
| 11:00 – 13:00 |  |  |  |  |
| 14:30 – 16:30 | NR17 (Johan) - 6.1.1, 6.1.3 -- continuation from morning if needed -- DCCA -- MUSIM -- Slicing | NR18 MT-SDT [0.5] (Diana) - 8.18 NR18 UAV [0.5] (Diana) - 8.8 | NR18 SL evolution [1] (Kyeongin) - 8.15 |  |
| 17:00 – 19:00 | NR17 (Johan) - 6.11 feMIMO, - 8.21 R18 MIMO incoming LS (if time) NR1516 - URLLC CP NR17 - 6.1.1, 6.1.3 -- eIAB (incl MAC BAP) -- QoE - 5.1.1 5.1.2 6.1.1 6.1.3 Continuation others if time. | NR18 NTN enh [1] (Sergio) - 8.7.1 - 8.7.3 - 8.7.4 | NRLTE1516 Pos (Nathan) - 4.3, 5.3 NR17 Pos (Nathan) - 6.7 |  |
| Tuesday |  |  |  |  |
| 08:30 – 10:30 | NR18 Mobile IAB [0.5] (Johan) - 8.12 NR17 Maint (Sergio) - 6.8 RedCap | EUTRA16+ (Tero) - 4.1 - 7.1: NB-IoT corrections (R2-2300845, R2-2301310) IF time allows: - 4.1: CIO for inter-RAT (R2-2301131, R2-2301132) NR18 MUSIM [0.5] (Tero) NR18 MUSIM [0.5] (Tero) - 8.17.1: R2-2300902: Work plan - 8.17.2: R2-2300773: Report of [Post119bis-e][212][MUSIM] IF time allows: - 8.17.3: R2-2301778 (MUSIM gap coordination), R2-2300754 (MUSIM band conflicts) | NR17 (Nathan) - 6.7 Pos - 6.5 SL relay |  |
| 11:00 – 13:00 | NR17 Maint (Sergio) - 7.2 Iot NTN - 6.6 NR NTN | NR18 eQoE [1] (Tero) - 8.14.1: Work plan, running CRs - 8.14.2: QoE for MBS in IDLE/INACTIVE - 8.14.5: QoE continuity for intra-5GC inter-RAT HO (R2-2300356, R2-2301756) | NR18 Pos [2] (Nathan) - 8.2 |  |
| 14:30 – 16:30 | NR18 Other [2] (Johan) - 8.21 NR17 continue if time (Johan) - TBD | NR18 XR [2] (Tero) - 8.5.1: LSs, TR updates, SA2/SA4 status updates - 8.5.2.1: PDU set information use, UL jitter | NR18 Pos [2] (Nathan) - 8.2 |  |
| 17:00 – 19:00 | NR18 feMob [2] (Johan) - 8.4 | NR18 XR [2] (Tero) - 8.5.2.4: Protocol stack impacts of PDU sets, in-sequence delivery - 8.5.2.2: PDU set prioritization and RAN2 impacts - 8.5.2.3: PSII, RAN2 impacts | NR17 (Nathan) - overflow from morning session ALT: NR17 (Kyeongin) - 6.10 NR18 SL relay [1.5] (Nathan) - 8.9 |  |
| Wednesday |  |  |  |  |
| 08:30 – 10:30 | NR18 feMob [2] (Johan) - 8.4 | NR18 NCR [0.5] (Sasha) - 8.1 NR17 MBS (Dawid) - 6.2 | NR18 IDC [1] (Yi) - 8.10 |  |
| 11:00 – 13:00 | NR18 Network Energy Saving [1] (Diana) - 8.3 | NR17 MBS (Dawid) - 6.2 NR 18 MBS [0.75] (Dawid) - 8.11 | NR17 SONMDT (HuNan) - 6.9 |  |
| 14:30 – 16:30 | NR18 Other [2] - 8.21 URLLC (Diana) - 8.21 Others (Johan) | NR18 RedCap [1] (Mattias) - 8.19 | NR18 SONMDT [1] (HuNan) - 8.13 |  |
| 17:00 – 19:00 | NR18 AIML [1] (Johan) - 8.16 | L18 IoT-NTN [1] (Sergio) - 8.6.1 - 8.6.2.2: R2-2301951 (offline 101) - 8.6.3 | NR18 SL relay [1.5] (Nathan)- - 8.9 |  |
| Thursday |  |  |  |  |
| 08:30 – 10:30 | CB NR151617 (Johan) | CB Diana | CB Kyeongin |  |
| 11:00 – 13:00 | NR18 TEI [0.5] - 8.20 Pos (Nathan) - 8.20 Other (Johan) | CB Diana | CB Kyeongin |  |
| 14:30 – 16:30 | CB NR17 (Johan) | NR18 NTN enh (Sergio) (1h) - 8.7.4.2 Tero NR QoE [1] (1h) (Tero) - 8.14.4: NR-DC support for QoE - 8.14.3: Rel-17 QoE leftovers | CB Nathan |  |
| 17:00 – 19:00 | CB NR17 (Johan) CB NR18 (Johan) | CB Tero NR18 XR [2] (Tero) - CBs from 8.5.2.X If time allows: - 8.5.3: DRX for XR power saving - 8.5.4: BSR and XR capability enhancements | CB Nathan |  |
| Friday |  |  |  |  |
| 08:30 – 10:30 | CB Dawid, Sasha, Tero TBD | CB Mattias (08:30 – 09:00) - CB eRedCap R17 Maint (Sergio) CB NR NTN - report from 103, 113, 114, 115, 116 CB IoT NTN - report from 111, 112 CB RedCap (might continue after lunch) - report from 105, 106, 107, 108, 109, 110 | CB Nathan, Kyeongin TBD |  |
| 11:00 – 13:00 | CB Johan | CB NR18 NTN enh (Sergio) - 8.7.4.1.2: R2-2301953 (offline 104) CB L18 IoT-NTN (Sergio) - 8.6.4: R2-2301952 (offline 102) - 8.6.3 (if time allows) | CB Yi CB HuNan |  |
| 14:30 – 16:00 | CB Johan | CB Tero - CB: 4.1 and 7.1 (if any) - CB: 8.17.2 (if any) - CBs from any other AIs | CB HuNan |  |
| 16:00 – 17:00 | CB and conclusion (Johan) |  |  |  |

List and status of offline email discussions

NOTE:  No offline email discussions will be kicked off before Monday Feb 27th, 09:00 EET

[AT121][101][IoT NTN enh] GNSS operation (CATT)

Initial scope: Discuss proposals in 8.6.2.2

Initial intended outcome: Summary of the offline discussion with e.g.:

- List of proposals for agreement (if any)

- List of proposals that require online discussions

Deadline for companies' feedback:  Wednesday 2023-03-01 06:00 EET

Deadline for rapporteur's summary (in R2-2301951): Wednesday 2023-03-01 12:00 EET

Status: Closed

[AT121][102][IoT NTN enh] Discontinuous coverage (Mediatek)

Initial scope: Discuss proposals in 8.6.4

Initial intended outcome: Summary of the offline discussion with e.g.:

- List of proposals for agreement (if any)

- List of proposals that require online discussions

Deadline for companies' feedback:  Wednesday 2023-03-01 06:00 EET

Deadline for rapporteur's summary (in R2-2301952): Wednesday 2023-03-01 12:00 EET

Status: Closed

[AT121][103][NR NTN] Neighbour cell measurements (Ericsson)

Initial scope: Initial F2F discussion on R2-2300910 and R2-2301137

Initial intended outcome: Oral summary of the initial F2F discussion

F2F offline time and location:  Tuesday 2023-02-28 10:30-11:00 EET, Main room

Updated scope: continue the discussion neighbour cell measurements

Updated intended outcome: Summary of the offline and/or agreeable CRs

F2F offline time and location:  Wednesday 2023-03-01 16:30-17:00 EET, Brk 1 room (Aphrodite III&IV)

Deadline for companies' feedback:  Thursday 2023-03-02 22:00 EET

Deadline for rapporteur's summary (in R2-2301972): Friday 2023-03-03 08:00 EET

Status: Closed

[AT121][104][NR NTN enh] NTN-NTN cell reselection (ZTE)

Initial scope: continue the discussion on trigger for measurements and cell reselection criteria enhancements based on the selected papers above

Initial intended outcome: Summary of the offline discussion with e.g.:

- List of proposals for agreement (if any)

- List of proposals that require online discussions

Deadline for companies' feedback:  Thursday 2023-03-02 22:00 EET

Deadline for rapporteur's summary (in R2-2301953): Friday 2023-03-03 08:00 EET

Status: Closed

[AT121][105][RedCap] RedCap & SDT (ZTE)

Scope: Discuss the Stage 3 details of option 2, including the possible impact on mobility.

Intended outcome: Summary of the offline and/or agreeable CRs

F2F offline time and location:  Wednesday 2023-03-01 16:30-17:00 EET, Brk 2 room (Omikron 2)

Deadline for companies' feedback:  Thursday 2023-03-02 22:00 EET

Deadline for rapporteur's summary (in R2-2301954): Friday 2023-03-03 08:00 EET

Status: Closed

[AT121][106][RedCap] CP corrections (QC)

Scope: Continue the discussion on changes 2 and 3 in R2-2300191

Intended outcome: Agreeable CR (and offline summary if needed)

Deadline for companies' feedback:  Thursday 2023-03-02 22:00 EET (F2F offline is also invited)

Deadline for revised CR (in R2-2301955) and rapporteur's summary (in R2-2301955): Friday 2023-03-03 08:00 EET

Status: Closed

[AT121][107][RedCap] RACH configuration (vivo)

Scope: Continue the discussion on R2-2300312

Intended outcome: Agreeable CR

Deadline for companies' feedback:  Thursday 2023-03-02 22:00 EET (F2F offline is also invited)

Deadline for revised CR (in R2-2301957): Friday 2023-03-03 08:00 EET

Status: Closed

[AT121][108][RedCap] RRC config & NeedForGaps (ZTE)

Scope: Continue the discussion on R2-2301316 and R2-2301688

Intended outcome: Agreeable CRs

Deadline for companies' feedback:  Thursday 2023-03-02 22:00 EET (F2F offline is also invited)

Deadline for revised CRs (in R2-2301958 and R2-2301960): Friday 2023-03-03 08:00 EET

Status: Closed

[AT121][109][RedCap] eDRX (Oppo)

Scope: Continue the discussion on the eDRX related CRs

Intended outcome: offline summary (and agreeable CRs, if any)

Deadline for companies' feedback:  Thursday 2023-03-02 22:00 EET

Deadline for rapporteur's summary (in R2-2301961): Friday 2023-03-03 08:00 EET

Status: Closed

[AT121][110][RedCap] BWP switch for SDT (vivo)

Scope: Continue the discussion on R2-2300313

Intended outcome: offline summary / agreeable CR

Deadline for companies' feedback:  Thursday 2023-03-02 22:00 EET

Deadline for offline summary / revised CR (in R2-2301962): Friday 2023-03-03 08:00 EET

Status: Closed

[AT121][111][IoT NTN] UP corrections (Mediatek)

Scope: Discuss the MAC CRs

Intended outcome: offline summary (and agreeable CRs, if any)

Deadline for companies' feedback:  Thursday 2023-03-02 22:00 EET

Deadline for rapporteur's summary (in R2-2301964): Friday 2023-03-03 08:00 EET

Status: Closed

[AT121][112][IoT NTN] CP corrections (Huawei)

Scope: Discuss the CP CRs

Intended outcome: offline summary (and agreeable CRs, if any)

Deadline for companies' feedback:  Thursday 2023-03-02 22:00 EET

Deadline for rapporteur's summary (in R2-2301965): Friday 2023-03-03 08:00 EET

Status: Closed

[AT121][113][NR NTN] (LG)

Scope: Continue the discussion on R2-2300124 and R2-2301636

Intended outcome: Agreeable merged CR

Deadline for companies' feedback:  Thursday 2023-03-02 22:00 EET

Deadline for revised CR (in R2-2301974): Friday 2023-03-03 08:00 EET

Status: Closed

[AT121][114][NR NTN] reply LS to RAN4 (Nokia)

Scope: Discussion the reply LS to RAN4 starting from R2-2300471

Intended outcome: Draft LS

Deadline for companies' feedback:  Thursday 2023-03-02 22:00 EET

Deadline for draft LS in R2-2301966: Friday 2023-03-03 08:00 EET

Status: Closed

[AT121][115][NR NTN] 38.304 CR on relaxed measurements (Huawei)

Scope: continue the discussion on R2-2300235

Intended outcome: Agreeable CR

Deadline for companies' feedback:  Thursday 2023-03-02 22:00 EET

Deadline for agreeable CR (in R2-2301970): Friday 2023-03-03 08:00 EET

Status: Closed

[AT121][116][NR NTN] CP corrections (Samsung)

Scope: Discuss the CRs in R2-2300125, R2-2300234 and R2-2300236

Intended outcome: offline summary (and agreeable CRs, if any)

Deadline for companies' feedback:  Thursday 2023-03-02 22:00 EET

Deadline for rapporteur's summary (in R2-2301973): Friday 2023-03-03 08:00 EET

Status: Closed

## 6.6	NR Non-Terrestrial Networks (NTN)

(NR_NTN_solutions-Core; leading WG: RAN2; REL-17; WID: RP-211557)

Tdoc Limitation: 3 tdocs

### 6.6.1	General and Stage 2 corrections

LSs and Stage 2 corrections.

Incoming LSs

Validity of assistance info

R2-2300018	Reply LS on validity of assistance information (R1-2212984; contact: OPPO)	RAN1	LS in	Rel-17	NR_NTN_solutions-Core	To: RAN2

- Noted (already considered in the previous meeting)

IOT bit for inter-satellite measurements

R2-2300043	LS to RAN2 on inter-operability testing (IOT) bit for inter-satellite measurement (R4-2220425; contact: MediaTek)	RAN4	LS in	Rel-17	NR_NTN_solutions-Core	To: RAN2

- Noted

- Discuss how to reflect this during the CP discussion

R2-2300271	[Draft] Reply LS on inter-operability testing (IOT) bit for inter-satellite measurement	MediaTek Inc.	LS out	Rel-17	NR_NTN_solutions	To:RAN4

Enhanced cell reselection requirements

R2-2300044	LS on capability description for enhanced cell reselection requirements in NTN (R4-2220427; contact: Nokia)	RAN4	LS in	Rel-17	NR_NTN_solutions-Core	To: RAN2

- Noted

R2-2300471	Draft Response LS on Enhanced Cell Reselection Requirements for NTN	Nokia, Nokia Shanghai Bell	LS out	Rel-17	NR_NTN_solutions-Core	To:RAN4

- Revised in R2-2301966

R2-2301966	Response LS on Enhanced Cell Reselection Requirements for NTN	Nokia, Nokia Shanghai Bell	LS out	Rel-17	NR_NTN_solutions-Core	To:RAN4

- Approved

[AT121][114][NR NTN] reply LS to RAN4 (Nokia)

Scope: Discussion the reply LS to RAN4 starting from R2-2300471

Intended outcome: Draft LS

Deadline for companies' feedback:  Thursday 2023-03-02 22:00 EET

Deadline for draft LS in R2-2301966: Friday 2023-03-03 08:00 EET

Measurement relaxation

R2-2300057	Reply LS on enhanced cell reselection requirements (R4-2220741; contact: Huawei)	RAN4	LS in	Rel-17	NR_NTN_solutions-Core	To: RAN2

- Noted

Stage 2 corrections

R2-2300123	Correction on Stage-2 descriptions for NR NTN	vivo, Nokia, Nokia Shanghai Bell	CR	Rel-17	38.300	17.3.0	0611	-	F	NR_NTN_solutions-Core

- HW thinks the second change is not needed. Oppo agrees

- 1st change agreed

- Revised in R2-2301967

R2-2301967	Correction on Stage-2 descriptions for NR NTN	vivo, Nokia, Nokia Shanghai Bell	CR	Rel-17	38.300	17.3.0	0611	-	F	NR_NTN_solutions-Core

- Agreed

R2-2300166	NTN Stage-2 correction	OPPO	CR	Rel-17	38.300	17.3.0	0612	-	F	NR_NTN_solutions-Core

- Ericsson thinks the original text is ok.

- Intel thinks this is not needed

- Apple thinks the current wording is ok

- ZTE thinks we should follow RAN1 agreement

- Come Back Friday

- Ericsson is ok in principle with the change but would like to further check the wording and come back to the next meeting

- Intel thinks that “K_mac can be even 0 if feeder link delay is fully compensated by Common TA broadcasted by the gNB” and that current spec is ok

- Postponed to the next meeting

R2-2301445	Corrections to 38.300 related to Section Scheduling and Timing 	THALES	CR	Rel-17	38.300	17.3.0	0630	-	F	NR_NTN_solutions-Core

- Samsung thinks the figure change is problematic and would like not to change this

- Intel thinks changes are editorial but can be agreed if we fix the coversheet

- Ericsson/QC think we don’t need the CR

- Not pursued

R2-2300881	Clarification on support of TN NTN mobility during RRC_INACTIVE	Qualcomm Incorporated	CR	Rel-17	38.300	17.3.0	0622	-	F	NR_NTN_solutions-Core

R2-2301139	Corrections on neighboring cell measurement	ZTE Corporation, Sanechips	CR	Rel-17	38.300	17.3.0	0626	-	F	NR_NTN_solutions-Core

R2-2301685	Corrections on the description related to the timing advance pre-compensation	CATT	CR	Rel-17	38.300	17.3.0	0636	-	F	NR_NTN_solutions-Core

### 6.6.2	UP corrections

R2-2300124	Corrections to NR Non-Terrestrial Networks (NTN) for TS 38.321	vivo	CR	Rel-17	38.321	17.3.0	1510	-	F	NR_NTN_solutions-Core

- LG has a similar CR for the first change

- QC/Mediatek agrees with the intention

- Apple prefer LG’s wording

- Continue in offline 113

R2-2301636	Clarification on HARQ feedback transmission after SPS activation	LG Electronics Inc.	CR	Rel-17	38.321	17.3.0	1560	-	F	NR_NTN_solutions-Core

- Continue in offline 113

[AT121][113][NR NTN] (LG)

Scope: Continue the discussion on R2-2300124 and R2-2301636

Intended outcome: Agreeable merged CR

Deadline for companies' feedback:  Thursday 2023-03-02 22:00 EET

Deadline for revised CR (in R2-2301974): Friday 2023-03-03 08:00 EET

R2-2301974	Corrections to NR NTN for 38.321	LG Electronics Inc., vivo	CR	Rel-17	38.321	17.3.0	1560	1	F	NR_NTN_solutions-Core

- Revised in R2-2301983 to remove “in a non-terrestrial network”

R2-2301983	Corrections to NR NTN for 38.321	LG Electronics Inc., vivo	CR	Rel-17	38.321	17.3.0	1560	2	F	NR_NTN_solutions-Core

- Agreed

R2-2300729	Clarification on CG-SDT Timer in NTN	Apple	CR	Rel-17	38.321	17.3.0	1522	-	F	NR_NTN_solutions-Core

- Nokia is not sure this is needed

- IDC thinks that if periodicity is properly configured there is no problem. Intel/Mediatek agree

- Apple is not sure existing periodicity values can solve the problem. HW thinks this is the case. IDC thinks the value range is huge and it gives the NW all the flexibility. Mediatek thinks this covers also the RTT for GEOs

- Apple thinks the behaviour is similar for the PUR timer. IDC thinks the question is about the difference between SDT and the regular retx timer and why we need a specific handling for SG-SDT

- Not pursued

R2-2301704	Preamble Group Selection for TA Reporting	Google Inc.	CR	Rel-17	38.321	17.3.0	1563	-	F	NR_NTN_solutions-Core

- LG thinks this is already covered by the specs. Google this is not yet covered

- IDC thinks this is not necessary. Nokia/QC agree.

- Not pursued

R2-2301658	Corrections on MAC procedure upon validity timer expiry	Nokia, Nokia Shanghai Bell	CR	Rel-17	38.321	17.3.0	1561	-	F	NR_NTN_solutions-Core

R2-2301157	Clarification on UE behaviour when the validity timer expires	Huawei, HiSilicon	discussion	Rel-17	NR_NTN_solutions-Core

### 6.6.3	CP corrections

IOT bit for inter-satellite measurements

R2-2300269	IOT bit for inter satellite measurement (38.306)	MediaTek Inc.	CR	Rel-17	38.306	17.3.0	0853	-	F	NR_NTN_solutions-Core

- Ericsson is ok in principle but wonders what is inter-satellite

- HW thinks the UE can derive this by comparing the ephemeris

- Revised in R2-2301988 to remove “

- Can come back to further clarify what inter-satellite really means

R2-2301988	IOT bit for inter satellite measurement (38.306)	MediaTek Inc.	CR	Rel-17	38.306	17.3.0	0853	1	F	NR_NTN_solutions-Core

- Agreed

R2-2300270	IOT bit for inter satellite measurement (38.331)	MediaTek Inc.	CR	Rel-17	38.331	17.3.0	3795	-	F	NR_NTN_solutions-Core

- Revised in R2-2301969 to fix the coversheet

R2-2301969	IOT bit for inter satellite measurement (38.331)	MediaTek Inc.	CR	Rel-17	38.331	17.3.0	3795	1	F	NR_NTN_solutions-Core

- Agreed

Moved from 6.6.1

R2-2301476	Discussion on IoT bit for inter-satellite measurement	Samsung Research America	discussion	Rel-17	NR_NTN_solutions-Core

Enhanced Cell Reselection Requirements

R2-2300470	38.306 CR On Enhanced Cell Reselection Requirements	Nokia, Nokia Shanghai Bell	CR	Rel-17	38.306	17.3.0	0856	-	F	NR_NTN_solutions-Core

- Intel is ok with this and thinks we should reply to RAN4

- Agreed

- Send a reply LS (offline 114)

R2-2300365	Correction on NR NTN UE capability for enhanced measurement requirements for cell reselection	Intel Corporation	CR	Rel-17	38.306	17.3.0	0854	-	F	NR_NTN_solutions-Core

Measurement relaxation

R2-2300235	CR to 38.304 on relaxed measurements	Huawei, HiSilicon	CR	Rel-17	38.304	17.3.0	0318	-	F	NR_NTN_solutions-Core

- HW thinks the RAN4 LS does not put any restriction on NW configuration and thinks there is no need for other changes than the reference to RAN4 specs

- LG thinks it’s clearer to add explicit restrictions in our specs

- Samsung wonders whether we need to add references to RAN4 specs in all the other places

- Intel supports HW CR (no need for other CRs)

- Nokia supports the CR but agrees that we need to aling other parts referring to RAN4 specs.

- Check if references need to be added to other parts of 304 (in offline 115)

- Revised in R2-2301970

- Apple thinks we need to have something in RRC CR.

R2-2301970	CR to 38.304 on relaxed measurements	Huawei, HiSilicon	CR	Rel-17	38.304	17.3.0	0318	1	F	NR_NTN_solutions-Core

[AT121][115][NR NTN] 38.304 CR on relaxed measurements (Huawei)

Scope: continue the discussion on R2-2300235

Intended outcome: Agreeable CR

Deadline for companies' feedback:  Thursday 2023-03-02 22:00 EET

Deadline for agreeable CR (in R2-2301970): Friday 2023-03-03 08:00 EET

R2-2301970	CR to 38.304 on relaxed measurements	Huawei, HiSilicon	CR	Rel-17	38.304	17.3.0	0318	1	F	NR_NTN_solutions-Core

- Tick the RAN box

- Revised in R2-2301981

- CATT thinks we should also have changes to 331. Ericsson/HW thinks this is covered by another CR

R2-2301981	CR to 38.304 on relaxed measurements	Huawei, HiSilicon	CR	Rel-17	38.304	17.3.0	0318	2	F	NR_NTN_solutions-Core

- Agreed

R2-2300730	Clarification on measurement relaxation in NTN	Apple	CR	Rel-17	38.331	17.3.0	3823	-	F	NR_NTN_solutions-Core

- Revised in R2-2301984 to fix the typo in “neighbour”

R2-2301984	Clarification on measurement relaxation in NTN	Apple	CR	Rel-17	38.331	17.3.0	3823	1	F	NR_NTN_solutions-Core

- Agreed

R2-2301478	Correction for NR NTN on relaxed measurement	Samsung	CR	Rel-17	38.331	17.3.0	3902	-	F	NR_NTN_solutions-Core

R2-2301686	Corrections on the relaxed cell reselection requirements	CATT	CR	Rel-17	38.304	17.3.0	0325	-	F	NR_NTN_solutions-Core

R2-2301847	Clarification on measurement relaxation target for NTN	LG Electronics Inc.	CR	Rel-17	38.304	17.3.0	0328	-	F	NR_NTN_enh-Core

R2-2301848	Clarification on measurement relaxation source for NTN	LG Electronics France	CR	Rel-17	38.331	17.3.0	3928	-	F	NR_NTN_enh-Core

Misc NBC CRs (in the current form)

R2-2300168	RRC correction on epochTime and distanceThreshold	OPPO	CR	Rel-17	38.331	17.3.0	3777	-	F	NR_NTN_solutions-Core

- Changes in field description for Epoch time are agreed, removing “and/”

- QC/Mediatek think we reason for change is ok but prefer not to change to avoid ASN.1 implication, we can live with a smaller range

- Nokia thinks we can live the smaller range

- HW thinks we need to add impact analysis

- Change 3 is not pursued. The value range is not modified

- Revised in R2-2301971 to reflect the comments above

R2-2301971	RRC correction on epochTime and distanceThreshold	OPPO	CR	Rel-17	38.331	17.3.0	3777	1	F	NR_NTN_solutions-Core

- Agreed

R2-2300201	Correction for RLC-Config-v1700	RadiSys	CR	Rel-18	38.331	17.3.0	3783	-	F	NR_NTN_enh-Core

- Not pursued (Rel-18 CRs not needed at the moment)

R2-2300202	Correction for RLC-Config-v1700	RadiSys	CR	Rel-17	38.331	17.3.0	3784	-	F	NR_NTN_solutions-Core

- QC thinks at lower level both IEs are optional so we don’t need to change anything. Mediatek agrees.

- Ericsson thinks it’s good to clarify that the NW does not configure both

- RAN2 understands that the NW will only configure one IE, not both.

- Not pursued

Moved from 6.6.2

R2-2300213	Additional IE inclusion within SI-SchedulingInfo-v1700	RadiSys	CR	Rel-18	38.331	17.3.0	3785	-	F	NR_NTN_solutions-Core

- Not pursued (Rel-18 CRs not needed at the moment)

R2-2300218	Additional IE inclusion within SI-SchedulingInfo-v1700	RadiSys	CR	Rel-17	38.331	17.3.0	3788	-	F	NR_NTN_solutions-Core

- The issue raised here can be further considered in offline 106 if needed

- Not pursued

R2-2300219	Correction in Need Code for cellSpecificKoffset-r17 and kmac-r17 in NTN-Config-r17	RadiSys	CR	Rel-18	38.331	17.3.0	3789	-	F	NR_NTN_solutions-Core

R2-2300301	Correction in Need Code for UplinkHARQ-mode-r17 and DownlinkHARQ-FeedbackDisabled-r17	RadiSys	CR	Rel-18	38.331	17.3.0	3797	-	A	NR_NTN_solutions-Core

- Not pursued (Rel-18 CRs not needed at the moment)

R2-2300302	Correction in Need Code for UplinkHARQ-mode-r17 and DownlinkHARQ-FeedbackDisabled-r17	RadiSys	CR	Rel-17	38.331	17.3.0	3798	-	F	NR_NTN_solutions-Core

Neighbour cell measurements

R2-2300910	NR NTN Rel-17 neighbor cell measurements 	Ericsson	discussion	Rel-17

- Continue the discussion in offline 103

R2-2301137	Clarification on neighboring cell measurements for NTN	ZTE Corporation, Sanechips	discussion	Rel-17

- Continue the discussion in offline 103

[AT121][103][NR NTN] Neighbour cell measurements (Ericsson)

Updated scope: continue the discussion neighbour cell measurements

Updated intended outcome: Summary of the offline and/or agreeable CRs

F2F offline time and location:  Wednesday 2023-03-01 16:30-17:00 EET, Brk 1 room (Aphrodite III&IV)

Deadline for companies' feedback:  Thursday 2023-03-02 22:00 EET

Deadline for rapporteur's summary (in R2-2301972): Friday 2023-03-03 08:00 EET

R2-2301972	[offline 103] Neighbour cell measurements	Ericsson	discussion	Rel-18	NR_NTN_solutions-Core

Proposal1 Revise SIB19 field description by adding sentence “If ntn-Config is absent for an entry in ntn-NeighCellConfigList, the ntn-Config provided in the previous entry in ntn-NeighCellConfigList applies”

- Oppo thinks this is a functional NBC change

- Apple thinks that if the majority is ok with the CR they can accept it provided it’s clearly marked as functional NBC in the coversheet

- QC thinks that although it’s a functional change this is essential and they can support it

- HW wonders if there is really a use case for signalling 8 cells from the same satellite. Nokia thinks we agreed to support up to 8

- Agreed (with a possible further clarification for the first entry)

Proposal 2 Inform in measurement object level on which satellite this measurement object applies to. Then, whole measurement object/carrier refers to same satellite, serving or one of the neighbors. This can be field description update for MO list.

- Can further check this in the discussion for the CR for introducing p1 above

R2-2301982	Corrections on neighboring cell measurement	Ericsson	CR	Rel-17	38.331	17.3.0	XXXX	-	F	NR_NTN_solutions-Core

- 1-week post meeting email disc

[POST121][101][NR NTN] Corrections on neighbour cell measurements (Ericsson)

Scope: Discuss the details of the 38.331 CR on Corrections on neighboring cell measurements reflecting meeting agreements

Intended outcome: Agreeable CR in R2-2301982

Deadline:  Short

R2-2301138	Corrections on neighboring cell measurement	ZTE Corporation, Sanechips	CR	Rel-17	38.331	17.3.0	3871	-	F	NR_NTN_solutions-Core

R2-2301703	Skip Measurements of a Neighbour Frequency or Cell	Google Inc.	CR	Rel-17	38.304	17.3.0	0326	-	F	NR_NTN_solutions-Core

SIB19 essentiality

R2-2301042	Clarification on essential SIB19 for NR NTN	Xiaomi, CAICT, Lenovo, Samsung, MediaTek, Apple, OPPO, CATT, CMCC, Qualcomm	CR	Rel-17	38.331	17.3.0	3861	-	F	NR_NTN_solutions-Core

- Come Back Friday

- HW understands a previous decision to capture the content of Chairman’s notes in a note in the spec but still wants to check if this correct (e.g. referring to the HAPS support) and would like to postpone this

- Agreed

PDD reporting

R2-2300125	Remaining issue on PDD reporting	vivo, Samsung, Huawei, HiSilicon	discussion	Rel-17	NR_NTN_solutions-Core

- Offline 116

- Draft CR on correction to PDD reporting

R2-2301980	Correction to PDD reporting	vivo, Samsung, Huawei, HiSilicon	CR	Rel-17	38.331	17.3.0	3937	-	F	NR_NTN_solutions-Core

- Agreed

SMTC

R2-2300234	Remaining issues on SMTC	Huawei, HiSilicon, vivo, Samsung	discussion	Rel-17	NR_NTN_solutions-Core

- Offline 116

Event D1

R2-2300236	CR to 38.331 on event D1	Huawei, HiSilicon	CR	Rel-17	38.331	17.3.0	3790	-	F	NR_NTN_solutions-Core

- Offline 116

- Not pursued

[AT121][116][NR NTN] CP corrections (Samsung)

Scope: Discuss the CRs in R2-2300125, R2-2300234 and R2-2300236

Intended outcome: offline summary (and agreeable CRs, if any)

Deadline for companies' feedback:  Thursday 2023-03-02 22:00 EET

Deadline for rapporteur's summary (in R2-2301973): Friday 2023-03-03 08:00 EET

R2-2301973	[offline 116] CP corrections	Samsung	discussion	Rel-18	NR_NTN_solutions-Core

For agreement:

Proposal 1 (15/16): Add a field description for the epochTime in otherConfig and agree TP in R2-2300125.

- Agreed

Proposal 4 (11/17): CR in R2-2300236 is not pursued.

- R2-2300236 is not pursued

Proposal 5: Agree CR R2-2300730.

- CATT have concern on P5, there is no description on the serving cell restriction, which is shown in RAN4 LS, as the relaxed intra-frequency measurements can be configured only by GSO serving cell. The relaxed inter-frequency measurements can be configured by GSO and NGSO serving cell.

- Agreed

For discussion:

Proposal 2-1: Two options on how to handle FL PDD for SMTC in SIB2/4:

•	Option 2 (10): Clarify in the field description of SMTC in SIB2/4 that the SMTC assumes feeder link PDD = 0ms (i.e. FL PDD is not compensated by NW).

•	Option 3 (5): Clarify in the field description of SMTC that the FL PDD corresponding to the epochTime in SIB19 is compensated (i.e. FL PDD at epoch time is compensated by NW).

- Google thinks none of this options really work

- Oppo thinks option 2 is aligned with our previous agreement and we should go for that.

- QC thinks option 2 is NBC and not acceptable. HW agrees

- Vivo thinks that the compensation for feeder link is for connected UEs, not idle

- Companies to check internally whether feeder link delay is compensated by the NW for SSB transmission

- Postponed to the next meeting

Proposal 2-2 (7/4): If Option 2 is agreed, add “feeder link propagation delay difference between the serving cell and neighbour cells equals to 0 ms” in field description of SMTC in SIB2/SIB4.

Proposal 3 (6/13): Clarify the feeder link propagation delay is not compensated by the transmitting node while exchanging the SMTC configuration in the inter-node message.

TN-NTN feature difference

R2-2300169	RRC correction on inactiveStateNTN	OPPO	CR	Rel-17	38.331	17.3.0	3778	-	F	NR_NTN_solutions-Core

R2-2300614	Handling of features with different UE support in TN and NTN	Intel Corporation	discussion	Rel-17	NR_NTN_solutions-Core

R2-2300880	Clarification on use of feature upon TN NTN mobility during RRC_INACTIVE	Qualcomm Incorporated	CR	Rel-17	38.331	17.3.0	3850	-	F	NR_NTN_solutions-Core

- Revised in R2-2301975

R2-2301975	Clarification on use of feature upon TN NTN mobility during RRC_INACTIVE	Qualcomm Incorporated, Intel Corporation, Oppo	CR	Rel-17	38.331	17.3.0	3850	1	F	NR_NTN_solutions-Core

- Apple thinks change 3 is not needed

- ZTE thinks we would need a similar statement for every feature. VDF agrees. Samsung also agrees

- Intel thinks this is the first time we have this situation: NW configures something thE UE does not support

- Revise the 2nd change to make it more generic

- Remove 3rd change

- RAN2 confirms that UE should only use/apply configurations of a given feature when UE supports the feature in the corresponding cell in which UE is camping, connecting, or resuming to.

- Revised in R2-2301995 to reflect the discussion

R2-2301995	Clarification on use of feature upon TN NTN mobility during RRC_INACTIVE	Qualcomm Incorporated, Intel Corporation, Oppo	CR	Rel-17	38.331	17.3.0	3850	2	F

- 1-week post meeting email disc

[POST121][102][NR NTN] TN NTN mobility during RRC_INACTIVE (Qualcomm)

Scope: Further check R2-2301995

Intended outcome: Agreeable CR

Deadline:  Short

epochTime and T430

R2-2301529	Clarification on T430 handling for target cell	ASUSTeK	CR	Rel-17	38.331	17.3.0	3905	-	F	NR_NTN_solutions-Core

R2-2301477	Correction for NR NTN on reconfiguration with sync	Samsung	CR	Rel-17	38.331	17.3.0	3901	-	F	NR_NTN_solutions-Core

R2-2300472	On T430 and epochTime related aspects	Nokia, Nokia Shanghai Bell	discussion	Rel-17	NR_NTN_solutions-Core

Missing reference

R2-2300267	Correction on missing referencing of the NTN spec in 38.331	MediaTek Inc.	CR	Rel-17	38.331	17.3.0	3793	-	F	NR_NTN_solutions-Core

R2-2301392	Correction on missing referencing of the NTN spec in 38.306	Mediatek Inc.	CR	Rel-17	38.306	17.3.0	0862	-	F	NR_NTN_solutions-Core

Capability related

R2-2300370	Clarification on NR NTN UE capability eventA4BasedCondHandover-r17	Intel Corporation	CR	Rel-17	38.306	17.3.0	0855	-	F	NR_NTN_solutions-Core

R2-2300879	Clarification on TN EUTRA capability reporting	Qualcomm Incorporated	CR	Rel-17	38.331	17.3.0	3849	-	F	NR_NTN_solutions-Core

HARQ field description

R2-2301436	Correction related to a missing description of a parameter of the number of HARQ processes	Thales	CR	Rel-17	38.331	17.3.0	3892	-	D	NR_NTN_solutions-Core

RAT-type

R2-2301687	Discussion on the RAT type of TN and NTN	CATT	discussion	Rel-17	NR_NTN_solutions-Core

Withdrawn

R2-2300268	Correction on missing referencing of the NTN spec in 38.306	MediaTek Inc.	CR	Rel-17	38.331	17.3.0	3794	-	A	NR_NTN_solutions-Core	Withdrawn

## 6.8	Reduced Capability

(NR_redcap-Core; leading WG: RAN1; REL-17; WID: RP-211574)

Tdoc Limitation: 3 tdocs

### 6.8.1	General

### 6.8.2	CP corrections

Includes also stage-2 corrections if needed

RedCap & SDT

Option 1: CG/RA-SDT can only be performed if the initial DL BWP includes the CD-SSB

Option 2: CG/RA-SDT can also be performed if the initial DL BWP does not include the CD-SSB but a NCD-SSB (to be signalled to the UE). A corresponding UE capability is introduced

Option 3: CG/RA-SDT can be performed even if the initial DL BWP does not include any SSB. It’s up to UE implementation whether to perform a new RSRP measurement on CB-SSB before CG transmission. A corresponding UE capability could be introduced

Option 4: If the network configures a REDCAP-specific initial DL BWP that does not include the CD-SSB, the UE monitors PDCCH on initialDownlinkBWP during the CG/RA-SDT procedure.

- Samsung thinks option 4 is a compromise but would prefer option 1 and cannot agree option 2

- QC think option 4 is the least favourable because of the need to switch among BWPs. Similar issues for option 3 that would increase implementation complexity. Would prefer 1 or 2

- LG shares QC view on option 4 and prefers option 1

- ZTE thinks there is no need for the UE to switch for option 2 and wants to understand if we support the use case. Thinks the NW needs to support NCD-SSB

- CMCC supports option 2 has they have a use case for this. VDF thinks the use case exists and we should find a way between option 3 and 2.

- Mediatek thinks option 4 is not a reasonable proposal to solve the issue. Considering the operators interest Mediatek thinks that there is only one option, i.e. option 2

- Option 4 is no longer considered

- Vivo agrees with QC. Also for option3 there is a need to switch so option 2 is the only reasonable one.

- Oppo also agrees that option 3 should be excluded. So it should be either option 1 or 2. But 2 requires more work. Xiaomi agrees. Huawei also agrees and think we can try option 2 and go for that if we can fix the details, otherwise fall back to option 1.

- Samsung thinks we already discussed NCD-SSB support for idle/inactive and decided not to do it.

- Apple likes what HW suggested and try with option 2 and if it does not work fall back to option 1.

- Nokia would like to exclude RA-SDT from the discussion

- Option 3 is no longer considered for this issue

- T-Mobile thinks the use case should be supported

- China Telecom would like to support the use case and prefers option 2

- QC prefers option 1 as a baseline for R17 and wonders whether we can have option 2 in R18 with early implementation

- ZTE thinks that it’s possible to have even less changes for option 2 if we don’t include the NCD-SSB configuration in the release message.

- Xiaomi thinks we need to verify the specification impacts

- Intel thinks RAN1 is discussing this and there are concerns with option 2

- Vivo supports the proposal to check the details of option 2 offline and then decide whether to go for that.

- Nokia thinks that at the end we need to consult with RAN1 and thinks that we should exclude RA-SDT.

- ZTE thinks that RedCap UEs have different requirements for reselection.

- Continue offline to check the details of option 2, including the possible impact on mobility, and if this can be included in R17 (offline 105)

Agreements:

- Option 4 is no longer considered

- Option 3 is no longer considered for this issue

Option 1

R2-2300173	Discussion on CG-SDT for RedCap Ues	OPPO	discussion	Rel-17	NR_redcap-Core

R2-2301727	SDT support in RedCap-specific initial BWP without CD-SSB	LG Electronics Inc.	discussion	Rel-17	NR_redcap-Core

Option 1 or Option 2

R2-2300190	Discussion on SSB for SDT procedures in RedCap-specific initial BWP	Qualcomm Incorporated	discussion	Rel-17	NR_redcap-Core

Option 2

R2-2300556	SDT for REDCAP UEs on BWP without CD-SSB	ZTE Corporation, Sanechips, vivo	discussion

R2-2300557	Corrections for SDT operation for REDCAP without CD-SSB	ZTE Corporation, Sanechips	CR	Rel-17	38.331	17.3.0	3817	-	F	NR_redcap-Core

- Revised in R2-2301954

Option 3

R2-2301284	CG-SDT on RedCap specific initial BWP	Nokia, Nokia Shanghai Bell	discussion	Rel-17	NR_redcap-Core

R2-2301689	Discussion on the CG-SDT configuration for RedCap UE	CATT	discussion	Rel-17	NR_redcap-Core

R2-2301694	Initial BWP for RedCap UEs configured with RA and CG-SDT	Ericsson	discussion	Rel-17	NR_redcap-Core

Option 4

R2-2300183	CG-SDT for RedCap UEs	Samsung Electronics Co., Ltd	discussion	Rel-17	NR_redcap-Core

[AT121][105][RedCap] RedCap & SDT (ZTE)

Scope: Discuss the Stage 3 details of option 2, including the possible impact on mobility.

Intended outcome: Summary of the offline and/or agreeable CRs

F2F offline time and location:  Wednesday 2023-03-01 16:30-17:00 EET, Brk 2 room (Omikron 2)

Deadline for companies' feedback:  Thursday 2023-03-02 22:00 EET

Deadline for CRs (in R2-2301954 and R2-2301976): Friday 2023-03-03 08:00 EET

R2-2301954	Corrections for SDT operation for REDCAP without CD-SSB	ZTE Corporation, Sanechips	CR	Rel-17	38.331	17.3.0	3817	1	F	NR_redcap-Core

- ZTE indicates that it would be possible to technically endorse the CRs and let the plenary decide on the release (R17 or R18)

- QC would like to minute that if RAN plenary decides that we only have option 2 in R18 we only have option 1 in R17.

- TMO-US thinks we should actually agree the CRs now for R17 as this is urgent. Any email disc should be to fix the details if not needed but we should agree the CRs now

- Intel is ok with the CRs and agrees with the request to have the minute that QC is suggested

- Mediatek , vivo support the CR and are ok to agree on it, and eventually to send this to RAN plenary for decision

- Samsung thinks we should discuss the release in the RAN plenary

- ZTE thinks it’s fair to allow further check but thinks there are no impacts to other groups.

- Ericsson doesn’t see the need to send this to RAN plenary

- Docomo does not support option 2 and thinks RAN1 should check this

- ZTE thinks that plenary can also discuss endorsed/agreed CRs

- RAN2 confirms it will focus only on option 2.

- Samsung would still like to only technically endorse the CRs from RAN2 perspective

- 1-week post meeting email discussion to check the technical details. The intention is to allow companies to check internally and attempt to have a RAN2 agreed CR at the end of the email disc. (of course, this does not preclude companies to raise concerns in the RAN plenary)

Agreements:

- RAN2 confirms it will focus only on option 2.

- 1-week post meeting email discussion to check the technical details of R2-2301954 and R2-2301956. The intention is to allow companies to check internally and attempt to have a RAN2 agreed CR at the end of the email disc. (Of course, this does not preclude companies to raise concerns in the RAN plenary)

R2-2301956	Corrections for SDT operation for REDCAP without CD-SSB	ZTE Corporation, Sanechips	CR	Rel-17	38.306	17.3.0	XXXX	-	F	NR_redcap-Core

- 1-week post meeting email discussion

[POST121][103][RedCap] SDT operation for RedCap without CD-SSB (ZTE)

Scope: Check the technical details of R2-2301954 and R2-2301956.

Intended outcome: Agreeable CRs

Deadline:  Short

Misc

R2-2300191	Correction to control plane procedures for RedCap UEs	Qualcomm Incorporated	CR	Rel-17	38.331	17.3.0	3780	-	F	NR_redcap-Core

- Oppo agrees change 1 is technically correct but there are others instances in the spec and wonder if we need to change everywhere

- HW thinks there is no ambiguity and we don’t need the change. At most we can put something in the minutes

- For change 1, ZTE thinks we already clarified this in the last meeting, no room for misunderstanding. Also think that second change is not needed. For change 3 agree with the intention but would prefer another way to solve it

- Change 1 in not pursued.

- Vivo supports second change.

- Oppo wonders if the change is really critical

- Apple agrees with ZTE for all 3 changes.

- HW thinks we don’t need change 2

- Continue the discussion for change 2 offline (offline 106)

- Lenovo agrees with the intention of change 3 but we need to do the change in a different way

- ZTE thinks we could use one of the spare values in SIB type info and then include a fake SI in the mandatory field. QC is not sure about this approach. Lenovo also doesn’t like this

- Continue the discussion offline on how to implement the change for issue 3 without NBC changes (offline 106)

- Revised in in R2-2301955

R2-2301955	Correction to control plane procedures for RedCap UEs	Qualcomm Incorporated	CR	Rel-17	38.331	17.3.0	3780	1	F	NR_redcap-Core

- Agreed

[AT121][106][RedCap] CP corrections (QC)

Scope: Continue the discussion on changes 2 and 3 in R2-2300191

Intended outcome: Agreeable CR (and offline summary if needed)

Deadline for companies' feedback:  Thursday 2023-03-02 22:00 EET (F2F offline is also invited)

Deadline for revised CR (in R2-2301955): Friday 2023-03-03 08:00 EET

R2-2300312	Correction on RACH configure for RedCap	vivo, Qualcomm, ZTE Corporation, Guangdong Genius	CR	Rel-17	38.331	17.3.0	3800	-	F	NR_redcap-Core

- HW is not sure about the need for the change or at least with the actual wording. LG has some sympathy for HW comment

- Continue offline (107) to find a possible better wording

- Revised in in R2-2301957

R2-2301957	Correction on RACH configure for RedCap	vivo, Qualcomm, ZTE Corporation, Guangdong Genius	CR	Rel-17	38.331	17.3.0	3800	1	F	NR_redcap-Core

- Agreed

[AT121][107][RedCap] RACH configuration (vivo)

Scope: Continue the discussion on R2-2300312

Intended outcome: Agreeable CR

Deadline for companies' feedback:  Thursday 2023-03-02 22:00 EET (F2F offline is also invited)

Deadline for revised CR (in R2-2301957): Friday 2023-03-03 08:00 EET

R2-2301316	Correction on RRC configuration for RedCap	ZTE Corporation, Sanechips	CR	Rel-17	38.331	17.3.0	3880	-	F	NR_redcap-Core

- HW thinks first change is needed (but there is a typo) and is fine to clarify something for the rest

- Continue offline (108)

- Revised in in R2-2301958

R2-2301958	Correction on RRC configuration for RedCap	ZTE Corporation, Sanechips	CR	Rel-17	38.331	17.3.0	3880	1	F	NR_redcap-Core

- Agreed

R2-2301513	Clarification on BWP capabilities of RedCap UEs 	T-Mobile USA Inc.	CR	Rel-17	38.306	17.3.0	0868	-	F	NR_redcap-Core

- Apple supports the CR but we should fix the coversheet

- Revised in R2-2301959

R2-2301959	Clarification on BWP capabilities of RedCap UEs 	T-Mobile USA Inc.	CR	Rel-17	38.306	17.3.0	0868	1	F	NR_redcap-Core

- Revised in R2-2302249

R2-2302249	Clarification on BWP capabilities of RedCap UEs 	T-Mobile USA Inc.	CR	Rel-17	38.306	17.3.0	0868	2	F	NR_redcap-Core

- Agreed

R2-2301688	Correction on the filed descriptions of NeedForGaps in 38.331	CATT, ZTE, vivo	CR	Rel-17	38.331	17.3.0	3917	-	F	NR_redcap-Core

- Continue offline (108)

- Revised in R2-2301960

R2-2301960	Correction on the filed descriptions of NeedForGaps in 38.331	CATT, ZTE, vivo	CR	Rel-17	38.331	17.3.0	3917	1	F	NR_redcap-Core

- Agreed

[AT121][108][RedCap] RRC config & NeedForGaps (ZTE)

Scope: Continue the discussion on R2-2301316 and R2-2301688

Intended outcome: Agreeable CRs

Deadline for companies' feedback:  Thursday 2023-03-02 22:00 EET (F2F offline is also invited)

Deadline for revised CRs (in R2-2301958 and R2-2301960): Friday 2023-03-03 08:00 EET

R2-2301728	Issues on dedicated configuration of RedCap-specific initial BWP	LG Electronics Inc.	discussion	Rel-17	NR_redcap-Core

eDRX

R2-2300172	Correction on eDRX	OPPO	CR	Rel-17	38.304	17.3.0	0317	-	F	NR_redcap-Core

- Offline 109

R2-2301330	Correction on eDRX	Nokia, Nokia Shanghai Bell	CR	Rel-17	38.304	17.3.0	0323	-	F	NR_redcap-Core

- Offline 109

- 1-week post meeting email discuss (Nokia)

[POST121][104][RedCap] Correction on eDRX (Nokia)

Scope: Revise CR on eDRX correction based on R2-2301330

Intended outcome: Agreeable CR

Deadline:  Short

R2-2300311	Correction on 38.304 for RedCap	vivo, Guangdong Genius	CR	Rel-17	38.304	17.3.0	0319	-	F	NR_redcap-Core, NR_SmallData_INACTIVE-Core

- Offline 109

- Not pursued

R2-2301134	Corrections for eDRX on IDLE eDRX cycle	Huawei, HiSilicon	CR	Rel-17	38.331	17.3.0	3869	-	F	NR_redcap-Core

- Offline 109

- 1st change and 2nd change are pursued by replacing “use” in the 1st change to “operate”. 3rd change is not pursued.

- Revised in R2-2302280

R2-2302280	Corrections for eDRX on IDLE eDRX cycle	Huawei, HiSilicon	CR	Rel-17	38.331	17.3.0	3869	1	F	NR_redcap-Core

- Agreed

R2-2301135	Correction for hyperSFN on SI update	Huawei, HiSilicon	CR	Rel-17	38.331	17.3.0	3870	-	F	NR_redcap-Core

- Offline 109

- Agreed

[AT121][109][RedCap] eDRX (Oppo)

Scope: Continue the discussion on the eDRX related CRs

Intended outcome: offline summary (and agreeable CRs, if any)

Deadline for companies' feedback:  Thursday 2023-03-02 22:00 EET

Deadline for rapporteur's summary (in R2-2301961): Friday 2023-03-03 08:00 EET

R2-2301961	[offline 109] eDRX	Oppo	discussion	Rel-18	NR_redcap-Core

Proposal 1: CR in R2-2300311 is not pursued.

- R2-2300311 is not pursued

Proposal 2: In CR R2-2301134, 1st change and 2nd change are pursued by replacing “use” in the 1st change to “operate”. 3rd change is not pursued.

- Agreed

Proposal 3: Discuss R2-2301135 online.

Proposal 4: Use CR in R2-2301330 as baseline and further discuss how to address the case where TeDRX, RAN is configured but not allowed in the current cell.

UE capability

R2-2301136	Correction on the RedCap UE capability of supportOfRedCap	Huawei, HiSilicon	CR	Rel-17	38.306	17.3.0	0861	-	F	NR_redcap-Core

Other

R2-2300157	Correction on offset for cell specific RSRP thresholds for 1Rx Redcap UE	OPPO	CR	Rel-17	38.331	17.3.0	3776	-	F	NR_redcap-Core

Withdrawn

R2-2300192	Correction to A1 and A2 events for RedCap UEs	Qualcomm Incorporated	CR	Rel-17	38.331	17.3.0	3781	-	F	NR_redcap-Core	Withdrawn

### 6.8.3	UP corrections

RedCap & SDT

R2-2300313	BWP switch for SDT with seperate initial BWP	vivo, Guangdong Genius	discussion	Rel-17	NR_redcap-Core

- Offline 110

[AT121][110][RedCap] BWP switch for SDT (vivo)

Scope: Continue the discussion on R2-2300313

Intended outcome: offline summary / agreeable CR

Deadline for companies' feedback:  Thursday 2023-03-02 22:00 EET

Deadline for offline summary / revised CR (in R2-2301962): Friday 2023-03-03 08:00 EET

R2-2301962	Correction SDT with separate initial BWP, Guangdong Genius	discussion	Rel-17	NR_redcap-Core

- Continue in the next meeting

R2-2300776	CR for Miscellaneous Corrections for SDT operation	LG Electronics	CR	Rel-17	38.321	17.3.0	1525	-	F	NR_SmallData_INACTIVE-Core

- Moved here from SDT session

- Postponed

R2-2301285	Correction on CG-SDT handling with RedCap specific initial BWP	Nokia, Nokia Shanghai Bell	CR	Rel-17	38.321	17.3.0	1542	-	F	NR_redcap-Core

Moved from 6.8.2

R2-2301695	Clarification on the validity of SS-RSRP measurement for RedCap UEs with CG-SDT	Ericsson	CR	Rel-17	38.321	17.3.0	1562	-	F	NR_redcap-Core

Other

R2-2301729	Correction on determination of whether the RedCap is applicable for RA procedure	LG Electronics.	CR	Rel-17	38.321	17.3.0	1564	-	F	NR_redcap-Core

## 7.2	NB-IoT and eMTC support for NTN

Tdoc Limitation: 3 tdocs

### 7.2.1	General and Stage 2 corrections

LSs and Stage 2 corrections.

Incoming LSs

R2-2300069	Response to “Response to “Reply to LS on UE capability signaling for IoT-NTN”” (S2-2211431; contact: Vodafone)	SA2	LS in	Rel-17	LTE_NBIOT_eMTC_NTN	To:RAN2, CT1, RAN3

- Noted

Stage 2

TDD support

R2-2300158	Discussion on TDD support in R17 IoT NTN	OPPO	discussion	Rel-17	LTE_NBIOT_eMTC_NTN

- ZTE thinks FDD is the main assumption for IoT-NTN but thinks that TDD should not be restricted and would like not to introduce changes for this

- Oppo thinks there would be an inconsistency with RAN1 CR and think we have no choice but to align to RAN plenary.

- Apple thinks the feature for IoT-NTN does not include HAPS so we should not care about that. Also HAPS band is for FDD

- ZTE thinks for MAC spec we added details for the TDD case.

- Introduce a clarification in Stage 2 about no TDD support

- Oppo wonders if we also need Stage 3 changes.

- We can further check the need for Stage 3 clarifications

R2-2300357	Stage-2 correction on TDD support for IoT NTN	OPPO	CR	Rel-17	36.300	17.3.0	1378	-	F	LTE_NBIOT_eMTC_NTN

- Agreed

R2-2300167	IoT-NTN Stage-2 correction	OPPO	CR	Rel-17	36.300	17.3.0	1377	-	F	LTE_NBIOT_eMTC_NTN

- Mediatek supports this and thinks this was also just agreed in RAN1

- HW thinks there is a misalignment with NR specs and we should check them as well.

- Ericsson agrees with the intention and align to 38.300

- Revised in R2-2301963 to keep only the change on Koffset

- Come back in the next meeting to fix the k_mac part, aligning to 38.300

R2-2301963	IoT-NTN Stage-2 correction	OPPO	CR	Rel-17	36.300	17.3.0	1377	-	F	LTE_NBIOT_eMTC_NTN

- Agreed

R2-2301158	CR to 36.300 on neighbour cell measurements	Huawei, HiSilicon	CR	Rel-17	36.300	17.3.0	1380	-	F	LTE_NBIOT_eMTC_NTN

- Mediatek is not sure we need this, IoT-NTN is different than NR-NTN.

- ZTE agrees with Mediatek. Samsung agrees

- Not pursued

AS deactivation due to discontinuous coverage

R2-2301393	Considerations on AS deactivation due to discontinuous coverage	Deutsche Telekom, Huawei, HiSilicon, Ericsson, Nokia, Nokia Shanghai Bell, Thales, Vodafone, Telit, BT, Telstra, Telecom Italia, Turkcell	discussion	Rel-17	LTE_NBIOT_eMTC_NTN

- QC thinks we discussed this before and thinks this CR does not consider devices deployed in areas where there is no TN coverage. Also thinks that SA2 already made changes aligned to previous RAN2 decision.

- Telit thinks that the UE should maintain AS connectivity

- QC thinks this goes back to UE implementation which is what the specs says at the moment

- Mediatek agrees this is UE implementation

- Samsung acknowledges the problem and thinks the solution could be to leave this to NW configuration

- CMCC think we should not leave this to UE implementation

- Novamint agrees with QC and Mediatek but is ok to consider concerns from operators

- Ericsson thinks the CR still would not restrict the UE implementation

- Mediatek think the CR is agreeable at a high level if we also address the issue of overhead for NTN-TN switching (registration and UE capabilities)

- Nokia/Ericsson think we should not link the two issues

- T-mobile thinks we should solve this in R17.

- Chair suggests to come back to this in Rel-18 in conjunction with the discussion to reduce the overhead for NTN-TN switching

- Come Back Friday (VC will check offline with interested companies and announce any updates)

R2-2300495	Further considerations on AS deactivation due to discontinuous coverage	Telit Communications S.p.A.	discussion

R2-2301380	Correction related to AS deactivation due to discontinuous coverage 	Deutsche Telekom, Nokia, Nokia Shanghai Bell, Ericsson, Huawei, HiSilicon, Thales, Vodafone, Telit, BT, Telstra, Telecom Italia, Turkcell	CR	Rel-17	36.300	17.3.0	1381	-	F	LTE_NBIOT_eMTC_NTN

- Revised in R2-2301863

R2-2301863	Correction related to AS deactivation due to discontinuous coverage 	Deutsche Telekom, Nokia, Nokia Shanghai Bell, Ericsson, Huawei, HiSilicon, Thales, Vodafone, Telit, BT, Telstra, Telecom Italia, Turkcell	CR	Rel-17	36.300	17.3.0	1381	1	F	LTE_NBIOT_eMTC_NTN	R2-2301380

- Revised in R2-2301976

R2-2301976	Correction related to AS deactivation due to discontinuous coverage 	Deutsche Telekom, Nokia, Nokia Shanghai Bell, Ericsson, Huawei, HiSilicon, Thales, Vodafone, Telit, BT, Telstra, Telecom Italia, Turkcell	CR	Rel-17	36.300	17.3.0	1381	2	F	LTE_NBIOT_eMTC_NTN

- Agreed

- Can come back in R18 to further improve the description, also based on the progress of the discussion on discontinuous coverage enhancements

### 7.2.2	UP corrections

Misc

R2-2300258	Misc corrections on MAC for IoT NTN	MediaTek Inc.	CR	Rel-17	36.321	17.3.0	1559	-	A	LTE_NBIOT_eMTC_NTN-Core

- Offline 111

- Agreed

R2-2301878	Correction for IoT NTN	Ericsson	CR	Rel-17	36.321	17.3.0	1563	-	F	LTE_NBIOT_eMTC_NTN

- Offline 111

- Agreed

R2-2301879	R17 IoT NTN user plane corrections	Ericsson	discussion	Rel-17	LTE_NBIOT_eMTC_NTN

- Offline 111

TDD support

R2-2300358	MAC correction on TDD support for IoT NTN	OPPO	CR	Rel-17	36.321	17.3.0	1560	-	F	LTE_NBIOT_eMTC_NTN

- Postponed

UE location Info in RLF report

Moved from 7.2.1

R2-2300886	NB-IoT UE location Info in RLF report	Qualcomm Incorporated	discussion	Rel-17	LTE_NBIOT_eMTC_NTN

- Offline 111

R2-2300887	Correction on UE location information in NB-IoT RLF report	Qualcomm Incorporated	CR	Rel-17	36.331	17.3.0	4906	-	F	LTE_NBIOT_eMTC_NTN

- Offline 111

- QC thinks we need a NW indication for this. Could also be discussed in legacy but the problem is for IoT. Would like to at least capture in the minutes that the UE may not send the location info. Apple agrees with QC

- Nokia thinks we should discuss this in the legacy session. HW agrees

- Samsung thinks RLF report itself is optional

- Ericsson would like to further check this. ZTE wonders what happens when an NB IoT NTN UE is served by a TN network

- Postponed to next meeting (check whether we need a general discussion for legacy NB-IoT or whether we can restrict this to NB-IoT NTN case)

Figure clarifying HARQ RTT timer

R2-2300888	Correction on figure clarifying HARQ RTT timer	Qualcomm Incorporated	CR	Rel-17	36.321	17.3.0	1561	-	F	LTE_NBIOT_eMTC_NTN

- Offline 111

- Changes are agreed, fixing the typo PDCC -> PDCCH in the figure

- Revised in R2-2301986

R2-2301986	Correction on figure clarifying HARQ RTT timer	Qualcomm Incorporated	CR	Rel-17	36.321	17.3.0	1561	1	F	LTE_NBIOT_eMTC_NTN

- Agreed unseen

Generation of TA reporting

R2-2301051	Clarification on the generation of TA reporting for IoT NTN	ZTE Corporation, Sanechips	CR	Rel-17	36.321	17.3.0	1562	-	F	LTE_NBIOT_eMTC_NTN-Core

- Offline 111

- Change “prior to the MAC PDU assembly” instead of “before a MAC PDU is assembled”

- Revised in R2-2301987

R2-2301987	Clarification on the generation of TA reporting for IoT NTN	ZTE Corporation, Sanechips	CR	Rel-17	36.321	17.3.0	1562	1	F	LTE_NBIOT_eMTC_NTN-Core

- Agreed

[AT121][111][IoT NTN] UP corrections (Mediatek)

Scope: Discuss the MAC CRs

Intended outcome: offline summary (and agreeable CRs, if any)

Deadline for companies' feedback:  Thursday 2023-03-02 22:00 EET

Deadline for rapporteur's summary (in R2-2301964): Friday 2023-03-03 08:00 EET

R2-2301964	[offline 111] UP corrections	Mediatek	discussion	Rel-18	LTE_NBIOT_eMTC_NTN-Core

Proposals with complete consensus

Proposal 1 (13/13): The CR in R2-2300258 for TS 36.321 R17 is agreed.

- Agreed

Proposal 5 (13/13): changes in CR R2-2300888 for TS 36.321 R17 are agreed. The corresponding figure needs some changes.

- Agreed

Proposal 6 (13/13): The changes in CR R2-2301051 for TS 36.321 R17 are agreed. The minor textual changes need to be taken into account.

- Agreed

Proposals with Vast Majority

Proposal 2 (12/13): The proposals P1 ~ P4 in R2-2301879 and corresponding changes in CR R2-2301878 for TS 36.321 R17 are agreed.

- Agreed

Proposal 3 (12/13): The proposal P5 in R2-2301879 and corresponding changes in CR R2-2301878 for TS 36.321 R17 are agreed.

- Agreed

Proposals need Discussion

Proposal 4 (8/13): The proposal P5 in R2-2300886 and corresponding changes in CR R2-2300887 for TS 36.321 R17 are not agreed. Time permitting RAN2 will discuss about these changes.

### 7.2.3	CP corrections

AS deactivation due to discontinuous coverage

Moved from 7.2.1

R2-2301388	Correction related to AS deactivation due to discontinuous coverage	Deutsche Telekom, Huawei, HiSilicon, Ericsson, Nokia, Nokia Shanghai Bell, Thales, Vodafone, Telit, BT, Telstra, Telecom Italia, Turkcell	CR	Rel-17	36.304	17.3.0	0862	-	F	LTE_NBIOT_eMTC_NTN

- Revised in R2-2301865

R2-2301865	Correction related to AS deactivation due to discontinuous coverage	Deutsche Telekom, Nokia, Nokia Shanghai Bell, Ericsson, Huawei, HiSilicon, Thales, Vodafone, Telit, BT, Telstra, Telecom Italia, Turkcell	CR	Rel-17	36.304	17.3.0	0862	1	F	LTE_NBIOT_eMTC_NTN	R2-2301388

- Revised in R2-2301977

R2-2301977	Correction related to AS deactivation due to discontinuous coverage	Deutsche Telekom, Nokia, Nokia Shanghai Bell, Ericsson, Huawei, HiSilicon, Thales, Vodafone, Telit, BT, Telstra, Telecom Italia, Turkcell	CR	Rel-17	36.304	17.3.0	0862	2	F	LTE_NBIOT_eMTC_NTN

- Samsung is fine in principle but wonders what idle mode tasks for NTN really means.

- Agreed

R2-2300367	Discussion on the deactivation of AS functions due to DC	Intel Corporation	discussion	Rel-17	LTE_NBIOT_eMTC_NTN-Core

R2-2301490	Discussion on deactivation of AS and out-of-coverage monitoring in discontinuous coverage	Samsung Electronics Benelux BV	discussion	Rel-17	LTE_NBIOT_eMTC_NTN

R2-2301491	Deactivation of AS and out-of-coverage monitoring in discontinuous coverage	Samsung Electronics Benelux BV	CR	Rel-17	36.304	17.3.0	0861	-	C	LTE_NBIOT_eMTC_NTN

UE capability update

R2-2300238	CR to 36.331 on UE capability update	Huawei, HiSilicon	CR	Rel-17	36.331	17.3.0	4904	-	F	LTE_NBIOT_eMTC_NTN

- Offline 112

- Postponed

T317

R2-2300237	Remaining issues on T317	Huawei, HiSilicon	discussion	Rel-17	LTE_NBIOT_eMTC_NTN

- Offline 112

- Draft CR to for modification to T317 in the Timers table

R2-2301979	Correction for T317 in the Timers table	Huawei, HiSilicon	CR	Rel-17	36.331	17.3.0	4919	-	F	LTE_NBIOT_eMTC_NTN-Core

- Agreed

R2-2301050	Correction on handling of T317 timer during HO	ZTE Corporation, Sanechips	CR	Rel-17	36.331	17.3.0	4908	-	F	LTE_NBIOT_eMTC_NTN-Core

- Offline 112

- Agreed

Epoch time

R2-2300927	Analysis on Reference time estimation issues of implicit Epoch time	Nokia, Nokia Shanghai Bell	discussion	Rel-17

- Offline 112

- RAN2 will not further discuss the issue in R2-2300927

R2-2301390	Discussion on epoch time	Mediatek Inc.	discussion

- Offline 112

- RAN2 will not further discuss the issue in R2-2301390

Pre-compensation gap configuration

R2-2300928	RRC parameter alignment with RAN1 specification for pre-compensation gap configuration	Nokia, Nokia Shanghai Bell	discussion	Rel-17

- Offline 112

- RAN2 will not further discuss the issue in R2-2300928

SIB32 acquisition in idle mode

R2-2301492	UE supporting discontinuous coverage acquiring SIB32 in idle mode	Samsung Electronics Benelux BV	CR	Rel-17	36.331	17.3.0	4918	-	F	LTE_NBIOT_eMTC_NTN

- Offline 112

- Not pursued

Misc

R2-2300259	Miscellaneous corrections to TS 36.331 for IoT NTN	MediaTek Inc.	CR	Rel-17	36.331	17.3.0	4900	-	A	LTE_NBIOT_eMTC_NTN-Core

- Offline 112

- Revised in R2-2301978 based on the comments in offline 112

R2-2301978	Miscellaneous corrections to TS 36.331 for IoT NTN	MediaTek Inc.	CR	Rel-17	36.331	17.3.0	4900	1	A	LTE_NBIOT_eMTC_NTN-Core

- Agreed

R2-2301389	Miscellaneous correction for IoT-NTN	Nokia, Nokia Shanghai Bell	CR	Rel-17	36.331	17.3.0	4913	-	F	LTE_NBIOT_eMTC_NTN

- Offline 112

- Not pursued

References

R2-2301049	Clarification on reference to TS 36.108 for IoT NTN	ZTE Corporation, Sanechips	CR	Rel-17	36.331	17.3.0	4907	-	F	LTE_NBIOT_eMTC_NTN-Core

- Offline 112

- Not pursued

R2-2301391	Reference Correction for IoT NTN in 36.306	Mediatek Inc.	CR	Rel-17	36.306	17.3.0	1867	-	A	LTE_NBIOT_eMTC_NTN-Core

- Offline 112

- Agreed

[AT121][112][IoT NTN] CP corrections (Huawei)

Scope: Discuss the CP CRs

Intended outcome: offline summary (and agreeable CRs, if any)

Deadline for companies' feedback:  Thursday 2023-03-02 22:00 EET

Deadline for rapporteur's summary (in R2-2301965): Friday 2023-03-03 08:00 EET

R2-2301965	[offline 112] CP corrections	Huawei	discussion	Rel-18	LTE_NBIOT_eMTC_NTN-Core

(14/14) Proposal 1: Second change of R2-2300237 (modifications to T317 in the Timers table) is agreed.

- Agreed

(14/14) Proposal 2: CR R2-2301050 is agreed.

- Agreed

(11/14) Proposal 3: RAN2 will not further discuss the issue in R2-2300927.

- Agreed

(8/14) Proposal 4: RAN2 will not further discuss the issue in R2-2301390.

- Agreed

(10/14) Proposal 5: RAN2 will not further discuss the issue in R2-2300928.

- Agreed

(10/14) Proposal 6: CR R2-2301492 is not pursued.

- R2-2301492 is not pursued

(14/14) Proposal 7: CR R2-2300259 is updated based on the comments.

- Agreed

(10/14) Proposal 8: CR R2-2301389 is not pursued.

- R2-2301389 is not pursued

(12/12) Proposal 9: CR R2-2301049 is not pursued.

- R2-2301049 is not pursued

(13/13) Proposal 10: CR R2-2301391 is agreed.

- Agreed

Withdrawn

R2-2300260	Discussion on epoch time	MediaTek Inc.	CR	Rel-17	36.331	17.3.0	4901	-	A	LTE_NBIOT_eMTC_NTN-Core	Withdrawn

R2-2300261	Correct the references for IoT NTN to 36.306	MediaTek Inc.	CR	Rel-17	36.331	17.3.0	4902	-	A	LTE_NBIOT_eMTC_NTN-Core	Withdrawn

## 8.6	IoT NTN enhancements

(IoT_NTN_enh-Core; leading WG: RAN1; REL-18; WID: RP-223519)

Time budget: 1 TU

Tdoc Limitation: 4 tdocs

### 8.6.1	Organizational

LSs, rapporteur inputs and other organizational documents. Rapporteur inputs and other pre-assigned documents in this AI do not count towards the tdoc limitation.

Initial running CRs are expected to be submitted for the next meeting:

CR rapporteurs:

36.300: Ericsson

36.331: Huawei

36.321: Mediatek

36.304: Nokia

36.306: Qualcomm

R2-2300273	List of RAN2 Agreements in IoT-NTN	MediaTek Inc.	discussion

- Noted

### 8.6.2	Performance Enhancements

R2-2301880	R18 IoT NTN performance enhancement	Ericsson	discussion	Rel-18	IoT_NTN_enh

#### 8.6.2.1	HARQ enhancements

R2-2300161	Discussion on HARQ enhancement for IoT NTN	OPPO	discussion	Rel-18	IoT_NTN_enh-Core

R2-2300203	Discussion on the HARQ handling in IoT NTN	CATT	discussion	Rel-18	IoT_NTN_enh-Core

R2-2300262	On Disabling HARQ Feedback in IoT-NTN	MediaTek Inc.	discussion

R2-2300579	Disabling HARQ feedback for IoT-NTN	Interdigital, Inc.	discussion	Rel-18	IoT_NTN_enh-Core

R2-2300889	Enhancement for UL and DL HARQ processes	Qualcomm Incorporated	discussion	Rel-18	IoT_NTN_enh-Core

R2-2301043	Discussion on HARQ enhancement	Xiaomi	discussion	Rel-18

R2-2301046	Draft LS on NPDCCH monitoring for HARQ mode B	Xiaomi	LS out	Rel-18 IoT_NTN_enh-Core	To:RAN1

R2-2301251	Discussion on the HARQ enhancement for IoT-NTN	CMCC	discussion	Rel-18	IoT_NTN_enh

R2-2301659	Discussion on Timing Advance Report MAC CE transmission in eMTC NTN	Nokia, Nokia Shanghai Bell, Huawei, HiSilicon	discussion	Rel-18	IoT_NTN_enh-Core

#### 8.6.2.2	GNSS operation enhancements

R2-2300175	Discussion on GNSS operation in connected mode	OPPO	discussion	Rel-18	IoT_NTN_enh-Core

R2-2300204	Discussion on GNSS operation in connected mode	CATT	discussion	Rel-18	IoT_NTN_enh-Core

R2-2300263	Enhancements on GNSS operation	MediaTek Inc.	discussion

R2-2300580	GNSS acquisition and reporting for IoT NTN	Interdigital, Inc.	discussion	Rel-18	IoT_NTN_enh-Core

R2-2300739	Improved GNSS Operation	Apple	discussion	Rel-18	IoT_NTN_enh-Core

R2-2300892	GNSS fix in RRC_CONNECTED	Qualcomm Incorporated	discussion	Rel-18	IoT_NTN_enh-Core

R2-2300979	Considerations on long GNSS operation in CONNECTED state	Lenovo	discussion	Rel-18

R2-2301041	Discussion on GNSS operation enhancement	Xiaomi	discussion	Rel-18

- Revised in R2-2301895

R2-2301895	Discussion on GNSS operation enhancement	Xiaomi	discussion	Rel-18

R2-2301053	Further discussion on GNSS enhancements	ZTE Corporation, Sanechips	discussion	IoT_NTN_enh-Core

R2-2301209	Discussion on the enhancement of GNSS operation	Huawei, Turkcell, HiSilicon	discussion	Rel-18	IoT_NTN_enh-Core

R2-2301252	Discussion on the GNSS enhancement for IoT-NTN	CMCC	discussion	Rel-18	IoT_NTN_enh

R2-2301493	On improved GNSS operation for IoT NTN	Samsung Electronics Benelux BV	discussion	Rel-18	IoT_NTN_enh

R2-2301660	On GNSS operation enhancements for IoT NTN	Nokia, Nokia Shanghai Bell	discussion	Rel-18	IoT_NTN_enh-Core

[AT121][101][IoT NTN enh] GNSS operation (CATT)

Initial scope: Discuss proposals in 8.6.2.2

Initial intended outcome: Summary of the offline discussion with e.g.:

- List of proposals for agreement (if any)

- List of proposals that require online discussions

Deadline for companies' feedback:  Wednesday 2023-03-01 06:00 EET

Deadline for rapporteur's summary (in R2-2301951): Wednesday 2023-03-01 12:00 EET

R2-2301951	[offline 101] GNSS operation enhancements	CATT	discussion	Rel-18	IoT_NTN_enh-Core

For email agreement:

(16/17) Proposal 1: For UE to report GNSS position fix time duration for measurement during the initial access, at least the following Msg5 message can be used:

RRCConnectionResumeComplete,RRCConnectionSetupComplete, RRCConnectionResumeComplete-NB, RRCConnectionSetupComplete-NB

FFS for RRCreestablishmentComplete and RRCConnectionReconfigurationComplete.

FFS for Msg3

-	HW thinks RAN1 only agreed to report this only in initial access and suggests to put the resume messages for FFS. Also HW doesn’t see the difference for this between the resume case and HO case

-	Samsung wonders if the time to perform GNSS measurement in initial access is the same as in connected

-	Oppo thinks the resume procedure is transparent to RAN1 and they did not intend to exclude this

- For UE to report GNSS position fix time duration for measurement during the initial access, at least the following Msg5 message can be used:

RRCConnectionSetupComplete, RRCConnectionSetupComplete-NB, RRCConnectionResumeComplete, RRCConnectionResumeComplete-NB

FFS for RRCreestablishmentComplete and RRCConnectionReconfigurationComplete.

RRCConnectionResumeComplete, RRCConnectionResumeComplete-NB,

FFS for Msg3

(1617) Proposal 6: By default, Rel-18 IoT NTN UE (supporting the improved GNSS operations) will leave RRC_CONNECTED state when current GNSS position becoming out-of-date, unless it has been indicated by network to perform GNSS measurement.

FFS for the case of a GNSS measurement is happening at/after the expiry of the GNSS validity duration

-	QC thinks it should be “configured” rather than indicated

-	IDC thinks this proposal says that if the R18 feature is not supported we fall back to legacy/ Intel agrees

-	Nokia thinks we need to keep the FFS and we can remove the unless part.

-	Ericsson wonders if this is a corner case

-	Oppo thinks the intention of the proposal is to clarify when the UE falls back to R17 behaviour

-	Intel thinks RAN1 agrees that the UE may re-acquire GNSS autonomously (when configured by the network) if UE does not receive eNB trigger to make GNSS measurement

- FFS whether the UE can stay in RRC_CONNECTED state when current GNSS position becoming out-of-date if the UE has initiated a new measurement

(16/17) Proposal 12: The value range {10s, 20s, 30s, 40s, 50s, 60s, 5 min, 10 min, 15 min, 20 min, 25 min, 30 min, 60 min, 90 min, 120 min, infinity} introduced in R17 is reused for connected UE GNSS validation duration report, unless modified by RAN1.

- Agreed

(14/17) Proposal 10: UE at least reports GNSS validity duration when the GNSS validity duration has changed.

FFS UE reports GNSS validity duration after each time of GNSS measurement

- Oppo thinks in R17 the UE reports the remaining validity duration.

- ZTE thinks we should report the whole validity duration. IDC agrees this should be the entire validity duration. HW also agrees

- Intel thinks that in RAN1 LS they report that Agreement GNSS assistance information that UE reports to eNB at least consists of: GNSS position fix time duration for measurement and GNSS validity duration

- QC/Mediatek think it should be the remaining time

- UE reports GNSS validity duration after GNSS measurement. FFS whether the UE reports every time or only if the validity duration changes. FFS if the duration is the remaining validity duration or the whole duration

(12/17) Proposal 11: New MAC CE should be defined for connected UE reporting GNSS validity duration.

- Ericsson disagrees and thinks we should not follow what RAN1 wants. Samsung agrees and generally think we should keep this in RRC. Nokia agrees

- IDC thinks we already had this discussion in the context of LTM  and we considered the security issue not to be a problem. Mediatek agrees

- Oppo thinks RAN1 is also aware of this

Agreements:

- For UE to report GNSS position fix time duration for measurement during the initial access, at least the following Msg5 message can be used:

RRCConnectionSetupComplete, RRCConnectionSetupComplete-NB,

RRCConnectionResumeComplete, RRCConnectionResumeComplete-NB,

FFS for RRCreestablishmentComplete and RRCConnectionReconfigurationComplete.

FS for Msg3

- FFS whether the UE can stay in RRC_CONNECTED state when current GNSS position becoming out-of-date if the UE has initiated a new measurement

- The value range {10s, 20s, 30s, 40s, 50s, 60s, 5 min, 10 min, 15 min, 20 min, 25 min, 30 min, 60 min, 90 min, 120 min, infinity} introduced in R17 is reused for connected UE GNSS validation duration report, unless modified by RAN1.

- UE reports GNSS validity duration after GNSS measurement. FFS whether the UE reports every time or only if the validity duration changes. FFS if the duration is the remaining validity duration or the whole duration

For further discussion:

(10/17) Proposal 2: Woking assumption: at least for eNB aperiodically triggering UE to make GNSS measurement, new MAC CE is used.

FFS for the security issue

(4/13) Proposal 3: Send LS to RAN1 and/or SA3 to inform/check security concern on using MAC CE for eNB to aperiodically UE to make GNSS measurement, unless RAN2 agrees to follow the RAN1 agreement on eNB aperiodically triggering UE to make GNSS measurement using MAC CE.

Postpone:

(13/17) Proposal 5: wait for progress of RAN1 on the issue of whether the GNSS position fix time duration for measurement may be changed during the long data connection.

(15/17) Proposal 7: RAN2 postpone the discussion of the UE capability indication of supporting GNSS operation enhancement.

Proposal 4: RAN2 to discuss the following options for GNSS measurement gap transmission:

- RRC signalling

- MAC CE triggering GNSS measurement

- No explicit GNSS measurement gap configuration

- Postpone the discussion

Proposal 8: For GNSS operation enhancement:

- UE capability indication is needed to indicate whether the UE supports GNSS operation enhancement

- Network indication（explicit/implicit）is needed to indicate whether the network supports GNSS operation enhancement

(14/17) Proposal 9: Postpone the discussion on the following issues:

- UE behaviour when the UE completes GNSS measurement

- UE behaviour when the UE can’t re-acquire GNSS position according to the configuration

- UE triggered GNSS measurement (can be discussed if time is allowed )

(14/15) Proposal 13: Postpone the discussion of the proposals listed in section 3.3

### 8.6.3	Mobility Enhancements

#### 8.6.3.1	Enhancements for neighbour cell measurements

Triggers for measurements

R2-2301054	Further discussion on neighbor cell measurement	ZTE Corporation, Sanechips	discussion	IoT_NTN_enh-Core

< Time-based trigger>

< quasi-Earth fixed cell >

Proposal 1a: For quasi-Earth fixed cell case, it’s suggested to provide the start time information of neighbor or upcoming cell (s).

- Panasonic wonders if this for the case of overlapping cells, in that case would make sense

- ZTE thinks this is the case of continuous coverage case

- Nokia is not sure this is essential. Mediatek agrees

- HE thinks we previously agreed that the exact time for starting measurements is up to UE implementation

- Intel thinks the start time is not necessary but the list of upcoming cells is.

- QC thinks this could be useful if this comes via dedicated configuration.

Proposal 1b: If the UE acquires the start time of upcoming cell (s) and is aware that it’s before t-Service, e.g., the time when the serving cell is going to stop serving the area, UE can exactly determine when to start the connected mode neighbor cell(s) measurement, e.g., upon the (start) time of the upcoming cell (s)’ coverage.

Proposal 1c: The timing information on when a neighbor/upcoming cell is going to start serving the area can be broadcast if the neighbor cell is a NTN quasi-Earth fixed cell. Such information can be provided as part of satellite assistance information for neighbour cells for IoT NTN.

< Earth moving cell >

Proposal 2: For earth moving cell, it’s no need to let UE calculate the exact time when loss of coverage of current cell might happen and therefore, no related assistance information, e.g., footprint information needs to be provided for UE.

< Location-based trigger>

< quasi-Earth fixed cell >

Proposal 3a: For quasi-earth fixed cell, the distance-based new trigger can be also supported. The reference location of the serving cell and the corresponding threshold are broadcast in system information.

- Huawei thinks we have agreed other mechanism for this and wonder whether we need this.

- Inmarsat thinks the situation is the same for earth-moving e fixed cells

- Location-based connected mode measurement initiation is supported in quasi-Earth-fixed cell (UE is not required to update the GNSS location for this). A serving cell reference location and a distance threshold/radius for detecting when to trigger connected mode measurements will be broadcast for quasi-Earth-fixed cell. FFS on whether the R17 IEs are reused or not.

- FFS if the same mechanism can also be used in idle (like in NR-NTN)

Proposal 3b: For quasi-Earth fixed cell case, if the UE can acquire the reference location of the serving cell and the corresponding threshold, UE can exactly determine when to start the connected mode neighbor cell(s) measurement, e.g., when the distance between UE and serving cell reference location is larger than the corresponding threshold and before the t-Service.

< Earth moving cell >

Proposal 4a: For earth moving cell case, the reference location of the serving cell and the corresponding threshold are broadcast in system information. UE can make use of this information to derive when loss of coverage of current cell might happen.

- Oppo thinks we have an agreement for idle/mode in NR NTN while here we are considering connected mode.

- Panasonic thinks feeder link switch could imply the need for a timer based solution

- Location-based connected mode measurement initiation is supported in earth-moving cell (UE is not required to update the GNSS location for this). A serving cell reference location and a distance threshold/radius for detecting when to trigger connected mode measurements will be broadcast for earth-moving cell. FFS on whether the R17 IEs are reused or not. FFS on whether additional information needs to be broadcast to inform the UE how the reference location moves over time or if this can be derived from other information (e.g. Epoch time and ephemeris).

- FFS if the same mechanism can also be used in idle (like in NR-NTN)

Proposal 4b: For earth moving cell case, if the UE can acquire the reference location of the serving cell and the corresponding threshold, UE can exactly determine when to start the connected mode neighbor cell(s) measurement, e.g., when the distance between UE and serving cell reference location is larger than the corresponding threshold.

Agreements:

- Location-based connected mode measurement initiation is supported in quasi-Earth-fixed cell (UE is not required to update the GNSS location for this). A serving cell reference location and a distance threshold/radius for detecting when to trigger connected mode measurements will be broadcast for quasi-Earth-fixed cell. FFS on whether the R17 IEs are reused or not. FFS if the same mechanism can also be used in idle (like in NR-NTN)

- Location-based connected mode measurement initiation is supported in earth-moving cell (UE is not required to update the GNSS location for this). A serving cell reference location and a distance threshold/radius for detecting when to trigger connected mode measurements will be broadcast for earth-moving cell. FFS on whether the R17 IEs are reused or not. FFS on whether additional information needs to be broadcast to inform the UE how the reference location moves over time or if this can be derived from other information (e.g. Epoch time and ephemeris). FFS if the same mechanism can also be used in idle (like in NR-NTN)

< configuration >

Proposal 5: The configuration framework for connected mode neighbor cell measurement via SIB can be reused for R18 IoT over NTN and can be further extended, e.g., to incorporate more possible new triggering conditions.

< connected mode measurement enhancements to eMTC NTN >

Observation 2a: Different from NB-IoT, it’s not easy to use serving cell stop time information as a new trigger for connected mode measurement for eMTC over NTN as UE may need to trigger measurement much earlier than this serving cell stop time in order to ensure target cell for handover can be found timely. But there is no clear rule on how to set this earlier timing amount.

Observation 2b: If enhancements connected mode neighbor cell measurement in R17 NB-IoT would be applied to eMTC over NTN, RAN2 needs to discuss whether the trigger conditions configuration framework in SIB can also be introduced for eMTC over NTN and how the new configuration coexist with the legacy measurement/report configuration.

Proposal 6: It’s suggested not to introduce new trigger conditions for connected mode neighbor cell measurement for eMTC over NTN.

R2-2300162	Discussion on measurement enhancement for IoT NTN	OPPO	discussion	Rel-18	IoT_NTN_enh-Core

< Location-based trigger>

Proposal 1	For NB-IoT, introduce location-based “s-measure criterion” and “low mobility criterion” for neighbouring cell measurement triggering in RRC_CONNECTED.

Proposal 2	For eMTC, introduce location-based “s-measure criterion” for neighbouring cell measurement triggering in RRC_CONNECTED.

Proposal 3	For both NB-IoT and eMTC, RRC_CONNECTED neighbouring cell measurement triggering is based on a combination of RSRP-based criterion and location-based criterion.

Proposal 4	For both quasi-earth fixed cell and earth moving cell, distance between UE and serving cell reference location is used for location-based “s-measure criterion”.

Proposal 5	For both quasi-earth fixed cell and earth moving cell, variance of distance between UE and serving cell reference location is used for location-based “low mobility criterion”.

- Inmarsat asks whether this is meant to take into account the speed of the UE.

- HW is not sure how this could be useful. ZTE/Panasonic agree.

- Ericsson wonders whether we need a low mobility criterion at all

- Intel wonders how UE derives its speed? by frequent GNSS acquisition?

- Apple is worried about the complexity of the solution.

- We don’t introduce any new low mobility criterion for enhanced mobility in Rel-18

Proposal 6	For location-based criterion for earth moving cell, RAN2 discuss solutions for UE to acquire the serving cell’s reference location which do not require frequent reconfiguration of reference locations.

< Time-based trigger>

Proposal 7	For eMTC NTN, a trigger for neighbour cell measurements based on T-service is supported for quasi-earth fixed cell.

Proposal 8	For eMTC NTN, for quasi-earth fixed cells, UE shall start intra/inter frequency measurement in connected mode before the t-Service if present. The exact time to start measurements in connected mode before t-Service is left to UE implementation.

Proposal 9	For IoT NTN, time-based neighbouring cell measurement triggering in RRC_CONNECTED is not supported for earth moving cell.

Agreements:

- We don’t introduce any new low mobility criterion for enhanced mobility in Rel-18

R2-2300581	Neighbour cell measurements before RLF	Interdigital, Inc.	discussion	Rel-18	IoT_NTN_enh-Core

Proposal 1a: The agreement in RAN2#120 (UE shall start intra/inter frequency measurement in connected mode before the t-Service if present) only applies to the case of overlapping coverage.

Proposal 1b: Measurements of a neighbouring NTN cell are triggered before t-service only if the incoming neighbour cell t-serviceStart is before t-service, or if no t-serviceStart is provided for the neighbour cell.

Proposal 2a: Measurements on TN carriers (if configured by the NW) can start before t-service independently of neighbouring NTN cell coverage.

Proposal 2b: For the hard switch coverage scenario, discuss whether UE should wait until t-Service before starting TN measurements, or whether UE should start TN measurements before t-Service.

Proposal 3: If the serving cell t-service expires, stop T310 and start T311 (i.e. perform cell search and re-establishment without attempting to recover on the current cell for the duration of T310).

Neighbour cell assistance information

(options regarding which SIB to use:

Option 1: new SIB

Option 2: SIB31

Option 2: other legacy SIB)

- Oppo thinks we could reuse the NT-NTN approach and then use SIB31. Mediatek/Lenovo/HW agree (potentially using some of the enhancements investigated for NR NTN)

- Samsung thinks we should define a new SIB considering the size. QC agrees

- IDC thinks the other option is to use SIB32, extending its scope

- ZTE prefers to use legacy signalling (SIB3, 4, 5). Nokia agrees

- QC has strong concerns with extending SIB31, which should be kept as small as possible. QC thinks the situation is not the same as in NR

- To be discussed in a Post-meeting discussion

[POST121][105][IoT NTN Enh] Neighbour cell assistance information (Qualcomm)

Scope: Discuss the possible content of neighbour cell assistance information and whether this should be included in an existing or new SIB

Intended outcome: report of the email discussion

Deadline:  Long

R2-2300891	Neighbour satellite and coverage information signalling	Qualcomm Incorporated	discussion	Rel-18	IoT_NTN_enh-Core

Proposal 1	Each neighbor satellite information in a list includes the mandatory orbital parameters and optionally validity duration, epoch time, common TA parameters.

Proposal 2	Size of neighbor satellite list is 6.

Proposal 3	Introduce new SIB (i.e., SIBX) to broadcast the neighbor satellite information.

Proposal 4	Introduce satellite identity to identify the satellite in the list.

Proposal 5	Include satellite identity in neighbor cell list or inter frequency list in SIB4/SIB5 and connected mode measurement frequency list (i.e., in measurement object for eMTC).

Proposal 6	A list of coverage area information is also broadcast in the new SIBx.

Proposal 7	The coverage area information broadcast in new SIBX can be list of footprint information, i.e., reference location and radius or a set of zones dividing coverage area.

Proposal 8	Coverage area information broadcast in the new SIBx can be applicable for TN or NTN frequency/cell.

Proposal 9	Introduce the coverage area ID and include it in a given neighbor cell list or inter-frequency list in SIB4/SIB5.

Proposal 10	If coverage Area ID is absent for a given neighbor frequency or cell, the UE performs the measurement of the neighbor frequency/cell regardless of its position.

Proposal 11	If coverage Area ID is present for a given neighbor frequency or cell, the UE performs the measurement of the neighbor frequency/cell when present in the area identified by the coverage Area ID.

R2-2300264	Enhancements on neighbour cell measurement	MediaTek Inc.	discussion

Proposal 1: Add the optional parameter of neighbour cell ephemeris in SIB31 in SSV and orbital format.

Proposal 2: Other IEs are needed together with the neighbour cell ephemeris:

- The frequency and optional cell id list

- Common TA

- UL sync validity duration

- Epoch time

R2-2301056	Further discussion on other mobility enhancements	ZTE Corporation, Sanechips	discussion	IoT_NTN_enh-Core

Proposal 1: It’s suggested to introduce satellite assistance information of neighbor cells into the existing system information messages which contains information relevant to intra-frequency cell re-selection, inter-frequency cell re-selection (e.g., SIB3, SIB4 or SIB5), and the measurement object configuration (e.g., measObjectEUTRA). Moreover, delta configuration can be allowed to reduce the signaling overhead.

Neighbour cell measurements in connected mode for LTE-M in NTN

R2-2301693	Discussion on neighbour cell measurements before RLF	Ericsson	discussion	Rel-18	IoT_NTN_enh-Core

Proposal 1	Introduce a mechanism to trigger neighbour cell measurements in connected mode for LTE-M in NTN.

Proposal 2	Introduce time-based criteria for quasi-earth fixed cells, based on T-service, to trigger neighbour cell measurements in connected mode for LTE-M in NTN. UE shall start intra/inter frequency measurement in connected mode before T-service, if present. The exact time to start measurements in connected mode before T-service is left to UE implementation.

- HW thinks the same behaviour could be achieved by changing the RSRP threshold via RRC reconfig when the NW knows that T-service is expiring.

- Nokia wonders if this is a replacement for CHO

Proposal 4	Introduce distance-based criteria to trigger neighbour cell measurements in connected mode for LTE-M in NTN.

Proposal 5	For earth-moving cells, UE shall start intra/inter frequency measurement in connected mode when its distance to a reference location is larger than threshold, if present. The exact time to start measurements in connected mode before that threshold is left to UE implementation.

- Ericsson thinks we should not assume we always have CHO and then the same enhancements as for NB-IoT could be used in LTE-M

- Oppo agrees with Ericsson.

- QC thinks we agreed something for IoT NTN in general, not only NB-IoT

- Ericsson thinks we only have RSRP measurements so far for LTE-M and we should introduce time and distance based triggers as well

- HW thinks we should clarify the motivation: is this for timely measurements or for power saving? Ericsson thinks this is for both of them.

- HW thinks that for NB-IoT is only for timely measurements

- CATT thinks that in TN, we have not introduce the mechanism for LTE-M, why it is important to introduce this mechanism for LTE-M in NTN?

- Intel thinks the new optimization is about when to initiate connected mode measurement, but in eMTC the measurement is already on

- Nokia wonders if this is to optimize the measurement framework.

- IDC thinks the motivation is the same for NB-IoT and LTE-M, even if the final mobility procedure is different

R2-2301494	On enhancements for neighbour cell measurements	Samsung Electronics Benelux BV	discussion	Rel-18	IoT_NTN_enh

Proposal 1: Introduce the same measurement triggers as for NB-IoT for eMTC, but for power saving purposes.

Proposal 2: RAN2 to consider use case-based / traffic-based conditions for not performing neighbour cell measurements.

Proposal 3: RAN2 to discuss options on broadcasted neighbour cell ephemeris.

Proposal 4: Target cell neighbour cell ephemeris is acquired in target cell when a handover is performed. Exact details are FFS.

Proposal 5: RAN2 to discuss how to improve NB-IoT RLF procedures by sending UE context in advance.

From other contributions:

Proposal x：RRC_CONNECTED measurement enhancements are not applicable to eMTC NTN.

R2-2300144	Enhancements for neighbour cell measurements	Huawei, HiSilicon, Turkcell	discussion	Rel-18	IoT_NTN_enh

R2-2300205	Enhancements for Neighbor Cell Measurements	CATT	discussion	Rel-18	IoT_NTN_enh-Core

R2-2300366	Discussion on neighbour cell measurements in IoT NTN	Intel Corporation	discussion	Rel-18	IoT_NTN_enh-Core

R2-2300750	Neighbour cell measurements before RLF for NB-IoT	Apple	discussion	Rel-18	IoT_NTN_enh

R2-2300980	CONNECTED neighbour cell measurement for NB-IoT in NTN	Lenovo	discussion	Rel-18

Moved from 8.6.3

R2-2300925	Analysis on mobility enhancements for IoT-NTN	Nokia, Nokia Shanghai Bell	discussion	Rel-18

R2-2301012	Enhancements for neighbor cell measurements	NEC	discussion	Rel-18	IoT_NTN_enh-Core

R2-2301187	Consideration on enhancements for the neighbour cell measurement	Xiaomi	discussion

R2-2301253	Discussion on enhancements for neighbour cell measurements	CMCC	discussion	Rel-18	IoT_NTN_enh

R2-2301602	Discussion on Enhancements for neighbour cell measurements	Transsion Holdings	discussion	Rel-18

Withdrawn

R2-2301624	Discussion on Enhancements for neighbour cell measurements	Transsion Holdings	discussion	Rel-18	Withdrawn

#### 8.6.3.2	Other

R2-2300145	Discussion on CHO enhancements	Huawei, HiSilicon, Turkcell	discussion	Rel-18	IoT_NTN_enh

Proposal 1: Same as NR NTN, time/location-based trigger condition is always configured together with one of the measurement-based trigger conditions (CHO events A3/A4/A5).

- Oppo thinks we should align to R17 NR NTN

- QC agrees

- Panasonic agrees. A RF measurement is absolutely crucial

- Ericsson does not agree. We should allow a time/location condition alone. Samsung/IDC agree. ZTE agrees

- QC thinks that at least it should be combined with A4.

- Nokia wonders how to select a target cell then. Mediatek wonders if it could be a cell with no signal

Proposal 2: Same as NR NTN, the network does not configure the location based CHO and time based CHO simultaneously for the same candidate cell.

R2-2301871	Conditional Handover in IoT NTN	Ericsson	discussion	Rel-18	IoT_NTN_enh

Observation 1	In NTN, a significant amount of (C)HO are due to service or feeder link switch, where RRM measurement trigger may not be required.

Proposal 1	It should be possible to configure location-based or time-based CHO triggers standalone for, eMTC devices in NTN, i.e., without a companion RRM measurement-based event.

R2-2300981	IDLE mobility for moving cells in IoT NTN	Lenovo	discussion	Rel-18

Proposal 1: For Earth-moving cells, RAN2 to discuss how to indicate UE/location-specific cell stop serving time for IDLE mobility.

Proposal 2: For Earth-moving cells, RAN2 to discuss how to indicate the feeder link switch time for IDLE mobility.

Proposal 3: RAN2 to discuss if distance-based enhancements are considered for IDLE mobility in IoT NTN.

R2-2300163	Discussion on mobility enhancements for eMTC NTN	OPPO	discussion	Rel-18	IoT_NTN_enh-Core

R2-2300265	On Mobility Enhancements in IoT-NTN	MediaTek Inc.	discussion

R2-2300749	Mobility enhancement in IoT NTN	Apple	discussion	Rel-18	IoT_NTN_enh

R2-2301495	Other mobility enhancements	Samsung Electronics Benelux BV	discussion	Rel-18	IoT_NTN_enh

### 8.6.4	Enhancements to discontinuous coverage

R2-2300206	Discussion on enhancements to discontinuous coverage	CATT	discussion	Rel-18	IoT_NTN_enh-Core

R2-2300266	On Enhancements to discontinuous coverage	MediaTek Inc.	discussion

R2-2300501	Impact of the UE Unreachability Periods on UE AS	Google Inc.	discussion	Rel-18

R2-2300582	IoT-NTN discontinuous coverage enhancements	Interdigital, Inc.	discussion	Rel-18	IoT_NTN_enh-Core

R2-2300654	Discussion on power saving enhancements for supporting discontinuous coverage	Spreadtrum Communications	discussion	Rel-18

R2-2300751	Support on discontinuous coverage in IoT NTN	Apple	discussion	Rel-18	IoT_NTN_enh

R2-2300878	Considerations on Supporting Discontinuous Coverage	NEC Europe Ltd	discussion	Rel-18	IoT_NTN_enh-Core

R2-2300890	RRC release procedure in discontinuous coverage	Qualcomm Incorporated	discussion	Rel-18	IoT_NTN_enh-Core

R2-2300926	On  IoT-NTN enhancements for discontinuous coverage	Nokia, Nokia Shanghai Bell	discussion	Rel-18

R2-2300982	On mobility and power saving issues for discontinuous coverage	Lenovo	discussion	Rel-18

R2-2301057	Discussion on discontinuous coverage enhancements	ZTE Corporation, Sanechips	discussion	IoT_NTN_enh-Core

R2-2301106	Discontinuous coverage enhancements	Samsung Electronics Nordic AB	discussion

R2-2301188	Discussion on enhancements to discontinuous coverage	Xiaomi	discussion

R2-2301210	Discussion on the discontinuous coverage	Huawei, Turkcell, HiSilicon	discussion	Rel-18	IoT_NTN_enh-Core

R2-2301254	Discussion on the discontinuous coverage for IoT-NTN	CMCC	discussion	Rel-18	IoT_NTN_enh

R2-2301603	Discussion on enhancement to discontinuous coverage for IoT NTN	Transsion Holdings	discussion	Rel-18

R2-2301862	Complementing discontinuous coverage with minimum support for discontinuous feeder link operation	Sateliot, GateHouse, Novamint, Intelsat, Airbus	discussion	Withdrawn

R2-2301870	IoT NTN Enhancements to discontinuous coverage	Ericsson	discussion	Rel-18	IoT_NTN_enh

R2-2301886	Complementing discontinuous coverage with minimum support for discontinuous feeder link operation	Sateliot, GateHouse, Novamint, Intelsat, Airbus, Hispasat, ESA, TNO	discussion

[AT121][102][IoT NTN enh] Discontinuous coverage (Mediatek)

Initial scope: Discuss proposals in 8.6.4

Initial intended outcome: Summary of the offline discussion with e.g.:

- List of proposals for agreement (if any)

- List of proposals that require online discussions

Deadline for companies' feedback:  Wednesday 2023-03-01 06:00 EET

Deadline for rapporteur's summary (in R2-2301952): Wednesday 2023-03-01 12:00 EET

R2-2301952	[offline 102] Enhancements to discontinuous coverage	Mediatek	discussion	Rel-18	IoT_NTN_enh-Core

Proposal 1: RAN2 will discuss if UE should provide out-of-coverage information as an assistance to the network (eNB)

Proposal 2a): For earth-moving cells, some assistance information (similar to NR-NTN) will be broadcast in SIB31 to assist the UE to verify if the remaining time of current cell’s coverage is sufficient to accommodate a new connection establishment.

Proposal 2b): The decision if UE will initiate the connection establishment if the remaining time in the current cell is not sufficient for a new connection establishment is left up to UE implementation.

Proposal 2c): Additional measurement assistance information (e.g., PCI or serving frequency) to help UE accelerate measurements and re-gain uplink sync more efficiently is left for FFS.

Proposal 3: RAN2 will check if dedicated RRC signalling can be used for providing satellite information corresponding to discontinuous coverage.

- HW/ZTE think that this kind of information is common for the UEs and should rather be broadcast

- RAN2 can continue to check whether dedicated RRC signalling can be used for providing satellite information corresponding to discontinuous coverage.

Proposal 4: RAN2 will discuss if RRC Release message needs any change/enhancement to enhance discontinuous coverage.

Proposal 5: RAN2 will discuss possible enhancements in paging and eDRX, while taking into account works of SA2 and CT1.

- RAN2 will support enhancements in paging and eDRX, in alignment with the work in SA2 and CT1. FFS on the details

Proposal 6: RAN2 will not include a new IE in SIB 31 to signal that the cell is operating in store and forward mode.

- QC thinks that if we have any indication it should preferably be at application layer (not AS layer)

- Companies supporting the store and forward approach can bring a proposal to the plenary for TEI18 or for updating the WID

Proposal: Enhancements for connected UE upon detecting discontinuous coverage (e.g., enter IDLE, suspend RLM, RLF detection, and RRC re-establishment process) are FFS (14/21).

- RAN2 may consider enhancements for connected UE upon detecting discontinuous coverage (e.g., suspend RLM, RLF detection, and RRC re-establishment process)

Agreements:

1.	RAN2 can continue to check whether dedicated RRC signalling can be used for providing satellite information corresponding to discontinuous coverage.

2.	RAN2 will support enhancements in paging and eDRX, in alignment with the work in SA2 and CT1. FFS on the details

3.	RAN2 may consider enhancements for connected UE upon detecting discontinuous coverage (e.g., suspend RLM, RLF detection, and RRC re-establishment process)

4.	Companies supporting the store and forward approach can bring a proposal to the plenary for TEI18 or for updating the WID

Withdrawn

R2-2301625	Discussion on enhancement to discontinuous coverage for IoT NTN	Transsion Holdings	discussion	Rel-18	Withdrawn

## 8.7	NR NTN enhancements

(NR_NTN_enh -Core; leading WG: RAN1; REL-18; WID: RP-223534)

Time budget: 1 TU

Tdoc Limitation: 4 tdocs

### 8.7.1	Organizational

LSs, rapporteur inputs and other organizational documents. Rapporteur inputs and other pre-assigned documents in this AI do not count towards the tdoc limitation.

Initial running CRs are expected to be submitted for the next meeting:

CR rapporteurs:

38.300: Thales

38.331: Ericsson

38.321: Interdigital

38.304: ZTE

38.306: Intel

Work plan

R2-2301344	R18 WI NR-NTN-enh work plan at RAN1, 2 and 3	THALES	Work Plan	Rel-18	R2-2210766

- Noted

Incoming LSs

RACH-less

R2-2300020	Reply LS on RACH-less handover in NTN (R1-2213001; contact: OPPO)	RAN1	LS in	Rel-18	NR_NTN_enh-Core 	To:RAN2, RAN4

- Noted

Latency impact for NTN verified UE location

R2-2300062	Reply LS on Latency impact for NTN verified UE location (S1-223539; contact: Xiaomi)	SA1	LS in	Rel-18	NR_NTN_enh-Core 	To:RAN2, SA2	Cc:RAN1, RAN3, RAN

- Noted

R2-2300066	LS Response on Latency impact for NTN verified UE location (S2-2211199; contact: Qualcomm)	SA2	LS in	Rel-18	5GSAT_ARCH	To:RAN2	Cc:SA1, RAN1, RAN3, RAN

- Noted

- RAN2 will work on a solution that ensures that location verification can be completed within a period of approximately 1 minute maximum and 30 seconds preferably.

### 8.7.2	Coverage Enhancements

Treated with lowest priority at this meeting, as the work is expected to progress in RAN1 first.

R2-2300347	Discussion on coverage enhancement for R18 NTN	vivo	discussion

R2-2301363	Blind Msg3 retransmission in Rel-18 NTN	InterDigital	discussion	Rel-18	NR_NTN_enh-Core

R2-2301524	Modification of Msg3 for coverage enhancements	NEC Telecom MODUS Ltd.	discussion

R2-2301637	Discussion on inital blind Msg3 retransmssion	LG Electronics Inc.	discussion	Rel-18	NR_NTN_enh-Core

R2-2301661	Discussion on Coverage Enhancement for NR NTN	Nokia, Nokia Shanghai Bell	discussion	Rel-18	NR_NTN_enh-Core

### 8.7.3	Network verified UE location

R2-2300528	discussion on network verified UE location	Ericsson	discussion	Rel-18	NR_NTN_enh-Core

Proposal 1	From a RAN2 point of view, it is the AMF that is in control of services for a UE and AMF will not allow the UE to run services before the network has verified the UE's location during the initial connection setup (i.e., attachment) phase.

- VC, Apple, Nokia don’t think that RAN2 needs to do anything about this

Proposal 2	NTN UE may be allowed by the network to continue operating the existing services before the network has verified the UE’s location after the initial connection setup (i.e., attachment) phase.

Proposal 3	UE’s TA report is not trusted in NTN and therefore is not used during the network initiated verification procedure of UE’s location.

- Oppo thinks this is up to SA3 to check but unfortunately we did not send an LS on this

- CMCC thinks TA should be trusted

- QC agrees with P3 suggesting to remove the “trusted” part

- Apple, Nokia thinks we could wait for RAN1.

- ZTE agrees with CMCC. Differently from GNSS measurements, TA reporting is defined in 3GPP specs

- Intel supports p3, it makes sense to minimize UE involvement

- CATT thinks TA should be trusted but this is not part of the WID

- Xiaomi doesn’t see the need for additionally reporting the TA

- China Telecom has concerns with P3

- RAN2 will wait for RAN1 decision on this

Proposal 4	In multi-RTT based positioning for NTN UE, a UE’s Rx-Tx time difference measurement shall not be directly derived from the TA applied by the UE.

- Oppo thinks this is up to RAN1

- RAN2 will wait for RAN1 decision on this

Proposal 5	For network verified UE location, the verification procedure can only be triggered by the CN.

- Agreed (as already decided also by SA2)

Proposal 6	As in the legacy LCS framework, NTN UE needs to be in RRC_CONNECTED in order to perform the network initiated verification procedure.

- Xiaomi, Oppo wonders why excluding Inactive UEs

- Network initiated verification procedure can be triggered by the NW when the UE is in RRC Connected. FFS whether the NTN UE can perform/report measurements also when in Inactive state.

Proposal 7	If the NTN UE’s location is not verified, the gNB is signaled of the existing failure cause i.e., “UE not in PLMN serving area” by the AMF and no new failure cause is needed.

- Indications to the AMF to be discussed in RAN3

Proposal 8	Reuse the existing UE capabilities on positioning to indicate whether the UE supports the feature of network verified UE location. A new additional UE capability is not needed.

- Apple is not sure how we can reuse existing capabilities

- Oppo thinks we can consider p8 as a Working Assumption

- Come back to this once the detailed mechanism will be defined

Proposal 9	For network verification of an NTN UE location (especially if multi-RTT is not supported by the UE), adopt UL-AoA based positioning as one candidate positioning method for the UE.

- CMCC wonders whether we have time to consider another method. QC agrees. CATT also agree

- HW thinks we should discuss first in RAN1. Intel agrees.

- Lenovo thinks we should wait for RAN1

- RAN2 will wait for RAN1 decision on this

Observation 7	For an NTN UE which doesn’t support the required RAT dependent positioning methods, it is up to NW (e.g., AMF) implementation on how to refrain the UE from accessing the corresponding service.

- RAN2 will not specify an AS mechanism to prevent UEs not supporting the required RAT dependent positioning methods to access the network.

Agreements:

- For network verified UE location, the verification procedure can only be triggered by the CN.

- Network initiated verification procedure can be triggered by the NW when the UE is in RRC Connected. FFS whether the NTN UE can perform/report measurements also when in Inactive state.

- RAN2 will not specify an AS mechanism to prevent UEs not supporting the required RAT dependent positioning methods to access the network

R2-2300272	On Network Verified UE Location in NR NTN	MediaTek Inc.	discussion

Proposal 1: If RAN1 agrees to Multiple-RTT method then RAN2 should discuss a procedure where the UE is configured to transmit UE-specific TA reports several times for a serving cell over a short period of time immediately after moving to connected.

R2-2300364	Discussion on the single satellite Multi-RTT positioning method in NTN	Intel Corporation	discussion	Rel-18	NR_NTN_enh-Core

Observation 1: the UE location verification procedure is triggered by the serving AMF, which is already supported in current specification.

Observation 2: the difference between legacy Multi-RTT and single satellite Multi-RTT is that, in NTN scenario single RTT value between a UE and the serving cell needs to be measured multiple times at different instances of time within a given serving cell.

Observation 3: It is already supported in current specification that for single TRP, UE can report the measurement results of Rx-Tx time difference for multiple measurement instances.

And we propose:

Proposal 1: Legacy signalling procedure of location service can be reused for the purpose of network verified UE location in NTN.

- Intel thinks the signalling procedure is the same for all positioning methods so we don’t need to clarify this further

- RAN2 assumes that, as a baseline, legacy signalling procedure of location service can be reused for the purpose of network verified UE location in NTN.

Proposal 2: Regarding the measurements of UE/gNB Rx-Tx Time Difference and measurement reporting, existing Multi-RTT mechanism can be reused to support network verified UE location in NTN.

- ZTE thinks that RAN1 is discussing this and we should wait for them. QC agrees. Vivo agrees. Ericsson also agrees,

- Wait for RAN1 progress on this

Proposal 3: an LS is sent to RAN3 to ask whether additional assistance data for Multi-RTT can be provided by gNB (instead of UE) to LMF, and at least serving satellite’s location and feeder link RTT are required.

- CATT thinks no LS is needed.

- Oppo thinks we should postpone this. Apple agrees

Proposal 4: RAN2 to assume that UE can finish Multi-RTT positioning procedure without cell change.

Agreement:

- RAN2 assumes that, as a baseline, legacy signalling procedure of location service can be reused for the purpose of network verified UE location in NTN.

R2-2300882	Single satellite Multi-RTT based positioning	Qualcomm Incorporated	discussion	Rel-18	NR_NTN_enh-Core

Proposal 1	Existing multi-RTT-based positioning method is used for network verified UE location. FFS on change on reporting format for  UE/gNB Rx-Tx time difference report based on RAN1 progress.

Proposal 2	Existing latency reduction techniques such as storing UE positioning capabilities at AMF and location scheduling in advance are applicable in NTN. FFS on the optimization in case of moving cell (i.e., frequency handover) for latency reduction such as optimization to Preconfigured Assistance Data.

Proposal 3	The TRP Information IE in response to TRP information request can be extended to carry additional information such as satellite information. FFS on other information like beam information and coarse UE location information. Discuss whether to send LS to RAN3 for feedback.

Proposal 4	The mirror point issue can be resolved by properly configuring neighbor cell measurement to UE,  for example, measurement of two neighbor cells in the opposite side of a satellite beam.

- ZTE thinks can be handled by implementation

- Huawei thinks cannot cover all cases.

- RAN2 assumes that in general the mirror point issue can be resolved by properly configuring neighbor cell measurement to UE, for example, measurement of two neighbor cells in the opposite side of a satellite beam. FFS if there are any cases that require anything in the specs

Proposal 5	RAN2 discuss whether Rel-17 positioning in RRC_INACTIVE is needed to be optimized for network verified UE location solution.

Agreement:

- RAN2 assumes that in general the mirror point issue can be resolved by properly configuring neighbor cell measurement to UE, for example, measurement of two neighbor cells in the opposite side of a satellite beam. FFS if there are any cases that require anything in the specs

R2-2300176	Discussion on network verified UE location	OPPO	discussion	Rel-18	NR_NTN_enh-Core

R2-2300207	Discussion on Network Verified UE Location	CATT	discussion	Rel-18	NR_NTN_enh-Core

R2-2300731	Network Verified UE Location	Apple	discussion	Rel-18	NR_NTN_enh-Core

R2-2301069	Discussion on NTN NW verified UE location	Lenovo	discussion	Rel-18

R2-2301119	Further discussion on Network Verified UE Location in NTN	Samsung Electronics Nordic AB	discussion

R2-2301140	Consideration on NW verified UE location	ZTE Corporation, Sanechips	discussion	Rel-18

R2-2301183	Discussion on network verified UE location	Xiaomi	discussion

R2-2301211	Further consideration on network verified UE location	Huawei, Turkcell, HiSilicon	discussion	Rel-18	NR_NTN_enh-Core

R2-2301354	On Network verified UE location	Nokia, Nokia Shanghai Bell	discussion

R2-2301837	Discussion on Network Verified Location	TCL Communication Ltd.	discussion	Rel-18

### 8.7.4	NTN-TN and NTN-NTN mobility and service continuity enhancements

#### 8.7.4.1	Cell reselection enhancements

##### 8.7.4.1.1	NTN-TN enhancements

TN coverage area

R2-2300363	Discussion on TN-NTN cell reselection enhancements	Intel Corporation	discussion	Rel-18	NR_NTN_enh-Core

Observation 1: The explicit description of geographical TN area seems more straightforward and with less spec impact.

And we propose:

Proposal 1: RAN2 adopts explicit description of geographical TN area, and focuses on the following options for further discussion:

Option 1: for each TN neighbour cell, the corresponding geographical area information is provided by network with location coordinates of cell center and cell radius.

Option 2: a boundary line is provided by network in the format of a list of location coordinates, additionally an indication can be used to indicate which side is the TN side

Option 6: for each TN area, a list of locations is provided by network, and the corresponding close shape could be illustrated by a polygon connecting these points within the list.

- HW thinks that whatever option is used needs to be associated with the frequency of the neighbour cells. Vivo thinks this should be a per frequency configuration

- TN coverage area information will be associated to the frequency information.

- Lenovo could be fine with p1 but has security concerns issues with option 1

- QC thinks we need to consider the signalling load

- RAN2 adopts explicit description of geographical TN area, and focuses on the following options for further discussion, taking the signalling overhead into account (FFS on the accuracy of the information):

Option 1: The corresponding geographical area information is provided by network with location coordinates of area center and radius.

Option 2: a boundary line is provided by network in the format of a list of location coordinates, additionally an indication can be used to indicate which side is the TN side

Option 6: for each TN area, a list of locations is provided by network, and the corresponding close shape could be illustrated by a polygon connecting these points within the list.

Agreements:

- TN coverage area information will be associated to the frequency information.

- RAN2 adopts explicit description of geographical TN area, and focuses on the following options for further discussion, taking the signalling overhead into account (FFS on the accuracy of the information):

Option 1: The corresponding geographical area information is provided by network with location coordinates of area center and radius.

Option 2: a boundary line is provided by network in the format of a list of location coordinates, additionally an indication can be used to indicate which side is the TN side

Option 6: for each TN area, a list of locations is provided by network, and the corresponding close shape could be illustrated by a polygon connecting these points within the list.

TN coverage area & NW type indication

R2-2300476	On Enhanced Cell Reselection in Rel-18 NTN	Nokia, Nokia Shanghai Bell	discussion	Rel-18	NR_NTN_enh-Core

Observation 1: The UE needs to know if it is close to the TN but does not require the exact coverage area.

Proposal 1: RAN selects the information comprising the reference location and a distance threshold of TN neighbour cells to inform the UE on the TN coverage.

Observation 2: The TN coverage does rarely change, and it is the same for all UEs.

Observation 3: Providing TN’s coverage via broadcast signalling is not practical for EMC as the information needs to be referenced to Earth-fixed points.

Proposal 2: Broadcast signalling is used to provide the information on the TN coverage area for UEs supporting NTN.

- As a baseline, broadcast signalling is used to provide the information on the TN coverage area for UEs supporting NTN.

Proposal 3:  UE-specific corrections to what has been delivered via broadcast signalling for TN coverage can be optionally provided via dedicated signalling.

- Vivo wonders if the UE-specific info would be provided by RRC or Higher layers)

- HW, Ericsson, Oppo think it’s too early to decide and we can come back to this after discussing the broadcast signalling

- Also based on the signalling overhead of the broadcast solution, RAN2 will further consider the option that UE-specific update can be optionally be provided via dedicated signalling, overriding the broadcast configuration (FFS if via RRC or higher layers. FFS on the validity time, if provided by RRC)

Observation 4: Explicit cell type indication (NTN versus TN) has been discussed during Rel-17 work. The option was not pursued.

Observation 5: In SIB19 NTN-capable UE can be provided with up to 8 neighbour NTN cells. The information includes cell’s PCI and carrier frequency.

Observation 6:  Thanks to the information available in SIB2, SIB3, SIB4, SIB5 and SIB19 UE knows on which frequency carriers there are TN or NTN neighbours.

Proposal 4: RAN2 does not pursue any solution for explicit indication to identify TN cells from inter-frequency list or inter-RAT frequency lists.

- Oppo/vivo support this

- QC thinks that based on the agreement that we associate the TN area coverage info with a frequency we don’t need to discuss this.

- Come Back Friday

- Postponed

Agreements:

- As a baseline, broadcast signalling is used to provide the information on the TN coverage area for UEs supporting NTN.

- Also based on the signalling overhead of the broadcast solution, RAN2 will further consider the option that UE-specific update can be optionally be provided via dedicated signalling, overriding the broadcast configuration (FFS if via RRC or higher layers. FFS on the validity time, if provided by RRC)

R2-2301479	Discussion on NTN-TN cell reselection enhancements	Samsung Research America	discussion	Rel-18	NR_NTN_enh-Core

Observation 1: For HAPS, NR operating band n1 is used, the UE cannot distinguish TN cell and NTN cell by the frequency band number.

Observation 2: The UE cannot know the cell type (i.e., TN or NTN) from the serving cell’s system information if the neighbor cell’s NTN assistance information is not included in the serving cell’s system information.

Observation 3: The UE cannot know the cell type by frequency band number or ARFCN or NTN assistance information in certain scenarios (e.g., NR band n1). This issue exists for both intra-frequency and inter-frequency neighbor cell measurement in cell reselection.

Observation 4: TN coverage information with accuracy/granularity up to multiples of TN cell size is sufficient for UE to identify TN coverage area.

Observation 5: Location-based cell reselection criteria needs to be introduced to compensate legacy cell reselection criterion and to align with RRC connected UE mobility.

Proposal 1: Cell type (i.e. TN or NTN) of a neighbor cell can be indicated explicitly for intra-frequency and inter-frequency cell reselection when the cell type cannot be implicitly indicated.

Proposal 2: System information is used to provided TN coverage information. FFS UE dedicated signal.

Proposal 3: For TN coverage information, reference location and distance threshold indicating TN coverage range are provided for UE to determine a TN coverage area.

Proposal 4: RAN2 to resume discussion on location-based cell reselection criteria.

TN neighbor cell measurement & Priority based TN and NTN cell reselection

R2-2300732	NTN-TN Cell Reselection Enhancement	Apple	discussion	Rel-18	NR_NTN_enh-Core

<UE operation on TN neighbor cell measurement>

Proposal 3: UE can change the frequency priority of TN neighbor cells based on whether the area has TN network coverage.

Proposal 3a: UE sets TN frequency as higher priority when it identifies it’s in the area where there is  TN coverage.

Proposal 3b: UE sets TN frequency as lower priority when it identifies it’s in the area where there is  no TN coverage.

Proposal 4: For UE power saving purpose, UE may assume there is no TN cell coverage if UE doesnot detect RS on TN neighbor frequency for some time.

<Priority based TN and NTN cell reselection>

Proposal 5: Introduce the cell reselection based on the priority of TN cell and NTN cell .

Proposal 5.1: NW can provide the priority between TN cell and NTN cell via the UE dedicated signaling or broadcast signaling.

Proposal 5.2: If both TN cell and NTN cell have good radio quality, UE selects the cell with higher priority to camp and establish the RRC connection.

Proposal 5.3: The priority between TN and NTN can be configured per slicing, per service type, per mobility state, or per UE type.

- Oppo thinks there is nothing else to do on top of what specified in Rel-17. Mediatek agrees.

- Apple and IDC this is about priorities for measurement

- Google wonders if this will complicate things

- Oppo thinks we don’t need to re-discuss measurement prioritization

- We don’t introduce additional cell reselection prioritization rules for NTN vs TN in Rel-18 (e.g. per service type, per mobility state, or per UE type) on top of what specified in Rel-17

Agreements:

- We don’t introduce additional cell reselection prioritization rules for NTN vs TN in Rel-18 (e.g. per service type, per mobility state, or per UE type) on top of what specified in Rel-17

R2-2300146	Discussion on NTN-TN cell reselection enhancements	Huawei, HiSilicon, Turkcell	discussion	Rel-18	NR_NTN_enh

R2-2300164	Discussion on NTN-TN cell reselection enhancement	OPPO	discussion	Rel-18	NR_NTN_enh-Core

R2-2300208	Discussion on Cell Reselection Enhancements in NTN-TN Scenario	CATT	discussion	Rel-18	NR_NTN_enh-Core

R2-2300345	Discussion on power saving for NTN-TN mobility	vivo	discussion

R2-2300511	Discussion on NTN-TN Mobility Enhancements in Idle State	Google Inc.	discussion	Rel-18

R2-2300798	Discussion on NTN-TN cell reselection enhancement	LG Electronics France	discussion	Rel-18	NR_NTN_enh

Moved from 8.7.4.1

R2-2300883	TN neighbour cell coverage information	Qualcomm Incorporated	discussion	Rel-18	NR_NTN_enh-Core

R2-2300983	IDLE mobility regarding NTN moving cells	Lenovo	discussion	Rel-18

R2-2300996	Discussion on NTN-TN Cell re-selection	ITL	discussion	Rel-18

R2-2301093	Cell selection/reselection enhancements in NTN-TN	Sony	discussion	Rel-18	NR_NTN_enh

R2-2301141	Consideration on cell reselection enhancements for NTN-TN	ZTE Corporation, Sanechips	discussion	Rel-18

R2-2301184	Cell reselection enhancements for NTN-TN mobility	Xiaomi	discussion

R2-2301225	Discussion on NTN-TN reselection	CMCC	discussion	Rel-18	NR_NTN_enh-Core

R2-2301365	NTN-TN mobility and service continuity	InterDigital	discussion	Rel-18	NR_NTN_enh-Core

R2-2301460	NTN-TN Mobility Cell Reselection	SHARP Corporation	discussion

R2-2301523	Details of the TN coverage data signalling	NEC Telecom MODUS Ltd.	discussion

R2-2301604	Further discussion on NTN-TN cell reselection enhancements	Transsion Holdings	discussion	Rel-18

R2-2301764	Further discussion on NTN-TN cell reselection enhancements	NTT DOCOMO, INC.	discussion	Rel-18

R2-2301869	TN NTN mobility enhancements	Ericsson	discussion	Rel-18	NR_NTN_enh

Withdrawn

R2-2301626	Further discussion on NTN-TN cell reselection enhancements	Transsion Holdings	discussion	Rel-18	Withdrawn

##### 8.7.4.1.2	NTN-NTN enhancements

Trigger for measurements

R2-2301142	Consideration on cell reselection enhancements for NTN-NTN	ZTE Corporation, Sanechips	discussion	Rel-18

Proposal 1: In R18, for earth-moving system, satellite with steerable beam is not considered as part of mobility enhancement in NTN.

- Agreed

Proposal 2: referenceLocation in SIB19 is used to broadcast the serving cell reference location for earth-moving cell.

- Panasonic does not agree with p2

- IDC supports this and doesn’t see the reason to introduce something different

- HW thinks we shouldn’t reuse the same IE we had for quasi-Earth fixed cells in R17. We should use a new IE.

- QC thinks reusing the same R17 IE could confuse R17 UEs

- A serving cell reference location and a distance threshold/radius will be broadcast for earth-moving cell. FFS on whether the R17 IEs are reused or not. FFS on whether additional information needs to be broadcast to inform the UE how the reference location moves over time or if this can be derived from other information (e.g. Epoch time and ephemeris).

Proposal 3: For earth-moving cell, epochTime of serving cell ephemeris is reused to indicate time information of serving cell reference location.

Proposal 4: Discussion on explicit indication to indicate earth-moving cell is postponed.

Proposal 5: No need to indicate t-service of serving cell for earth-moving cell.

Proposal 6: location-based measurement initiation is supported in earth-moving cell

- Agreed as: “For cell selection/reselection, location-based measurement initiation is supported in earth-moving cell”

Proposal 7: distanceThresh defined for location based measurement initiation for quasi-fixed system in R17 is reused in earth-moving system.

Proposal 8: For earth moving system, RAN2 further discuss if a new IE to indicate the distance threshold used for serving cell coverage is needed or not.

Proposal 9: Location based cell reselection should be supported by introducing a distance threshold.

Proposal 9a: For cell reselection, cell ranked on R-criterion is applied first and then the distance threshold is applied to down scope the candidate cells for reselection with the following steps in detail:

-  Step 1: UE perform cell ranking based on the R-criterion.

-  Step 2: Among the highest ranked N cells.

- For cells provided with reference location: only those whose distance to UE shorter than the distance threshold will be considered by UE as candidate cells.

- Cells not provided with reference location will not be considered as candidate cell for reselection

- Step 3: Among all the candidate cells decided by on the distance threshold in step 2, UE reselect to the highest ranked cell based on R-criterion.

Proposal 9b: Down select from the following options on how to determine the highest ranked N cells in Step 2:

Option 1: The N is not a fixed value and the highest ranked N cells are cells whose R value is within rangeToBestCell of the R value of the highest ranked cell.

Option 2: N is a fixed value in specification or configured by NW.

Proposal 10: Among neighbor cells with similar RSRP/RSRQ, those with longer valid time should be prioritized during cell reselection.

Proposal 10a: Down select from the following solutions on how to prioritize neighbor cells with longer valid time during cell reselection:

Solution 1: Broadcast a threshold of the valid time along with QoffsetTime as adjustment to cell-ranking criterion Rs for serving cell and Rn for neighboring cells so that cells with serving time longer than the threshold will get a bonus by adding the QoffsetTime in the R value calculation.

Solution 2: Broadcast a threshold of the valid time along with CellReselectionPriorityOffset as adjustment to the cell reselection priority so that the cells with serving time longer than the threshold will be prioritized by adding the CellReselectionPriorityOffset to the cell reselection priority configured.

Solution 3: A rangeToBestCellNTN is broadcast in system information. UE rank the neighbor cells based on the R-criterion while the cells whose R value is within range to best cell of the R value of the highest ranked cell will be considered as candidate cells. Among all these candidate cells, UE will reselect to the cell with longest serving time.

Agreements:

- In R18, for earth-moving system, satellite with steerable beam is not considered as part of mobility enhancement in NTN.

- A serving cell reference location and a distance threshold/radius will be broadcast for earth-moving cell. FFS on whether the R17 IEs are reused or not. FFS on whether additional information needs to be broadcast to inform the UE how the reference location moves over time or if this can be derived from other information (e.g. Epoch time and ephemeris).

- For cell selection/reselection, location-based measurement initiation is supported in earth-moving cell

R2-2300344	Discussion on cell reselection enhancements for earth-moving cell	vivo	discussion

Proposal 1: For the earth-moving cell, if the distance between the UE and reference location of the serving cell is larger than a configured distance threshold, the UE shall perform neighbour cell measurement for cell reselection.

Proposal 2: For the earth-moving cell, the existing parameter distanceThresh can be reused to provide the distance threshold for the location-based cell reselection.

Proposal 3: For the earth-moving cell, the reference location is signalled with an associated validity timer in the system information.

Proposal 4: If the validity timer of the reference location expires, UE re-acquires the reference location from the network.

Proposal 5: For the earth-moving cell, a new timer should be introduced as the validity timer for the reference location of the serving cell.

Proposal 6: An explicit cell type indication (i.e., quasi-fixed cell or earth-moving cell) to the UE is not needed.

Proposal 7: For the time-based measurement initiation in the earth-moving cell, two types of stop time (which intends to handle the stop time due to service-link switching and feeder-link switching respectively) should be supported.

Proposal 8: Similar to t-Service, the gNB configures a cell-level serving cell stop time that covers the stop time due to feeder link switching in the earth-moving cell.

Proposal 9: Similar to the location-based criterion, the UE calculates the stop time based on the distance between the UE and reference location of serving cell to cover the stop time due to service-link switching.

Proposal 10: The UE should start neighbour cell measurements before either type of the stop time is reached.

Cell re-selection criteria enhancements

R2-2300799	Discussion on NTN-NTN cell reselection enhancement	LG Electronics France	discussion	Rel-18	NR_NTN_enh

Proposal 1	In earth moving scenario, serving cell provides multiple reference locations and its time information.

Proposal 2	Distance threshold representing, e.g., edge of cell coverage should be provided to calculate RST of serving cell.

< distance-based cell reselection >

Proposal 3	Introduce distance-based cell reselection, i.e., UE should consider distance between UE location and reference location of neighbor cell to discriminate a neighbor cell that UE locates at the edge of the cell coverage.

Proposal 4	Distance-based cell reselection procedure should be introduced as follows in Rel-18 NTN.

Step-1. Exclude neighbor cell from candidate neighbor cell list if the distance between UE and its

reference location is larger than distance threshold.

Step-2. Cell ranking evaluation based on R-value criterion.

Step-3. UE reselects to the highest ranked cell.

< time-based cell reselection >

Proposal 5	Introduce time-based cell reselection, i.e., UE should consider remaining service time of neighbor cell to prevent UE from performing cell reselection to a neighbor cell about to terminate service.

Proposal 6	To adopt one of the following options for time-based reselection.

Opt-1) Filtering neighbor cell with RST.

Step-1. UE calculates RST of neighbor cells.

Step-2. UE treats a neighbor cell as barred if RST of neighbor cell is shorter than time threshold.

Step-3. UE performs R-value-based ranking evaluation to neighbor cells.

Step-4. UE performs cell reselection to the highest ranked cell.

Opt-2) Filtering neighbor cell with R-value.

Step-1. UE performs R-value-based ranking evaluation to neighbor cells.

Step-2. UE treats a neighbor cell as barred if R-value of neighbor cell is less than threshold.

Step-3. UE performs cell reselection to neighbor cell having the longest RST.

Opt-3) Introduce RST-based R-value offset, in which neighbor cell having longer RST gets bigger value of R-value offset.

Step-1. UE calculates RST of neighbor cells.

Step-2. UE performs R-value-based ranking evaluation to neighbor cells.

Step-3. UE derives new R-value by adding RST-based R-value offset to R-value derived in Step-2.

Step-4. UE performs cell reselection to highest ranked cell.

R2-2301226	Discussion on NTN-NTN reselection	CMCC	discussion	Rel-18	NR_NTN_enh-Core

Proposal 1: The real-time reference location of earth moving cell could already be estimated with the existed ephemeris information and no additional information is required.

Proposal 2: Kindly suggest RAN2 to consider down selection one solution among the 3 options for NTN-NTN reselection power consumption reduction:

Option 1: Introduce a distance threshold.

Cell ranked on R-criterion first and then the distance threshold applies to down scope the candidate cells for reselection.

Option 2: Introduce a distance threshold.

Distance threshold applies to decide the candidate cells and then rank the candidate cells based on R-criterion to decide the target cell for reselection.

Option 3: Cell ranked on R-criterion first and then the distance criteria apply to decide the target cell for reselection.

Proposal 3: It is proposed to support option 2 with alt.1 to enhance the cell reselection procedure for NTN-NTN scenario considering power consumption reduction.

R2-2301364	Cell reselection enhancements for Earth moving cell	InterDigital	discussion	Rel-18	NR_NTN_enh-Core

Proposal 1:	Prioritize location-based cell (re)selection enhancements for Earth-moving cells.

Proposal 2:	RAN2 to confirm the distance threshold provided for Earth-moving cells does not change frequently over time like the reference point.

Proposal 3:	For Earth-moving cells, the reference location is periodically updated to compensate for cell movement.

Proposal 4:	For Earth-moving cells, a change in reference location will not trigger system information change notification nor a modification of valueTag in SIB1.

Proposal 5:	Multiple future reference location coordinates and associated timestamp can be broadcast simultaneously to reduce frequency of SIB re-aquisition.

Proposal 6:	Study using serving cell ephemeris data to estimate additional reference location(s) (e.g. between broadcasted reference points).

Proposal 7:	Cell reselection enhancements based on the distance between a UE and neighbouring cell reference point(s) are supported in Rel-18.

Proposal 8:	In Earth-moving cell, the reference location and distance threshold of neighbouring cell(s) are provided by network for UE to estimate when one or more neighbouring cell(s) stop/start providing coverage at the present UE location.

Proposal 9:	In Earth-moving cell, NTN-NeighCellConfig can optionally contain reference location(s) and distance threshold(s) of neighbouring cell(s).

[AT121][104][NR NTN enh] NTN-NTN cell reselection (ZTE)

Initial scope: continue the discussion on trigger for measurements and cell reselection criteria enhancements based on the selected papers above

Initial intended outcome: Summary of the offline discussion with e.g.:

- List of proposals for agreement (if any)

- List of proposals that require online discussions

Deadline for companies' feedback:  Thursday 2023-03-02 22:00 EET

Deadline for rapporteur's summary (in R2-2301953): Friday 2023-03-03 08:00 EET

R2-2301953	[offline 104] NTN-NTN cell reselection	ZTE	discussion	Rel-18	NR_NTN_enh

For Agreements:

P3: For earth-moving cell, new IE is introduced to indicate the reference location of serving cell. (19/26)

P4: For earth-moving cell, the location-based cell measurement rules of quasi-fixed cell is reused, i.e., for cell reselection in earth-moving cell, UE initiates measurements when its location to serving cell reference location is larger than the configured distance threshold. (26/27)

- Agreed

P5: For cell (re)selection in earth-moving system, a distance threshold is introduced for location-based measurement  initiation, which reuses distanceThresh in SIB19. (16/26)

P6: For cell (re)selection in earth-moving system, time-based measurement initiation is used to address feeder-link switch case. (19/24)

Online discussion:

P 1: For earth-moving cell with fixed beam, the trajectory of serving cell reference location can be derived by UE based on satellite’s ephemeris and ephochTime.  (19/28)

- Samsung would like to understand what fixed beams means in the previous agreement

- QC suggests to remove the fixed beams part

- Vivo thinks this method does not work

- ZTE understand the possible concern is about the different ephemeris format but think that both provide equivalent information

P2. If confirmed UE can derive the trajectory of serving cell reference location based on satellite’s ephemeris and epochTime, no additional assisting information is needed.

P 7: RAN2 further discuss whether to support location-based cell reselection criteria. (support: 12, not support: 11)

P 8: Time-based cell reselection criteria is not pursued in R18. (support: 8, not support:15 )

- Long post meeting email disc on NTN-NTN cell reselection (ZTE)

Agreements:

- For earth-moving cell, the location-based cell measurement rules of quasi-fixed cell is reused, i.e., for cell reselection in earth-moving cell, UE initiates measurements when its location to serving cell reference location is larger than the configured distance threshold.

[POST121][106][NR NTN Enh] NTN-NTN cell reselection (ZTE)

Scope: Continue the discussion on NTN-NTN cell reselection aspects (triggers for measurements, derivation of trajectory of serving cell reference location, cell reselection criteria enhancements, etc.)

Intended outcome: report of the email discussion

Deadline:  Long

R2-2300147	Discussion on NTN-NTN cell reselection enhancements	Huawei, HiSilicon, Turkcell	discussion	Rel-18	NR_NTN_enh

R2-2300165	Discussion on NTN-NTN cell reselection enhancement	OPPO	discussion	Rel-18	NR_NTN_enh-Core

R2-2300241	Issues on NTN Mobility	Lockheed Martin	discussion	Rel-18

R2-2300362	Discussion on NTN-NTN cell reselection enhancements	Intel Corporation	discussion	Rel-18	NR_NTN_enh-Core

R2-2300466	Neighbour cell signalling overhead reduction	PANASONIC	discussion

Moved from 8.7.4.1

R2-2300451	Discussion on cell reselection in earth moving cell	Quectel	discussion	Rel-18	Late

R2-2300509	Discussion on NTN-NTN Mobility Enhancements in Idle State	Google Inc.	discussion	Rel-18	R2-2212893

R2-2300655	Discussion on NTN-NTN cell reselection enhancements	Spreadtrum Communications	discussion	Rel-18

R2-2300733	NTN-NTN  Cell Reselection Enhancement	Apple	discussion	Rel-18	NR_NTN_enh-Core

Moved from 8.7.4.1

R2-2300884	Neighbor cell measurement relaxation	Qualcomm Incorporated	discussion	Rel-18	NR_NTN_enh-Core

R2-2300984	Measurement for cell reselection in NTN with TN cells involved	Lenovo	discussion	Rel-18

R2-2300995	Discussion on NTN-NTN Cell re-selection	ITL	discussion	Rel-18

R2-2301185	Cell reselection enhancements for NTN-NTN mobility	Xiaomi	discussion

R2-2301480	Discussion on Cell Reselection with Earth-moving Cell	Samsung Research America	discussion	Rel-18	NR_NTN_enh-Core

R2-2301535	Discussion on reference location for moving cell	ASUSTeK	discussion	Rel-18	NR_NTN_enh-Core

R2-2301605	Further discussion on NTN-NTN  cell reselection enhancements	Transsion Holdings	discussion	Rel-18

R2-2301868	NTN NTN mobility enhancements	Ericsson	discussion	Rel-18	NR_NTN_enh

Withdrawn

R2-2301627	Further discussion on NTN-NTN  cell reselection enhancements	Transsion Holdings	discussion	Rel-18	Withdrawn

#### 8.7.4.2	Handover enhancements

group HO (“UE specific pre-configuration of the target cell + group HO indication”) / broadcast common (C)HO configuration / enhancements for Earth-moving cells

R2-2300177	Discussion on NTN handover enhancements	OPPO	discussion	Rel-18	NR_NTN_enh-Core

< UE specific pre-configuration of the target cell + short (possibly broadcast) HO command >

Proposal 1	Network can trigger handover by simply indicating target cell’s ID/index. The target cell’s configuration can be pre-configured to the UE, e.g. via CHO configuration. FFS on the triggering signaling, e.g. MAC CE or RRC message. FFS whether it can be sent in a groupcast manner and needs RAN1’s involvement.

- ZTE thinks this is similar to what we have for CHO and we could have some CHO ID without using other signalling

- HW wonders what the benefit is and only sees complexity

- Nokia wonders which scenario is addressed. VDF agrees. LG also and thinks the real problem in feeder link switch is RACH congestion, but this proposal doesn’t solve it. Sequans also has similar concerns

- Oppo thinks this could be used in case of feeder link switch. Also for load balancing reasons

From R2-2300885:

Proposal 1	Support group HO command that includes HO commands for multiple UEs to reduce signaling overhead from the source cell.

< broadcast common (C)HO configuration >

Proposal 2	As a starting point, t304 and spCellConfigCommon within ReconfigurationWithSync can be delivered to UEs in common signalling. FFS on other information.

- LG thinks this doesn’t have much benefit

- QC thinks this information should not be broadcast, the UE should not acquire SI before executing HO, fine if this provided by groupcast signalling

- VDF thinks that broadcast signalling in the NW is very expensive

- Ericsson sees benefits but would like to see how much we would save first. Mediatek agrees

- Nokia is fine to identify IEs which would be common and could be sent only once.

- CMCC thinks some common information could be broadcast to all UEs

- Continue in the next meeting, to show the possible signalling gain of the proposal to have some common (C)HO configuration. FFS the number of cells that could be signalled. FFS whether broadcast or groupcast signalling could be used.

< location-based CHO enhancements for Earth-moving cells >

Observation 1	With the existing condEventD1, for earth-moving cells, network may have to frequently update the reference locations for both serving cell and candidate cells, which will increase a lot of signalling overhead.

Proposal 3	For location-based CHO execution to work for earth-moving cells, RAN2 discuss solutions for UE to acquire the serving cell and candidate cell’s reference locations which do not require frequent reconfiguration of reference locations.

- Mediatek can support CHO enhancements for Earth-moving cells if on top of existing measurements

- For location-based CHO for earth-moving cells we follow the solution being investigated for cell reselection to allow the UE to derive the serving cell’s reference locations as the cells move. FFS whether the same mechanism can also be used for the candidate cell’s reference location

Agreements:

- Continue in the next meeting, to show the possible signalling gain of the proposal to have some common (C)HO configuration. FFS the number of cells that could be signalled. FFS whether broadcast or groupcast signalling could be used.

- For location-based CHO for earth-moving cells we follow the solution being investigated for cell reselection to allow the UE to derive the serving cell’s reference locations as the cells move. FFS whether the same mechanism can also be used for the candidate cell’s reference location

RACH-less

R2-2301867	Handover enhancements	Ericsson	discussion	Rel-18	NR_NTN_enh

Proposal 1	RAN2 to prioritize CHO enhancements.

Proposal 2	RAN2 to study and quantify the signalling gains of broadcasting (C)HO common target cell configuration in System Information.

Proposal 3	Support RACH-less Handover in Rel-18.

- Ericsson informs that RAN4 indicated that “RAN4 would like to thank RAN1 for the liaison in R1-2213001 regarding RACH-less handover in NTN. For NTN-NTN FR1-FR1 handover, RAN4 confirms that Note 1 in the LS (R1-2213001) is correct, i.e. the timing requirement specified in Table 7.1C.2-1 of TS 38.133 applies to the first UL transmission, including PUCCH, PUSCH, SRS, PRACH, and msgA, in the target cell, provided that At least one SSB is available at the UE during the last 160 ms. UE is provided with information such that the UE has valid N_"TA,adj" ^"common" and N_"TA,adj" ^"UE" upon handover execution. The network and UE have a common understanding of NTA component upon HO execution.”

- Support RACH-less Handover in Rel-18.

Proposal 4	RACH-less Handover in NR NTN is based on LTE’s RACH-less Handover which implies L3 mobility.

- Oppo thinks we can rephrase saying the we follow the LTE principle

- CMCC and HW thinks that, if combined with the unchanged PCI case, this would not require L3 mobility

- RACH-less Handover in NR NTN is a L3 mobility procedure (FFS if this is combined with the unchanged PCI approach, if supported) and uses the LTE’s RACH-less Handover procedure as a baseline. FFS on TA acquisition

Proposal 5	In NTN RACH-less handover, network indicates whether TA in the target cell is identical to the source cell or needs to be calculated by UE from the provided satellite assistance information.

- LG thinks that UE precompensation might not be sufficient

- IDC/Nokia/Mediatek support the proposal

- In NTN RACH-less handover, network indicates (implicitly or explicitly) whether NTA in the target cell is identical to the source cell or explicitly provided by the NW.

Proposal 6	Confirm the assumption that service link switch implies L3 mobility in Rel-18.

From R2-2300885:

Proposal 2	Support dynamic grant from the target cell for RACH-less PUSCH transmission to reduce random access congestion in the target cell. FFS whether to limit the solution to same feeder link/gateway scenario.

- QC thinks this is as in LTE

- Support dynamic grant from the target cell for RACH-less PUSCH transmission to reduce random access congestion in the target cell. FFS whether to limit the solution to same feeder link/gateway scenario.

Agreements:

- Support RACH-less Handover in Rel-18.

- RACH-less Handover in NR NTN is a L3 mobility procedure (FFS if this is combined with the unchanged PCI approach, if supported) and uses the LTE’s RACH-less Handover procedure as a baseline. FFS on TA acquisition

- In NTN RACH-less handover, network indicates (implicitly or explicitly) whether NTA in the target cell is identical to the source cell or explicitly provided by the NW.

- Support dynamic grant from the target cell for RACH-less PUSCH transmission to reduce random access congestion in the target cell. FFS whether to limit the solution to same feeder link/gateway scenario

R2-2300734	NTN specific Handover Enhancement	Apple	discussion	Rel-18	NR_NTN_enh-Core

Proposal 1: Consider the following solutions to reduce the signaling overhead of the target cell’s common configuration.

•	Option 1: Broadcast or groupcast the common config of the target cell in source cell;

•	Option 2: Request UE to acquire the common config of the target cell by itself;

•	Option 3: Provide the common config of the target cell by the delta signaling method.

Proposal 2: Consider the group handover scheme of UE specific pre-configuration of the target cell + group HO command as the solution to optimize the HO signaling overhead in NTN.

Proposal 3: Subsequent CHO should be considered for the HO in NTN moving cell deployment.

Proposal 4: In feeder link switching scenario, the handover is not transparent to UE if the security key is changed.

Proposal 5: In feeder link switching scenario, if feeder link latency is changed, UE is required to re-sync to the NW after the feeder link switching; otherwise, the re-sync operation can be skipped.

Proposal 6: In feeder link switching scenario, if no handover is triggered but mobility gap is configured, UE should suspend the activity in Uu interface during the gap.

Proposal 7: Consider the mechanism to mitigate the HO congestion, e.g., delay HO with a random time.

R2-2300477	On Enhanced NTN Connected-mode Mobility in Rel-18	Nokia, Nokia Shanghai Bell	discussion	Rel-18	NR_NTN_enh-Core

Proposal 1: RAN2 is asked to study if the fact that the next serving cells can be largely predicted in NTN Earth-moving scenario can be used to improve the mobility performance – for both the network and the UEs.

Proposal 2: RAN2 is asked to study how the UE can be provided with CHO configurations for cells beyond the next cell change (future candidate cells).

Proposal 3: Group HO or HO command pre-configuration is not pursued as a part of Rel-18 NTN enhancements unless clear gains are shown.

Proposal 4: Reusing PCI after satellite switches is not pursued as a part of Rel-18 NTN, unless the gains are clearly proven and identified issues can be easily resolved.

Proposal 5: When defining Rel-18 RACH-less HO for NTN, RAN2 considers the following aspects: how UE obtains UL grant? How it sets the initial UL Tx power? Can RACH-less be combined with CHO?

Proposal 6: Reconsider if CHO Recovery beyond the duration window of time-based CHO shall be allowed.

PCI unchanged

R2-2301269	Service Link Switching with PCI unchanged	CMCC,CATT,Huawei,HiSilicon,Lenovo,vivo	discussion	Rel-18	NR_NTN_enh

Proposal 1: RAN2 should aim to address the issue of the frequent and unavoidable handover for UEs (seconds, tens of seconds or hundreds of HO frequency), at least for quasi-earth fixed cell scenario, which will obviously reduce significant signaling overhead, power consumption, as well as the impact on the service performances caused by service interruption due to HO signaling latency.

Proposal 2: RAN2 assume no technical problem exists to implement PCI unchanging after satellites switching and an LS may be needed to RAN1 to confirm the view above, if needed.

Proposal 3: It is proposed to firstly make decision on how to address the issue of DL/UL re-sync awareness of the UE to avoid UL/DL out of synchronization during the satellite switching, either by notification by gNB via implicit information or explicit information. How to re-sync to a new arriving satellite is FFS.

Proposal 4: At least for hard satellite switching in quasi-earth fixed cell, satellites switching without PCI changing is feasible.

- Google thinks that different satellites might have different coverage and then wonders how this would work. HW thinks this would still work

- Nokia thinks that there would be RAN1 impacts and also wonders whether we would gain much as we would still need random access. CMCC thinks this would not have RAN1 impact

- Ericsson thinks we are introducing a new scenario (hard service link switch) we did not consider before. HW thinks this is not a new scenario and would not require RAN1 involvement.

- Samsung has strong similar concerns as Ericsson and Nokia and in case of hard switch there would always be interruption

- Sequans also has similar concerns as Ericsson and wonders how a R17 UE would behave in this kind of deployment

- Vivo agrees with Huawei and CMCC. Vivo thinks the target deployment (hard service link switch) is already there

- ZTE thinks both deployment are possible

- Working Assumption: in quasi-earth fixed cell, for hard satellite switch in the same SSB frequency and same gNB (no key change), satellite switching without PCI changing (not requiring L3 mobility) is supported.

Proposal 5: An LS to RAN1 may be needed to confirm the feasibility of the soft switching case.

From R2-2300885:

Proposal 3	Same PCI with hard satellite switch can be considered to reduce signaling overhead in quasi-earth fixed cell for the scenario of satellite switch with same gateway/gNB. Further discuss the details on interruption during the satellite switch.

From R2-2301867:

Proposal 6	Confirm the assumption that service link switch implies L3 mobility in Rel-18.

Working Assumption:

- In quasi-earth fixed cell case, for hard satellite switch in the same SSB frequency and same gNB (no key change), satellite switching without PCI changing (not requiring L3 mobility) is supported.

R2-2300885	Further handover enhancement for NTN	Qualcomm Incorporated	discussion	Rel-18	NR_NTN_enh-Core

Proposal 1	Support group HO command that includes HO commands for multiple UEs to reduce signaling overhead from the source cell.

Proposal 2	Support dynamic grant from the target cell for RACH-less PUSCH transmission to reduce random access congestion in the target cell. FFS whether to limit the solution to same feeder link/gateway scenario.

Proposal 3	Same PCI with hard satellite switch can be considered to reduce signaling overhead in quasi-earth fixed cell for the scenario of satellite switch with same gateway/gNB. Further discuss the details on interruption during the satellite switch.

R2-2300148	Discussion on NTN handover enhancements	Huawei, HiSilicon, Turkcell	discussion	Rel-18	NR_NTN_enh

R2-2300209	Discussion on PCI unchanged scenario	CATT	discussion	Rel-18	NR_NTN_enh-Core

R2-2300210	Discussion on NTN HO Enhancements	CATT	discussion	Rel-18	NR_NTN_enh-Core

R2-2300274	Handover Enhancement in LEO NTN with Earth-moving Cells	MediaTek Inc.	discussion

R2-2300346	On handover enhancement for siganlling overhead reduction in NR NTN	vivo	discussion

R2-2300361	Discussion on NTN 2-step handover	Intel Corporation	discussion	Rel-18	NR_NTN_enh-Core

R2-2300450	Discussion on NTN HO enhancnment	CAICT	discussion	Rel-18	NR_NTN_enh-Core

R2-2300514	SMTC and Measurement Gap Enhancements for Connected UEs	Google Inc.	discussion	Rel-18

R2-2300516	Discussion on NTN-TN Mobility Enhancements in Connected State	Google Inc.	discussion	Rel-18	R2-2212894

R2-2300800	Discussion on handover enhancement	LG Electronics France	discussion	Rel-18	NR_NTN_enh

R2-2300856	Discussion of HO common signaling reduction in NTN	China Telecom	discussion	Rel-18	NR_NTN_enh

R2-2300885	Further handover enhancement for NTN	Qualcomm Incorporated	discussion	Rel-18	NR_NTN_enh-Core

R2-2300985	Considerations on common signalling for CONNECTED mobility in NTN	Lenovo	discussion	Rel-18

R2-2301013	NTN-NTN handover enhancement	NEC	discussion	Rel-18	NR_NTN_enh-Core

R2-2301094	Signaling overhead reduction and group handover during NTN-NTN HOs	Sony	discussion	Rel-18	NR_NTN_enh

R2-2301143	Consideration on HO enhancements in NTN	ZTE Corporation, Sanechips	discussion	Rel-18

R2-2301186	Discussion on handover enhancements for NTN-NTN mobility	Xiaomi	discussion

R2-2301366	NTN mobility enhancements for RRC_CONNECTED	InterDigital	discussion	Rel-18	NR_NTN_enh-Core

R2-2301481	Discussion on NTN handover enhancements	Samsung Research America	discussion	Rel-18	NR_NTN_enh-Core

R2-2301504	Discussion on Handover enhancements for NTN	Fujitsu Limited	discussion	Rel-18	NR_NTN_enh-Core

R2-2301536	Discussion on RACH-less handover for NTN	ASUSTeK	discussion	Rel-18	NR_NTN_enh-Core

R2-2301537	Discussion on handover enhancement with common signalling	ASUSTeK	discussion	Rel-18	NR_NTN_enh-Core

R2-2301606	Further discussion on NTN-NTN handover enhancements	Transsion Holdings	discussion	Rel-18

R2-2301766	Further discussion on NTN-NTN handover enhancements	NTT DOCOMO, INC.	discussion	Rel-18

R2-2301821	Discussion on handover enhancements	Sharp	discussion	Rel-18	NR_NTN_enh-Core	R2-2212560

R2-2301864	NTN-NTN handover enhancements	Sequans Communications	discussion	Rel-18	NR_NTN_enh-Core

R2-2301866	HO/CHO Signaling Overhead Reduction by NTN-config omission	Sequans Communications	discussion	Rel-18	NR_NTN_enh-Core	R2-2212721

Withdrawn

R2-2300467	Handover timing improvement	PANASONIC	discussion	Late	Withdrawn

R2-2301628	Further discussion on NTN-NTN handover enhancements	Transsion Holdings	discussion	Rel-18	Withdrawn

# Summary

Agreed CRs

NR-NTN

R2-2301967	Correction on Stage-2 descriptions for NR NTN	vivo, Nokia, Nokia Shanghai Bell	CR	Rel-17	38.300	17.3.0	0611	-	F	NR_NTN_solutions-Core

- Agreed

R2-2301983	Corrections to NR NTN for 38.321	LG Electronics Inc., vivo	CR	Rel-17	38.321	17.3.0	1560	2	F	NR_NTN_solutions-Core

- Agreed

R2-2301988	IOT bit for inter satellite measurement (38.306)	MediaTek Inc.	CR	Rel-17	38.306	17.3.0	0853	1	F	NR_NTN_solutions-Core

- Agreed

R2-2301969	IOT bit for inter satellite measurement (38.331)	MediaTek Inc.	CR	Rel-17	38.331	17.3.0	3795	1	F	NR_NTN_solutions-Core

- Agreed

R2-2300470	38.306 CR On Enhanced Cell Reselection Requirements	Nokia, Nokia Shanghai Bell	CR	Rel-17	38.306	17.3.0	0856	-	F	NR_NTN_solutions-Core

- Intel is ok with this and thinks we should reply to RAN4

- Agreed

R2-2301981	CR to 38.304 on relaxed measurements	Huawei, HiSilicon	CR	Rel-17	38.304	17.3.0	0318	2	F	NR_NTN_solutions-Core

- Agreed

R2-2301984	Clarification on measurement relaxation in NTN	Apple	CR	Rel-17	38.331	17.3.0	3823	1	F	NR_NTN_solutions-Core

- Agreed

R2-2301971	RRC correction on epochTime and distanceThreshold	OPPO	CR	Rel-17	38.331	17.3.0	3777	1	F	NR_NTN_solutions-Core

- Agreed

R2-2301042	Clarification on essential SIB19 for NR NTN	Xiaomi, CAICT, Lenovo, Samsung, MediaTek, Apple, OPPO, CATT, CMCC, Qualcomm	CR	Rel-17	38.331	17.3.0	3861	-	F	NR_NTN_solutions-Core

- Agreed

R2-2301980	Correction to PDD reporting	vivo, Samsung, Huawei, HiSilicon	CR	Rel-17	38.331	17.3.0	3937	-	F	NR_NTN_solutions-Core

- Agreed

IoT-NTN

R2-2300357	Stage-2 correction on TDD support for IoT NTN	OPPO	CR	Rel-17	36.300	17.3.0	1378	-	F	LTE_NBIOT_eMTC_NTN

- Agreed

R2-2301963	IoT-NTN Stage-2 correction	OPPO	CR	Rel-17	36.300	17.3.0	1377	-	F	LTE_NBIOT_eMTC_NTN

- Agreed

R2-2301976	Correction related to AS deactivation due to discontinuous coverage 	Deutsche Telekom, Nokia, Nokia Shanghai Bell, Ericsson, Huawei, HiSilicon, Thales, Vodafone, Telit, BT, Telstra, Telecom Italia, Turkcell	CR	Rel-17	36.300	17.3.0	1381	2	F	LTE_NBIOT_eMTC_NTN

- Agreed

R2-2300258	Misc corrections on MAC for IoT NTN	MediaTek Inc.	CR	Rel-17	36.321	17.3.0	1559	-	A	LTE_NBIOT_eMTC_NTN-Core

- Agreed

R2-2301878	Correction for IoT NTN	Ericsson	CR	Rel-17	36.321	17.3.0	1563	-	F	LTE_NBIOT_eMTC_NTN

- Agreed

R2-2301986	Correction on figure clarifying HARQ RTT timer	Qualcomm Incorporated	CR	Rel-17	36.321	17.3.0	1561	1	F	LTE_NBIOT_eMTC_NTN

- Agreed unseen

R2-2301987	Clarification on the generation of TA reporting for IoT NTN	ZTE Corporation, Sanechips	CR	Rel-17	36.321	17.3.0	1562	1	F	LTE_NBIOT_eMTC_NTN-Core

- Agreed

R2-2301977	Correction related to AS deactivation due to discontinuous coverage	Deutsche Telekom, Nokia, Nokia Shanghai Bell, Ericsson, Huawei, HiSilicon, Thales, Vodafone, Telit, BT, Telstra, Telecom Italia, Turkcell	CR	Rel-17	36.304	17.3.0	0862	2	F	LTE_NBIOT_eMTC_NTN

- Agreed

R2-2301979	Correction for T317 in the Timers table	Huawei, HiSilicon	CR	Rel-17	36.331	17.3.0	4919	-	F	LTE_NBIOT_eMTC_NTN-Core

- Agreed

R2-2301050	Correction on handling of T317 timer during HO	ZTE Corporation, Sanechips	CR	Rel-17	36.331	17.3.0	4908	-	F	LTE_NBIOT_eMTC_NTN-Core

- Agreed

R2-2301978	Miscellaneous corrections to TS 36.331 for IoT NTN	MediaTek Inc.	CR	Rel-17	36.331	17.3.0	4900	1	A	LTE_NBIOT_eMTC_NTN-Core

- Agreed

R2-2301391	Reference Correction for IoT NTN in 36.306	Mediatek Inc.	CR	Rel-17	36.306	17.3.0	1867	-	A	LTE_NBIOT_eMTC_NTN-Core

- Agreed

RedCap

R2-2301955	Correction to control plane procedures for RedCap UEs	Qualcomm Incorporated	CR	Rel-17	38.331	17.3.0	3780	1	F	NR_redcap-Core

- Agreed

R2-2301957	Correction on RACH configure for RedCap	vivo, Qualcomm, ZTE Corporation, Guangdong Genius	CR	Rel-17	38.331	17.3.0	3800	1	F	NR_redcap-Core

- Agreed

R2-2301958	Correction on RRC configuration for RedCap	ZTE Corporation, Sanechips	CR	Rel-17	38.331	17.3.0	3880	1	F	NR_redcap-Core

- Agreed

R2-2302249	Clarification on BWP capabilities of RedCap UEs 	T-Mobile USA Inc.	CR	Rel-17	38.306	17.3.0	0868	2	F	NR_redcap-Core

- Agreed

R2-2301960	Correction on the filed descriptions of NeedForGaps in 38.331	CATT, ZTE, vivo	CR	Rel-17	38.331	17.3.0	3917	1	F	NR_redcap-Core

- Agreed

R2-2302280	Corrections for eDRX on IDLE eDRX cycle	Huawei, HiSilicon	CR	Rel-17	38.331	17.3.0	3869	1	F	NR_redcap-Core

- Agreed

R2-2301135	Correction for hyperSFN on SI update	Huawei, HiSilicon	CR	Rel-17	38.331	17.3.0	3870	-	F	NR_redcap-Core

- Agreed

Approved LSs out

R2-2301966	Response LS on Enhanced Cell Reselection Requirements for NTN	Nokia, Nokia Shanghai Bell	LS out	Rel-17	NR_NTN_solutions-Core	To:RAN4

Endorsed CRs

[Post121] Email discussions

Short

[POST121][101][NR NTN] Corrections on neighbour cell measurements (Ericsson)

Scope: Discuss the details of the 38.331 CR on Corrections on neighboring cell measurements reflecting meeting agreements

Intended outcome: Agreeable CR in R2-2301982

Deadline:  Short

[POST121][102][NR NTN] TN NTN mobility during RRC_INACTIVE (Qualcomm)

Scope: Further check R2-2301995

Intended outcome: Agreeable CR

Deadline:  Short

[POST121][103][RedCap] SDT operation for RedCap without CD-SSB (ZTE)

Scope: Check the technical details of R2-2301954 and R2-2301956.

Intended outcome: Agreeable CRs

Deadline:  Short

[POST121][104][RedCap] Correction on eDRX (Nokia)

Scope: Revise CR on eDRX correction based on R2-2301330

Intended outcome: Agreeable CR

Deadline:  Short

Long

[POST121][105][IoT NTN Enh] Neighbour cell assistance information (Qualcomm)

Scope: Discuss the possible content of neighbour cell assistance information and whether this should be included in an existing or new SIB

Intended outcome: report of the email discussion

Deadline:  Long

[POST121][106][NR NTN Enh] NTN-NTN cell reselection (ZTE)

Scope: Continue the discussion on NTN-NTN cell reselection aspects (triggers for measurements, derivation of trajectory of serving cell reference location, cell reselection criteria enhancements, etc.)

Intended outcome: report of the email discussion

Deadline:  Long