3GPP TSG-RAN WG2 Meeting #121-bis-e		R2-230XXXX

- Online, 17 – 26 April, 2023

Agenda item:	6.5.3

Source:	Samsung

Title:	Summary of agenda item 6.5.3 (Samsung)

Document for:	Discussion & Decision

# Introduction

This is to summarize the company contributions [1-3] submitted to agenda item 6.5.3 User plane corrections.

# Discussion

There are 1 PDCP CR, 1 RLC CR, and 1 SRAP CR submitted under the AI 6.5.3.

PDCP CR

| R2-2303490 | Clarification on the services expected from SRAP layer | Huawei, HiSilicon | - In clause 2, - Add reference for SRAP layer - In clause 4.3.2, - Add a bullet for SRAP layer, to clarify that the PDCP entity expects the data transfer service from SRAP entity if the UE is a L2 U2N Remote UE. |
| --- | --- | --- | --- |

The 1st change is necessary. For the 2nd change, Rapporteur thinks that it is reasonable to clarify the expected services from SRAP entity since this change is aligned with the agreement in R2-2302139 (for the case that the transmitting PDCP entity is associated with an SRAP entity) at RAN2 #121 meeting. So Rapporteur suggests to agree the changes in R2-2303490.

Proposal 1. The 38.323 CR in R2-2303490 is agreeable.

RLC CR

| R2-2303491 | Clarification on the maximum Data field size for L2 U2N relay | Huawei, HiSilicon | - In clause 6.2.3.2, - calrify that in case the upper layer is SRAP, the maximum Data field size can be larger than the maximum size of a PDCP PDU. |
| --- | --- | --- | --- |

Rapporteur thinks that the proposed change is reasonable to support the case that SRAP is located over RLC sublayer which is similar to the case of BAP over RLC sublayer in IAB. So Rapporteur suggests to agree the change in R2-2303491.

Proposal 2. The 38.322 CR in R2-2303491 is agreeable.

SRAP CR

| R2-2304036 | Corrections on SRAP for SL relay | NEC | - In clause 5.3.1.2, - clarify how to determine the egress PC5 relay RLC channel for sending SRB1 via default SL-RLC1 - else if the SRAP Data PDU is for SRB1 and if there is not an entry in sl-SRAP-ConfigRemote, whose sl-RemoteUE-RB-Identity matches the SRB identity of the SRAP Data PDU, or if there is an entry in sl-SRAP-ConfigRemote without the corresponding sl-EgressRLC-ChannelPC5: - Determine the egress PC5 Relay RLC channel in the link with U2N Relay UE corresponding to logicalChannelIdentity for SL-RLC1 as specified in TS 38.331 [3]; |
| --- | --- | --- | --- |

Rapporteur thinks that the proposal is reasonable, for example to align with the associated procedure texts in clause 5.3.5.16 in TS 38.331. In the yellow highlighted texts below, there are UE procedures to apply the specified SL-RLC1 in a case that SRB1 is not included in sl-SRAP-ConfigRemote or in a case that SRB1 is included but sl-EgressRLC-ChannelPC5 is not configured. But the corresponding SRAP procedure text is missing as the proponent points out.

#### 5.3.5.16 L2 U2N Remote UE configuration The network configures the L2 U2N Remote UE with relay operation related configurations, e.g. SRAP configuration. The L2 U2N Remote UE shall: 1> if sl-SRAP-ConfigRemote is set to setup: 2> if the sl-L2RemoteUE-Config contains the sl-SRAP-ConfigRemote: 3> if no SRAP entity has been established: 4> establish a SRAP entity as specified in TS 38.351 [66]; 3> configure the parameters to SRAP entity in accordance with the sl-SRAP-ConfigRemote; 3> if SRB1 is included in sl-MappingToAddModList, and sl-EgressRLC-ChannelPC5 is configured: 4> release SL-RLC1, if established; 4> associate the PC5 Relay RLC channel as indicated by sl-EgressRLC-ChannelPC5 with SRB1; 3> else: (i.e. SRB1 is not included in sl-MappingToAddModList, or SRB1 is included in sl-MappingToAddModList, but sl-EgressRLC-ChannelPC5 is not configured) 4> if SL-RLC1 is not established: 5> apply the default configuration of SL-RLC1 as specified in clause 9.2.4 and associate it with the SRB1; 2> if the sl-L2RemoteUE-Config contains the sl-UEIdentityRemote: 3> use the value of the sl-UEIdentityRemote as the C-RNTI in the PCell. 1> else if sl-SRAP-ConfigRemote is set to release: 2> release the relay operation related configurations.

So Rapporteur suggests to agree the change in R2-2304036.

Proposal 3. The 38.351 CR in R2-2304036 is agreeable.

# Conclusion

We have the following proposals:

# References

[1] R2-2303490, Clarification on the services expected from SRAP layer, Huawei, HiSilicon

[2] R2-2303491, Clarification on the maximum Data field size for L2 U2N relay, Huawei, HiSilicon

[3] R2-2304036, Corrections on SRAP for SL relay, NEC