3GPP TSG-RAN WG2 Meeting #122	R2-2305592

Incheon, South Korea, 22– 26 May 2023

Agenda item:	7.9.5

Source:	Nokia, Nokia Shanghai Bell

Title:	Considerations on paging for sidelink relay

WID/SID:	NR_SL_relay_enh-Core - Release 18

Document for:	Discussion and Decision

# 1	Introduction

In RAN2-121bis-e meeting, the following agreements were made for the paging monitoring by the L2 relay UE for the remote UE using is as the RRC_IDLE state or using the eDRX paging cycle in Rel-17:

Agreements:

RAN2 not pursue further work to enable L2 U2N SL Relay UE paging monitoring for L2 Remote UE using i_s as for RRC_IDLE state in Rel-17.

RAN2 not pursue further work to enable L2 U2N SL Relay UE paging monitoring for L2 Remote UE using the eDRX paging cycle in Rel-17.

This paper discussed the potential solution in Rel-18 for the same issue.

# 2	Discussion

## 2.1	Discussion of the solution on the issue of paging monitoring by the L2 relay UE for the remote UE using eDRX paging cycle

It has already been identified in the email discussion of paging monitoring issue in Rel-17 [R2-2304291] that L2 relay UE is not able to monitor the paging message of the remote UE if the remote UE’ paging message is sending by the NW using eDRX paging cycle while the L2 relay UE doesn’t support the eDRX feature and cannot monitor the remote UE’s paging message using eDRX paging cycle. The agreement for Rel-17 is not to pursue further work to enable L2 U2N SL Relay UE paging monitoring for L2 remote UE using the eDRX paging cycle. This agreement basically means the NW can disable the eDRX feature for the UE in the cell using the setting of eDRX-AllowedIdle IE and eDRX-AllowedInacitve IE in SIB1 if the cell supports L2 U2N relay functionality. In this way, all the UEs in the cell including the remote UE will disable the eDRX feature. However, the eDRX feature was introduced to enhance the power saving for IoT devices. Disable the feature using SIB1 means all the UEs in the cell cannot use eDRX feature for power saving.

Observation 1: All the UEs in the cell cannot use eDRX feature to enhance the power saving if NW disables the eDRX feature in the cell using SIB1.

The agreement can be achieved in another way by specifying in the specification to disable eDRX feature for all the remote UEs even if the NW enables the eDRX feature for the cell using SIB1, but regardless whether the relay UE supports the eDRX feature or not. As the remote UE in U2N relay use cases may be IoT or sensor device, of which power saving is one of the essential requirements to extend the battery life. If the remote UE’s eDRX feature needs to be always disabled when it is connected to the relay UE as agreed in Rel-17, the power saving gain achieved by eDRX cannot be utilized for the remote UE.

Observation 2: Remote UE’s power consumption will be higher if eDRX feature will always be disabled when it is connected to the relay UE.

To provide better power saving performance for the remote UE or all the IoT UEs in the cell that L2 U2N relay is supported, the enhancement should be introduced in Rel-18 for handling the issue of paging monitoring due to the mismatch of the relay UE’s and remote UE’s capability on support of eDRX feature. The potential solutions include:

- Solution#1: introduce relay UE’s capability on support of eDRX in relay discovery message so that the remote UE selects only those relay UEs that support the eDRX feature. This solution requires the change of relay discovery message in Rel-18. It may also prevent the remote UE to select the relay UE for U2N relay if all the discovered relay UEs doesn’t support eDRX feature so that the remote UE is not able to connect to the network via U2N relay .

- Solution#2: the relay UE and the remote UE can exchange their eDRX capability using SL RRC signaling after SL unicast connection for U2N relay is established between the remote UE and the selected relay UE. If the mismatch of the eDRX capability between the relay UE and the remote UE is identified, the relay UE or the remote UE may be triggered to indicate the mismatch to the NW so that the NW can update the paging occasion calculation of the remote UE’s paging message according to the relay UE’s capability. In this way the relay UE can monitor the remote UE’s paging message without any problem even the relay UE doesn’t support eDRX feature. This solution doesn’t need to introduce the change on relay discovery message and the new relay selection criteria for the remote UE. It only needs to introduce the new indication from the relay UE or the remote UE to update the paging occasion calculation in the NW side, which seems has less specification and UE implementation impact.

RAN2 should discuss the potential solutions to handle the issue of paging monitoring by the relay UE due to the mismatch of eDRX feature capability between the relay UE and the remote UE in Rel-18 in order to provide better power saving performance for the remote UE.

Proposal 1: RAN2 to discuss the potential solutions to handle the issue of paging monitoring by the relay UE due to the mismatch of eDRX feature capability between the relay UE and the remote UE in Rel-18.

# 3	Conclusion

This document has made the following observations and proposals:

Observation 1: All the UEs in the cell cannot use eDRX feature to enhance the power saving if NW disables the eDRX feature in the cell using SIB1.

Observation 2: Remote UE’s power consumption will be higher if eDRX feature will always be disabled when it is connected to the relay UE.

Proposal 1: RAN2 to discuss the potential solutions to handle the issue of paging monitoring by the relay UE due to the mismatch of eDRX feature capability between the relay UE and the remote UE in Rel-18.