3GPP TSG RAN WG2#122	R2-2304625

Incheon, Korea, 22nd - 26th May, 2023

3GPP TSG-RAN WG3 Meeting #119-bis-e	R3-232047

Online, 17 – 26 Apr 2023

Title:	LS on the feasibility of introducing assistance information for handling of QoE reporting during RAN overload

Response to:	-

Release:	Release 18

Work Item:	NR_QoE_enh-Core

Source:	RAN3

To:	SA5

Cc:	RAN2

Contact Person:

#### Name:	Man Zhang

E-mail Address:	zhang.man4@zte.com.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

During the Rel-18 NR QoE enhancement WI phase, RAN3 is discussing enhancements for handling of QoE reports during RAN overload.

In Rel-17, the following feature was introduced for handling the QoE reporting to an NG-RAN node that is in overload:

- The NG-RAN node can send an RRC message to the UE with a pause indication, to temporarily stop UE from sending QoE reports associated with one or multiple QMC configurations.

- After the overload has been resolved, the NG-RAN node could also send an RRC message with a resume indication, to notify the UE to resume the QoE reporting to the NG-RAN node.

In Rel-18, RAN3 has discussed for several meetings whether the OAM can provide some assistance information (per QoE measurement configuration) to the NG-RAN node for handling QoE reporting in a RAN overload scenario more efficiently. The motivation behind introducing such assistance information is to assist the NG-RAN node in choosing those QoE measurement configurations for which QoE reporting can be paused and those whose reporting can be exempted from pausing during RAN overload.

However, there is still no consensus in RAN3 on the assistance information that can be provided by the OAM to assist with RAN overload handling.

During the discussion, most companies preferred to introduce a priority per QoE configuration that can be used as assistance information. One concern raised with introducing this priority was that there could be different consumers for receiving the QoE report(s) (pursuing the role of the MCE) where each consumer might have its own priority for receiving these QoE reports. Consequently, it is claimed that the OAM may not be able to set these priorities in an accurate way.

Another type of assistance information which has been proposed in RAN3 is whether the OAM can indicate the “type of consumer” that will receive the QoE reports, or some “characteristics of reporting” as assistance information in the QoE configuration.

RAN3 therefore have the following questions:

Q1: Can there be multiple types of consumers for receiving the QoE reports (pursuing the role of the MCE)? If yes, what are those potential consumers as supported by the current specifications?

Q2: If the answer to the first part of  Q1 is “yes”, and if different consumers can have different priorities in receiving the QoE reports, is the OAM able to compare and rank by priority the preferences of different consumers or rank the consumers themselves? Can the OAM make the final decision regarding setting the reporting priorities? Can the OAM coordinate with the consumers and inform the consumers if the intended priority is not met?

Q3: Is there any other issue(s) related to sending such an explicit priority per QoE configuration as assistance information to the NG-RAN node?

Q4: Can the OAM indicate the “type of consumer” (as in Q1) or “characteristics of reporting” (e.g., the loop cycle, reporting periodicity, expected number of reports or amount of data to be reported etc.) as assistance to the NG-RAN node in the QoE configuration?

2. Actions:

To SA5.

ACTION: 	RAN3 respectfully asks SA5 to provide feedback on the above questions.

3. Date of Next TSG-RAN WG3 Meetings:

RAN3#120	from 2023-05-22	to 2023-05-26		Incheon, Republic of Korea

RAN3#121	from 2023-08-21	to 2023-08-25		Toulouse, France