3GPP TSG-RAN WG2 Meeting #123	R2-2307140

Toulouse, France, August 21-25, 2023 	(Revision of R2-2304676)

Agenda Item:	7.16.2.1

Source:	NEC

Title:	General aspects of AI/ML framework

Document for:	Discussion, Decision

# Introduction

In RAN2#122 meeting, there is no discussion on the general aspects for AIML, but in RAN2#121bis-e meeting, the following agreements and working assumptions are achieved for the general aspects of AIML:

Agreement at RAN2#121bis-e

- Model ID can be used to identify model or models for the following LCM purposes:

- model selection/activation/deactivation/switching (or identification, if that will be supported as a separate step). (e.g. for so called “model ID based LCM”)

- If model transfer/delivery is supported, model ID can be used for model transfer/delivery LCM purpose.

- How to achieve globality of the Model ID is FFS.

  - Initial discussion in RAN2: the following global unique model ID definition directions can be considered as a starting point:

  - Direction1: Pre-defined/hard-coded global unique model ID

  - Direction3: Assigned global unique model ID via specific ID management node.

  - Note: Other global unique model ID definition is not precluded.

  - Model ID structure, if any, is FFS

In RAN1#113 meeting, the following agreements and working assumptions are achieved for the general aspects of AIML.

Agreement at RAN1#113:

Agreement Consider at least the following aspects and if applicable, the corresponding potential specification impact related to data collection: - Measurement configuration and reporting - Contents, type and format of data including: - Data related to model input - Data related to ground truth - Quality of the data - Other information - Signaling of assistance information for categorizing the data - Note: The study should consider the feasibility of disclosure of proprietary information - Signaling for data collection procedure - Note 1: Use-case specific details can be studied in respective agenda items - Note 2: Signaling mechanism details can be studied by appropriate working groups. Agreement For model identification of UE-side or UE-part of two-sided models, categorize model identification types as follows, and further study relevant aspects, necessity, and specification impact (if any). - Type A: Model is identified to NW (if applicable) and UE (if applicable) without over-the-air signaling - The model may be assigned with a model ID during the model identification, which may be referred/used in over-the-air signaling after model identification. - FFS: Spec impact to other WGs - Type B: Model is identified via over-the-air signaling, - Type B1: - Model identification initiated by the UE, and NW assists the remaining steps (if any) of the model identification - the model may be assigned with a model ID during the model identification - FFS: details of steps - Type B2: - Model identification initiated by the NW, and UE responds (if applicable) for the remaining steps (if any) of the model identification - the model may be assigned with a model ID during the model identification - FFS: details of steps - Note: The support and applicability of each model identification Type is a separate discussion. This study does not imply that model identification is necessary. Agreement For functionality/model-ID based LCM, - Once functionalities/models are identified, the same or similar procedures may be used for their activation, deactivation, switching, fallback, and monitoring. Agreement Once models are identified, UE can indicate supported AI/ML model IDs for a given AI/ML-enabled Feature/FG in a UE capability report as starting point. - FFS: applicability to model identification, Type A, type B1 and type B2 - FFS: Using a procedure other than UE capability report - Note: model identification using capability report is not precluded for type B1 and type B2 Agreement Study how to handle the impact of UE’s internal conditions such as memory, battery, and other hardware limitations on functionality/model operations and AI/ML-enabled Feature. Note: it does not preclude any existing solutions Agreement Revise the following terminologies for model activation, model deactivation, and model switching as follows Model activation; Enable an AI/ML model for a specific function AI/ML-enabled feature. Model deactivation; Disable an AI/ML model for a specific function AI/ML-enabled feature. Model switching; Deactivating a currently active AI/ML model and activating a different AI/ML model for a specific function AI/ML-enabled feature. Agreement In model delivery/transfer Case z4, the “known model structure” means an exact model structure as has been previously identified between NW and UE and for which the UE has explicitly indicated its support. In model delivery/transfer Case z5, the “unknown model structure” means any other model structure not covered in z4, including any model structure that is only partially known. Agreement For the purpose of activation/selection/switching of UE-side models/UE-part of two-sided models /functionalities (if applicable), study necessity, feasibility and potential specification impact for methods to assess/monitor the applicability and expected performance of an inactive model/functionality, including the following examples: - Assessment/Monitoring based on the additional conditions associated with the model/functionality - Assessment/Monitoring based on input/output data distribution - Assessment/Monitoring using the inactive model/functionality for monitoring purpose and measuring the inference accuracy - Assessment/Monitoring based on past knowledge of the performance of the same model/functionality (e.g., based on other UEs) FFS: Requirements for the assessment/monitoring to be reliable (e.g., sufficient data coverage during evaluation) FFS: Additional aspects specific to the case where the inactive model has never been activated before, if any.

In this contribution, we provide our views on general aspects of AI/ML framework, in particular on lifecycle management (LCM).

# Discussion

## Model ID based LCM

If more than one AI/ML models are available for the same functionality, for example, different AI/ML models have been trained for different configurations or scenarios, LCM would be extended to multi-model management, including model activation, deactivation, selection, switching, and so on.

Two different LCM frameworks, model-ID-based LCM and functionality-based LCM are under active discussions in several meetings at RAN1, but the relationship between them are still not clear. To our understanding, if both LCM frameworks will be supported eventually in Rel-19, we don’t need to discuss the relationship since they may work independently. If the two LCM frameworks would work jointly, the granularities of different terminologies may be ordered as the following: sub-use-case >= functionality >= AI/ML model.

On the other hand, if only one LCM framework will be supported, we prefer to support model-ID-based LCM since functionality-based LCM may not be able to support all collaboration levels between NW and UE, especially for level Z with model transfer. It could be difficult to deliver or to transfer multiple models for the same functionality without involving model ID. To have one and only complete solution, model-ID-based LCM should be given a higher priority.

Proposal 1: Study model-ID based lifecycle management with a higher priority (within model-ID-based LCM and functionality-based LCM) .

Model-ID can be quite useful to control the signalling overhead. Even the AI/ML models are NW-sided models, model-ID is also needed to let UE know that the applied AI/ML model has been changed/updated and corresponding measurement and report for the UE to provide for the input of model inference may also be updated.

Proposal-2: Support model-ID based lifecycle management also for NW-sided model, and details of usage of model-ID is FFS.

Furthermore, the globality of the Model-ID is being under discussion and it is useful for e.g., model deployment or registration, but global model ID may be too long to be used suitably for some specific LCM procedure, e.g., model switching. Model index which is associated to each global model ID and unique within one UE can be further considered. For instance, if there are multiple models for the same functionality configured to one UE and need to be switched, only a model index (which is low overhead) may be enough to indicate then.

Model ID is also a RAN2 discussion point and the previous agreements suggested that Model ID can be “global”, which is particularly useful considering model transfer and test certification. We think that the Model identification procedure is used to translate the global model ID to the local model index of an AI/ML model.

For a two-sided AI/ML model (e.g., for the use case of CSI compression), since NW part (e.g., CSI reconstruction part) and UE part (e.g., CSI generation part) may not be trained jointly, for example, via model training collaboration type 2 or 3, and multiple NW parts and UE parts may be trained even for one AI/ML model. In this case, as a part of the model ID, identification of NW parts and UE parts may also be useful. Meanwhile, additional procedures may apply to select one of NW parts to be best matched with one of the UE parts before effective Model Inference.

Proposal-3: For a two-sided model, discuss methods to align NW part and UE part of one AI/ML model, e.g., assigning NW part ID and UE part ID in addition to model-ID.

## Data collection

Data collection is needed for different LCM stages, for example, model training, model inference and model monitoring. For different representative sub use cases, studies are needed to identify whether the legacy CSI/BM/positioning framework can be sufficient for each sub use case respectively.

A hierarchical structure could be useful to link the AI/ML framework and the legacy CSI/BM/positioning framework. Multiple alternatives can be considered and we need to study their pros and cons in terms of complexity, flexibility, requirement on UE capability, latency, as well as the spec impact. Taking CSI report as an example, one method is to configure one legacy CSI ReportConfig for each stage of LCM of an AI/ML model, as shown in the figure (a) below. The other way is to use RRC to reconfigure the target AI/ML model and LCM stage for a CSI ReportConfig, as shown in the figure (b) below.

Figure 1: Using legacy CSI framework to collected data needed for an AI/ML model

(a) dedicated ReportConfig for LCM; (b) defining the target LCM for one ReportConfig

Proposal 4: Study a hierarchical structure to configure the linkage between the AI/ML framework and the legacy CSI/BM/positioning framework.

## Model selection, activation, deactivation, switching, and fallback

As discussed, if more than one AI/ML models are available for the same functionality, they may be trained specifically for different configurations or different scenarios. Adaptive selection of applied AI/ML model can greatly reduce the required LCM signalling on explicit model selection, activation, deactivation, switching, and fallback. For example, if different AI/ML models are available for LOS/NLOS, high/low SINR, high/low velocity, more/less antenna ports/beams respectively, model switching can be adaptive to the change of configurations or the detection of change of the scenarios.

Proposal-5: Discuss adaptive model selection, activation, deactivation, switching, and fallback without explicit LCM signaling.

As most mechanisms currently assumed for model selection/activation/deactivation is by using UE dedicated signalling which can result in significantly high overhead and increased burden on network management, we might also consider the mechanisms by which some of AI/ML models can be activated autonomously e.g. based on broadcast signalling. For example, UE can get information about the AI/ML model to be activated through broadcast signalling (e.g., SIB) and can activate the model on its own if no additional information is required from the network for model selection. This would be beneficial for AI/ML models whose parameters are mainly derived based on network characteristics for e.g. beam prediction.

Proposal-6: Support autonomous model activation procedure for AI/ML models with assistance of network broadcast signaling.

## Model monitoring

As discussed, if more than one AI/ML models are available for the same functionality, LCM would be extended to multi-model management. On the other hand, usually one of the AI/ML models is applied for model inference. When the applied AI/ML model cannot meet the performance requirement, for example, model failure is declared, model switching might be needed. To determine which model would be the better one to use, a test procedure can be done by comparing the model inference performance of multiple candidate models. To accelerate the procedure, monitoring of multiple AI/ML models for the same functionality can be considered. In this case, model switching can be done right after or even before the real model failure occurs, by comparing the monitoring results of multiple candidate models.

Proposal-7: Consider to support model monitoring of multiple AI/ML models for the same functionality.

For AI/ML model performance feedback, methods should be identified to support the monitoring of AI/ML model performance and the required feedback signalling. In previous RAN1 discussion, a number of methods were identified by RAN1, each method has its own application scenario, since the cause of model failure may be different. Supporting more than one monitoring methods as discussed by RAN1 seems inevitable. If the monitoring method is determined at network side, the configuration information of model monitoring method should be provided to UE. If the monitoring method is determined at UE solely, together with the model monitoring results, it may report to the network, the applied monitoring method, or it may report the cause of model failure if the model monitoring results implying the model has been failed.

Proposal-8: Information of model monitoring methods can be provided to NW or UE. If model failure occurs, the cause of model failure may also be reported.

Moreover, it is expected that for UE sided monitoring, if the model fails or if model is not performing well then, the report may need to be provided to the network with minimal latency, especially for the AI/ML models which are delay sensitive (e.g., channel reporting). Hence, L1/L2 based mechanisms framework should be defined to allow reporting from the UE to the network about model failure or deteriorating model performance to reduce the delay as much as possible. If L1/L2 based mechanism is defined then signalling overhead is a serious concern and hence further discussion is required on the information content which can be included within L1/L2 signalling.

Proposal-9: Define L1/L2 based mechanism for UE reporting of model failure for UE sided model monitoring.

## Model update

In the process of model monitoring, when the model performance (e.g., accuracy of model inference) is detected to deteriorate, it may be essential to perform model updating. For example, UE or gNB can optimize the existing model in combination with the latest local or field data (e.g., fine-tuning). Optionally, UE or gNB can switch to another model (e.g., for better generalization performance). Specifically, the first method requires longer processing delay, but the performance of the updated model may be better. For the second method, although the performance of the new model switched may not be as good as that of the optimized model, it can save the delay of model updating (e.g., fine-tuning). But multiple models may need to be allocated to the target case in advance for the second method. In addition, there may be other potential methods to achieve model updating. Therefore, the reasonable methods on model updating should be discussed to ensure normal model inference and the updating of AI/ML model should cause as less interruption of AI/ML model inference as possible.

Proposal-10: Discuss the methods to update AI/ML model with minimum interruptions of AI/ML model inference.

# Conclusion and Proposal

We have the following proposals:

Proposal 1: Study model-ID based lifecycle management with a higher priority (within model-ID-based LCM and functionality-based LCM) .

Proposal-2: Support model-ID based lifecycle management also for NW-sided model, and details of usage of model-ID is FFS.

Proposal-3: For a two-sided model, discuss methods to align NW part and UE part of one AI/ML model, e.g., assigning NW part ID and UE part ID in addition to model-ID.

Proposal 4: Study a hierarchical structure to configure the linkage between the AI/ML framework and the legacy CSI/BM/positioning framework.

Proposal-5: Discuss adaptive model selection, activation, deactivation, switching, and fallback without explicit LCM signaling.

Proposal-6: Support autonomous model activation procedure for AI/ML models with assistance of network broadcast signaling.

Proposal-7: Consider to support model monitoring of multiple AI/ML models for the same functionality.

Proposal-8: Information of model monitoring methods can be provided to NW or UE. If model failure occurs, the cause of model failure may also be reported.

Proposal-9: Define L1/L2 based mechanism for UE reporting of model failure for UE sided model monitoring.

Proposal-10: Discuss the methods to update AI/ML model with minimum interruptions of AI/ML model inference.

# Reference

- R2-2301440  Summary of RAN2 post-meeting discussion on AIML on model transfer/delivery

- R2-2301576  Summary of RAN2 post-meeting discussion on data collection

- TR37.817