3GPP TSG-RAN WG2 Meeting #123		   R2-2307292

Toulouse, France, 21st – 25th Aug. 2023

Source:	vivo

Title:	Discussion on security aspects for selective SCG based on SA3 reply LS

Agenda item:	7.4.3

Document for:		Discussion & Decision

# 1. Introduction

In RAN2 #120 meeting, the security issues were discussed in RAN2 and an LS to SA3 was agreed to be sent[1]. SA3 discussed about the security for selective SCG activation on their #110 and #111 meeting, and two reply LSs were sent back to RAN2[2][3], where their main point is that the same S-KgNB shall not be used by UE when the UE is going back to previous SN, and they concluded to have a ‘SN Counter based solution for fresh key derivation’ for selective SCG activation.

This contribution will discuss the possible RAN2 impact based on SA3 LS including:

- How should MN configure multiple sk-counters to be stored by UE along with its subsequent CPAC configuration;

- Whether it is possible that the UE uses these sk-counters in an order which is not monotonically incremented and whether there are security concerns;

- What should be the baseline security procedure in subsequent CPAC.

# 2. Discussion

The detailed solution provided by SA3 is as follows[3]:

SA3 has considered different alternative solutions to address the issue and concluded to have a SN Counter based solution for fresh key derivation for selective SCG activation. Following is the solution details from security perspective: 1. The MN provides to the UE the SN Counter values for each SN, and the UE stores these values along with the CPC. 2. The UE derives the KSN using the KgNB together with an unused SN Counter value pre-provisioned by the MN. The UE changes the SN Counter upon every SN change.

In legacy PSCell change, the network can decide whether to provide sk-counter to UE in RRC reconfiguration based on e.g. whether the PDCP termination point of the SN has changed compared to previous PSCell. However, in subsequent CPC procedure, the network has no idea how the subsequent CPC procedure would be performed especially about the order in which UE accesses to different PSCells. Therefore, it is not clear how the network can determine whether a sk-counter should be included in a certain conditional reconfiguration.

Observation 1: In subsequent CPAC procedure, the network does not know in advance when provisioning sk-counters whether the UE is going to perform intra-SN or inter-SN CPAC.

We understand from the network’s point of view, the only choice is to provide sk-counters in every subsequent CPAC configuration. In this way, the UE can have available sk-counter in case an inter-SN CPAC is performed in subsequent CPAC procedure. Meanwhile, as SA3 replied that the same SN counter/ S-KgNB cannot be reused while connected to SN #1 before and after being connected to SN #2, more than one sk-counters may be needed in the conditional configuration in case the UE goes back to a previous PSCell.

Proposal 1: One or more sk-counter are provided in the conditional configuration for a PSCell for subsequent CPAC, since the network cannot determine in advance whether a candidate cell will be taken by UE as the target cell for intra-SN CPC or inter-SN CPC.

The next question would be, when would the UE change the sk-counter to derive a new key? In SA3 reply LS, they mentioned ‘The UE changes the SN Counter upon every SN change’. However, our understanding is that there is no information to the UE whether one CPC procedure is intra-SN or inter-SN. So, there are only two possible ways forward on this question:

Option 1: The network informs the UE about the SN information of each PSCell.

With this information, The UE can know whether one CPC procedure is intra-SN or inter-SN. How the UE can know the SN to which a PSCell belongs, can be FFS, e.g. it can be some kind of SN identifier.

Option 2: The UE always changes the sk-counter to derive a new key upon every CPC in subsequent CPAC procedure

With this option, the UE uses a new sk-counter upon every CPC in subsequent CPAC procedures no matter it is indeed intra-SN or inter-SN CPC (as the UE anyway does not know that). The problem may be that the sk-counter can be consumed faster, but the security can be guaranteed on the other hand.

We think RAN2 can discuss the above two options to further progress the security aspects in subsequent CPAC. Considering option-1 can have more spec impact which is also not aligned with legacy design, we prefer to adopt option-2.

Observation 2: The UE also does not have any information about whether a CPC procedure is intra-SN or inter-SN.

Proposal 2: Due to not knowing a CPC procedure is intra-SN or inter-SN, a UE uses an unused sk-counter to derive the new key upon every CPC in subsequent CPC procedures.

In subsequent CPAC procedure, as we discussed in Observation 1, the UE may access to different PSCells in random order and the sk-counter used by UE may not be in an ascending order. However, in our understanding, the MN still monotonically increment SN Counter for each additional calculated KSN in conditional configuration preparation phase. We think on the UE side, as long as the sk-counter(s) for different PSCells are one-to-one mapping to the SN counter on MN side and are not repetitive, there should not be any security problem. In RAN2 #120 meeting, companies agreed to move the related issue from LS to SA3 in a last minute as follows. A number of companies think it is reasonable to ask the point but due to a lack of time, and the intention to simplify the LS, it is not included. However, we think this is important to finalize the subsequent CPAC design for security.

Remove: “the order in which the sk-counter values are used upon successive S-KgNB change depend on the order in which PSCells are selected by the UE (i.e. the sk-counter value used is not monotonically incremented as specified in TS 33.501).”

Based on SA3 reply LS[3], we understand it is already allowed that UE uses multiple sk-counters to derive/update secondary key in an order which is not monotonically incremented. We can confirm if this is the RAN2 common understanding.

Proposal 3: RAN2 to confirm whether it is the common understanding that a UE can use multiple sk-counters to derive/update secondary key in an order which is not monotonically incremented with no security concern based on SA3 reply LS.

At last, based on the discussions above, the whole procedure for key derivation/update in subsequent CPAC procedure can be as follows:

- Step 1: Network provide multiple CPAC configuration to UE, in which each PSCell is associated to one or more sk-counter. All the sk-counters are not repetitive.

- Step 2: During CPAC execution and UE accesses to a new PSCell or a previously accessed PSCell, UE uses an unused sk-counter to derive the secondary key. If no unused sk-counter is available, the access to the concerned PSCell is not allowed.

Proposal 4: RAN2 to consider the following baseline procedure for security aspect in subsequent CPAC:

- Step 1: Network provide multiple CPAC configuration to UE, in which each PSCell is associated to one or more sk-counter. All the sk-counters are not repetitive.

- Step 2: During CPAC execution and UE accesses to a new PSCell or a previously accessed PSCell, UE uses an unused sk-counter to derive the secondary key. If no unused sk-counter is available, the access to the concerned PSCell is not allowed.

# 3. Conclusion

In this contribution, we discussed about RAN2 impact based on SA3 LS. We think the sk-counter should always be provided in conditional reconfiguration and it is associated to one PSCell.

We have the following observations and proposals:

Observation 1: In subsequent CPAC procedure, the network does not know in advance when provisioning sk-counters whether the UE is going to perform intra-SN or inter-SN CPAC.

Observation 2: The UE also does not have any information about whether a CPC procedure is intra-SN or inter-SN.

Proposal 1: One or more sk-counter are provided in the conditional configuration for a PSCell for subsequent CPAC, since the network cannot determine in advance whether a candidate cell will be taken by UE as the target cell for intra-SN CPC or inter-SN CPC.

Proposal 2: Due to not knowing a CPC procedure is intra-SN or inter-SN, a UE uses an unused sk-counter to derive the new key upon every CPC in subsequent CPC procedures.

Proposal 3: RAN2 to confirm whether it is the common understanding that a UE can use multiple sk-counters to derive/update secondary key in an order which is not monotonically incremented with no security concern based on SA3 reply LS.

Proposal 4: RAN2 to consider the following baseline procedure for security aspect in subsequent CPAC:

- Step 1: Network provide multiple CPAC configuration to UE, in which each PSCell is associated to one or more sk-counter. All the sk-counters are not repetitive.

- Step 2: During CPAC execution and UE accesses to a new PSCell or a previously accessed PSCell, UE uses an unused sk-counter to derive the secondary key. If no unused sk-counter is available, the access to the concerned PSCell is not allowed.

# 4. References

R2-2213337, LS on security for selective SCG activation

R2-2301943, Reply LS R2-2213337 LS on security for selective SCG activation

R2-2307070, LS on Security Solution for Selective SCG