3GPP TSG-RAN WG2 Meeting #123	R2-2307454

Toulouse, France, 21st – 25th August, 2023

Agenda Item:	7.17.2

Source:	Huawei, HiSilicon

Title:	Discussion on proactive and reactive approaches

Document for: Discussion and Decision

# Introduction

UE capability restrictions were discussed in RAN2#121bis meeting [1] and made the following agreements with FFS:

Consider “proactive” approach (wherein the UE can request capability restrictions which can be independent of current RRC configuration if allowed by the NW) to MUSIM capability restrictions in addition to the reactive approach (which has been agreed previously). Such a mechanism shall still be under NW control, i.e. it is up to network whether to allow such signalling. FFS on the details – should aim for a common framework for the reactive and proactive approach. FFS on UE capabilities UE can explicitly request specific serving cells or serving cell group to be released for Rel-18 MUSIM purpose. FFS how/whether this works for the proactive case.

In this contribution, we discuss the open issues about proactive and reactive approaches based on the above agreements.

# Discussion

## Reactive approach

In previous meeting, it was agreed that impacted capabilities that should be indicated to the NW include supported band combinations for CA/DC, MIMO layers, and measurement gap. Firstly, we analyze the reactive approach for these capabilities.

1) CA/DC capability

For reactive approach on CA/DC capability, there are following agreements:

For dual-active MUSIM, UE signaling will support the request for release (and reversal) of SCells and SCG. The signaling details (e.g. implicit or explicit request of each SCell or SCG) is FFS. UE can explicitly request specific serving cells or serving cell group to be released for Rel-18 MUSIM purpose. FFS how/whether this works for the proactive case.

In RAN2#121bis meeting, it was agreed to support UE-requested release (and reversal) of specific SCells and SCG explicitly. One scenario is to start the RRC connection in NW-B, the resources on specific SCells/SCG are temporarily restricted in NW-A; when the RRC connection in NW-B is released, the restriction in NW-A is removed. As a baseline, if the temporary capability restriction is allowed by the NW, the UE can indicate the SCell index(s)/SCG to be released with UAI based on current RRC configuration when resource conflict happens between the two SIMs. This could be considered as reactive approach, which is the most straightforward to address the resource conflict. In this way, there is less workload for both the UE and the NW, and less signalling overhead for UAI reporting.

In the previous meeting, there was discussion on whether to introduce UE-requested SCell/SCG deactivation. Compared with SCell/SCG release, the benefit is the SCell/SCG could be deactivated/activated quickly when the conflict happens/stops. In some cases, the NW may consider the deactivation/activation as more efficient, for example, the NW would like to activate the SCell/SCG quickly if possible after the restriction is removed. In our view, it should be up to NW whether to deactivate or release the SCell/SCG according to the UAI. Although there was no consensus to support UE-initiated SCell deactivation for MUSIM, in order to not restrict the NW’s choice to deactivate the SCell/SCG, we think as a compromise the UE can indicate the specific SCell index(s)/SCG “to be affected” instead of “to be released” because of resource conflict in MUSIM. Then it is left to NW how to handle the affected SCell(s)/SCG. The NW can either release the SCell(s)/SCG, or deactivate the SCell(s)/SCG with no RRM measurement (including no RLM/BFD for SCG), or configure gap-based RRM measurement for the deactivated SCell(s)/SCG.

Proposal 1: For reactive approach, the UE indicates the SCell index(s)/SCG “to be affected” instead of “to be released” for Rel-18 MUSIM purpose, and  is up to NW whether to deactivate or to release the SCell(s)/SCG.

2） MIMO Layer

In the last meeting, it was agreed the UE can indicate the change of the maximum MIMO layer. As we analyzed in [4], the MIMO layer is related to the available RF resource on certain band/CC. Thus, in reactive approach, a change of maximum DL/UL MIMO layer capability for a specific serving cell can be indicated in the UAI when the resource conflict happens/stops due to MUSIM dual active operation.

Proposal 2: The UE indicates the maximum supported DL/UL MIMO layer for a specific serving cell for R18 MUSIM purpose in the UAI.

3) Measurement gap

In the last meeting, it was agreed the UE can indicate the change of measurement gap requirement due to MUSIM dual active operation. Currently, the measurement gap requirement (i.e. IE needForGapsInfoNR) is reported in RRCReconfigurationComplete message and RRCResumeComplete message, where the UE can indicate whether measurement gap is required for each target band. In MUSIM scenario, we cannot always rely on RRCReconfigurationComplete message to indicate the current measurement gap requirement, since if the UE indicates MIMO/SCell/SCG change in UAI, it is possible that the NW does not send RRCReconfiguration message to the UE, e.g. the current configuration does not exceed the UE temporary capability or the NW decides to use MAC CE signaling instead of RRC configuration. To address the case that the UE’s measurement gap capability has been changed but the UE does not receive the RRCReconfiguration message from the NW, we prefer to add measurement gap capability in UAI message as other temporary capabilities and the existing needForGapInfoNR can be reused.

Besides, as the measurement gap is used for the current RRC configuration, we consider there is no need to proactively indicate the change of measurement gap before receiving a RRC configuration. In other words, the change of measurement gap capability is only needed to indicate in the reactive approach.

Proposal 3: The UE can indicate the temporary capability restriction of measurement gap for R18 MUSIM purpose in the UAI by using the existing needForGapInfoNR.

Proposal 4: The UE indicates the change of measurement gap capability based on the current RRC configuration. No proactive approach is needed for the change of measurement gap capability.

In general, after receiving the UAI for temporary capability restriction in reactive approach, it is expected that the NW responds as quickly as possible to provide a better user experience. For the NW/UE behavior after the UAI for capability restriction has been reported, there are the following options:

- The UE reduces its capability after receiving the RRC reconfiguration message from the NW. If no reconfiguration is received, the UE cannot reduce its capability autonomously.

- The UE reduces its capability after receiving the RRC reconfiguration message. If no reconfiguration is received, the UE reduces its capability autonomously after a timer expires. The value of the timer is configured by the NW.

- It is up to UE implementation when to reduce its capability after initiating the UAI for capability restriction.

In MUSIM scenario, only UE knows the QoS requirements of the services in each NW as there is no interaction between the NWs serving the MUSIM UE. Hence if the temporary capability reporting is allowed by NW, UE should be responsible for the capability split between the SIMs to achieve a better overall QoS. However, with option 1, the UE may be unable to setup the RRC connection in NW-B, or suffer from a bad user experience in NW-B during the procedures like RRC reconfiguration, handover, re-direction and so on. With option 2 or option 3, the UE can split the resources between the two SIMs, which is more efficient considering the overall performance. To make the usable capability aligned between the UE and the NW, we think option 2 is a more feasible way forward.

Proposal 5: The UE starts a timer after it sends the UAI to indicate the temporary capability restriction for R18 MUSIM purpose, and the UE reduces its capability after receiving the RRC reconfiguration message and stops the timer. If no reconfiguration is received until the timer expires, the UE reduces its capability autonomously. The value of the timer is configured by the NW.

## Proactive approach

In previous meeting, there is the following agreement for proactive approach:

Consider “proactive” approach (wherein the UE can request capability restrictions which can be independent of current RRC configuration if allowed by the NW) to MUSIM capability restrictions in addition to the reactive approach (which has been agreed previously). Such a mechanism shall still be under NW control, i.e. it is up to network whether to allow such signalling. FFS on the details – should aim for a common framework for the reactive and proactive approach. FFS on UE capabilities

Generally, we think the proactive approach is just an enhancement for the reactive approach, which is optional to be supported by the UE and the NW. Besides, the complexity and signalling overhead should be taken into account, with a limited spec impact. The benefit of the proactive approach is, the UE signalling could be initiated before the conflict happens between two SIMs. The proactive approach aims to avoid the possible NW configuration exceeding the temporary UE capability in advance. Compared with the reactive approach, the key difference is the proactive approach is for future configuration in NW-A, not for the current configuration.

In the following table, we listed four scenarios for temporary capability restriction.

| Scenario | UE state in NW-A | UE state in NW-B | UE behavior |
| --- | --- | --- | --- |
| #1 | RRC_CONNECTED | RRC_IDLE/RRC_INACTIVE Request to setup/resume RRC connection; | Initiate UAI signalling in NW-A (e.g. indicate the affected SCell/SCG) |
| #2 | RRC_CONNECTED | RRC_CONNECTED RRC reconfiguration (including handover, re-direction) results in resource conflict between SIMs | Initiate UAI signalling in NW-A (e.g. indicate the affected SCell/SCG) |
| #3 | RRC_IDLE/RRC_INACTIVE Request to setup/resume RRC connection; | RRC_CONNECTED | 1) Early indication in msg3/msg5; or 2) Initiate UAI signalling in NW-A (e.g. indicate the affected SCell/SCG) after acquiring RRC configuration |
| #4 | RRC_CONNECTED RRC reconfiguration (including handover, re-direction) results in resource conflict between SIMs | RRC_CONNECTED | Initiate UAI signalling in NW-A (e.g. indicate the affected SCell/SCG) |

The above scenarios can be categorized into two groups:

- Group 1: Scenarios that handle future behavior in NW B. Scenario #1 and Scenario #2 come under this category

- Group 2: Scenarios that handle future behavior in NW A. Scenario #3 and Scenario #4 comes under this category.

In this contribution, we focus on the cases for scenarios #1, #2, and #4. The analysis for scenario #3 is in [3].

- Group#1: For the future behavior in NW-B

In some conditions, the UE could be able to predict the possible future behavior in NW-B. For example, the camped cell in RRC idle/inactive state is possibly to be the PCell in RRC connected state, or the intra/inter frequencies with good measurement results are possibly to be configured as SCell/SCG. With such prediction, the UE may proactively provide capability restriction to the NW-A for possible future configuration in NW-B. If the UE can know what is about to be configured in NW-B, the UE can update its capability earlier in NW-A, thus, the delay to setup RRC connection in NW-B can be improved. However, this prediction is not accurate enough and may bring unnecessary capability restriction in NW-A.

In our view, the timing to initiate the UAI is left to UE implementation. The signalling solution for Group#1 is based on the current RRC configuration in NW-A, so from signaling perspective, it is the same as in the reactive approach. Moreover, there is no additional spec impact with reactive approach for Group#1.

Observation 1: The signalling solution for the reactive approach can be reused for the proactive approach in Group#1.

- Group#2: For the future behavior in NW-A

If the UE is able to indicate the possible restricted capabilities for future configuration in NW-A proactively, the risk of data loss/configuration delay in NW-A could be avoided.

For Group#2, there are two options on how the proactive approach works for the temporary capability reporting (e.g. CA/DC capability, maximum MIMO layers).

- Option 1: The UE indicates the affected  frequency bands and the corresponding  band combination (BC) based on the candidate frequency bands/band combinations indicated by the NW

In [2], it was proposed to indicate the affected frequencies possibly used by the second transceiver so that NW-A is prevented to use them. We understand that frequencies indicated to NW-A should not be the exact frequencies used by NW-B but the affected frequencies in NW-A sharing the same resources with frequencies in NW-B. In addition, the indicated frequencies should be the frequency bands that belong to at least one of the band combinations supported in the NW-A to avoid the ambiguity for the NW-A. The reason is that whether a frequency band is affected or not differs for different band combinations. For example, let’s assume that band-C can be configured in 4 band combinations:

- BC1: {band A + band B + band C}

- BC2: {band A + band C}

- BC3: {band C + band D} and

- BC4: intra-band {band C}

If band C is affected in BC1, but not in BC2, BC3 and BC4, and the UE indicates band C to NW-A, it’s not clear for the NW-A to determine which BC has issue. This may result in the over limitation of configurable BCs for the NW-A. To avoid this, UE should indicate both the affected frequency band(s) and the corresponding affected band combination. The BC could be referred by the index of the reported BC list in the UE capability information.

For the affected bands in a band combination, there are two cases:

Case 1: The band is not supported to be configured as serving cells;

Case 2: The band is still supported to be configured as serving cells, but the capability on the band is restricted (e.g. maximum MIMO layers)

For example, when band C is not supported due to resource conflict between SIMs in the band combination BC1, the UE can indicate the band C is affected for the band combination BC1.  In addition, when band A is supported with restricted MIMO layers, the UE can additionally indicate a change of maximum MIMO layer on the CCs of band A, and it is assumed other capabilities of band A are unchanged. For the band(s) that are not indicated to be affected in the BC, it is assumed the capability of these band(s) are unchanged.

Moreover, to decrease signalling overhead of the UAI and to decrease the UE complexity, it is preferred that the NW can provide the candidate serving frequency bands, or candidate band combinations to the UE. With such information from the NW, only the frequency bands and band combinations consisting of the requested candidate frequency bands/band combinations are considered when reporting the UAI.

- Option 2: The UE indicates that a frequency is not supported to be configured as SCell/SCG based on the purpose of corresponding measurement configuration from the NW.

In our view, the typical scenario for proactive approach is that the candidate SCell/SCG may not be supported by the UE due to dual active MUSIM operation. In the current spec, the SCell/SCG addition/modification/release is usually configured based on the measurement results reported by the UE. In order to avoid an affected frequency added as SCell/SCG, when providing the measurement configuration to the UE, the NW can indicate the purpose of the measurement (e.g. for SCell/SCG addition). With the purpose of the measurement indicated by the NW, the UE can decide whether the corresponding frequency is supported to be added as SCell/SCG in advance, and indicate to the NW if the frequency is not supported to be added as SCell/SCG. Then the NW will add an SCell/SCG by taking the indication from the UE into account to avoid possible resource conflict and is not expected to add a SCell/SCG blindly for the UE.

Proposal 6: Consider the following signaling solutions for proactive approach:

1) The UE indicates the affected  frequency band(s) and the corresponding  band combination (BC) based on the candidate frequency band/band combinations configured by the NW;

2) The UE indicates that a frequency is not supported to be configured as SCell/SCG based on the measurement purpose of that frequency configured by the NW.

## Common for reactive and proactive approach

    - Removal of Capability Restrictions

For both reactive and proactive approaches, the UE can indicate the removal of CA/DC capability or DL/UL MIMO layers restriction by omitting the affected fields in the UAI. For example, when the NW-B leaves RRC connected state, the UE can initiate the UAI reporting procedure to send the UAI message with affected SCell (s)/SCG information or DL/UL MIMO layers being empty. Then the NW-A can understand that the UE has reverted to the full CA/DC capability.

Proposal 7: The UE indicates removal of capability restriction (i.e. CA/DC capability, DL/UL MIMO layers) by not including the corresponding fields in the UAI for R18 MUSIM purpose.

    - Prohibit Timer

In the previous meeting, it was still FFS whether or not to define prohibit timer for the capability restriction UAI. Irrespective of proactive and reactive approaches, the UE cannot predict how long the current configuration is maintained in the NW-B. When the NW B changes its configuration, the available resource for NW-A may change, and another UAI is necessary to inform the changes to the NW-A. This is beneficial to achieve a better user experience for the MUSIM operation. Hence, there is no need for prohibit timer for Rel-18 MUSIM scenario.

Proposal 8: No prohibit timer is defined for the UAI for R18 MUSIM purpose.

# Conclusion

In this contribution, we further discussed MUSIM temporary capability restrictions and have the following proposals:

Reactive approach

Proposal 1: For reactive approach, the UE indicates the SCell index(s)/SCG “to be affected” instead of “to be released” for Rel-18 MUSIM purpose, and  is up to NW whether to deactivate or to release the SCell(s)/SCG.

Proposal 2: The UE indicates the maximum supported DL/UL MIMO layer for a specific serving cell for R18 MUSIM purpose in the UAI.

Proposal 3: The UE can indicate the temporary capability restriction of measurement gap for R18 MUSIM purpose in the UAI by using the existing needForGapInfoNR.

Proposal 4: The UE indicates the change of measurement gap capability based on the current RRC configuration. No proactive approach is needed for the change of measurement gap capability.

Proposal 5: The UE starts a timer after it sends the UAI to indicate the temporary capability restriction for R18 MUSIM purpose, and the UE reduces its capability after receiving the RRC reconfiguration message and stops the timer. If no reconfiguration is received until the timer expires, the UE reduces its capability autonomously. The value of the timer is configured by the NW.

Proactive approach

Proposal 6: Consider the following signaling solutions for proactive approach:

1) The UE indicates the affected  frequency band(s) and the corresponding  band combination(BC) based on the candidate frequency band/band combinations configured by the NW;

2) The UE indicates that a frequency is not supported to be configured as SCell/SCG based on the measurement purpose of that frequency configured by the NW.

Common to both approaches

Proposal 7: The UE indicates removal of capability restriction (i.e. CA/DC capability, DL/UL MIMO layers) by not including the corresponding fields in the UAI for R18 MUSIM purpose.

Proposal 8: No prohibit timer is defined for the UAI for R18 MUSIM purpose.

# References

- R2-2306553, Report of 3GPP TSG RAN WG2 meeting #121bis-e, Online.

- R2-2303639 Overall Dual-RX/TX MUSIM procedure, Ericsson.

- R2-2307450 Discussion on early MUSIM indication

- R2-2307451 Details of allowed MUSIM temporary capability restrictions