3GPP TSG-RAN WG2 #123 	R2-2307471

Toulouse, France 21 – 25 August 2023

Agenda Item:	7.19.3

Source:	NEC

Title:	Discussion on issue of decoding Msg4

Document for:	Discussion, Decision

Introduction

RAN2#122 meeting discussed the issue of decoding Msg4 raised by RAN1 LS [1], however, no consensus was achieved. This contribution further discusses this issue.

Discussion

Point here is how to define eRedCap UE behavior when eRedCap UE detects a DCI scheduling a Msg4 PDSCH transmission with a larger bandwidth than it can receive or process. Followings are some options discussed by RAN1:

Option 1: The UE considers the contention resolution as not successful.

Option 2: The UE discards the DCI and continues monitoring the DCI until ra-ContentionResolutionTimer expires.

Option 3: The UE behavior is up to the UE implementation.

Since TS38.213 Clause 10.1 specified that “If a UE detects a DCI format with inconsistent information, the UE discards all the information in the DCI format”, we firstly observe that Rel-18 eRedCap UE discards all the information in the DCI format and continues monitoring the DCI until ra-ContentionResolutionTimer expires when the UE detects a DCI scheduling a Msg4 PDSCH transmission with a larger bandwidth, i.e., Option 2 is the eRedCap UE behavior specified by current specification.

Option 2 (The UE discards the DCI and continues monitoring the DCI until ra-ContentionResolutionTimer expires.) is the UE behaviour specified by current specification.

- Then let us review the current specification on Msg4 PDSCH transmission/reception in TS38.321 Clause 5.1.5 Contention Resolution from RAN2 perspective. Procedures are summarized below:

Once Msg3 is transmitted the MAC entity shall:

- start or restart the ra-ContentionResolutionTimer (details are specified for the Msg3 transmission (i.e. initial transmission or HARQ retransmission) is scheduled with PUSCH repetition Type A, non-terrestrial network etc.)

- monitor the PDCCH while the ra-ContentionResolutionTimer is running regardless of the possible occurrence of a measurement gap;

- if notification of a reception of a PDCCH transmission of the SpCell is received from lower layers:

  - specify ra-ContentionResolutionTimer timer stop and TEMPORARY_C-RNTI discard bahaviour based on Msg3 content (either C-RNTI MAC UE was included or the CCCH SDU was included)

- specified behaviour upon ra-ContentionResolutionTimer expiring:

  - consider the Contention Resolution not successful

- specified behaviour when the Contention Resolution is considered not successful:

As highlighted in yellow above, currently, the condition for UE to consider the Contention Resolution not successful is only upon ra-ContentionResolutionTimer expiring. To support Option 1 (The UE considers the contention resolution as not successful), it seems necessary to enhance another condition such as “the eRedCap UE detects a DCI scheduling a Msg4 PDSCH transmission with a larger bandwidth than it can receive or process”. However, as observed previously, the UE physical layer discards the DCI. Additionally, RAN1 has agreed that “The UE is not required to process a Msg4 PDSCH with a larger number of PRBs than 25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS.”, therefore it is still unclear that how MAC layer knows the new condition is met or not.

Considering nothing is broken as analyzed in Observation 1, we think no additional UE behavior definition is needed in the RAN2 specifications.

For the case when the UE detects a DCI scheduling a Msg4 PDSCH transmission with a larger bandwidth than it can receive or process, no additional UE behavior definition is needed in the RAN2 specifications.

Summary

This contribution discussed the issue of decoding Msg4.

Observation 1 Option 2 (The UE discards the DCI and continues monitoring the DCI until ra-ContentionResolutionTimer expires.) is the UE behaviour specified by current specification.

Proposal For the case when the UE detects a DCI scheduling a Msg4 PDSCH transmission with a larger bandwidth than it can receive or process, no additional UE behavior definition is needed in the RAN2 specifications.

- References

[1] R1-2304262 LS on Msg4 PDSCH transmission to Rel-18 eRedCap UEs