3GPP TSG-RAN WG2 Meeting #123	R2-2307504

Toulouse, France, 21st – 25th August 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.355 | CR |  | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Missing error cause code for DL PRS Measurements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Fraunhofer IIS, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh-Core |  |  |  |  |  | Date: |  |  | 2023-08-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Release-17 provides possibility to perform PRS measurements without having the need of gap. To allow this to happen a PRS processing window (PPW) has been specified where UE should be able to process both PRS and data subject to the UE capability. In such PPW, NW provides the prioritization between PRS and different data types. It can happen that during the PPW, UE may have to skip the PRS because UE had to handle high priority data. The performed PRS measurements may not meet the requirements if data is continoulsy prioritized in such PPW. From NW observability perspective, it is necessary to understand if PRS is being skipped and UE is unable to perform measurement. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A cause code indicating that PRS measurement had to be skipped is added in the target device error cause when DL PRS related measurements are reported for DL-TDOA, DL-AoD, Multi-RTT Positioning methods. Impact Analysis Impacted 5G architecture options: NR SA, (NG)EN-DC, NE-DC,NR-DC Impacted functionality: PRS Measurement Observability NR DL-PRS Info Field Descriptions Inter-operability: - No Inter-operability issue seen |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - PRS measurement failure observability would be missing from operator NW when PPW feature is deployed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.10, 6.5.11, 6.5.12 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# *** First Change ***

### 6.5.10	NR DL-TDOA Positioning

This clause defines the information elements for NR downlink TDOA positioning (TS 38.305 [40]).

#### 6.5.10.1	NR DL-TDOA Assistance Data

<Skip Unmodified Changes>

#### –	NR-DL-TDOA-TargetDeviceErrorCauses

The IE NR-DL-TDOA-TargetDeviceErrorCauses is used by the target device to provide NR DL-TDOA error reasons to the location server.

-- ASN1START

NR-DL-TDOA-TargetDeviceErrorCauses-r16 ::= SEQUENCE {

cause-r16		ENUMERATED {	undefined,

assistance-data-missing,

unableToMeasureAnyTRP,

attemptedButUnableToMeasureSomeNeighbourTRPs,

thereWereNotEnoughSignalsReceivedForUeBasedDL-TDOA,

locationCalculationAssistanceDataMissing,

...,

prs-MeasurementSkipped-v17xy

},

...

}

-- ASN1STOP

*** Next Changes ***

### 6.5.11	NR DL-AoD Positioning

This clause defines the information elements for NR downlink AoD positioning (TS 38.305 [40]).

#### 6.5.11.1	NR DL-AoD Assistance Data

#### –	NR-DL-AoD-ProvideAssistanceData

<Skip Unmodified Changes>

#### –	NR-DL-AoD-TargetDeviceErrorCauses

The IE NR-DL-AoD-TargetDeviceErrorCauses is used by the target device to provide NR DL-AoD error reasons to the location server.

-- ASN1START

NR-DL-AoD-TargetDeviceErrorCauses-r16 ::= SEQUENCE {

cause-r16		ENUMERATED {	undefined,

assistance-data-missing,

unableToMeasureAnyTRP,

attemptedButUnableToMeasureSomeNeighbourTRPs,

thereWereNotEnoughSignalsReceivedForUeBasedDL-AoD,

locationCalculationAssistanceDataMissing,

...,

prs-MeasurementSkipped-v17xy

},

...

}

-- ASN1STOP

*** Next Changes ***

### 6.5.12	NR Multi-RTT Positioning

This clause defines the information elements for NR Multi-RTT positioning (TS 38.305 [40]).

#### 6.5.12.1	NR Multi-RTT Assistance Data

#### –	NR-Multi-RTT-ProvideAssistanceData

<Skip Unmodified Changes>

#### –	NR-Multi-RTT-TargetDeviceErrorCauses

The IE NR-Multi-RTT-TargetDeviceErrorCauses is used by the target device to provide NR Multi-RTT error reasons to the location server.

-- ASN1START

NR-Multi-RTT-TargetDeviceErrorCauses-r16 ::= SEQUENCE {

cause-r16		ENUMERATED {	undefined,

dl-assistance-data-missing,

unableToMeasureAnyTRP,

attemptedButUnableToMeasureSomeNeighbourTRPs,

ul-srs-configuration-missing,

unableToTransmit-ul-srs,

...,

prs-MeasurementSkipped-v17xy

},

...

}

-- ASN1STOP

*** End Of Changes ***