3GPP TSG-RAN WG2 Meeting #123	R2-2307545

Toulouse 21th -25th Aug, 2023

Source: 			ZTE Corporation, Sanechips

Title: 	Consideration on the interBandMRDC-WithOverlapDL-Bands-r16

Agenda item:	5.1.3.2

Document for: 	Discussion and Decision

- Introduction

In the last meeting, a LS [1] was sent from RAN4 on the interBandMRDC-WithOverlapDL-Bands-r16 with the below information:

During 107th meeting, RAN4 discussed the MRTD/MTTD requirement applicability for inter-band EN-DC/NE-DC with overlapping DL frequency. After discussion, RAN4 reached the following consensus: · For TDD-TDD inter-band EN-DC with overlapping frequency, o apply the MTTD/MTRD requirement for synchronous operation in 7.5.2.1/7.6.2.1 of 38.133 for EN-DC · For FDD-FDD inter-band EN-DC with overlapping frequency, o apply the MTTD/MTRD requirement for synchronous/asynchronous operation in 7.5.2/7.6.2 of 38.133 for EN-DC Furthermore, “Type 1 UE” and “Type 2 UE” are not defined terminologies in RAN4 specification. Instead, Type 2 UE is described as “a UE indicating capable of interBandMRDC-WithOverlapDL-Bands-r16” and Type 1 UE is the conventional UE supporting general requirements for inter-band collocated EN-DC in RAN4 specifications. RAN4 would like to share the above agreements which is thought to be relevant for the signalling description “interBandMRDC-WithOverlapDL-Bands-r16” with RAN2. RAN2 is invited to consider whether and how to update the signalling in 38.306

Meanwhile, a CR on this interBandMRDC-WithOverlapDL-Bands-r16 was also agreed in RAN4 [2], in which it further clarified the MRTD/MTTD requirement applicability when this capability is not reported as below:

MTTD: For E-UTRA TDD-NR TDD inter-band EN-DC with overlapping DL bands, only synchronized operation is assumed. The UE shall be capable of handling a maximum uplink transmission timing difference between E-UTRA PCell and PSCell as shown in Table 7.5.2.1-1 provided that UE indicates that it is capable of interBandMRDC-WithOverlapDL-Bands-r16, and in Table 7.5.3-1 provided that it is not capable of interBandMRDC-WithOverlapDL-Bands-r16. MRTD: For E-UTRA TDD-NR TDD inter-band EN-DC with overlapping DL bands, only synchronized operation is assumed. The UE shall be capable of handling at least a relative receive timing difference between subframe timing of signal from a E-UTRA cell belonging to the MCG and slot timing of signal from a cell belonging to the SCG at the UE receiver as shown in Table 7.6.2.1-1 provided that UE indicates that it is capable of interBandMRDC-WithOverlapDL-Bands-r16, and in Table 7.6.3-1 provided that it is not capable of interBandMRDC-WithOverlapDL-Bands-r16.

In this paper, we’d like to share our understanding on this capability based on RAN4’s progress.

- Discussion

- TDD-TDD inter-band EN-DC

According to the LS [1] and RAN4 agreed CR [2] as above, for TDD-TDD inter-band EN-DC with overlapping frequency, only synchronized operation is assumed and the MTTD/MTRD requirement for synchronous operation in 7.5.2.1/7.6.2.1 of 38.133 would be applied for EN-DC. Furthermore, if this capability is not reported, the MTTD/MTRD requirement for intra-band EN-DC as in 7.5.3/7.6.3 of 38.133 shall be applied.

Table 1: MTTD/MRTD requirement for the TDD-TDD inter-band EN-DC

| TDD-TDD inter-band EN-DC with overlapping frequency | Support interBandMRDC-WithOverlapDL-Bands-r16 | Not support interBandMRDC-WithOverlapDL-Bands-r16 |
| --- | --- | --- |
| synchronous | MTTD: 7.5.2.1 of 38.133 MRTD: 7.6.2.1 of 38.133 | MTTD: Table 7.5.3-1 of 38.133 MRTD: Table 7.6.3-1 of 38.133 |
| asynchronous | N/A | N/A |

Observation 1: On the MTTD/MRTD requirement for the TDD-TDD inter-band EN-DC with overlapping frequency, Ran4 spec has given clear clarification as summarized in Table 1.

- FDD-FDD inter-band EN-DC

For the FDD-FDD inter-band EN-DC with overlapping frequency, according to the LS, it’s clear that if the UE support the interBandMRDC-WithOverlapDL-Bands-r16, it would apply the MTTD/MTRD requirement for synchronous/asynchronous operation in 7.5.2/7.6.2 of 38.133 for EN-DC.

However, for the case that the UE does not support the interBandMRDC-WithOverlapDL-Bands-r16, it’s still unclear on what requirement shall be applied. According to the previous feature list. For the MRTD, the intra-band MR-DC requirements with MRTD<3us (i.e. Synchronous operation) would apply. But, in the 38.133 chapter 7.6.3, for the intra-band EN-DC, different MRTD requirements adopted for the asynchronous and synchronous operation.

| 2-19 | FDD-FDD or TDD-TDD inter-band MR-DC with overlapping or partially overlapping DL spectrum | Type 1 UE: supports FDD-FDD or TDD-TDD inter-band operation with overlapping or partially DL bands with MRTD<3us and intra-band MR-DC requirements apply. Type 2 UE: supports FDD-FDD or TDD-TDD inter-band operation with overlapping or partially overlapping DL bands with an MR-DC MRTD according to clause 7.6.2 in 38.133 and applicable inter-band RF requirements. If absent the UE is a type 1 UE. |
| --- | --- | --- |
| 38.133 7.6.3 Minimum Requirements for intra-band EN-DC The UE shall be capable of handling at least a relative receive timing difference between subframe timing of signal from a E-UTRA cell belonging to the MCG and slot timing of signal from a cell belonging to the SCG as shown in Table 7.6.2-1 for E-UTRA FDD-NR FDD intra-band EN-DC provided the UE indicates that it is capable of asynchronous EN-DC operation [2]. Table 7.6.2-1: Maximum receive timing difference requirement for asynchronous EN-DC Sub-carrier spacing of E-UTRA cell in MCG (kHz); DL Sub-carrier spacing of cell in SCG (kHz) (Note 1); Maximum receive timing difference (µs). 15; 15; 500. 15; 30; 250. 15; 60; 125. 15; 120Note2; 62.5. The UE shall be capable of handling at least a relative receive timing difference between subframe timing of signal from a E-UTRA cell belonging to the MCG and slot timing of signal from a cell belonging to the SCG as shown in Table 7.6.3-1 for E-UTRA FDD-NR FDD and E-UTRA TDD-NR TDD intra-band EN-DC provided the UE does not indicate that it is capable of asynchronous FDD-FDD EN-DC operation [16]. Table 7.6.3-1 Maximum receive timing difference requirement for intra-band synchronous EN-DC Sub-carrier spacing of E-UTRA cell in MCG (kHz); DL Sub-carrier spacing of cell in SCG (kHz) Note1; Maximum receive timing difference (µs). 15; 15; 3. 15; 30; 3. 15; 60; 3. |

From the above discussion, when the UE is not capable of interBandMRDC-WithOverlapDL-Bands-r16, for the FDD-FDD inter-band EN-DC with overlapping frequency, the intra-band MR-DC requirements with MRTD<3us (i.e. Synchronous operation) would apply for the synchronous operation, but for the asynchronous operation, there is a gap between the RAN4 feature description and the RAN4 spec (i.e. 38.133 7.6.3). Thus this shall be confirmed with RAN4 for the asynchronous operation.

Proposal 1:Ran2 to ask RAN4 to confirm whether the MRTD as in Table 7.6.2-1 of 38.133 (instead of the Table 7.6.3-1) shall be adopted for asynchronous operation with FDD-FDD inter-band EN-DC with overlapping frequency when the UE is not capable of interBandMRDC-WithOverlapDL-Bands-r16.

Similarly for the MTTD, from both the LS and the feature list table below, it doesn’t mention which requirement shall be supported for the FDD-FDD inter-band EN-DC with overlapping frequency when the UE is not capable of interBandMRDC-WithOverlapDL-Bands-r16.

7.5.3 Minimum Requirements for intra-band EN-DC The UE shall be capable of handling a maximum uplink transmission timing difference between E-UTRA PCell and PSCell as shown in Table 7.5.2-1 for E-UTRA FDD-NR FDD intra-band EN-DC provided the UE indicates that it is capable of asynchronous EN-DC operation [2]. Table 7.5.2-1 Maximum uplink transmission timing difference requirement for asynchronous EN-DC Sub-carrier spacing in E-UTRA PCell (kHz); UL Sub-carrier spacing for data in PSCell (kHz); Maximum uplink transmission timing difference (µs). 15; 15; 500. 15; 30; 250. 15; 60; 125. 15; 120Note1; 62.5. The UE shall be capable of handling a maximum uplink transmission timing difference between E-UTRA PCell and PSCell as shown in Table 7.5.3-1 for E-UTRA TDD-NR TDD and E-UTRA FDD-NR FDD intra-band EN-DC provided the UE does not indicate that it is capable of asynchronous FDD-FDD EN-DC operation [16]. Table 7.5.3-1: Maximum uplink transmission timing difference requirement for intra-band synchronous EN-DC Sub-carrier spacing in E-UTRA PCell (kHz); UL Sub-carrier spacing for data in PSCell (kHz); Maximum uplink transmission timing difference (µs). 15; 15; 5.21Note1,Note 2. 15; 30; 5.21Note 2. 15; 60; 5.21 Note 2. NOTE 1: This is not applicable for a UE which indicates the capability of only supporting single UL timing (ul-TimingAlignmentEUTRA-NR is signalled). Single UL timing for E-UTRA and NR cell is assumed for this UE. NOTE 2: If the transmission timing difference exceeds the cyclic prefix length of the UL Sub-carrier spacing for data in PSCell, NR UE Tx EVM degradation is expected for the symbol that is overlapping the LTE subframe boundary.

Proposal 2:Ran2 to ask RAN4 to confirm the MTTD requirement for the FDD-FDD inter-band EN-DC with overlapping frequency for the asynchronous and synchronous operations respectively when the UE is not capable of interBandMRDC-WithOverlapDL-Bands-r16.

In short the cases that need to be confirmed can be summarized as below.

Table 2: MTTD/MRTD requirement for the FDD-FDD inter-band EN-DC

| FDD-FDD inter-band EN-DC with overlapping frequency | Support interBandMRDC-WithOverlapDL-Bands-r16 | Not support interBandMRDC-WithOverlapDL-Bands-r16 |
| --- | --- | --- |
| synchronous | MTTD: 7.5.2.1 of 38.133 MRTD: 7.6.2.1 of 38.133 | MTTD: Table 7.5.3-1 of 38.133?? MRTD: Table 7.6.3-1 of 38.133 |
| asynchronous | MTTD: 7.5.2 of 38.133 MRTD: 7.6.2 of 38.133 | MTTD: Table 7.5.2-1 of 38.133?? MRTD: Table 7.6.2-1 of 38.133?? |

Proposal 3: Send a Reply LS to RAN4 to include the above proposal 1 and proposal 2.

Proposal 3a: Approve the draft Reply LS in [3].

- Conclusion and proposals

With the above analysis, we have the following proposals:

Observation 1: On the MTTD/MRTD requirement for the TDD-TDD inter-band EN-DC with overlapping frequency, Ran4 spec has given clear clarification as summarized in Table 1.

Table 1: MTTD/MRTD requirement for the TDD-TDD inter-band EN-DC

| TDD-TDD inter-band EN-DC with overlapping frequency | Support interBandMRDC-WithOverlapDL-Bands-r16 | Not support interBandMRDC-WithOverlapDL-Bands-r16 |
| --- | --- | --- |
| synchronous | MTTD: 7.5.2.1 of 38.133 MRTD: 7.6.2.1 of 38.133 | MTTD: Table 7.5.3-1 of 38.133 MRTD: Table 7.6.3-1 of 38.133 |
| asynchronous | N/A | N/A |

Proposal 1:Ran2 to ask RAN4 to confirm whether the MRTD as in Table 7.6.2-1 of 38.133 (instead of the Table 7.6.3-1) shall be applied for asynchronous operation with FDD-FDD inter-band EN-DC with overlapping frequency when the UE is not capable of interBandMRDC-WithOverlapDL-Bands-r16.

Proposal 2:Ran2 to ask RAN4 to confirm the MTTD requirement for the FDD-FDD inter-band EN-DC with overlapping frequency for the asynchronous and synchronous operations respectively when the UE is not capable of interBandMRDC-WithOverlapDL-Bands-r16.

Proposal 3: Send a Reply LS to RAN4 to include the above proposal 1 and proposal 2.

Proposal 3a: Approve the draft Reply LS in [3].

- Reference

[1] R4-2310170  LS on update for “interBandMRDC-WithOverlapDL-Bands-r16” in 38.306 	RAN4 Apple

[2] R4-2310169 Clarification on MRTD/MTTD and interruption requirement for EN-DC/NE-DC in Rel-16 Apple, Huawei, HiSilicon, Nokia, Ericsson, Samsung

[3] R2-2308907 (draft) Reply LS on update for “interBandMRDC-WithOverlapDL-Bands-r16” in 38.306  ZTE Corporation, Sanechips