3GPP TSG-RAN WG2 Meeting #123                                                        R2-2307559

Toulouse, France, 21st–25th Aug, 2023

Source:              CATT

Title:	             Discussion on NR Sidelink CA

Agenda Item:	7.15.6

Document for:	Discussion and Decision

Introduction

In this contribution, we mainly focus on the below issues on NR sidelink CA, and our views will be given based on the analysis.

- The new event for SL carrier (re-)selection triggers;

- The configuration for DTX is UE specific or carrier specific;

- The NBC issue;

- Whether SL CA is applied for PC5-S signaling;

- When SL CA is applied for PC5-RRC signaling;

- Details of PDCP duplication.

Discussion

The SL carrier (re-)selection triggers

Regarding to the SL carrier (re-)selection, the issue on triggers for NR SL carrier (re-)selection needs to be discussed. In the past meetings, we have reached below agreements to trigger the SL carrier (re)selection or perform SL carrier (re-)selection.

RAN2#121bis-e Agreement: Proposal 10: For TX carrier (re)selection triggers in NR sidelink CA, reuse the triggers for TX carrier (re)selection per sidelink process in LTE sidelink CA as follows at least for GC/BC - if the resource (re)selection is triggered with the sidelink process. - if there is no sidelink grant associated with the sidelink process on any carrier allowed for the STCH as indicated by upper layers (i.e., RRC layer and V2X layer). - FFS on unicast case. Proposal 7 For LCP, only allow the LCHs having a priority whose associated CBR threshold for reselection is no lower than the CBR of the carrier when the carrier is (re-)selected. FFS on how to determine the per-carrier CBR at least for GC/BC. FFS on unicast case. Proposal 5 NR SL CA TX carrier (re)selection follows LTE CA solution, i.e., define 1) per-carrier-per-priority CBR threshold for carrier (re)selection, and 2) per-carrier-per-priority CBR threshold for carrier keeping. And final carrier selection is done based on the lowest CBR value across carriers. Where the priority is the LCH priority. FFS on unicast case. RAN2#122 Agreement: Agreements on TX carrier (re)selection triggers, LCP impact, and CBR-based carrier reselection/keeping for UC - Agreements made for GC/BC (RAN2#121bis-e) are also applicable for UC. TX carrier reselection is done among the carriers that peer UE also supports. Agreement on DTX based SL RLF in SL CA - The counting is calculated per carrier. - Legacy SL RLF is not declared when the counting is reached to sl-MaxnumConsecutiveDTX) for carrier(s) and the UE has other available SL carrier(s) for SL CA.

Besides above carrier re-selection triggers, considering SL unicast is introduced in NR, more SL carrier re-selection triggers can be further considered based on the new features of NR SL unicast. For example, if the SL-RSRP of current SL carrier is below the configured threshold, SL carrier re-selection can be considered to find a carrier with high channel quality to improve the spectral efficiency. For another example, indication from peer UE to request carrier re-selection can also be considered, since peer UE has timely cognition to which carrier is better. Moreover, the SL carrier re-selection can be considered if consecutive DTX is received on a carrier; otherwise, the data transmission over this SL carrier will be low quality. .

Proposal 1: For SL UC, the following new SL carrier re-selection triggers can be considered:

- SL-RSRP is below a threshold;

- The indication from peer UE to request carrier reselection;

- The number of consecutive DTX on a carrier is above a threshold.

The configuration on DTX

In RAN2#122 meeting, RAN2 agreed that the counting on DTX is calculated per carrier. But the parameter for DTX, such as maxNumConsecutiveDTX, is suggested to be common to each carrier.  Carrier specific maxNumConsecutiveDTX is not needed from the view of fairness. Hence, the configuration of  maxNumConsecutiveDTX should be same as legacy, which could be configured by system information, RRC dedicated signaling and pre-configuration.

Proposal 2: No enhancement is needed on the configuration of  maxNumConsecutiveDTX.

NBC issue

In RAN2#122 meeting, NBC issue for GC/BC was raised. If the Rx UE is legacy UE, Tx UE should not apply SL CA in order to avoid the data loss at legacy Rx UE side. Regarding to this issue, the following agreement was reached in the last RAN2 meeting:

RAN2#122 Agreement: Agreements on backward compatibility issue in SL CA (for GC/BC) 1: Consider a case that a V2X service which needs to be mapped into multiple carriers while there is at least one legacy UE to receive this V2X service. RAN2 further discuss whether TX profile approach can be supported.

Based on the above agreement, it is obvious that even if there is legacy Rx UE, multiple carriers can be configured for one V2X service, hence, it can rely on upper layer configuration to solve the NBC issue. Considering that the V2X layer may have full picture about RX UE(s) which are interested to a V2X service, similar to the legacy reason of introducing Tx profile in Rel-17 SL DRX, it is suggested to use TX profile to indicate whether SL CA is applied or not.

Proposal 3: Tx profile can be used to solve the NBC issue.

SL CA for PC5-S

There are two types of PC5-S signaling:

- SL-SRB4: it is related to sidelink discovery.

- SL-SRB0/1/2: they are related to sidelink communication.

In LTE, SL CA is introduced only for SL communication. In NR, both SL discovery and SL communication are supported. It is unclear whether SL CA is also applied to SL discovery. In [1], it is noted that SL CA work in Rel-18 mainly targets some V2X use cases. For V2X use cases, which do not include the SL discovery use case.

Proposal 4: For PC5-S signalling used for SL discovery (SL-SRB4), SL CA is not applied.

On the issue that whether PC5-S message could apply the SL CA, based on the SA2 LS reply [1] as below, it can be seen that the AS layer cannot know the frequency information of the service triggering PC5-S message during PC5-UC link establishment procedure.

SA2 discussed the questions asked by RAN2 and would like to provide the following answers. - Question 1: According to TS 24.588, V2X layer is only provisioned with a mapping between service identifier and initial L2 address used for unicast. But service identifier is invisible to AS-layer, and the initial L2 ID will only be used in DCR and be replaced by a self-chosen L2 ID in PC5-S link establishment procedure. Then, after L2 ID changes, whether/how can UE's AS layer obtain the mapping between L2 ID and frequencies? - Answer 1 with question for clarification: According to discussion in SA2, most companies' understanding is Question 1 is about how the AS layer can know frequency information associated with the PC5-S messages sent during the PC5 unicast link establishment procedure. Anyhow some companies think that Question 1 is related to the scenario after the PC5 unicast link is established. Therefore, SA2 would like to ask RAN2 about what the scenario Question 1 is related to. SA2 also discussed how to let the AS layer know frequency information associated with the PC5-S messages sent for PC5 unicast link establishment, by providing frequency information together with the PC5-S message to the AS layer from the V2X layer, however there was no corresponding CR approved yet in SA2.

According to the LS reply, one possible enhanced method is that V2X layer will provide frequency information together with the PC5-S message to the AS layer, then AS transmits the PC5-S message over these carriers by SL CA. In our view, there is no strong requirement to transmit PC5-S message using SL CA, and it is not backward compatible. If SL CA is introduced for PC5-S, it will bring new NBC issue and it should also consider whether PDCP duplication should be used for PC5-S. It will not only impact RAN2, but also impact SA2. Considering the time limitation, it is suggested not to introduce the SL CA for PC5-S message.

Proposal 5: For PC5-S signaling used for sidelink communication (SL-SRB0/1/2), SL CA is not applied.

SL CA for PC5-RRC

In RAN2#122 meeting, RAN2 reached the following work assumption:

RAN2#122 Agreement Agreement on PDCP duplication/SL CA for SL SRB 1: Working assumption: SL CA/PDCP duplication is applied to PC5-RRC after SL link is established. FFS on exact time when it can be started.

According to above agreement and SA2 LS [2], after the SL link is established, the Tx UE could determine the carrier set for SL UC after UE receives peer UE capability information and the carrier set for service from the high layer, so it is feasible to apply the  SL CA/PDCP duplication to PC5-RRC.

Proposal 6: Confirm the following work assumption:

- SL CA/PDCP duplication is applied to PC5-RRC after SL link is established.

One remaining issue is when the PC5-RRC message will apply the SL-CA. Based on the definition of PC5-RRC signaling in annex, it can be seen that the PC5-RRC signaling include several messages, such as, the measurementReportSidelink, rrcReconfigurationSidelink and the messages for UE Capability exchange for sidelink communication. The carriers used for SL CA needs to be determined based on UE capability and the SL carriers corresponding to a L2 ID based on the information from upper layer. The Tx UE could receive the peer UE capability information and then send the SL CA/duplication configuration to Rx UE, and apply the SL CA/PDCP duplication after receiving RRCReconfigurationCompleteSidelink. So the Tx UE will apply the SL CA/PDCP duplication for P5-RRC after receiving RRCReconfigurationCompleteSidelink associated to prior SL CA/duplication configuration from Rx UE.

Proposal 7: For SL UC, Tx UE should send SL CA/duplication configuration to Rx UE.

Proposal 8: Tx UE will apply the SL CA/PDCP duplication for PC5-RRC after receiving RRCReconfigurationCompleteSidelink associated to prior SL CA/duplication configuration from Rx UE.

Details of PDCP duplication

Regarding to the PDCP duplication, the following open issues should be discussed:

- Issue 1: How to configure PDCP duplication for SL-DRB and SL-SRB?

- Issue 2: Whether PDCP duplication activation/deactivation is needed for SL-DRB and SL-SRB?

- Issue 3: How to define the LCID used for PDCP duplication?

In the following, the above issues will be discussed one by one:

Issue 1: The PDCP duplication configuration for SL-SRB and SL-DRB

In the last RAN2 meeting, the following agreements were reached:

RAN2#122 Agreement: Agreement on criterion for packet duplication 1: SLRB configures whether PDCP duplication is used or not

Based on the above agreement, it is agreed that SLRB configures whether PDCP duplication is used or not. Here, more details on how to configure the PDCP duplication is discussed.

For SL DRB, similar to the BOOLEAN parameter pdcp-Duplication used for DRB in Uu interface, when UE in connected mode, the network may have the QoS information of SL DRB, so the network could configure the SL PDCP duplication is used or not by RRC dedicated signaling. For UE in Idle/Inactive mode, network could configure it by system information. For OOC UE, the pre-configuration could be used to configure PDCP duplication.

Proposal 9: For SL DRB，the PDCP duplication configuration could be configured by the dedicated RRC signaling, SIB or pre-configuration.

For SL SRB, if the SL CA/PDCP duplication is agreed to be applied, the configuration of PDCP duplication could be specified in 9.1.1 of TS38.331. For example, if SL SRB3 is agreed to apply the SL PDCP duplication, the PDCP duplication configuration could be added in below table as in 9.1.1 of TS38.331, which is used as SCCH configuration for sidelink PC5-RRC message. Besides, whether to apply the PDCP duplication could be considered as UE implementation, it means that, the Tx UE could apply the PDCP duplication if the channel quality is poor, and not apply it if channel quality is good. PDCP duplication for SL SRB is selected by the transmitting UE, up to UE implementation.

| Name | Value | Semantics description | Ver |
| --- | --- | --- | --- |
| PDCP configuration |  |  |  |
| >t-Reordering | Undefined | Selected by the receiving UE, up to UE implementation |  |
| >pdcp-SN-Size | 12 |  |  |
| RLC configuration |  | AM RLC |  |
| >sn-FieldLength | 12 |  |  |
| >t-Reassembly | Undefined | Selected by the receiving UE, up to UE implementation |  |
| >t-PollRetransmit | Undefined | Selected by the transmitting UE, up to UE implementation |  |
| >pollPDU | Undefined | Selected by the transmitting UE, up to UE implementation |  |
| >pollByte | Undefined | Selected by the transmitting UE, up to UE implementation |  |
| >maxRetxThreshold | Undefined | Selected by the transmitting UE, up to UE implementation |  |
| >t-StatusProhibit | Undefined | Selected by the receiving UE, up to UE implementation |  |
| >logicalChannelIdentity | 3 |  |  |
| MAC configuration |  |  |  |
| >priority | 1 |  |  |
| >prioritisedBitRate | infinity |  |  |
| >logicalChannelGroup | 0 |  |  |
| >schedulingRequestId | 0 | The scheduling request configuration with this value is applicable for this SCCH if configured by the network. |  |
| >sl-HARQ-FeedbackEnabled | Undefined | Selected by the transmitting UE, up to UE implementation |  |

Proposal 10: For SL SRB, if SL CA/PDCP duplication is agreed to be applied, the PDCP duplication configuration could be specified in 9.1.1 of TS38.331, and the PDCP duplication configuration could be that selected by the transmitting UE and up to UE implementation.

Issue 2: PDCP duplication activation/deactivation for SL-SRB and SL-DRB

For SL DRB, based on above analysis, PDCP duplication activation/deactivation could be introduced based on a BOOLEAN parameter in PDCP duplication configuration. In Uu interface, MACCE based activation/deactivation for PDCP duplication is applied to make the PDCP duplication could be activated/deactivated quickly since the channel status may change. For SL PDCP duplication, regarding the SL UE may be in IDLE or inactive mode, the network cannot receive the channel quality information or sense the status of SL RB when UE is not in connected mode, and there is no feedback in SL GC/BC, so it is suggested that MACCE based activation/deactivation for PDCP duplication is not introduced for SL RB.

Proposal 11: For SL DRB, MAC CE based activation/deactivation for PDCP duplication in Uu interface is not introduced.

For SL SRB, considering the rule that PDCP duplication is always used for Uu SRB if configured, and there is no QoS requirement for SL SRB, SL SRB could reuse the PDCP duplication rule as Uu SRB does. Based on above analysis to PDCP duplication configuration for SL SRB, the PDCP duplication could be applied for SL SRB if configured for SL SRB3.

Proposal 12: For SL SRB3, PDCP duplication will be used if configured, there is no explicit PDCP duplication activation/deactivation.

Issue 3: LCID for SL PDCP duplication

By below agreement, one remaining issue is to design the specific SL LCID values for the duplicated sidelink LCHs.

RAN2#121bis Agreement: Proposal 16: For NR sidelink PDCP duplication, reuse the hard-coded way for paired sidelink LCID to identify duplicated sidelink LCHs (i.e. for a unified design for all Bcast/Gcast). The specific SL LCID values occupied are left to Stage-3. FFS on Unicast case. RAN2#122 Agreement: Agreement on LCID to identify duplicated SL LCHs for UC 1: Agreement made for GC/BC (RAN2#121bis-e) is also applicable for UC.

Based on below table from the TS 38.321, LCID 20–55 are reserved, so it could be used for PDCP duplication realted to logical channle 4–19 and logical channle 3 for SCCH carrying PC5-RRC messages if  SL duplication is supproted for PC5-RRC message.

Table 6.2.4-1 Values of LCID for SL-SCH

| Index | LCID values |
| --- | --- |
| 0 | SCCH carrying PC5-S messages that are not protected |
| 1 | SCCH carrying PC5-S messages "Direct Security Mode Command" and "Direct Security Mode Complete" |
| 2 | SCCH carrying other PC5-S messages that are protected |
| 3 | SCCH carrying PC5-RRC messages |
| 4–19 | Identity of the logical channel |
| 20–55 | Reserved |
| 56 | SCCH carrying RRC messages delivered via SL-RLC0 as specified in TS 38.331 [5] |
| 57 | SCCH carrying RRC message delivered via SL-RLC1 as specified in TS 38.331 [5] |
| 58 | SCCH for Sidelink Discovery Messages |
| 59 | Sidelink Inter-UE Coordination Request |
| 60 | Sidelink Inter-UE Coordination Information |
| 61 | Sidelink DRX Command |
| 62 | Sidelink CSI Reporting |
| 63 | Padding |

Proposal 13: The logical channel LCID 20–36 could be used for PDCP duplication assocatiated the logical channle 4–19 and logical channle 3 for SCCH carrying PC5-RRC messages if agreed.

Conclusion

According to the analysis in section 2, it is proposed:

Proposal 1: For SL UC, the following new SL carrier re-selection triggers can be considered:

- SL-RSRP is below a threshold;

- The indication from peer UE to request carrier reselection;

- The number of consecutive DTX on a carrier is above a threshold.

Proposal 2: No enhancement is needed on the configuration of  maxNumConsecutiveDTX.

Proposal 3: Tx profile can be used to solve the NBC issue.

Proposal 4: For PC5-S signalling used for SL discovery (SL-SRB4), SL CA is not applied.

Proposal 5: For PC5-S signaling used for sidelink communication (SL-SRB0/1/2), SL CA is not applied.

Proposal 6: Confirm the following work assumption:

- SL CA/PDCP duplication is applied to PC5-RRC after SL link is established.

Proposal 7: For SL UC, Tx UE should send SL CA/duplication configuration to Rx UE.

Proposal 8: Tx UE will apply the SL CA/PDCP duplication for PC5-RRC after receiving RRCReconfigurationCompleteSidelink associated to prior SL CA/duplication configuration from Rx UE.

Proposal 9: For SL DRB，the PDCP duplication configuration could be configured by the dedicated RRC signaling, SIB or pre-configuration.

Proposal 10: For SL SRB, if SL CA/PDCP duplication is agreed to be applied, the PDCP duplication configuration could be specified in 9.1.1 of TS38.331, and the PDCP duplication configuration could be that selected by the transmitting UE and up to UE implementation.

Proposal 11: For SL DRB, MAC CE based activation/deactivation for PDCP duplication in Uu interface is not introduced.

Proposal 12: For SL SRB3, PDCP duplication will be used if configured, there is no explicit PDCP duplication activation/deactivation.

Proposal 13: The logical channel LCID 20–36 could be used for PDCP duplication assocatiated the logical channle 4–19 and logical channle 3 for SCCH carrying PC5-RRC messages if agreed.

Reference

- RP-230077 WID revision: NR sidelink evolution.

- S2-2307794, Reply LS on carrier mapping for unicast SL CA, Source: SA2, To: RAN2.

Annex

6.6.1	General message structure

#### –	SCCH-Message

The SCCH-Message class is the set of RRC messages that may be sent from the UE to the UE for unicast of NR sidelink communication on SCCH logical channel.

-- ASN1START

-- TAG-SCCH-MESSAGE-START

SCCH-Message ::=             SEQUENCE {

message                         SCCH-MessageType

}

SCCH-MessageType ::=         CHOICE {

c1                              CHOICE {

measurementReportSidelink                MeasurementReportSidelink,

rrcReconfigurationSidelink               RRCReconfigurationSidelink,

rrcReconfigurationCompleteSidelink       RRCReconfigurationCompleteSidelink,

rrcReconfigurationFailureSidelink        RRCReconfigurationFailureSidelink,

ueCapabilityEnquirySidelink              UECapabilityEnquirySidelink,

ueCapabilityInformationSidelink          UECapabilityInformationSidelink,

uuMessageTransferSidelink-r17            UuMessageTransferSidelink-r17,

remoteUEInformationSidelink-r17          RemoteUEInformationSidelink-r17

},

messageClassExtension           CHOICE {

c2                              CHOICE {

notificationMessageSidelink-r17 NotificationMessageSidelink-r17,

ueAssistanceInformationSidelink-r17 UEAssistanceInformationSidelink-r17,

spare6 NULL, spare5 NULL, spare4 NULL, spare3 NULL, spare2 NULL, spare1 NULL

},

messageClassExtensionFuture-r17    SEQUENCE {}

}

}

-- TAG-SCCH-MESSAGE-STOP

-- ASN1STOP