3GPP TSG-RAN WG2 Meeting #123		R2-2307562

Toulouse, France, 21 – 25 August, 2023                   Revision of R2-2304850

Agenda item:	5.2

Source:	Huawei, HiSilicon, vivo, Ericsson

Title:	Potential issue caused by using destination index

Document for:	Discussion and Decision

# Introduction

In current TS 38.331 [1], for SL RSRP measurement configuration and unicast SL DRX configuration, gNB can provide such dedicated configuration associated with a destination index to Tx UE, whereas the destination index is used to indicate the unicast connection based on the destination ID list reported in SidelinkUEInformation message.

#### 5.3.5.14 Sidelink dedicated configuration … 1> if sl-MeasConfigInfoToReleaseList is included in sl-ConfigDedicatedNR within RRCReconfiguration: 2> for each SL-DestinationIndex included in the received sl-MeasConfigInfoToReleaseList that is part of the current UE configuration: 3> remove the entry with the matching SL-DestinationIndex from the stored NR sidelink measurement configuration information; 1> if sl-MeasConfigInfoToAddModList is included in sl-ConfigDedicatedNR within RRCReconfiguration: 2> for each sl-DestinationIndex included in the received sl-MeasConfigInfoToAddModList that is part of the current stored NR sidelink measurement configuration: 3> reconfigure the entry according to the value received for this sl-DestinationIndex from the stored NR sidelink measurement configuration information; 2> for each sl-DestinationIndex included in the received sl-MeasConfigInfoToAddModList that is not part of the current stored NR sidelink measurement configuration: 3> add a new entry for this sl-DestinationIndex to the stored NR sidelink measurement configuration. 1> if sl-DRX-ConfigUC-ToReleaseList is included in sl-ConfigDedicatedNR within RRCReconfiguration: 2> for each SL-DestinationIndex included in the received sl-DRX-ConfigUC-ToReleaseList that is part of the current UE configuration: 3> remove the entry with the matching SL-DestinationIndex from the stored NR sidelink DRX configuration information; 1> if sl-DRX-ConfigUC-ToAddModList is included in sl-ConfigDedicatedNR within RRCReconfiguration: 2> for each sl-DestinationIndex included in the received sl-DRX-ConfigUC-ToAddModList that is part of the current stored NR sidelink DRX configuration: 3> reconfigure the entry according to the value received for this sl-DestinationIndex from the stored NR sidelink DRX configuration information; 2> for each sl-DestinationIndex included in the received sl-DRX-ConfigUC-ToAddModList that is not part of the current stored NR sidelink DRX configuration: 3> add a new entry for this sl-DestinationIndex to the stored NR sidelink DRX configuration. …

Then UE applies such dedicated configuration based on the destination index according to above specification, i.e., by comparing the destination index associated with dedicated configuration with the destination index from the stored configuration. In this paper, we discuss potential issue caused by using destination index, considering the destination ID corresponding to one certain destination index can be changed when UE updates its destination ID list in SidelinkUEInformation message.

# Discussion

UE can update the destination ID list in the SUI message, e.g., add some new destination ID(s) and/or delete some existing destination ID(s). Taking an example as in Table 1, compared with old SUI, UE deletes destination ID#y and adds destination ID#m in new SUI. It can be found that, the destination index corresponding to destination ID#z is changed from #2 to #1, and the destination ID in old SUI is different from that in new SUI for both destination index #1 and destination index #2.

Table 1: destination index changing due to destination ID list updates

| Original reported info. in old SUI |  | Updated info. In new SUI |  |
| --- | --- | --- | --- |
| DST ID list | Associated DST Index | DST ID list | Associated DST Index |
| Destination ID #x | #0 | Destination ID #x | #0 |
| Destination ID #y | #1 | Destination ID #z | #1 |
| Destination ID #z | #2 | Destination ID #m | #2 |

Observation 1: The destination index for certain destination ID in old SUI can be different from that for same destination ID in new SUI, due to UE updating destination ID list.

There was one offline discussion during RAN2#122 meeting regarding this issue [2][3], some companies thought that the UE and the gNB can maintain the correct mapping between destination index and destination ID. However, it should be emphasized that, while UE applies dedicated SL configuration based on the destination index rather than destination ID, the destination index associated with stored SL configuration shall not be automatically assumed as to be changed by UE autonomous behavior. According to RRC procedure principle, the stored SL configuration is under gNB’s control via dedicated RRC configuration, without which the UE should not changed the stored configuration.

Observation 2: In current specification, UE applies dedicated SL configuration based on the destination index rather than destination ID.

Observation 3: The destination index associated with stored SL configuration can’t be changed by UE itself, as the stored SL configuration is under gNB’s control via dedicated RRC configuration.

Then for dedicated SL RSRP measurement configuration and unicast SL DRX configuration, problem will be caused, to be more specific, based on latest SUI message:

- If gNB use destination index #1 (to provide SL configuration for destination ID #z)

- In this case, the UE had released the SL configuration for destination index #1, as the destination ID #y is deleted, then all SL configuration for this destination will be released.

- Then, the UE will treat such SL configuration for destination index #1 as a new SL configuration, as it is not part of the current stored SL configuration. However, such SL configuration for destination may be a delta configuration instead of complete configuration for destination ID #z, then configuration failure may be introduced (i.e. UE regards a delta configuration as a complete configuration).

Observation 4: When UE updating destination ID list, for dedicated SL configuration anchored at destination index, configuration failure may be introduced (i.e. UE regards a delta configuration as a complete configuration).

From our perspective, two options can be considered to solve this issue.

- Option 1: use NOTE to clarify that the UE is expected to update the mapping between the destination L2 ID and the destination index for the stored NR sidelink configuration after the UE updates the destination list and reports to the gNB.

- Option 2: apply dedicated SL configuration using destination ID corresponding to destination index in procedure part.

Regarding above two options, option 1 may be preferred, as a NOTE is introduced to clarify that UE can update the destination index for the stored SL configuration after the UE updates the destination list and reports to the gNB, even without a dedicated RRC signalling for this purpose.

Proposal 1: RAN2 to agree the companion CRs to solve the issue for dedicated SL configuration using destination index.

# Conclusion

In this paper, we discuss potential issue for dedicated SL configuration cause using destination index and have the following observation and proposals.

Observation 1: The destination index for certain destination ID in old SUI can be different from that for same destination ID in new SUI, due to UE updating destination ID list.

Observation 2: In current specification, UE applies dedicated SL configuration based on the destination index rather than destination ID.

Observation 3: The destination index associated with stored SL configuration can’t be changed by UE itself, as the stored SL configuration is under gNB’s control via dedicated RRC configuration.

Observation 4: When UE updating destination ID list, for dedicated SL configuration anchored at destination index, configuration failure may be introduced (i.e. UE regards a delta configuration as a complete configuration).

Proposal 1: RAN2 to agree the companion CRs to solve the issue for dedicated SL configuration using destination index.

# References

TS 38.331.

Report of 3GPP TSG RAN WG2 meeting #122, Incheon, Korea.

R2-2306701, Summary of [AT122][501][V2X/SL] V2X corrections, vivo