3GPP TSG-RAN WG2 Meeting #123	R2-2307723

Toulouse, France, August 21-25, 2023

Agenda Item:	7.15.2

Source: 	Xiaomi

Title:  	Discussion on SL consistent LBT failure

Document for:	Discussion and Decision

1	Introduction

In last RAN2 meeting, RAN2 discussed the consistent LBT for SL-U and achieved some WAs and agreements [1].

In this contribution, we would like to discuss some remaining issues related to consistent LBT failure and have corresponding proposals.

2	Discussion

2.1 SL-specific LBT failure indication

In RAN2#121 meeting, RAN2 agreed that SL LBT failure indication granularity is per SL RB set.

Agreements on SL LBT failure indication granularity 1: SL LBT failure indication granularity is per SL RB set.

In addition, RAN1 agreed that multi-channel transmission is supported which means a SL transmission may cross multiple RB sets. In this case, different from NR-U, where only one LBT failure indication is provided from PHY to MAC, in SL-U, more than one LBT failure indication may be delivered from PHY to MAC for a SL transmission if this transmission is not transmitted due to LBT failure.

Agreement For dynamic channel access mode with multi-channel case in SL-U, NR-U UL channel access procedure is considered as baseline for transmission on multiple channels - FFS: whether transmission of PSFCH and/or S-SSB on a subset of RB sets is supported (using the NR-U DL channel access procedure as baseline) - FFS any necessary enhancement and modification for the SL-U operation

Proposal 1: For a multi-channel SL transmission, PHY can deliver more than one LBT failure indication if this SL transmission is not transmitted due to LBT failure.

In last RAN2 meeting, we discussed about the consistent LBT failure counting for SSB/ PSFCH and have the following agreements.

Agreements on SL C-LBT failure and S-SSB

1: 	Counting LBT failure indication regardless of whether LBT failure was provided because of S-SSB transmission or data transmission when RB set for S-SSB transmission belongs to the selected TX resource pool.

Agreements on SL C-LBT failure and PSFCH

1: 	Counting LBT failure indication regardless of whether LBT failure was provided because of PSFCH transmission or not when RB set for PSFCH transmission belongs to the selected TX resource pool. FFS when multiple PSFCH occasions are configured.

Actually during the online discussion, there was some concern for the case that RB set for S-SSB/PSFCH transmission and RB set for the selected resource pool are different, i.e., S-SSB-specific RB set. From our understanding, there is no agreement in RAN1 that RB set used for S-SSB transmission cannot transmit data, even the case is valid, i.e., some RB sets only allow S-SSB transmission, the LBT failure for S-SSB should be counted as well, otherwise, the RB set used for S-SSB transmission cannot declare consistent LBT failure even the channel condition is congested.

Proposal 2: Counting LBT failure indication of S-SSB when RB set for S-SSB/PSFCH transmission and RB set for the selected resource pool are different.

While for the PSFCH case, the situation is not quite the same as S-SSB, as the PSFCH transmission should belong to the selected TX resource pool, i.e., there is no PSFCH specific RB set. Regarding multiple PSFCH occasion, we think some additional mechanism is needed for the LBT failure indication counting, otherwise, consistent LBT failure may be triggered frequently when LBT fails in multiple PSFCH occasions consecutively. Since the PSFCH occasions are quite consecutive in time and frequency domain, it is reasonable to count only once if LBT fails in all the PSFCH occasions.

Proposal 3: LBT failure counts only once when LBT fails in all the PSFCH occasions.

2.2 SL-specific consistent LBT failure detection

In NR-U, RRC controls the consistent LBT failure detection and recovery by configuring a maximum count and a detection timer, which are configured and maintained per BWP. In previous RAN2 meeting, we have agreed to reuse NR-U parameters and variables as a baseline. However the granularity of the parameters are not determined.

5:	As in NR-U, introduce the following parameters and variables for the SL-specific consistent LBT failure detection in SL-U as the baseline:

- An SL-specific LBT failure indication counter (e.g. SL_LBT_COUNTER);

- An SL-specific maximum LBT failure instance count threshold (e.g. sl-LBT-FailureInstanceMaxCount);

- An SL-specific LBT failure detection timer (e.g. sl-LBT-FailureDetectionTimer).

In last RAN2 meeting, we agreed SL-specific consistent LBT failure is detected per RB set.

Agreement

SL C-LBT failure is declared per RB-set

In this case, the counter and timer should be maintained per RB set, i.e. for each RB set, if LBT failure indication is received from the lower layer, the SL-specific LBT failure indication counter associated with the RB set (e.g. SL_LBT_COUNTER) is incremented by one and the SL-specific LBT failure detection timer (e.g. sl-LBT-FailureDetectionTimer) associated with the RB set is started or restarted.

Proposal 4: The SL-specific LBT failure indication counter and the SL-specific LBT failure detection timer is maintained per RB set.

Regarding the granularity of the configuration of the count threshold/detection timer, based on the existing RRC structure, it is possible to configure the count threshold/detection timer per resource pool or per BWP. However, even the consistent LBT failure is detected per RB set, there is no need to configure different values of the threshold/timer for RB set within different resource pool. A common value configured per SL BWP and applied to all the RB sets associated with the BWP is enough.

Proposal 5: The SL-specific LBT failure indication count threshold and the SL-specific LBT failure detection timer is configured per SL BWP.

2.3 SL-specific consistent LBT failure recovery

2.3.1 SL-specific consistent LBT failure reporting

In RAN2#121 meeting, we confirmed that UE uses MAC CE to report consistent LBT failure to the network and the MAC CE indicates the RB set(s) where consistent LBT failure happens.

Confirm the following working assumption:

UE uses the MAC CE to report consistent LBT failure to the gNB

Agreement:

Uu MAC CE indicates RB set(s) where C-LBT failure happens.

In NR-U, the MAC CE includes a bitmap to indicate the serving cell where SL-specific consistent LBT failure has been triggered and not cancelled. However with the granularity as RB set, if any RB set within the same BWP has a unique index, single bitmap indication is enough, otherwise, if RB set(s) within different resource pools can have the same index, additional resource pool index is needed to identify the RB set triggered consistent LBT failure. It is suggested RAN2 to send LS to RAN1 to check whether RB set(s) can have unique index or not.

Proposal 6: MAC CE includes resource pool index to identify the RB set if RB set does not have unique index.

Proposal 7: Send LS to RAN1 to check if RB set(s) within the same BWP can have unique index.

In NR-U, a dedicated SR configuration was introduced for consistent LBT failure reporting. The motivation is to assist the network to acknowledge the occurrence of consistent LBT failure through the dedicated SR resource and then recover accordingly. Similarly, since we have already agreed to support the report of SL consistent LBT failure to the network for UE operating in both mode 1 and RRC_CONNECTED mode 2, in case there is no resource available upon triggering of the report, SR needs to be triggered to request uplink resource. Therefore RAN2 needs to discuss whether to define a dedicated SR configuration for SL consistent LBT reporting.

Proposal 8: RAN2 to discuss the SR configuration associated with SL-specific consistent LBT failure reporting.

2.3.2 Change of resource pool/RB set

During last RAN2 meeting, RAN2 discussed about how to recovery from SL consistent LBT failure for mode 2 idle/inactive UEs and achieved the following agreement

Agreements on SL C-LBT failure recovery (mode 2, RRC ide/inactive UE)

1: 	Exclusion of RB set(s) that SL C-LBT failure was detected in candidate resource selection + resource pool (re)selection

2:	The UE performs resource pool (re)selection

-  When SL C-LBT failure was detected for all RB-sets within a selected resource pool or;

-  Up to UE implementation although the above condition is not met

However for connected mode 2 UEs, SL consistent LBT failure reporting to the NW is also allowed.

Agreements on SL C-LBT failure recovery (mode 2, RRC connected UE)

1: 	RAN2 confirms that SL C-LBT failure indication is reported to the gNB also for mode 2, RRC connected UE.

In this case, it remains unclear whether mode 2 connected UEs also allow the change of resource pool/RB set upon detection of SL consistent LBT failure. Actually the detailed recovery behaviour also has impact on the cancellation of SL consistent LBT failure. In last meeting, there was some proposed mechanisms on the cancellation for mode 2 idle/inactive UEs, e.g., based on timer or measurement. In this case, if both are allowed for mode 2 connected UEs, it is hard to determine when to cancel the SL consistent LBT failure. In addition, since mode 2 connected UE already reports the SL consistent LBT failure to the NW, even without resource reselection, it can still rely on the NW for recovery, e.g., reconfigure the resource pool/RB set etc. Therefore, we think for mode 2 connected UEs, there is no need to support change of resource pool/RB set as mode 2 idle/inactive UEs.

Proposal 9: Mode 2 connected UE does not support change of resource pool/RB set as mode 2 idle/inactive UEs upon SL-specific consistent LBT failure detection.

2.4 Cancellation of SL-specific consistent LBT failure

In last RAN2 meeting, we have agreed whether/how the triggered consistent SL LBT failure is cancelled is FFS:

Working assumption: If SL LBT failure granularity is resource pool/RB set, support the change of resource pool/RB set of which consistent SL LBT failure has not been triggered from SL consistent LBT failure by TX UE upon consistent LBT failure detection. FFS whether/how the triggered consistent SL LBT failure is cancelled

In NR-U, UE needs to cancel the consistent LBT failure on the serving cells if any in the following cases.

- Case 1: upon deactivation of the SCell, cancel, if any, triggered consistent LBT failure for the SCell.

- Case 2: upon MAC reset, cancel, if any, triggered consistent LBT failure

- Case 3: upon reception of a PDCCH for BWP switching, cancel, if any, triggered consistent LBT failure for this Serving Cell

- Case 4: upon successful transmission of the consistent LBT failure MAC CE, cancel all the triggered consistent LBT failure(s) in SCell(s) for which consistent LBT failure was indicated

- Case 5: upon complete of RACH on the SpCell which has triggered consistent LBT failure, cancel all the triggered consistent LBT failure(s) in the SpCell

- Case 6: upon reconfiguration of the consistent LBT failure related parameters for this serving cell, cancel all the triggered consistent LBT failure(s) in this serving cell

When it comes to SL-U, generally we should reuse the NR-U cancellation mechanism as much as possible. Besides case 1, case 3 and case 5, i.e., BWP switching and RACH are not supported on SL, all the others cases should be supported. Case 2/6 should be applied commonly to UEs operating in both mode 1 and mode 2 while for case 4, it should be the cancellation of SL-specific consistent LBT failure in RB sets for which SL-specific consistent LBT failure was indicated, secondly this case only applies to UE operating in mode 1 and RRC_connected UE operating in mode 2. In addition, if the resource pool/RB set is reconfigured, similar as reconfiguration of consistent LBT failure related parameters, triggered consistent LBT failure should be cancelled. In addition, since the resource allocation mode is controlled by the network via RRC configuration, when the resource allocation mode is switched from mode 1 to mode 2 or vice versa, similar as the above reconfiguration cases, UE should cancel the triggered SL consistent LBT failure for the corresponding RB sets.

Proposal 10a: The SL-specific consistent LBT failure is cancelled in the following cases for UEs operating in both mode 1 and mode 2.

- Case 1: upon MAC reset, cancel, if any, triggered SL-specific consistent LBT failure.

- Case 2: upon reconfiguration of the consistent LBT failure related parameters, cancel if any the triggered consistent LBT failure(s) for the RB sets.

- Case 3: upon reconfiguration of resource pool, cancel, if any the triggered consistent LBT failure(s) for the RB sets associated with the resource pool.

- Case 4: upon resource allocation mode switch, cancel, if any the triggered consistent LBT failure for the RB sets.

Proposal 10b: The SL-specific consistent LBT failure is cancelled in the following case for UEs operating in mode 1 and RRC_CONNECTED UEs operating in mode 2.

- Case 5: upon successful transmission of the SL-specific consistent LBT failure MAC CE to the network, cancel all the triggered consistent LBT failure(s) in RB sets for which consistent LBT failure was indicated.

However, for UE operating in mode 2 but in RRC_IDLE/INACTIVE states, since reporting to the NW is not supported, upon SL-specific consistent LBT failure detection, there is no other means to cancel this consistent LBT failure status unless the carrier is deactivated or MAC is reset or related parameters are reconfigured. In this case, some other mechanism should be introduced to allow the UE to cancel the consistent LBT failure and reuse the resources for SL communication. In last meeting, there were some proposed mechanisms on the cancellation for mode 2 idle/inactive UEs, e.g., based on timer or measurement.

Actually we think whether the channel condition recovers or not does not depend on how long time passing away, but should be based on the measured degree of congestion. Therefore it is more reasonable to rely on the measurement to judge whether the link can be recovered, however, the existing CBR/RSSI measurement is performed per resource pool or per cell, while the consistent LBT failure detection is per RB set. Also it is possible that a RB set may belong to multiple resource pools, for which case it is hard to judge whether a RB set can recovery from consistent LBT failure based on measurement. Then from this perspective, maybe timer based solution is more suitable and straightforward. But as mentioned above, a single timer alone is not enough to reflect the channel link, at least one successful LBT is required after the expiry of the timer to determine the RB set can be recovered.

In this case, for RRC_IDLE/INACTIVE UEs operating in mode 2, upon detection of SL consistent LBT failure on a RB set, start the recovery timer for this RB set and upon the expiry of the timer, UE performs a LBT on this RB set and if LBT is successful, UE considers this RB set recovers from SL consistent LBT failure. Similar as the SL consistent LBT failure detection timer, this recovery timer is maintained per RB set and configured per BWP.

Proposal 10c: The SL-specific consistent LBT failure is cancelled in the following case for RRC_IDLE/INACTIVE UEs operating in mode 2.

- Case 6: upon expiry of the associated SL consistent LBT failure recovery timer and a successful LBT on the RB set, cancel if any the triggered SL consistent LBT failure for the RB set.

3	Conclusion

In this contribution, we discussed about SL consistent LBT failure and provide corresponding observations and proposals:

Proposal 1: For a multi-channel SL transmission, PHY can deliver more than one LBT failure indication if this SL transmission is not transmitted due to LBT failure.

Proposal 2: Counting LBT failure indication of S-SSB when RB set for S-SSB/PSFCH transmission and RB set for the selected resource pool are different.

Proposal 3: LBT failure counts only once when LBT fails in all the PSFCH occasions.

Proposal 4: The SL-specific LBT failure indication counter and the SL-specific LBT failure detection timer is maintained per RB set.

Proposal 5: The SL-specific LBT failure indication count threshold and the SL-specific LBT failure detection timer is configured per SL BWP.

Proposal 6: MAC CE includes resource pool index to identify the RB set if RB set does not have unique index.

Proposal 7: Send LS to RAN1 to check if RB set(s) within the same BWP can have unique index.

Proposal 8: RAN2 to discuss the SR configuration associated with SL-specific consistent LBT failure reporting.

Proposal 9: Mode 2 connected UE does not support change of resource pool/RB set as mode 2 idle/inactive UEs upon SL-specific consistent LBT failure detection.

Proposal 10a: The SL-specific consistent LBT failure is cancelled in the following cases for UEs operating in both mode 1 and mode 2.

- Case 1: upon MAC reset, cancel, if any, triggered SL-specific consistent LBT failure.

- Case 2: upon reconfiguration of the consistent LBT failure related parameters, cancel if any the triggered consistent LBT failure(s) for the RB sets.

- Case 3: upon reconfiguration of resource pool, cancel, if any the triggered consistent LBT failure(s) for the RB sets associated with the resource pool.

- Case 4: upon resource allocation mode switch, cancel, if any the triggered consistent LBT failure for the RB sets.

Proposal 10b: The SL-specific consistent LBT failure is cancelled in the following case for UEs operating in mode 1 and RRC_CONNECTED UEs operating in mode 2.

- Case 5: upon successful transmission of the SL-specific consistent LBT failure MAC CE to the network, cancel all the triggered consistent LBT failure(s) in RB sets for which consistent LBT failure was indicated.

Proposal 10c: The SL-specific consistent LBT failure is cancelled in the following case for RRC_IDLE/INACTIVE UEs operating in mode 2.

- Case 6: upon expiry of the associated SL consistent LBT failure recovery timer and a successful LBT on the RB set, cancel if any the triggered SL consistent LBT failure for the RB set.

4	Reference

[1] RAN2#122 meeting minutes.