# 3GPP TSG RAN WG2 Meeting #123      	                           R2-2307809
Toulouse, France, Aug 21- Aug 25

Agenda item:	7.3.4

Source:	Apple

Title:	Detailed mechanism of legacy UE barring in NES cell

WID/SID:	Netw_Energy_NR-Core– Release 18

Document for:	Discussion and Decision

# 1 Introduction

- Specify mechanism(s) to prevent legacy UEs camping on cells adopting the Rel-18 NES techniques, if necessary [RAN2]

The Network Energy saving (NES) WID RP-223540 was agreed in RAN#98-e [1], the WI objective on legacy UE barring is copied below:

In RAN2#122 [2], legacy UE barring mechanism was discussed, and it was agreed to introduce separate camping restrictions for NES-capable and non-NES UEs:

Agreements:

- We will define UE capabilities with signaling.  Details are FFS and will be discussed later during the WI phase.

- Separate camping restrictions for NES-capable and non-NES UEs will be defined.  FFS if it is a single bit or more.

In this contribution, we share our view on mechanism of preventing legacy UEs camping on Rel-18 NES cells.

# 2 Discussion

### 6.5.1	Cell selection/reselection

For backward compatibility, there is a need to allow NES cells to prevent legacy UEs from camping. NES cells should be able to configure whether to prevent legacy UEs, while allowing NES-capable UEs to camp on. Possible solutions may include but not limited to:

-	Use IntraFreqExcludedCellList/InterFreqExcludedCellList

-	Use the cellBarred or cell reservation fields in MIB/SIB

The definition of NES cell will be discussed in the WI phase.

The NW should be able to configure NES-capable UEs to prioritize/down-prioritize a specific NES cell or NES cells on a specific frequency. It is left to the WI phase whether the existing mechanism for cell (re)selection is sufficient according to the NES techniques specified.

From RAN2 perspective, legacy UEs and NES-capable UEs can be handled via cell selection/reselection techniques in the presence of NES cells.

The legacy UE barring issue has been extensively discussed as part of cell (re)selection in study item phase, and captured in TR 38.864 [3]. We copy the related part below:

In our understanding, above conclusion of study item identified 3 solutions:

- Solution 1: Set cellBarred in MIB as "barred" and introduce a new barring indication in SIB1 to allow NES capable UE camping (e.g. 1bit cellBarredNES set as "notBarred").

- Solution 2: Set cellReservedForOtherUse and cellReservedForFutureUse in SIB1 and introduce a new barring indication (1 or 2 bits) in SIB1 to allow NES capable UE camping.

- Solution 3: Introduce a NES dedicated IntraFreqExcludedCellList/InterFreqExcludedCellList in SIB3 and SIB4.

Observation 1: Study item identified 3 solutions of legacy UE barring:

- Solution 1: Set cellBarred in MIB as "barred" and introduce a new barring indication in SIB1 to allow NES capable UE camping (e.g. 1bit cellBarredNES set as "notBarred").

- Solution 2: Set cellReservedForOtherUse and cellReservedForFutureUse in SIB1 and introduce a new barring indication (1 or 2 bits) in SIB1 to allow NES capable UE camping.

- Solution 3: Introduce a NES dedicated IntraFreqExcludedCellList/InterFreqExcludedCellList in SIB3/4.

We compare these 3 solutions in table.1. Please note that solution 2 has two variants (solution 2a and solution 2b), depending on whether need to differentiate NES-capable/NPN-incapable UE and NES-capable/NPN-capable UE. This is because the existing field cellReservedForOtherUse is used to bar NPN incapable UE:

Observation 2: Solution 2 may need to differentiate NES-capable/NPN-incapable UE and NES-capable/NPN-capable UE. This is because the existing field cellReservedForOtherUse is used to bar NPN incapable UE.

|  | ASN.1 change | Spec changes on Rel-18 NES capable UE behaviour | Applicable scenario | Notes |
| --- | --- | --- | --- | --- |
| Solution 1 | Add 1bit in SIB1 for all NES-capable UEs (e.g. cellBarredNES) | - Always ignore legacy cellBarred bit in MIB - Determine whether the cell is barred via new cellBarredNES bit in SIB1 | Both cell selection and cell reselection | - Similar solution as NTN/Redcap. - Compared with solution 2, higher Rel-18 NES UE power consumption because it always needs to decode SIB1 even if the target cell bars all UEs. |
| Solution 2a | Add 1bit in SIB1 for all NES-capable UEs (e.g. cellBarredNESUE) | Define new UE behaviour for below 4 combinations of cell reservation fields: a) If (cellReservedForOtherUse=F, cellReservedForFutureUse=F, cellBarredNESUE=F) All UEs can camp. b) If (cellReservedForOtherUse=T, cellReservedForFutureUse=F, cellBarredNESUE=F) NPN-capable UEs and NES-capable UEs (irrespective of NPN-capable) can camp. c) If (cellReservedForOtherUse=T, cellReservedForFutureUse=T, cellBarredNESUE=F) NES-capable UE (irrespective of NPN-capable) can camp. d) If (cellReservedForOtherUse=T, cellReservedForFutureUse=T, cellBarredNESUE=T) All UEs are barred. | Both cell selection and cell reselection | - Compared with solution 1, lower Rel-18 NES UE power consumption if the target cell bars all UEs (i.e. the UE doesn't need to decode SIB1 if the target cell's cellBarred is true). - Compared with solution 2b, it can't differentiate NES-capable/NPN-incapable UE and NES-capable/NPN-capable UE. |
| Solution 2b | Add 2bit in SIB1 for NES-capable/NPN-incapable UEs (e.g. cellBarredNESnonNPNUE) and NES-capable/NPN-capable UEs (e.g. cellBarredNESNPNUE) | Define new UE behaviour for below 6 combinations of cell reservation fields: a) If (cellReservedForOtherUse=F, cellReservedForFutureUse=F, cellBarredNESnonNPNUE =F, cellBarredNESNPNUE=F) All UEs can camp. b) If (cellReservedForOtherUse=T, cellReservedForFutureUse=F, cellBarredNESnonNPNUE =F, cellBarredNESNPNUE=F) NPN-capable UEs and NES-capable UEs (irrespective of NPN-capable) can camp. c) If (cellReservedForOtherUse=T, cellReservedForFutureUse=T, cellBarredNESnonNPNUE =F, cellBarredNESNPNUE=F) NES-capable UE can camp. d) If (cellReservedForOtherUse=T, cellReservedForFutureUse=T, cellBarredNESnonNPNUE =T, cellBarredNESNPNUE=F) NES-capable / NPN-capable UE can camp. e) If (cellReservedForOtherUse=T, cellReservedForFutureUse=T, cellBarredNESnonNPNUE =F, cellBarredNESNPNUE=T) NES-capable / NPN-incapable UE can camp. f) If (cellReservedForOtherUse=T, cellReservedForFutureUse=T, cellBarredNESnonNPNUE =T, cellBarredNESNPNUE=T) All UEs are barred. | Both cell selection and cell reselection | - Compared with solution 1, lower Rel-18 NES UE power consumption if the target cell bars all UEs (i.e. the UE doesn't need to decode SIB1 if the target cell's cellBarred is true). - Compared with solution 2a, it can differentiate NES-capable/NPN-incapable UE and NES-capable/NPN-capable UE (i.e. case d and e), but it makes the UE behaviour complex. |
| Solution 3 | Add two NES cell lists in SIB3 and SIB4, respectively (IntraFreqExcludedCellList /InterFreqExcludedCellList) | In cell reselection, ignore two legacy cell list and apply the NES dedicate one if configured | Only cell reselection |  |

Table.1 Comparison of legacy UE barring solutions

The key observations from Table.1:

- Solution 3 can only apply to cell reselection, but solution 1 and solution 2a/2b can apply to both cell selection and cell reselection.

- Compare Solution 1 with solution 2a/2b:

  - Pros:

    - Less spec impact (especially compared with solution 2b).

    - Similar to legacy UE barring solution of NTN/Redcap.

  - Cons

    - It may increase Rel-18 NES capable UE's power consumption during cell (re)selection because it always needs to decode neighbour cells' SIB1 even if the target cell's cellBarred=true.

    - Compared with Solution 2b, it can't have separate camping restrictions for NES-capable/NPN-incapable UE and NES-capable/NPN-capable UE.

- Compare solution 2a with 2b

  - Pros:

    - Simpler than solution 2b. The spec impact / UE impact are similar to solution 1.

  - Cons

    - It can't have separate camping restrictions for NES-capable/NPN-incapable UE and NES-capable/NPN-capable UE.

Observation 3: Solution 3 can only apply to cell reselection, but solution 1 and solution 2 can apply to both cell selection and cell reselection.

Observation 4: Compared with solution 2a/2b, solution 1:

- Pros: less spec impact (especially compared with solution 2b). Similar to NTN/Redcap barring solution.

- Cons:

  - It may increase Rel-18 NES capable UE's power consumption during cell (re)selection because it always needs to decode neighbour cells' SIB1 even if the target cell's cellBarred=true.

  - Compared with Solution 2b, it can't have separate camping restrictions for NES-capable/NPN-incapable UE and NES-capable/NPN-capable UE.

Observation 5: Compared with solution 2b, solution 2a:

- Pros: Simpler than solution 2b. The spec impact / UE impact are similar to solution 1.

- Cons: it can't have separate camping restrictions for NES-capable/NPN-incapable UE and NES-capable/NPN-capable UE.

According to observation 3, we prefer to specify a single solution applicable to both cell selection and cell reselection. Thus, we would like to preclude solution 3.

Proposal 1: RAN2 don't pursue the solution to introduce NES dedicated IntraFreqExcludedCellList / InterFreqExcludedCellList in SIB3/4.

Then, based on Observation 3 and Observation 4, we think that RAN2 need to first determine whether we have requirement to differentiate NES-capable/NPN-incapable UE and NES-capable/NPN-capable UE. If yes, we think solution 2b can be introduced. Otherwise, we prefer to introduce solution 1 to minimize specification impacts.

Proposal 2: Before down-selection of UE barring solution, RAN2 discuss whether need to have separate camping restrictions for NES-capable/NPN-incapable UE and NES-capable/NPN-capable UE.

Proposal 3a: If RAN2 conclude the differentiation requirement in Proposal 2, introduce below two new cell reservations bits in SIB1 to work with existing fields cellReservedForOtherUse and cellReservedForFutureUse:

- cellBarredNESnonNPNUE: it bars NES-capable/NPN-incapable UEs if set to be "true"

- cellBarredNESNPNUE: it bars NES-capable/NPN-capable UEs if set to be "true"

Proposal 3b: If RAN2 conclude no need for differentiation requirement in Proposal 2, introduce NTN similar solution, i.e., introduce a 1bit cellBarredNES in SIB1 and NES capable UE ignores legacy cellBarred bit in MIB.

# 3 Conclusion

In this contribution, we discuss legacy UE barring mechanism. Our observations are:

Observation 1: Study item identified 3 solutions of legacy UE barring:

- Solution 1: Set cellBarred in MIB as "barred" and introduce a new barring indication in SIB1 to allow NES capable UE camping (e.g. 1bit cellBarredNES set as "notBarred").

- Solution 2: Set cellReservedForOtherUse and cellReservedForFutureUse in SIB1 and introduce a new barring indication (1 or 2 bits) in SIB1 to allow NES capable UE camping.

- Solution 3: Introduce a NES dedicated IntraFreqExcludedCellList/InterFreqExcludedCellList in SIB3/4.

Observation 2: Solution 2 may need to differentiate NES-capable/NPN-incapable UE and NES-capable/NPN-capable UE. This is because the existing field cellReservedForOtherUse is used to bar NPN incapable UE.

Observation 3: Solution 3 can only apply to cell reselection, but solution 1 and solution 2 can apply to both cell selection and cell reselection.

Observation 4: Compared with solution 2a/2b, solution 1:

- Pros: less spec impact (especially compared with solution 2b). Similar to NTN UE barring solution.

- Cons:

  - It may increase Rel-18 NES capable UE's power consumption during cell (re)selection because it always needs to decode neighbour cells' SIB1 even if the target cell's cellBarred=true.

  - Compared with Solution 2b, it can't have separate camping restrictions for NES-capable/NPN-incapable UE and NES-capable/NPN-capable UE.

Observation 5: Compared with solution 2b, solution 2a:

- Pros: Simpler than solution 2b. The spec impact / UE impact are similar to solution 1.

- Cons: it can't have separate camping restrictions for NES-capable/NPN-incapable UE and NES-capable/NPN-capable UE.

Based on observations, our proposals can be found below.

Proposal 1: RAN2 don't pursue the solution to introduce NES dedicated IntraFreqExcludedCellList / InterFreqExcludedCellList in SIB3/4.

Proposal 2: Before down-selection of UE barring solution, RAN2 discuss whether need to have separate camping restrictions for NES-capable/NPN-incapable UE and NES-capable/NPN-capable UE.

Proposal 3a: If RAN2 conclude the differentiation requirement in Proposal 2, introduce below two new cell reservations bits in SIB1 to work with existing fields cellReservedForOtherUse and cellReservedForFutureUse:

- cellBarredNESnonNPNUE: it bars NES-capable/NPN-incapable UEs if set to be "true"

- cellBarredNESNPNUE: it bars NES-capable/NPN-capable UEs if set to be "true"

Proposal 3b: If RAN2 conclude no need for differentiation requirement in Proposal 2, introduce NTN similar solution, i.e., introduce a 1bit cellBarredNES in SIB1 and NES capable UE ignores legacy cellBarred bit in MIB

# 4 References

[1] RP-223540, New WID: Network energy savings for NR, Huawei

[2] RAN2#122, Chair Notes

[3] TR 38.864-v1.0.0, Study on Network energy savings for NR, 2022.