3GPP TSG-RAN WG2 #123	R2-2307903

Toulouse, France, August 21 – 25, 2023

Agenda Item:	7.15.3

Source:	Ericsson, Xiaomi, Nokia, Nokia Shanghai Bell, vivo

Title:	LCP enhancement for COT sharing

Document for:	Discussion, Decision

# 1	Introduction

Studying and specifying support of sidelink on unlicensed spectrum (i.e., SL-U) is one of the objectives of SL evolution in Rel.18. In this paper, we continue the discussion from RAN2#122 on the enhanced LCP procedure.

# 2	Discussion

## 2.1 Background

In RAN2#121bis, RAN2 has made further agreements on consistent LBT failure handling as shown in the below

Agreement

SL C-LBT failure is declared per RB-set

UE uses the MAC CE to report consistent LBT failure to the gNB

Uu MAC CE indicates RB set(s) where C-LBT failure happens.

UE triggers SL RLF for all UC connections when UE has triggered consistent SL LBT failure in all RB sets.

In RAN2#121bis, RAN2 has made further agreements on U2U COT sharing and LCP as shown in the below

Agreement

RAN2 will wait for more conclusion from RAN1 on the assistance information for COT sharing.

If the resource to be used is within a shared COT, and if PDU not generated before COT arrival, and there is data in buffer satisfying COT requirement, at least enhanced LCP should be allowed. FFS on the condition for UE to use enhanced LCP. FFS on spec impact.

If a UE decides to use the resource in a shared COT, and when enhanced LCP is decided to be used, for destination selection step in enhanced LCP, at least further restrict the destinations to be the candidates allowed by the COT (as defined by RAN1).

In RAN#122, RAN2 has made the below work assumption regarding enhanced LCP procedure

Agreements on SL enhanced LCP

1: 	Working assumption: For shared COT, CAPC restriction is applicable to enhanced LCP according RAN1 agreement on CAPC requirement.

## 2.2 Discussion

The existing LCP procedure contains two steps:

Step 1: Selection of destination.

Step 2: Selection of logical channel(s) for the selected destination.

In order to support the enhanced LCP for COT sharing purpose, Step 1 will be affected. In the enhanced Step 1, the UE intended to share the COT needs to prioritize a destination indicated by the COT information over other destinations. Details of the enhancement will be up to RAN2 depending on further RAN1 progress.

In the enhanced LCP procedure, how to select the destination will be up to RAN2 depending on  further RAN1 progress.

Regarding whether Step 2 needs to be enhanced, companies have diverse views. RAN2 has made the below work assumption in RAN2#122,

Agreements on SL enhanced LCP

1: 	Working assumption: For shared COT, CAPC restriction is applicable to enhanced LCP according RAN1 agreement on CAPC requirement.

In our view, the further enhancement for Step 2, i.e., exclude the LCHs in the LCP whose CAPC values are higher than the CAPC value in the COT information, is not well motivated due to the below issues

- in the WID, it has been captured that,

- Channel access mechanisms from NR-U shall be reused for sidelink unlicensed operation

  - Assess the applicability of sidelink resource reservation from Rel-16/Rel-17 to sidelink unlicensed operation within the boundaries of unlicensed channel access mechanism and operation

    - No specific enhancements for Rel-17 resource allocation mechanisms

    - If the existing NR-U channel access framework does not support the required SL-U functionality, WGs will make appropriate recommendations for RAN approval.

The WID says clearly that the NR-U rules of channel access shall be reused for SL-U. There is no clear motivation to break this rule.

- There is no exclusion of LCHs in the LCP procedure for NR-U. In an area containing both NR-U UEs and SL-U UEs sharing the same unlicensed band, if an SL-U UE excludes the LCHs in the LCP whose CAPC values are higher than the CAPC value in the COT information, it would become easier for this SL-U UE to access the channel than NR-U UEs and other SL-U UEs who perform the legacy LCP procedure. This would cause the below issues

  - Misaligned UE behaviors between NR-U UEs and SL-U UEs in terms of COT sharing

  - Misaligned UE behaviors between SL-U UEs performing the enhanced LCP procedure and SL-U UE performing the legacy LCP procedure.

The above issues would result in that SL-U UEs performing the enhanced LCP procedure have more opportunities to grasp the channels, than NR-U UEs and other SL-U UEs performing the legacy LCP procedure. Specifically, SL-U UEs performing the enhanced LCP procedure towards SL UEs with higher CAPC priority have more opportunities to grasp the channels, than towards SL UEs with lower CAPC priority data. As a result, the lower CAPC priority UEs may be de-prioritised to such an extension that they will not receive the data.

It is not aligned with the WID if UE excludes LCHs according to the CAPC value in the enhanced LCP procedure.

Exclusion of LCHs in Step 2 would lead to misaligned UE behaviors between SL-U UEs and NR-U UEs, and between SL-U UEs performing the enhanced LCP procedure and SL-U UEs performing the legacy LCP procedure.

Based on the above observations, it is necessary to further discuss the above working assumption, rather directly confirm the working assumption.

Meanwhile, we understand the concerns and intentions of proponent companies for this working assumption. In order to not delay the discussion, we suggest RAN2 to confirm the above working assumption with a condition that the network can configure whether Mode 1 SL-U UEs are allowed to apply the enhanced LCP. This is because that the network has full knowledge on whether there are NR-U UEs competing the same unlicensed channels with SL-U UEs, based on that the network can decide when to enable the enhanced LCP procedures for SL-U UEs. For Mode 2 SL-U UEs, the UEs can perform the enhanced LCP procedure when the conditions (e.g., the data meets the COT requirements) are met. Therefore, we make the below proposals for Mode 1 UEs and Mode 2 UEs respectively.

RAN2 to agree that:

  - For shared COT, CAPC restriction is applicable to the enhanced LCP procedure, i.e., UE should exclude LCHs whose CAPC values are higher than the CAPC value in the COT information.

  - The network configures whether Mode 1 UEs are allowed to apply the enhanced LCP procedure.

For mode 2 UEs, it is up to UE implementation whether to apply the enhanced LCP procedure.

# 3 Conclusion

In the previous sections we made the following observations:

Observation 1	In the enhanced LCP procedure, how to select the destination will be up to RAN2 depending on  further RAN1 progress.

Observation 2	It is not aligned with the WID if UE excludes LCHs according to the CAPC value in the enhanced LCP procedure.

Observation 3	Exclusion of LCHs in Step 2 would lead to misaligned UE behaviors between SL-U UEs and NR-U UEs, and between SL-U UEs performing the enhanced LCP procedure and SL-U UEs performing the legacy LCP procedure.

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN2 to agree that:

a.	For shared COT, CAPC restriction is applicable to the enhanced LCP procedure, i.e., UE should exclude LCHs whose CAPC values are higher than the CAPC value in the COT information.

b.	The network configures whether Mode 1 UEs are allowed to apply the enhanced LCP procedure.

Proposal 2	For mode 2 UEs, it is up to UE implementation whether to apply the enhanced LCP procedure.