3GPP TSG-RAN WG2 Meeting #123                      	                                    R2-2308307

Toulouse, France, August 21-25, 2023

Agenda Item:	7.19.2

Source: 	CMCC

Title:  	Discussion on eDRX in RRC_INACTIVE

WID/SID:	         NR_redcap_enh-Core

Document for:	Discussion and Decision

Introduction

In the last RAN2 meeting, the following items have been achieved:

UE can support Rel-18 INACTIVE eDRX (which comprises eDRX cycles and PTWs), even if it doesn’t support Rel-17 INACTIVE eDRX. A cell can allow Rel-18 INACTIVE eDRX (which comprises eDRX cycles and PTWs), even if it doesn’t allow Rel-17 INACTIVE eDRX, but the cell must allow IDLE eDRX. We confirm the working assumption: UEs configured with Rel-18 enhanced INACTIVE eDRX should fall back to use Rel-17 INACTIVE eDRX (if capable and configured with Rel-17 INACTIVE eDRX) if the Rel-18 enhanced INACTIVE eDRX is not allowed but the Rel-17 INACTIVE eDRX is allowed by the current cell. gNB has the possibility to configure both Rel-17 INACTIVE eDRX and Rel-18 INACTIVE eDRX, allowing the UE to fall back to use Rel-17 INACTIVE eDRX. A UE configured with Rel-18 INACTIVE eDRX will fallback to use INACTIVE RAN DRX if it is either not configured with Rel-17 INACTIVE eDRX or the cell does not allow Rel-18 INACTIVE eDRX and Rel-17 INACTIVE eDRX.

Based on the discussion progress, we provide our view on the extended DRX design for the case eDRX cycle in RRC_INACTIVE >10.24s in this paper.

Discussion

From the RAN2 #121bis meeting, the following agreements relative to PTW in INACTIVE eDRX cycle was confirmed:

Use the same UE_ID_H as IDLE eDRX for calculating the PH for RAN paging when INACTIVE eDRX is longer than 10.24s. Use TeDRX_RAN instead of TeDRX_CN to calculate the PH for RAN paging when TeDRX_RAN is longer than 10.24s. For the overlapping PH, RAN PTW starting location is determined based on CN eDRX cycle. For the non-overlapping PH, PTW starting location for RAN PTW is determined based on the CN eDRX cycle. In an overlapped or non-overlapped PH: Within RAN PTW and outside CN PTW, T = RAN configured DRX cycle If this is even a valid case (we will decide later): In an overlapped PH: Within CN PTW and outside RAN PTW, T = min {CN configured DRX cycle, default paging cycle broadcast in system information}. In an overlapped PH: Within both CN PTW and RAN PTW, T = min {CN configured DRX cycle, RAN configured DRX cycle, default paging cycle broadcast in system information}.

The solutions on the PTW for eDRX cycle in RRC_INACTIVE >10.24s need to be further discussed for eRedCap UE.

In [1], the PTW is determined by a Paging Hyperframe (PH), a starting position within the PH (PTW_start) and an ending position (PTW_end).  The following formula was used to calculate the PH, PTW_start and PTW_end for the eDRX in RRC_IDLE in R17:

PH, PTW_start and PTW_end are given by the following formula: The PH for CN is the H-SFN satisfying the following equations: H-SFN mod TeDRX_CN= (UE_ID_H mod TeDRX_CN), where - UE_ID_H: 13 most significant bits of the Hashed ID. - TeDRX_CN: UE-specific eDRX cycle in Hyper-frames, (TeDRX_CN = 2, …, 1024 Hyper-frames) configured by upper layers. PTW_start denotes the first radio frame of the PH that is part of the PTW and has SFN satisfying the following equation: SFN = 128 * ieDRX_CN, where - ieDRX_CN = floor(UE_ID_H /TeDRX_CN) mod 8 PTW_end is the last radio frame of the PTW and has SFN satisfying the following equation: SFN = (PTW_start + L*100 - 1) mod 1024, where - L = Paging Time Window (PTW) length (in seconds) configured by upper layers

From the agreement of RAN2 #121bis meeting, the same formula of PH/PTW in R17 could be reused for enhanced INACTIVE eDRX. Because the eDRX cycle of the CN and RAN paging may be different, the PTW starting SFN determined by the formula above may be different when RAN and CN paging coincide in the same PH.  Considering the power saving issue, it is agreed that the PTW starting locations should be the same when RAN and CN paging coincide in the same PH. The PTW starting locations for INACTIVE eDRX cycles longer than 10.24s is designed and determined by CN eDRX cycle. The PTW length of CN eDRX is already negotiated by RAN and CN, and configured by CN to UE.  There is no need to restrict the PTW length of CN eDRX and RAN eDRX.

Proposal 1: The PTW length of CN eDRX is already negotiated by RAN and CN, and configured by CN to UE.  There is no need to restrict the PTW length of CN eDRX and RAN eDRX.

If the PTW length of CN eDRX and RAN eDRX is not restricted, the PTW length in INACTIVE eDRX cycle can be longer or shorter than or equal to the CN PTW length when RAN and CN paging coincide in the same PH in the case that IDLE eDRX cycle >10.24s and INACTIVE eDRX cycle>10.24s. UE should monitor both the paging within the PTW of IDLE and INACTIVE eDRX. In the case of RAN and CN paging coincide in the same PH: Within CN PTW and outside RAN PTW, T is determined by the shortest of the CN configured DRX value (s), if configured by RRC and/or upper layers, and a default DRX value broadcast in system information.

Proposal 2: In the case of RAN and CN paging coincide in the same PH: Within CN PTW and outside RAN PTW, T is determined by the shortest of the CN configured DRX value (s), if configured by RRC and/or upper layers, and a default DRX value broadcast in system information.

Conclusion

In this contribution, we discussed eDRX cycle in RRC_INACTIVE. Following are our proposals:

Proposal 1: The PTW length of CN eDRX is already negotiated by RAN and CN, and configured by CN to UE.  There is no need to restrict the PTW length of CN eDRX and RAN eDRX.

Proposal 2: In the case of RAN and CN paging coincide in the same PH: Within CN PTW and outside RAN PTW, T is determined by the shortest of the CN configured DRX value (s), if configured by RRC and/or upper layers, and a default DRX value broadcast in system information.

Reference

[1] 3GPP TS 38.304 V17.3.0 User Equipment (UE) procedures in idle mode and RRC Inactive state (Release 17)