3GPP TSG-RAN2 Meeting #123-bis	R2-2311310

# Xiamen, China, 9th Oct 2023 - 13th Oct 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.331 | CR | 4962 | rev | 1 | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on ul-SyncValidityDuration in SIB31 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation, Sanechips |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NBIOT_eMTC_NTN-Core |  |  |  |  |  | Date: |  |  | 2023-10-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The ul-SyncValidityDuration is validity duration of the satellite ephemeris data and common TA parameters. Hence, only if the value of the ephemeris data, common TA or epochTime change, the value of ul-SyncValidityDuration needs to be updated. Such restriction has been mentioned in NR NTN but missed in IoT NTN. |  |  |  |  |  |  |  |  |
| Summary of change: |  | To add the restrictions for change of ul-SyncValidityDuration in the description of ul-SyncValidityDuration in SIB31. Impact Analysis Impacted functionality: The change of ul-SyncValidityDuration Inter-operability: No inter-operability issue can be seen. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It may cause unreasonable configuration. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

6.3.1	System information blocks

<skip unrelated parts>

–	SystemInformationBlockType31

The IE SystemInformationBlockType31 contains satellite assistance information for the serving cell. SystemInformationBlockType31 is only signalled in a NTN cell.

SystemInformationBlockType31 information element

-- ASN1START

SystemInformationBlockType31-r17 ::= SEQUENCE {

servingSatelliteInfo-r17		ServingSatelliteInfo-r17,

lateNonCriticalExtension		OCTET STRING					OPTIONAL,

...

}

ServingSatelliteInfo-r17 ::=	SEQUENCE {

ephemerisInfo-r17			CHOICE {

stateVectors				EphemerisStateVectors-r17,

orbitalParameters			EphemerisOrbitalParameters-r17

},

nta-CommonParameters-17			SEQUENCE {

nta-Common-r17					INTEGER (0..8316827)		OPTIONAL,	-- Need OP

nta-CommonDrift-r17				INTEGER (-261935..261935)	OPTIONAL,	-- Need OP

nta-CommonDriftVariation-r17	INTEGER (0..29479)			OPTIONAL	-- Need OP

},

ul-SyncValidityDuration-r17		ENUMERATED {s5, s10, s15, s20, s25, s30, s35, s40,

s45, s50, s55, s60, s120, s180, s240, s900},

epochTime-r17					SEQUENCE {

startSFN-r17					INTEGER (0..1023),

startSubFrame-r17				INTEGER (0..9)

}																OPTIONAL,	-- Need OP

k-Offset-r17					INTEGER (0..1023),

k-Mac-r17						INTEGER (1..512)				OPTIONAL,	-- Need OP

...

}

-- ASN1STOP

SystemInformationBlockType31 field descriptions

epochTime Epoch time of the satellite ephemeris data and common TA parameters, see TS 36.213 [23]. The reference point for epoch time of the serving satellite ephemeris and Common TA parameters is the uplink time synchronization reference point. epochTime is the starting time of a DL subframe indicated by startSFN and startSubframe. For serving cell, the startSFN indicates the current SFN or the next upcoming SFN after the frame where the message indicating the epochTime is received. If the field is absent, the UE uses the starting time of the DL subframe corresponding to the end of the SI window during which the SI message carrying SIB31(-NB) is transmitted. E-UTRAN always includes epochTime when SIB31(-NB) is provided through dedicated signalling. In case of handover or conditional handover, this field is based on the timing of the target cell, i.e. the startSFN and startSubFrame number indicated in this field refers to the SFN and sub-frame of the target cell, and UE considers the target cell epoch time (indicated by the startSFN and startSubFrame in this field) to be the frame nearest to the frame where RRCConnectionReconfiguration message is received.

k-Mac Scheduling offset used when downlink and uplink frame timing are not aligned at the eNB, see TS 36.213 [23]. Unit in ms. If the field if absent, the UE uses the (default) value of 0.

k-Offset Scheduling offset used in the timing relationships in NTN, see TS 36.213 [23]. Unit in ms.

nta-Common Network-controlled common TA, see TS 36.213 [23]. Unit of μs. Step of 32.55208 ×10-3 μs. Actual value = field value * 32.55208 ×10-3. If the field is absent, the UE uses the (default) value of 0.

nta-CommonDrift Drift rate of the common TA, see TS 36.213 [23]. Unit of μs/s. Step of 0.2 ×10-3 μs/s. Actual value = field value * 0.2 ×10-3. If the field is absent, the UE uses the (default) value of 0.

nta-CommonDriftVariation Drift rate variation of the common TA, see TS 36.213 [23]. Unit of μs/s2. Step of 0.2 ×10-4 μs/s2. Actual value = field value * 0.2 ×10-4. If the field is absent, the UE uses the (default) value of 0.

orbitalParameters Instantaneous values of the satellite orbital parameters. The signalled values are only valid for the duration as defined by ul-SyncValidityDuration and epochTime.

stateVectors Instantaneous values of the satellite state vectors. The signalled values are only valid for the duration as defined by ul-SyncValidityDuration and epochTime.

ul-SyncValidityDuration Validity duration of the satellite ephemeris data and common TA parameters, i.e. maximum time duration (from epochTime) during which the UE can apply the satellite ephemeris without acquiring new satellite ephemeris, see TS 36.213 [23]. Unit in second. Value s5 corresponds to 5 seconds, value s10 corresponds to 10 seconds and so on. The ul-SyncValidityDuration is only updated when at least one of epochTime, nta-CommonParameters, ephemerisInfo is updated.