3GPP TSG RAN WG2#124	R2-2311720

Chicago, USA, 13th - 17th November 2023

# 3GPP TSG-RAN WG1 Meeting #114bis	R1-2310681

Xiamen, China, October 9th – 13th, 2023

Title:	Reply LS on Data Collection Requirements and Assumptions

Response to:	LS R2-2306906/R1-2306388 on Data Collection Requirements and Assumptions (from RAN2)

Release:	Release 18

Work Item:	Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (FS_NR_AIML_air)

Source:	TSG RAN WG1

To:	TSG RAN WG2

Cc:

Contact person:	Taesang Yoo

taesangy@qti.qualcomm.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

# 1	Overall description

RAN1 thanks RAN2 for the LS on Data Collection Requirements and Assumptions.

Regarding Part A: RAN2 Assumptions on data collection that require RAN1 confirmation, RAN1 has already provided reply in R1-2308730.

Regarding Part B: Aspects of data collection that require RAN1 feedback/inputs, RAN2 asked

Part B: Aspects of data collection that require RAN1 feedback/inputs To facilitate the discussion on data collection in RAN2 for further progress, RAN2 would like RAN1 to provide feedback/inputs on the following essential aspects: - Data content - Typical data size (value or value range) of the identified data content - Reporting type (e.g., periodic, event triggered, other) of the identified data content - Typical latency requirement (value or value range) to transfer the identified data content RAN2 would require RAN1 feedback/inputs on the data collection requirements per LCM purpose (i.e., model training, inference and monitoring) for each (sub)use case, and the LCM sidedness should also be considered. Besides, RAN2 would also like to know to what extent the data would / should be specified (in detail).

This LS reply is meant to capture existing RAN1 agreements/conclusions/observations and discussions for the purpose of replying the RAN2 LS; The LS reply does not serve as additional agreements/conclusions/observations beyond what RAN1 has already agreed/concluded/observed.

Please find RAN1’s reply as follows.

For CSI compression

| LCM purpose | Data content | Typical data size (per data sample) | Typical latency requirement | Notes |
| --- | --- | --- | --- | --- |
| Training | Target CSI | See Notes 1, 2 | Relaxed | This row applies to Type 1, Type 2, and the first or second stage of described procedure of Type 3 separate training. |
|  | CSI Feedback | See Note 3 | Relaxed | This is for dataset delivery for the second stage of described procedure of Type 3 separate training (either from Network side to UE side, or from UE side to Network side) and forward propagation information for Type 2 training. See Note 7 |
|  | Gradients for CSI Feeback | No agreement | Relaxed | This is for backward propagation for Type 2 training See Note 7 |
| Inference | CSI Feedback | See Note 3 | Time-critical | Can use L1 report similar to legacy CSI |
| Monitoring | Reconstructed CSI from NW to UE See Note 6 | No agreement; [expected to be similar to target CSI for monitoring] | Near-real-time | This is called “UE-sided monitoring” in RAN1. |
|  | Calculated performance metrics See Note 6 | See Note 4 | Near-real-time | This is called “UE-sided monitoring” in RAN1. |
|  | Target CSI See Note 6 | See Notes 1, 2 | Near-real-time | This is called “NW-sided monitoring” in RAN1. |

Note 1: Target CSI may be precoding matrix or channel matrix. RAN1’s reply for data size is based on precoding Matrix which has been more widely evaluated than channel matrix.

Note 2: Data size for target CSI depends on the format. There is no agreement on the format or necessary precision of the target CSI. Some examples based on companies’ evaluations are: eType-II format (up to ~1000 bits), eType-II-like format (~ a few 1000 bits), and float32 format (up to ~ 150K bits). The data size may also vary depending on the configuration, and the captured value indicates the order of magnitude of the typical data size per sample as a guideline.

Note 3: There is no agreement on the CSI feedback size. Values in the order of eType II payload size may be assumed (up to ~ 1000 bits) for RAN2 discussion.

Note 4: There is no agreement on the exact metric or reporting format. An example based on companies’ evaluations is: SGCS (10s of bits)

Note 5: There are no agreements on the reporting type.

Note 6: Feasibility and necessity of the monitoring schemes listed in the table are under discussion

Note 7: RAN1 has agreed to deprioritize Type 2 training over the air interface.

For CSI prediction at UE side

| LCM purpose | Data content | Typical data size (per data sample) | Typical latency requirement | Notes |
| --- | --- | --- | --- | --- |
| Training | Target CSI in observation and prediction window | See Notes 1, 2 | Relaxed |  |
| Inference | Predicted CSI feedback (AI/ML output) | See Note 3 | Time-critical | Can use L1 report similar to legacy CSI |
| Monitoring | ground truth (i.e., target CSI) corresponding to predicted CSI See Note 6 | See Notes 1, 2 | Near-real-time |  |
|  | Calculated performance metrics / Performance monitoring output See Note 6 | See Note 5 | Near-real-time |  |

Note 1: Target CSI may be precoding matrix or channel matrix. RAN1’s reply for data size is based on channel matrix which has been more widely evaluated than precoding Matrix.

Note 2: Data size for target CSI depends on the format. There is no agreement on the format or precision of the target CSI. The data size may also vary depending on the configuration, and the captured value indicates the order of magnitude of the typical data size per sample as a guideline. One example based on companies’ evaluations is up to around 1.5Mbits, assuming float 32 and 10 CSI-RS observation instances as input to predict one future CSI instance.

Note 3: There is no agreement on the predicted CSI feedback size. Values in the order of eType II payload size may be assumed (up to ~ 1000 bits) for RAN2 discussion.

Note 4: There are no agreements on the reporting type.

Note 5: There is no agreement on the performance metric or monitoring output details.

Note 6: Feasibility and necessity of the monitoring schemes listed in the table are under discussion.

For Beam management

| LCM purpose | UE-side/NW-side models | Data content | Typical data size (per data sample) | Typical latency requirement | Notes |
| --- | --- | --- | --- | --- | --- |
| Training | UE-side, NW-side | L1-RSRPs and/or beam-IDs | See Note 1 for L1-RSRPs | Relaxed |  |
| Inference | UE-side | Beam prediction results | Small (10s of bits) | Time-critical | RAN1 has agreed to consider L1 signalling for this reporting |
|  | NW-side | L1-RSRPs, and Beam-IDs if needed, for Set B | See Note 1 for L1-RSRPs | Time-critical |  |
| Monitoring | UE-side | Event occurrence and/or calculated performance metrics (from UE to NW) See Note 4 | Small (10s of bits) | Near-real-time |  |
|  | UE-side | L1-RSRP(s) and/or beam-ID(s) See Note 4 | Up to 10 bits, or up to 100 bits, or up to hundreds of bits. See Note 1 for L1-RSRPs | Near-real-time |  |
|  | NW-side | L1-RSRP(s) and/or beam-ID(s) See Note 4 | Up to 10 bits, or up to 100 bits, or up to hundreds of bits. See Note 1 for L1-RSRPs | Near-real-time |  |

Note 1: There is no agreement on the data size of L1-RSRPs for Set A or Set B, but the following typical data size is provided as guidance for RAN2 discussion. Based on existing L1-RSRP reporting methodology, i.e., 7 bits for the strongest beam and 4 bits for the remaining beams, for Set B = 16 as an example, the typical data size would be 67 (hence up to ~100 bits), and for Set A = 128 as an example, the typical data size would be 515 (hence up to ~500 bits) if all beams in Set A were to be collected. For BM Case 2, the data size L1-RSRPs for Set A and Set B represents the data size per predicted future time instance and per history measurement time instance, respectively. Payload size may not be fixed.

Note 2: There are no agreements on the reporting type.

Note 4: Feasibility and necessity of the monitoring schemes listed in the table are under discussion.

Note 5: For BM Case 2, the typical value of the number of history measurement time instance used in evaluations is up to 8 and typical value of the number of predicted future time instance is 1~4.

For positioning

| LCM purpose | Case | Data content | Typical data size (per data sample) | Typical latency requirement | Notes |
| --- | --- | --- | --- | --- | --- |
| Training | All Cases | Measurements (corresponding to model input): timing, power, and/or phase info See Note 2 | Size depends on number of PRS/SRS resources, measurement type (timing, power, and/or phase info) and report format: ~100 bits to 1000s bits per PRS/SRS resource See Note 3 | Relaxed |  |
|  | Direct AI/ML positioning | Label: Location coordinates as model output | 56 to 144 bits See Note 3 | Relaxed |  |
|  | AI/ML assisted positioning | Label: Intermediate positioning measurement (timing info, LOS/NLOS indicator) as model output See Note 2 | 10s bits to 100s bits per PRS/SRS resource See Note 3 | Relaxed |  |
| Inference | 1 | Location coordinates as model output | 56 to 144 bits See Note 3 | See Note 5 |  |
|  | 2a, 3a | Intermediate positioning measurement (timing info, LOS/NLOS indicator) as model output See Note 2 | 10s bits to 100s bits per PRS/SRS resource See Note 3 | See Note 5 |  |
|  | 2b, 3b | Measurements (corresponding to model input): Timing, power, and/or phase info See Note 2 | Size depends on number of PRS/SRS resources, measurement type (timing, power, and/or phase info) and report format: ~100 bits to 1000s bits per PRS/SRS resource See Note 3 | See Note 5 |  |
| Monitoring | All Cases | See Note 8 | See Note 8 | Near-real-time | See Note 6 and 7 |

Note 1: The necessity and feasibility of difference cases (Case1 to Case3b) needs further discussion/conclusion.

Note 2: For measurements as model input, no agreement on measurement types (i.e., time, power, and/or phase) in RAN1 for all cases (i.e., Case1 to Case3b). Measurement types (including their necessity) and sizes/dimension needs to be further discussed. Candidate measurement types discussed/evaluated for model input include CIR (contains timing, power and phase information), PDP (contains timing and power information), DP (contains timing information). For labels (i.e., model output) of AI/ML assisted positioning (Case2a, Case3a), RAN1 identified an initial listing of candidates that provide performance benefits (i.e., timing info, LOS/NLOS indicator). RSRP/RSRPP is for further discussion.

Note 3: The measurement size of one data sample = (measurement data size of one PRS/SRS resource)*(number of PRS/SRS resources needed for model input). The label size of one data sample = (label data size of one PRS/SRS resource)*(number of PRS/SRS resources needed for model output). The quantization and bit representation of time, power, and phase information (including their necessity) still need to be further discussed.  Existing specification allows reporting of up to 64 PRS/SRS resources per frequency layer for one positioning fix. For evaluations, most companies considered up to 18 TRPs. It should be noted that AI/ML positioning is not restricted to work only with maximum of 18 TRPs.

- Example of calculation on a potential lower bound on measurement size per PRS/SRS resource:

  - A potential lower bound on measurement size per PRS/SRS resource can be calculated as follows (assuming timing only for 9 measurements per PRS/SRS resource): 16 + 9*8 = 88 bits. The total lower bound can be 88*N bits, where N is number of PRS/SRS resources used as model input for obtaining a positioning fix. This is based on the assumption of timing info as 16 bits for first arrival and 9 bits for relative timing.

- Example of calculation of a potential upper bound on measurement size per PRS/SRS resource:

  - A potential upper bound on measurement size per PRS/SRS resource can be calculated as follows (assuming timing, power, and phase for 256 measurements per PRS/SRS resource and assuming 8 bit representation of each real number): 2*(8*256) = 4096 bits. The total upper bound can be 4096*N bits, where N is number of PRS/SRS resources used as model input for obtaining a positioning fix.

- For location coordinates (corresponding to model output)

  - The bit representation of location coordinates depends on the type of shape, resolution, and uncertainty used to indicate the location (e.g., ellipsoid point, ellipsoid point with uncertainty circle, high accuracy ellipsoid with uncertainty ellipsoid, etc.) as listed in TS 23.032. The range of bit representation for location coordinates can be 7 bytes to 18 bytes (i.e., 56 to 144 bits). The location information report in existing specifications may contain additional information besides location coordinates (e.g., velocity, location error, integrity info, etc.)

- For intermediate positioning measurement (corresponding to model output):

  - The quantization and bit representation of time, [RSRP/RSRPP], and LOS/NLOS information (including their necessity) as model output still need to be discussed in an appropriate working group. As a reference to existing timing representation in Rel17 [TS 37.355], an example on the label size can be of 21 bits per PRS/SRS resource while assuming model output produces one timing of 21 bits per PRS/SRS resource. The label size can be 21*N bits, where N is number of PRS/SRS resources for which intermediate positioning measurement has been generated. If LOS/NLOS indicator (1 bit per PRS/SRS resource assuming hard value for LOS/NLOS indicator) is included, the label size becomes 22*N bits.

Note 4: No agreement on reporting types (i.e., periodicity, event-triggered/on-demand, etc.).

Note 5: There are no agreements on the reporting latency.

Note 6: RAN1 agreed on an initial listing of entities that can derive the monitoring metric for AI/ML positioning for different cases (Case1 to Case3b):

-1: At least UE derives monitoring metric

-2a: At least UE derives monitoring metric

- LMF (if monitoring based on ground truth)

-3a: At least gNB/TRP derives monitoring metric

- LMF (if monitoring based on ground truth)

-2b and 3b: At least LMF derives monitoring metric

Note 7: No agreement yet on a monitoring decision entity or their mapping to other entities (e.g., entity running the inference, entity deriving the monitoring metric, etc.).

Note 8: RAN1 has studied several types of related statistics where potential request/report of Monitoring related statistics and its necessity are for further discussion.

Common Notes for all sub-use-cases:

- In answering latency requirements, RAN1 used the following descriptions:

  - Relaxed (e.g., minutes, hours, days, or no latency requirement)

  - Near-real-time (e.g., several tens of msecs to a few seconds)

  - Time-critical (e.g., a few msecs)

- In the reply, RAN1 captured the typical data size per each data sample.

- Model training is assumed to be offline training.

- In RAN1’s answer, RAN1 did not list assistance information. RAN1 has informed RAN2 of related conclusions/agreements/observations regarding assistance information in the RAN1 response to Part A.

- There may be other information identified for training not included in the tables. For example, in positioning enhancement, some information has been considered as potential spec impact (e.g., quality indicators, time stamps, RS configuration(s)).

- In this reply for Part B, the term 'NW-side monitoring' is not explicitly used since RAN1’s understanding of the term is not fully aligned with RAN2 terminology. Rather, RAN1 explained directly the data contents for monitoring. It should also be noted that in the RAN1 response to part A, RAN1 used the term ‘NW-sided monitoring’ aligned with RAN2.

- For monitoring, RAN1 provided replies only for near-real-time monitoring. The requirements for data collection for relaxed monitoring, if necessary, can be considered to be similar to offline training requirements.

# 2	Actions

To RAN2

ACTION: 	RAN1 respectively asks RAN2 to take the above information into account in RAN2’s discussions.

# 3	Dates of next TSG RAN WG1 meetings

RAN1#115	13-17  November 2023	Chicago, US