3GPP TSG-RAN WG2 Meeting #125                                                        R2-2400113

Athens, Greece, Feb. 26th – Mar. 1st, 2024

Source:              CATT

Title:	             Discussion on open issues for UAV UE capabilities

Agenda Item:	7.8.4

Document for:	Discussion and Decision

Introduction

In this paper, we discuss the following open issues for NR UAV UE capabilities listed by Rapporteurs.

- To address the following Editor’s Notes from NR UE capability CR (R2-2313640) nr-NS-PmaxListAerial-r18 ENUMERATED {supported} OPTIONAL, -- Editor’s Note: Understanding is that a UE that doesn’t support any frequency band that requires an aerial specific NS value doesn’t need to implement the procedure for aerial specific NS value. Whether indication is needed is still FFS. This is only shown as placeholder. sl-A2X-Service-r18 ENUMERATED {brid, daa, bridAndDAA} OPTIONAL -- Editor’s Note: Granularity of this capability, e.g. per UE/band/FS is still FFS. Depending on the conclusion, this may need to be moved.

Discussion

Issue 1

Issue: Whether nr-NS-PmaxListAerial-r18 is needed?

To comply with out of band emission (OOBE) requirements, NR supports the aerial UE-specific NS values in R18. According to ECC Decision (22)07 as below, the need for OOBE limits for aerial UE only applies in some frequency bands, i.e., 1710-1785 MHz, 2500-2570 MHz / 2570-2620 MHz.

And RAN2 “Understanding is that a UE that doesn’t support any frequency band that requires an aerial specific NS value doesn’t need to implement the procedure for aerial specific NS value.”

The first discussion point is whether a UE capability is needed. Since it’s possible that a UE operating in other places than Europe may not support this OOBE requirements, it’s necessary to let network know whether UE has this capability. For example, in case a handover may happen between a CEPT country and a non-CEPT country, a suitable handover decision could be made accordingly. Or a UE not supporting this OOBE requirements already got access to a cell where ECC Decision (22)07 applies, a RRCRelease message could be sent to UE to terminate this connection when the gNB knows this UE can’t comply with the OOBE requirements.

The second discussion point is about the granularity of this UE capability. Since according to current ECC Decision (22)07, the affected bands are only n3, n7 and n38 as specified in TS 38.101-1, the corresponding UE capability can be indicated in a per-band manner, i.e., defined in IE BandNR. A TP is provided in Annex.

Table 5.2-1: NR operating bands in FR1

| NR operating band | Uplink (UL) operating band BS receive / UE transmit FUL_low – FUL_high | Downlink (DL) operating band BS transmit / UE receive FDL_low – FDL_high | Duplex Mode |
| --- | --- | --- | --- |
| n3 | 1710 MHz – 1785 MHz | 1805 MHz – 1880 MHz | FDD |
| n7 | 2500 MHz – 2570 MHz | 2620 MHz – 2690 MHz | FDD |
| n38 | 2570 MHz – 2620 MHz | 2570 MHz – 2620 MHz | TDD |

Proposal 1: a per band UE capability nr-NS-PmaxListAerial-r18 is defined in IE BandNR for the support of aerial specific NS value.

Issue 2

Issue: Granularity of sl-A2X-Service-r18

According to current R18 TS 38.331 as below, sl-A2X-Service-r18 is a per UE capability which indicates whether UE supports A2X service(s) which include BRID, DAA or both.

AerialParameters-r18 ::=                      SEQUENCE {

<omitted part>

-- Support of A2X service(s) using PC5 Sidelink and dedicated resource pool for A2X service(s)

sl-A2X-Service-r18                            ENUMERATED {brid, daa, bridAndDAA}                   OPTIONAL,

-- Editor's Note: Granularity of this capability, e.g. per UE/band/FS is still FFS. Depending on the conclusion, this may need to

-- be moved.

...

}

And the corresponding field description is as follows:

| sl-A2X-Service-r18 Indicates whether the UE supports A2X service(s) which include BRID, DAA or both using A2X communication as specified in TS 38.331 [9]. This field also indicates whether the UE supports the dedicated resource pools as specified in TS 38.331 for the corresponding A2X service(s). | UE | No | No | No |
| --- | --- | --- | --- | --- |

In our view, since it’s related to service level capability, it’s sufficient to make it a per UE capability. Considering this is on top of NR side link, a UE supporting this feature shall also support NR sidelink. A TP is also provided in Annex.

Proposal 2: clarify in TS 38.306 that a UE supporting sl-A2X-Service-r18 shall also supports NR sidelink.

Since there are similar issues for LTE UAV, if P1 and P2 can be agreed, the same handling can be adopted for LTE UAV capabilities, i.e., per band UE capability multiNS-PmaxAerial-r18 is defined, and a UE supporting sl-A2X-Service-r18 shall also support LTE sidelink.

Proposal 3: if P1 and P2 can be agreed, the same handling can be adopted for LTE UAV capabilities, i.e., per band UE capability multiNS-PmaxAerial-r18 is defined, and clarify that a UE supporting sl-A2X-Service-r18 shall also support LTE sidelink.

Conclusion

In this paper, we discuss the open issues for UAV UE capabilities, and we propose:

Proposal 1: a per band UE capability nr-NS-PmaxListAerial-r18 is defined in IE BandNR for the support of aerial specific NS value.

Proposal 2: clarify in TS 38.306 that a UE supporting sl-A2X-Service-r18 shall also supports NR sidelink.

Proposal 3: if P1 and P2 can be agreed, the same handling can be adopted for LTE UAV capabilities, i.e., per band UE capability multiNS-PmaxAerial-r18 is defined, and clarify that a UE supporting sl-A2X-Service-r18 shall also support LTE sidelink.

Annex

TP for 38.331

#### –	AerialParameters

The IE AerialParameters is used to convey the capabilities supported by the UE for aerial operation.

AerialParameters information element

-- ASN1START

-- TAG-AERIALPARAMETERS-START

AerialParameters-r18 ::=                      SEQUENCE {

-- Support of Aerial UE features

aerialUE-Capability-r18                       ENUMERATED {supported}                               OPTIONAL,

-- Support of altitude measurement and event H1/H2-triggered reporting

altitudeMeas-r18                              ENUMERATED {supported}                               OPTIONAL,

-- Support of altitude based measurement configuration of SSB-ToMeasure

altitudeBasedSSB-ToMeasure-r18                ENUMERATED {supported}                               OPTIONAL,

-- Support of events A3H1, A3H2, A4H1, A4H2, A5H1, A5H2

eventAxHy-r18                                 ENUMERATED {supported}                               OPTIONAL,

-- Support of flight path reporting

flightPathReporting-r18                       ENUMERATED {supported}                               OPTIONAL,

-- Support of flight path availability indication via UAI

flightPathAvailabilityIndicationUAI-r18       ENUMERATED {supported}                               OPTIONAL,

-- Support of numberOfTriggeringCells for eventA3, eventA4, and eventA5, and additionally, if the UE supports eventAxHy-r18,

-- support of numberOfTriggeringCells for eventA3H1, eventA3H2, eventA4H1, eventA4H2, eventA5H1, and eventA5H2

multipleCellsMeasExtension-r18                ENUMERATED {supported}                               OPTIONAL,

Support of reporting only the measurement report corresponding to the event with the smallest value between the

-- altitude of the UAV and the altitude threshold for which the altitude-related entering condition e.g. A3H1-2 is satisfied, when

-- multiple events of the same type (Hx or AxHy) for the same MO (for AxHy) are triggered simultaneously.

simulMultiTriggerSingleMeasReport-r18         ENUMERATED {supported}                               OPTIONAL,

-- Support of A2X service(s) using PC5 Sidelink and dedicated resource pool for A2X service(s)

sl-A2X-Service-r18                            ENUMERATED {brid, daa, bridAndDAA}                   OPTIONAL,

-- Editor's Note: Granularity of this capability, e.g. per UE/band/FS is still FFS. Depending on the conclusion, this may need to

-- be moved.

...

}

-- TAG-AERIALPARAMETERS-STOP

-- ASN1STOP

BandNR ::=                          SEQUENCE {

<omitted part>

[[

nr-NS-PmaxListAerial-r18                      ENUMERATED {supported}                               OPTIONAL

]]

<omitted part>

}

TP for 38.306

- 4.2.7.2	BandNR parameters

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| nr-NS-PmaxListAerial-r18 Indicates whether the UE supports aerial specific NS value. | Band | No | N/A | N/A |

- 4.2.24	Aerial UE Parameters

| sl-A2X-Service-r18 Indicates whether the UE supports A2X service(s) which include BRID, DAA or both using A2X communication as specified in TS 38.331 [9]. This field also indicates whether the UE supports the dedicated resource pools as specified in TS 38.331 for the corresponding A2X service(s). A UE supporting this feature shall also support NR sidelink. | UE | No | No | No |
| --- | --- | --- | --- | --- |