3GPP TSG-RAN WG2 #125	R2-2400236

Athens, Greece, February 2024

Agenda Item:	7.25.2

Source:	OPPO

Title:	Left issue on Tx Switching

Document for:	Discussion, Decision

Introduction

This is to discuss the following left issue

•	Supporting the advanced capability of the switching period can be improved to min {max(Tswitch_A-C, Tswitch_B-D), max(Tswitch_A-D, Tswitch_B-C)} .

Discussion

As triggered by R4-2317609, there is a need to indicate the switching period of

min {max(Tswitch_A-C, Tswitch_B-D), max(Tswitch_A-D, Tswitch_B-C)}

Firstly, based on the existing capability signaling

ULTxSwitchingBandPair-r18 ::=                             SEQUENCE {

bandIndexUL1-r18                                           INTEGER(1..maxSimultaneousBands),

bandIndexUL2-r18                                           INTEGER(1..maxSimultaneousBands),

uplinkTxSwitchingOptionForBandPair-r18                     ENUMERATED {switchedUL, dualUL, both},

-- R1 49-X: Supported switching option for each band pair in the band combination for UL Tx switching across more than 2 bands

uplinkTxSwitchingPeriodForBandPair-r18                     SEQUENCE {

switchingPeriodFor2T-r18                                 ENUMERATED {n35us, n140us, n210us}                       OPTIONAL,

switchingPeriodFor1T-r18                                 ENUMERATED {n35us, n140us, n210us}

},

uplinkTxSwitching-DL-Interruption-r18                      BIT STRING (SIZE(1..maxSimultaneousBands))                   OPTIONAL,

uplinkTxSwitchingPeriodUnaffectedBandDualUL-List-r18       SEQUENCE (SIZE (1..maxSimultaneousBands-2-r18)) OF

SwitchingPeriodUnaffectedBandDualUL-r18            OPTIONAL

}

Network can already be aware of the component of

Tswitch_A-C,

Tswitch_B-D,

Tswitch_A-D,

Tswitch_B-C

And thus is able to derive both

min {max(Tswitch_A-C, Tswitch_B-D), max(Tswitch_A-D, Tswitch_B-C)}

max {max(Tswitch_A-C, Tswitch_B-D), max(Tswitch_A-D, Tswitch_B-C)}

And based on

UplinkTxSwitchingAdditionalPeriodDualUL-r18::=            SEQUENCE {

uplinkTxSwitchingBetweenBandPairs-r18                      SEQUENCE {

bandPairIndex1-r18                                         INTEGER(1.. maxULTxSwitchingBandPairs),

anotherBandPairOrBand-r18                                  CHOICE {

bandPairIndex2-r18                                          INTEGER(1.. maxULTxSwitchingBandPairs),

bandIndex-r18                                               INTEGER(1..maxSimultaneousBands)

}

},

switchingAdditionalPeriodDualUL-r18                        ENUMERATED {n35us, n140us, n210us}

}

UE can report a switching period for switching from A+B to C+D, which is different from the default max {max(Tswitch_A-C, Tswitch_B-D), max(Tswitch_A-D, Tswitch_B-C)}.

-	switchingAdditionalPeriodDualUL-r18 indicateds the length of switching period for switching between one band pair indicated by bandPairIndex1-r18 and another band pair indicated by bandPairIndex2-r18 or another band indicated by bandIndex-r18.

I.e., this field can be used

- either when the switching period is larger than the default one, i.e., original reason to introduce this field as requested in R4-2310495

- or when the switching period equals to min {max(Tswitch_A-C, Tswitch_B-D), max(Tswitch_A-D, Tswitch_B-C)}, as requested in R4-2317609

The only left issue is whether to put restriction to the field of switchingAdditionalPeriodDualUL-r18.

- Rely on switchingAdditionalPeriodDualUL-r18 to report min {max(Tswitch_A-C, Tswitch_B-D), max(Tswitch_A-D, Tswitch_B-C)}.

- R2 clarify in 306 that when switchingAdditionalPeriodDualUL-r18 is used to report min {max(Tswitch_A-C, Tswitch_B-D), max(Tswitch_A-D, Tswitch_B-C)}, the default max {max(Tswitch_A-C, Tswitch_B-D), max(Tswitch_A-D, Tswitch_B-C)} is also applicable (but for different switching cases, as stated in R4-2317609).

- R2 clarify in 306 that when switchingAdditionalPeriodDualUL-r18 is used to report a value larger than max(Tswitch_A-D, Tswitch_B-C)}, the default max {max(Tswitch_A-C, Tswitch_B-D), max(Tswitch_A-D, Tswitch_B-C)} is not applicable.

Conclusion

We have the following proposals:

Proposal 1	Rely on switchingAdditionalPeriodDualUL-r18 to report min {max(Tswitch_A-C, Tswitch_B-D), max(Tswitch_A-D, Tswitch_B-C)}.

Proposal 2	R2 clarify in 306 that when switchingAdditionalPeriodDualUL-r18 is used to report min {max(Tswitch_A-C, Tswitch_B-D), max(Tswitch_A-D, Tswitch_B-C)}, the default max {max(Tswitch_A-C, Tswitch_B-D), max(Tswitch_A-D, Tswitch_B-C)} is also applicable (but for different switching cases, as stated in R4-2317609).

Proposal 3	R2 clarify in 306 that when switchingAdditionalPeriodDualUL-r18 is used to report a value larger than max(Tswitch_A-D, Tswitch_B-C)}, the default max {max(Tswitch_A-C, Tswitch_B-D), max(Tswitch_A-D, Tswitch_B-C)} is not applicable.