3GPP TSG-RAN WG2 Meeting #125	R2-2400628

# Athens, Greece, 26th February – 1st March 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.306 | CR | 1876 | rev |  | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Removal of references to unknown RAN4 specification |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Lenovo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ext_to_71GHz-Core |  |  |  |  |  | Date: |  |  | 2024-02-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - In RAN2#117-e the UE capability CRs to 36.306/36.331 were agreed in R2-2204087/R2-2203954 (and approved in RP#95-e in CR package RP-220475) in which a reference to an unknown RAN4 specification has been added (“as specified in TS 38.101-x [xx]”) in the description of below UE capabilities: - eutra-5GC-HO-ToNR-TDD-FR2-r15 - eutra-EPC-HO-ToNR-TDD-FR2-r15 - ims-VoiceOverNR-FR2-r15 - ce-EUTRA-5GC-HO-ToNR-FDD-FR2-r16 - eutra-5GC-HO-ToNR-TDD-FR2-2-r17 - eutra-EPC-HO-ToNR-TDD-FR2-2-r17 - ims-VoiceOverNR-FR2-2-r17 - ce-EUTRA-5GC-HO-ToNR-TDD-FR2-2-r17 The references to the unknown RAN4 specification have been added intentionally as placeholder for the case when RAN4 decides to specify requirements for these UE capabilities. However, neither inter-RAT handover requirements to NR FR2-1/FR2-2 nor requirements for IMS voice over NR FR2-1/FR-2 have been specified yet by RAN4. Considering the fact that Rel-17 work has been completed since a while, the references to an unknown RAN4 specification should be removed from the concerned capability descriptions. - A further mistake needs to be fixed. Instead of ce-EUTRA-5GC-HO-ToNR-FDD-FR2-r16 it should be clarified that the capability ce-EUTRA-5GC-HO-ToNR-TDD-FR2-r16 indicates the support of handover from E-UTRA/5GC to NR TDD FR2-1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - The reference to an unknown RAN4 specification has been removed from the below UE capabilities: - eutra-5GC-HO-ToNR-TDD-FR2-r15 - eutra-EPC-HO-ToNR-TDD-FR2-r15 - ims-VoiceOverNR-FR2-r15 - ce-EUTRA-5GC-HO-ToNR-FDD-FR2-r16 - eutra-5GC-HO-ToNR-TDD-FR2-2-r17 - eutra-EPC-HO-ToNR-TDD-FR2-2-r17 - ims-VoiceOverNR-FR2-2-r17 - ce-EUTRA-5GC-HO-ToNR-TDD-FR2-2-r17 - It has been clarified that the capability ce-EUTRA-5GC-HO-ToNR-TDD-FR2-r16 indicates the support of handover from E-UTRA/5GC to NR TDD FR2-1. Impact analysis Impacted 5G architecture options: NR SA Impacted functionality: UE capability signaling for the support of NR 71GHz Inter-operability: - There are no interoperability issues. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | References to an unknown RAN4 specification remain in the specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.34.7, 4.3.34.11, 4.3.34.14, 4.3.34.22, 4.3.34.23, 4.3.34.25, 4.3.34.26, 4.3.34.27, 4.3.34.28 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 36.331 CR4984 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of changes

### 4.3.34	Inter-RAT Parameters NR

#### 4.3.34.1	en-DC-r15

This field indicates whether UE supports E-UTRA NR Dual Connectivity as specified in TS 37.340 [38].

#### 4.3.34.2	supportedBandListEN-DC-r15

Only applicable if the UE supports E-UTRA NR Dual Connectivity or NG-RAN E-UTRA-NR Dual Connectivity. This field includes the supported NR bands as defined in TS 38.101-1 [33] and TS 38.101-2 [34]. The presence of this field also indicates that the UE can perform both NR SS-RSRP and SS-RSRQ measurement in the included NR band(s) as specified in TS 38.215 [36].

#### 4.3.34.3	supportedBandListNR-SA-r15

This field indicates whether UE supports standalone NR, as specified in TS 38.331 [35], and includes the supported NR bands as defined in TS 38.101-1 [33] and TS 38.101-2 [34]. The presence of this field also indicates that the UE can perform both NR SS-RSRP and SS-RSRQ measurement in the included NR band(s) as specified in TS 38.215 [36].

#### 4.3.34.4	eutra-5GC-HO-ToNR-FDD-FR1-r15

This field indicates whether the UE supports handover from E-UTRA/5GC to NR FDD FR1. It is mandatory for UEs of this release of the specification if the UE supports the associated RATs and if the UE supports eutra-5GC-r15.

#### 4.3.34.5	eutra-5GC-HO-ToNR-TDD-FR1-r15

This field indicates whether the UE supports handover from E-UTRA/5GC to NR TDD FR1. It is mandatory for UEs of this release of the specification if the UE supports the associated RATs and if the UE supports eutra-5GC-r15.

#### 4.3.34.6	eutra-5GC-HO-ToNR-FDD-FR2-r15

This field indicates whether the UE supports handover from E-UTRA/5GC to NR FDD FR2. It is mandatory for UEs of this release of the specification if the UE supports the associated RATs and if the UE supports eutra-5GC-r15.

#### 4.3.34.7	eutra-5GC-HO-ToNR-TDD-FR2-r15

This field indicates whether the UE supports handover from E-UTRA/5GC to NR TDD FR2-1. It is mandatory for UEs of this release of the specification if the UE supports the associated RATs and if the UE supports eutra-5GC-r15.

#### 4.3.34.8	eutra-EPC-HO-ToNR-FDD-FR1-r15

This field indicates whether the UE supports handover from E-UTRA/EPC to NR FDD FR1. It is mandatory for UEs of this release of the specification if the UE supports the associated RATs.

#### 4.3.34.9	eutra-EPC-HO-ToNR-TDD-FR1-r15

This field indicates whether the UE supports handover from E-UTRA/EPC to NR TDD FR1. It is mandatory for UEs of this release of the specification if the UE supports the associated RATs.

#### 4.3.34.10	eutra-EPC-HO-ToNR-FDD-FR2-r15

This field indicates whether the UE supports handover from E-UTRA/EPC to NR FDD FR2. It is mandatory for UEs of this release of the specification if the UE supports the associated RATs.

#### 4.3.34.11	eutra-EPC-HO-ToNR-TDD-FR2-r15

This field indicates whether the UE supports handover from E-UTRA/EPC to NR TDD FR2-1. It is mandatory for UEs of this release of the specification if the UE supports the associated RATs.

#### 4.3.34.12	sa-NR-r15

This field indicates whether the UE supports standalone NR as specified in TS 38.331 [35].

#### 4.3.34.13	ims-VoiceOverNR-FR1-r15

This field indicates whether the UE supports IMS voice over NR FR1.

#### 4.3.34.14	ims-VoiceOverNR-FR2-r15

This field indicates whether the UE supports IMS voice over NR FR2-1.

#### 4.3.34.15	eventB2-r15

This field defines whether the UE supports event B2. In this release of specification, it is mandatory for a UE supporting NR SA operation to support eventB2-r15.

#### 4.3.34.16	ss-SINR-Meas-NR-FR1-r15

This field indicates whether the UE can perform NR FR1 SS-SINR measurement as specified in TS 38.215 [36].

#### 4.3.34.17	ss-SINR-Meas-NR-FR2-r15

This field indicates whether the UE can perform NR FR2 SS-SINR measurement as specified in TS 38.215 [36].

#### 4.3.34.18	ng-EN-DC-r15

This field indicates whether UE supports NG-RAN E-UTRA-NR Dual Connectivity as specified in TS 37.340 [38].

#### 4.3.34.19	nr-HO-ToEN-DC-r16

This field indicates whether the UE supports inter-RAT handover from NR to EN-DC while NR-DC or NE-DC is not configured as defined in TS 37.340 [38]. It is mandatory to support inter-RAT handover from NR to EN-DC if the UE supports E-UTRA NR Dual Connectivity.

#### 4.3.34.20	ce-EUTRA-5GC-HO-ToNR-FDD-FR1-r16

This field indicates whether the UE supports handover from E-UTRA/5GC in coverage enhancement mode A or B to NR FDD FR1. A UE indicating support of ce-EUTRA-5GC-HO-ToNR-FDD-FR1-r16 shall also indicate support of ce-EUTRA-5GC-r16. This feature is only applicable if the UE supports a UE Category other than Category M1 and M2.

#### 4.3.34.21	ce-EUTRA-5GC-HO-ToNR-TDD-FR1-r16

This field indicates whether the UE supports handover from E-UTRA/5GC in coverage enhancement mode A or B to NR TDD FR1. A UE indicating support of ce-EUTRA-5GC-HO-ToNR-TDD-FR1-r16 shall also indicate support of ce-EUTRA-5GC-r16. This feature is only applicable if the UE supports a UE Category other than Category M1 and M2.

#### 4.3.34.22	ce-EUTRA-5GC-HO-ToNR-FDD-FR2-r16

This field indicates whether the UE supports handover from E-UTRA/5GC in coverage enhancement mode A or B to NR FDD FR2. A UE indicating support of ce-EUTRA-5GC-HO-ToNR-FDD-FR2-r16 shall also indicate support of ce-EUTRA-5GC-r16. This feature is only applicable if the UE supports a UE Category other than Category M1 and M2.

#### 4.3.34.23	ce-EUTRA-5GC-HO-ToNR-TDD-FR2-r16

This field indicates whether the UE supports handover from E-UTRA/5GC in coverage enhancement mode A or B to NR TDD FR2-1. A UE indicating support of ce-EUTRA-5GC-HO-ToNR-TDD-FR2-r16 shall also indicate support of ce-EUTRA-5GC-r16. This feature is only applicable if the UE supports a UE Category other than Category M1 and M2.

#### 4.3.34.24	extendedBand-n77-r16

This field is only applicable for UEs that indicate support for band n77. If present, the UE supports the restriction to 3450 - 3550 MHz and 3700 - 3980 MHz ranges of band n77 in the USA as specified in Note 12 of Table 5.2-1 in TS 38.101-1 [33]. If absent, the UE supports only restriction to the 3700 - 3980 MHz range of band n77 in the USA. A UE that indicates this field shall also support NS value 55 as specified in TS 38.101-1 [33]. A UE supporting NS value 55 shall indicate this field.

#### 4.3.34.25	eutra-5GC-HO-ToNR-TDD-FR2-2-r17

This field indicates whether the UE supports handover from E-UTRA/5GC to NR TDD FR2-2. A UE that indicates this field also supports eutra-5GC-r15. A UE supporting handover from E-UTRA/5GC to NR TDD FR2-2 shall also support the RRM measurements for FR2-2 as specified in TS 36.331 [5].

#### 4.3.34.26	eutra-EPC-HO-ToNR-TDD-FR2-2-r17

This field indicates whether the UE supports handover from E-UTRA/EPC to NR TDD FR2-2. A UE supporting handover from E-UTRA/EPC to NR TDD FR2-2 shall also support the RRM measurements for FR2-2 as specified in TS 36.331 [5].

#### 4.3.34.27	ims-VoiceOverNR-FR2-2-r17

This field indicates whether the UE supports IMS voice over NR FR2-2.

#### 4.3.34.28	ce-EUTRA-5GC-HO-ToNR-TDD-FR2-2-r17

This field indicates whether the UE supports handover from E-UTRA/5GC in coverage enhancement mode A or B to NR TDD FR2-2. A UE indicating support of ce-EUTRA-5GC-HO-ToNR-TDD-FR2-2-r17 shall also indicate support of ce-EUTRA-5GC-r16. This feature is only applicable if the UE supports a UE Category other than Category M1 and M2.

#### 4.3.34.29	extendedBand-n77-2-r17

This field is only applicable for UEs that indicate support for band n77. If present, the UE supports the restriction to 3450 - 3650 MHz and 3650 - 3980 ranges of band n77 in Canada as specified in Note 12 of Table 5.2-1 in TS 38.101-1 [33]. If absent, the UE supports only restriction to the 3450 - 3650 MHz range of band n77 in Canada. A UE that indicates this field shall also support NS value 57 as specified in TS 38.101-1 [33]. A UE supporting NS value 57 shall indicate this field.

End of changes