3GPP TSG-RAN WG2 Meeting #125                                    	R2-240853

Athens, Greece: February 26 – March 1, 2024

Agenda item:	7.7.3

Source:	Qualcomm Incorporated

Title:	DL sync and switch time in Satellite switch with re-sync

Document for:	Discussion and Decision

# Introduction

In this document, we further discuss the following FFS remaining in TS 38.331. This document also addresses the resolution for the RIL H001 [UE should switch to new satellite upon t-Service, rather than leaving the NW unaware of the exact switching time].

- Editor's Note: FFS whether in the soft-switch scenario a UE can obtain DL synchronization from the target satellite without losing UL synchronization to the source satellite.

# Discussion

RAN2 has agreed to support soft satellite switch with PCI unchanged and indicate time to UE when the target satellite starts providing service. As per current procedure, the UE can leave the source satellite and go to synchronize to the target satellite any time between t-ServiceStart and t-Service. This means the gap between t-ServiceStart and t-Service is supposed to be small and good enough for UE to be able to be synchronized with the target satellite. The larger overlapping gap may not be resource efficient as both satellites are being used and may bring difficulty for network to make sure the two SSBs do not interfere at UEs for longer duration due to time variant service link and feeder link.

Figure 1 Current soft satellite switch procedure

The gap between t-ServiceStart and t-Service is supposed to be small and good enough for UE to be able to be synchronized with the target satellite.

This means the network is supposed to avoid UL/DL scheduling during the gap, therefore the gap should be configured as minimum as possible. However, there is still concern raised on the interruption.

We can leave this to RAN4 to define the DL synchronization duration or minimum DL synchronization period. Whether be it for soft satellite switch or hard satellite switch, the interruption must be considered. Compared to hard satellite switch, the only difference for satellite switch is that UE has chance to perform early synchronization to the target cell.

Whether UE can perform measurement of different satellite in the same frequency without scheduling restriction depends on the following UE capability, i.e., parallelMeasurementWithoutRestriction-r17.

| parallelMeasurementWithoutRestriction-r17 Indicates whether the UE supports measurements on cells belonging to different satellites as the serving cell in parallel with normal operation (i.e. data/control transmission and/or reception, and L1 measurements) of serving cell without scheduling restrictions. The feature is applicable only when the serving satellite is NGSO. If the serving cell belongs to GSO satellite, the scheduling restriction is not applied on the premise that a mixed type of satellites on the same frequency layer is not supported in this release. If not reported, for measurements in parallel with normal operation of serving cell scheduling restrictions shall apply. | Band | No | FDD only | FR1 only |
| --- | --- | --- | --- | --- |

However, there is a difference between capability of measurement vs capability of obtaining DL synchronization. A UE may need more accurate samples of measurement to obtain proper synchronization with the target satellite, therefore, if the UE is given sufficient longer duration of synchronization time, it is possible that UE can obtain DL synchronization of the target cell without scheduling restriction.

We think the UE can still be allowed to disconnect with serving satellite and connect to target satellite, if a UE can do so early. In this case, the UE should send some UL signal, such as SR or RACH to the gNB via new satellite. If the UE knows SR is pending or would be triggered (e.g., new DL data arrival, or TA report triggered and SR is configured to be triggered via TA report), UE should be able to perform UL synchronization (e.g., RACH or UL transmission) with target cell earlier than t-Service. Otherwise, UE may choose to disconnect from source and connects to target at t-Service.

However, this could still mean some UEs would be connected to target satellite and some are still with the old satellite creating network ambiguity. Therefore, we think it may be simple just to follow hard switch and all UEs leave the source satellite at the same time, i.e., t-Service. In this case, if the UE does not support parallelMeasurementWithoutRestriction-r17, there is no difference between soft and hard satellite switch. If the UE supports parallelMeasurementWithoutRestriction-r17, then at t-Service, the interruption would be much less in soft satellite switch case as UE would have already obtained the DL measurements and/or the DL synchronization with the target satellite.

Text proposal-2: ##### 5.2.2.4.21 Actions upon reception of SIB19 Upon receiving SIB19 in an NTN cell, the UE in RRC_CONNECTED or the UE after transitioning to RRC_CONNECTED shall: 1> start or restart T430 for serving cell with the timer value set to ntn-UlSyncValidityDuration for the serving cell from the subframe indicated by epochTime for the serving cell; 1> if SatSwitchWithReSync and t-Service are included, and the UE supports hard satellite switch with resynchronization; 2> if t-ServiceStart is included and the UE supports soft satellite switch with resynchronization and parallelMeasurementWithoutRestriction-r17: 3> start measurement of the SpCell served by the satellite indicated by ntn-Config in SatSwitchWithReSync for DL synchronization at t-ServiceStart; 2> else: 23> perform the satellite switch with resynchronization as specified in 5.7.19 at the time indicated by t-Service for the serving cell. NOTE: UE should attempt to re-acquire SIB19 before the end of the duration indicated by ntn-UlSyncValidityDuration and epochTime by UE implementation.

- The UE is assumed connected to serving satellite while obtaining DL synchronization with the target satellite, i.e., performing target satellite measurement only if the UE supports parallelMeasurementWithoutRestriction-r17.

- In soft satellite switch case, all UEs leave the source satellite at t-Service and starts UL synchronization (i.e., UL synchronization restored at MAC layer) with the target satellite after t-Service.

- Remove editor’s note on DL synchronization in section 5.7.19.

- Adopt the text proposal provided in Annex.

# Conclusion

Following proposals are made.

Observation 1.	The gap between t-ServiceStart and t-Service is supposed to be small and good enough for UE to be able to be synchronized with the target satellite.

Proposal 1	The UE is assumed connected to serving satellite while obtaining DL synchronization with the target satellite, i.e., performing target satellite measurement only if the UE supports parallelMeasurementWithoutRestriction-r17.

Proposal 2	In soft satellite switch case, all UEs leave the source satellite at t-Service and starts UL synchronization (i.e., UL synchronization restored at MAC layer) with the target satellite after t-Service.

Proposal 3	Remove editor’s note on DL synchronization in section 5.7.19.

Proposal 4	Adopt the text proposal provided in Annex.

# Annes: TS 38.331 Text proposal

<<start of change>>

#### 5.2.2.4	Actions upon receipt of System Information

##### 5.2.2.4.1	Actions upon reception of the MIB

Upon receiving the MIB the UE shall:

1>	store the acquired MIB;

1>	if the UE is in RRC_IDLE or in RRC_INACTIVE, or if the UE is in RRC_CONNECTED while T311 is running:

2>	if the access is not for NTN or the UE is not capable of NTN or the UE is not capable of NES cell DTX/DRX; and

2>	if the access is not for ATG or the UE is not capable of ATG; and

2>	if the cellBarred in the acquired MIB is set to barred:

3>	if the UE is an (e)RedCap UE and ssb-SubcarrierOffset indicates SIB1 is transmitted in the cell (TS 38.213 [13]):

4>	acquire the SIB1, which is scheduled as specified in TS 38.213 [13];

3>	consider the cell as barred in accordance with TS 38.304 [20];

3>	perform cell re-selection to other cells on the same frequency as the barred cell as specified in TS 38.304 [20];

2>	else:

3>	apply the received systemFrameNumber, pdcch-ConfigSIB1, subCarrierSpacingCommon, ssb-SubcarrierOffset and dmrs-TypeA-Position.

NOTE 1:	A UE capable of NTN access should acquire SIB1 to determine whether the cell is an NTN cell.

NOTE 2:	A UE capable of ATG access should acquire SIB1 to determine whether the cell is an ATG cell.

NOTE 3:	A UE capable of NES cell DTX/DRX should acquire SIB1 to determine the cell barring status when the cellBarred in MIB is set to barred.

<<skipped>>

##### 5.2.2.4.21	Actions upon reception of SIB19

Upon receiving SIB19 in an NTN cell, the UE in RRC_CONNECTED or the UE after transitioning to RRC_CONNECTED shall:

1>	start or restart T430 for serving cell with the timer value set to ntn-UlSyncValidityDuration for the serving cell from the subframe indicated by epochTime for the serving cell;

1>	if SatSwitchWithReSync and t-Service are included, and the UE supports hard satellite switch with resynchronization;

2>	if t-ServiceStart is included and the UE supports soft satellite switch with resynchronization and parallelMeasurementWithoutRestriction-r17:

3> start measurement of the SpCell served by the satellite indicated by ntn-Config in SatSwitchWithReSync for DL synchronization at t-ServiceStart;

2>	perform the satellite switch with resynchronization as specified in 5.7.19 at the time indicated by t-Service for the serving cell.

NOTE:	UE should attempt to re-acquire SIB19 before the end of the duration indicated by ntn-UlSyncValidityDuration and epochTime by UE implementation.

<<skipped>>

### 5.7.19	Satellite switch with re-synchronization in RRC_CONNECTED

The UE shall:

1>	stop timer T430 if running;

1>	inform lower layers that UL synchronisation is lost due to satellite switch with re-synchronization;

1>	start re-synchronising to the DL of the SpCell served by the satellite indicated by ntn-Config in SatSwitchWithReSync, if needed;

1>	start timer T430 with the timer value set to ntn-UlSyncValidityDuration from the subframe indicated by epochTime in ntn-Config in SatSwitchWithReSync;

1>	inform lower layers when UL synchronisation is obtained.

<<End of the change>>