3GPP TSG-RAN WG2 #125	Tdoc R2-2401339

Athens, Greece, 26 Feb – 1 Mar, 2024

Agenda Item:	7.17.3

Source:	Ericsson

Title:	Modification of MUSIM UE capability

Document for:	Discussion

# 1	Introduction

In this document we discuss the UE capability for Rel-18 MUSIM enhancements, and argue it should not only indicate the support of the Rel-18 MUSIM enhancements, but also cover that the UE is registered to more than one PLMNs.

# 2	Discussion

RAN2 agreed that at resume, in case the UE cannot support the configuration used before UE went to RRC_Inactive, UE can (based on implementation) choose to apply only a part of the resulting configuration. This gives a mismatch between what the network assumes and what the UE actually uses, that will remain until RCReconfiguration after UAI”.

To avoid this, a network could at resume select to always use a defensive/limited configuration. With existing UE capability definition, the Nw would have to use that defensive/limited configuration with all UEs that indicate support of the Rel-18 MUSIM enhancements. If the capability was rephrased to cover that UE is also registered with more than one USIM to networks, less UEs would be subject to such defensive/limited configuration.

We note that UE performs the NAS Registration procedure with the Registration type set to Mobility Registration Update and includes "UE Radio Capability Update" to keep the musim-CapabilityRestriction up to date.

The UE capability in TS 38.306 is proposed to be revised as follows:

| musim-CapabilityRestriction-r18 Indicates whether the UE, equipped with more than one USIM by which the UE is registered to one or more networks, supports providing MUSIM assistance information with temporary capability restriction and capability restriction indication (i.e., musim-CapabilityRestrictionIndication), as defined in TS 38.331 [9]. | UE | No | No | No |
| --- | --- | --- | --- | --- |

RAN2 to agree the revised musim-CapabilityRestriction above.

# Conclusion

Based on the discussion in this document we propose the following:

Proposal 1	RAN2 to agree the revised musim-CapabilityRestriction above.