3GPP TSG-RAN WG2 Meeting#125	   R2-2401397

Athens, Greece, 26th February-1st March 2024

Agenda item:		7.11.2.1

Source:	vivo

Title:	Remaining Issues on UE Triggered RRC Resumption

Document for:		Discussion and Decision

1 Introduction

In this contribution, we would like to provide our view on the potential remaining issues on UE triggered RRC resumption conditions.

In RRC_INACTIVE state, if configured with MBS multicast reception in RRC_INACTIVE, the UE shall:

1>	if the RRC connection resume procedure is triggered for multicast reception at reception of SIB1, as specified in 5.2.2.4.2; or

1>	if the PTM configuration is not available in the cell after cell selection or reselection for a multicast session that the UE has joined for which the UE is not indicated to stop monitoring the G-RNTI; or

1>	if mbs-NeighbourCellList acquired in the previous cell indicates that a multicast session that the UE has joined for which the UE is not indicated to stop monitoring the G-RNTI is not provided for RRC_INACTIVE in the current serving cell; or

1>	if the measured RSRP or RSRQ for serving cell as specified in TS 38.304 [20] is below the threshold indicated by thresholdIndex for a multicast session that the UE has joined:

2>	initiate RRC connection resume procedure in 5.3.13.2 with resumeCause set to mt-Access.

2 Discussion

As the UE may join more than one MBS session, generally, when executing the per MBS session level condition check, the terms “at least one of the MBS session(s)” or “at least one multicast session” are widely used to indicate the UE has to do the condition check for each MBS session that it has joined. For example, in sub-clause 5.3.2.3 group paying part, the UE checks whether the indicated TMGI is matched with any of MBS session(s) it joined.

1> if in RRC_INACTIVE and the UE has joined one or more MBS session(s) indicated by the TMGI(s) included in the pagingGroupList: 2> if the UE is not configured to receive multicast in RRC_INACTIVE or if inactiveReceptionAllowed is not included for at least one of the MBS session (s) indicated by the TMGI(s) that the UE has joined: 3> if PagingRecordList is not included in the Paging message; or 3> if none of the ue-Identity included in any of the PagingRecord matches the UE identity allocated by upper layers or the UE's stored fullI-RNTI: 4> initiate the RRC connection resumption procedure according to 5.3.13 with resumeCause set as below: 5> if the UE is configured by upper layers with Access Identity 1: 6> set resumeCause to mps-PriorityAccess; 5> else if the UE is configured by upper layers with Access Identity 2: 6> set resumeCause to mcs-PriorityAccess; 5> else if the UE is configured by upper layers with one or more Access Identities equal to 11-15: 6> set resumeCause to highPriorityAccess; 5> else: 6> set resumeCause to mt-Access; 3> else if the ue-Identity included in any of the PagingRecord matches the UE identity allocated by upper layers: 4> forward the TMGI(s) to the upper layers; 2> else: 3> start monitoring the G-RNTI(s) corresponding to the TMGI(s), if configured; 3> if the UE was notified to stop monitoring the G-RNTI(s) for all the joined multicast sessions that are configured for reception in RRC_INACTIVE: 4> start monitoring the Multicast MCCH-RNTI; 4> acquire the MBSMulticastConfiguration message on multicast MCCH, if present; 3> else if the UE was notified to stop monitoring the G-RNTI for at least one multicast session for which the PTM configuration was not included in RRCRelease message: 4> acquire the MBSMulticastConfiguration message on multicast MCCH; 1> if the UE is acting as a L2 U2N Relay UE, for each of the PagingRecord, if any, included in the Paging message: 2> if the ue-Identity included in the PagingRecord in the Paging message matches the UE identity in sl-PagingIdentityRemoteUE included in sl-PagingInfo-RemoteUE received in RemoteUEInformationSidelink message from a L2 U2N Remote UE: 3> inititate the Uu Message transfer in sidelink to that UE as specified in 5.8.9.9;

It seems a spontaneous logic to reuse this style of expression for the conditions for UE triggered RRC resumption. With this, it is clearer that the UE checks whether PTM configuration or MSB session will be provided in the target cell for each of the joined multicast session(s). Otherwise, the existing term “a multicast session that the UE has joined” seems that the UE only needs to execute the condition check for only one multicast session. It is also not clear which multicast session should be chosen for the check. Thus, we have the following proposal,

Proposal 1: For clause 5.3.13.1d, change “a multicast session that the UE has joined for which the UE is not indicated to stop monitoring the G-RNTI” to “at least one of the multicast session(s) that the UE has joined for which the UE is not indicated to stop monitoring the G-RNTI”.

Further, regarding the RSRP(Q)-based RRC resumption condition, our understanding is that the intended UE behavior is to compare the measured RSRP(Q) with the configured threshold(s) of the ongoing multicast session(s). And if the RSRP(Q) is below at least one of the corresponding thresholds, the UE initiates the RRC resume request procedure. However, the existing condition only mentions a single threshold of a multicast session as the thresholdIndex only links to a threshold value. Also, from the UE point of view, it is still not clear which multicast session should be chosen for the check.

TS 38.331 5.3.13.1d 1> if the measured RSRP or RSRQ for serving cell as specified in TS 38.304 [20] is below the threshold indicated by thresholdIndex for a multicast session that the UE has joined: thresholdIndex Indicates the index of thresholdMBS entry in thresholdMBS-List that is used for RRC connection resume for a UE receiving the corresponding multicast session in RRC_INACTIVE. Value 0 corresponds to the first entry in thresholdMBS-List, the value 1 corresponds to the second entry in thresholdMBS-List and so on.

To make it clear, we propose,

Proposal 2: RAN2 clarifies that if the measured RSRP(Q) is below at least one of the threshold(s) related to a multicast session that the UE has joined for which the UE is not indicated to stop monitoring the G-RNTI, UE initiates RRC connection resume procedure.

The corresponding TP is given in the Annex.

Proposal 3: RAN2 to adopt the text proposal in the Annex.

3 Conclusion

In this contribution, we have discussed the remaining issues on UE triggered RRC resumption conditions. All the proposals are summarized below:

Proposal 1: For clause 5.3.13.1d, change “a multicast session that the UE has joined for which the UE is not indicated to stop monitoring the G-RNTI” to “at least one of the multicast session(s) that the UE has joined for which the UE is not indicated to stop monitoring the G-RNTI”.

Proposal 2: RAN2 clarifies that if the measured RSRP(Q) is below at least one of the threshold(s) related to a multicast session that the UE has joined for which the UE is not indicated to stop monitoring the G-RNTI, UE initiates RRC connection resume procedure.

Proposal 3: RAN2 to adopt the text proposal in the Annex.

4 Annex

Text proposal for TS 38.331 ver 18.0.0:

START OF the CHANGE

5.3.13.1d	RRC connection resume for multicast reception

In RRC_INACTIVE state, if configured with MBS multicast reception in RRC_INACTIVE, the UE shall:

1>	if the RRC connection resume procedure is triggered for multicast reception at reception of SIB1, as specified in 5.2.2.4.2; or

1>	if the PTM configuration is not available in the cell after cell selection or reselection for at least one of the multicast session(s) that the UE has joined for which the UE is not indicated to stop monitoring the G-RNTI; or

1>	if mbs-NeighbourCellList acquired in the previous cell indicates that at least one of the multicast session(s) that the UE has joined for which the UE is not indicated to stop monitoring the G-RNTI is not provided for RRC_INACTIVE in the current serving cell; or

1>	if the measured RSRP or RSRQ for serving cell as specified in TS 38.304 [20] is below at least one of the threshold(s) indicated by thresholdIndex for a multicast session that the UE has joined for which the UE is not indicated to stop monitoring the G-RNTI:

2>	initiate RRC connection resume procedure in 5.3.13.2 with resumeCause set to mt-Access..

END OF the CHANGE