3GPP TSG RAN WG2 Meeting #125	                                                    R2-2401412

# Athens, Greece, 26th Feb. – 1st Mar., 2024

Agenda item:		7.24.1

Source:			Huawei, HiSilicon, Qualcomm, CATT, Lenovo, Orange, China Telecom, CMCC

Title:		Discussion on issues about resumeIndication in RRCRelease

Work Item:		TEI18

Document for:	 	Discussion and decision

# Introduction

During RAN2#124 meeting, RAN2 discussed issues about SDT signaling optimization for partial context transfer, and agreed the following:

Agreements - RAN2 agrees to introduce a resumeIndication in the RRCRelease message sent to terminate SDT procedure, allowing the UE to initiate the RRC resume procedure immediately after receiving this RRCRelease message and after performing cell selection. Same UE requirements as receiving paging. - This functionality is an optional UE capability with signalling. - Send LS to RAN3 after the CR agreable

A corresponding post email discussion “[POST124][022][SDT signaling opt] CRs (Huawei)” took place triggered and one open issue was agreed to be discussed in the maintenance phase: “To be discussed in the maintenance phase: whether/how to address the possibility of UE selecting a cell belonging to different RAT or RNA after moving to RRC_INACTIVE”, in this paper, we further discuss this issue.

# Discussion

Upon reception of RRCRelease with suspendConfig, UE enters RRC_INACTIVE and performs cell selection. As indicated in the above agreement, UE should perform resume procedure according to the resumeIndication after performing cell selection.

The possibility of UE selecting a cell belonging to different RAT or RNA after moving to RRC_INACTIVE is quite remote because the condition for initiating the SDT procedure in a cell itself requires that the RSRP threshold for SDT is met which in turn ensures that at the time of initiating the SDT procedure the UE is in the center of the cell. In addition to this, since the duration of the SDT procedure is limited to 4 seconds this probability reduces further.  Hence, unless the UE is fast moving, the possibility of it selecting a different cell when the last serving gNB terminates the SDT procedure before 4 seconds would be quite low and the probability of this cell belonging to another RAT or RNA is even smaller.

Secondly, a smart network implementation will not set the indication to resume immediately together with different priorities for frequencies or RAT. The network also sets RNA to the UE in a dedicated manner. Hence, the network can easily prevent the UE from selecting a cell belonging to different frequency, another RNA or different RAT when it wants the UE to initiate the resume procedure immediately due to non-SDT DL data arrival.

Observation 1: Network implementation can easily prevent the UE from selecting a cell belonging to different frequency, another RNA or different RAT when it wants the UE to initiate the resume procedure immediately due to non-SDT DL data arrival.

Based on the discussion above, we propose to agree on the following:

Proposal 1: There is no need to address in specifications the possibility of UE selecting a cell belonging to different RAT or RNA after receiving RRCRelease with resumeIndication included.

# Conclusion

Based on the discussion in this paper, the following observations and proposals are made:

Observation 1: Network implementation can easily prevent the UE from selecting a cell belonging to different frequency, another RNA or different RAT when it wants the UE to initiate the resume procedure immediately due to non-SDT DL data arrival.

Proposal 1: There is no need to address in specifications the possibility of UE selecting a cell belonging to different RAT or RNA after receiving RRCRelease with resumeIndication included.