3GPP TSG-RAN WG2 Meeting #125	R2-2401495

# Athens 27th Feb– 2nd Mar, 2024

Source:              ZTE Corporation, Sanechips

Title:                   [RIL-Z102] MUSIM Gap UAI Processing

# Agenda item:     7.17.2

# Document for:   Discussion and Decision

- Introduction

In this paper, we discuss the [RIL-Z102] issue on MUSIM gap UAI processing, a draft TP was also provided.

- Discussion

As described in the [RIL-Z102], according to the current description as below, when execute the first if condition, the timer T346h may have been started, it would impact the transmission of the musim-GapPriorityPreferenceList.

1> if configured to provide MUSIM assistance information for gap preference: 2> if the UE has a preference on the MUSIM gap(s) and the UE did not transmit a UEAssistanceInformation message with musim-GapPreferenceList since it was configured to provide MUSIM assistance information for gap preference; or 2> if the current musim-GapPreferenceList is different from the one indicated in the last transmission of the UEAssistanceInformation message including musim-GapPreferenceList and the timer T346h is not running: 3> initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide the current musim-GapPreferenceList; 3> start or restart the timer T346h with the timer value set to the musim-GapProhibitTimer. NOTE 3: The UE does not need to initiate transmission of the UEAssistanceInformation message if the difference between the current musim-GapPreferenceList and the last transmission of the UEAssistanceInformation message including musim-GapPreferenceList is only due to removal of an ended aperiodic gap. 1> if configured to provide MUSIM assistance information for gap priority preference: 2> if the UE has a preference on the MUSIM gap(s) priority and the UE did not transmit a UEAssistanceInformation message with musim-GapPriorityPreferenceList since it was configured to provide MUSIM assistance information for gap priority preference; or 2> if the current musim-GapPriorityPreferenceList is different from the one indicated in the last transmission of the UEAssistanceInformation message including musim-GapPriorityPreferenceList and the timer T346h is not running: 3> initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide the musim-GapPriorityPreferenceList and/or musim-GapKeepPreference; 3> start or restart the timer T346h with the timer value set to the musim-GapProhibitTimer.

To solve this problem, the two “if conditions” may need to be merged according to the logic in the Fig 1:

Fig 1: MUSIM Gap UAI Processing

In which the Codition1/2 are defined as below:

Condition 1: 2> if the UE has a preference on the MUSIM gap(s) and the UE did not transmit a UEAssistanceInformation message with musim-GapPreferenceList since it was configured to provide MUSIM assistance information for gap preference; or 2> if the current musim-GapPreferenceList is different from the one indicated in the last transmission of the UEAssistanceInformation message including musim-GapPreferenceList and the timer T346h is not running:

Condition 2: 2> if the UE has a preference on the MUSIM gap(s) priority and the UE did not transmit a UEAssistanceInformation message with musim-GapPriorityPreferenceList since it was configured to provide MUSIM assistance information for gap priority preference; or 2> if the current musim-GapPriorityPreferenceList is different from the one indicated in the last transmission of the UEAssistanceInformation message including musim-GapPriorityPreferenceList and the timer T346h is not running:

Based on the Fig 1, the procedure for the MUSIM Gap UAI transmission can be modified as below:

1> if configured to provide MUSIM assistance information for gap preference: 2> if the UE has a preference on the MUSIM gap(s) and the UE did not transmit a UEAssistanceInformation message with musim-GapPreferenceList since it was configured to provide MUSIM assistance information for gap preference; or if the current musim-GapPreferenceList is different from the one indicated in the last transmission of the UEAssistanceInformation message including musim-GapPreferenceList and the timer T346h is not running: 2> if the current musim-GapPreferenceList is different from the one indicated in the last transmission of the UEAssistanceInformation message including musim-GapPreferenceList and the timer T346h is not running: 3> initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide the current musim-GapPreferenceList; 3> start or restart the timer T346h with the timer value set to the musim-GapProhibitTimer. NOTE 3: The UE does not need to initiate transmission of the UEAssistanceInformation message if the difference between the current musim-GapPreferenceList and the last transmission of the UEAssistanceInformation message including musim-GapPreferenceList is only due to removal of an ended aperiodic gap. 1> 2> else if configured to provide MUSIM assistance information for gap priority preference: 2> 3> if the UE has a preference on the MUSIM gap(s) priority and the UE did not transmit a UEAssistanceInformation message with musim-GapPriorityPreferenceList since it was configured to provide MUSIM assistance information for gap priority preference; or 2> 3> if the current musim-GapPriorityPreferenceList is different from the one indicated in the last transmission of the UEAssistanceInformation message including musim-GapPriorityPreferenceList and the timer T346h is not running: 3> 4> initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide the musim-GapPriorityPreferenceList and/or musim-GapKeepPreference; 3> 4> start or restart the timer T346h with the timer value set to the musim-GapProhibitTimer.

Proposal 1: Merge the two “if condition” for the MUSIM Gap UAI processing according to the Fig 1.

Besides, some modifications are also needed for the “Actions related to transmission of UEAssistanceInformation message in the 5.7.4.3, for that the priority and keep based reporting are only for the periodic Gap. A TP was also provided as in the Annex.

Proposal 2: Agree the TP as in the Annex.

- Conclusion and proposals

With the above analysis, we have the following proposal:

Proposal 1: Merge the two “if condition” for the MUSIM Gap UAI processing according to the Fig 1.

Proposal 2: Agree the TP as in the Annex.

- Annex

---First Change #### 5.7.4.2 Initiation /***********************Omit the Unrelated part*******************************************/ 1> if configured to provide MUSIM assistance information for gap preference: 2> if the UE has a preference on the MUSIM gap(s) and the UE did not transmit a UEAssistanceInformation message with musim-GapPreferenceList since it was configured to provide MUSIM assistance information for gap preference; or if the current musim-GapPreferenceList is different from the one indicated in the last transmission of the UEAssistanceInformation message including musim-GapPreferenceList and the timer T346h is not running: 2> if the current musim-GapPreferenceList is different from the one indicated in the last transmission of the UEAssistanceInformation message including musim-GapPreferenceList and the timer T346h is not running: 3> initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide the current musim-GapPreferenceList; 3> start or restart the timer T346h with the timer value set to the musim-GapProhibitTimer. NOTE 3: The UE does not need to initiate transmission of the UEAssistanceInformation message if the difference between the current musim-GapPreferenceList and the last transmission of the UEAssistanceInformation message including musim-GapPreferenceList is only due to removal of an ended aperiodic gap. 1> 2> else if configured to provide MUSIM assistance information for gap priority preference: 2> 3> if the UE has a preference on the MUSIM gap(s) priority and the UE did not transmit a UEAssistanceInformation message with musim-GapPriorityPreferenceList since it was configured to provide MUSIM assistance information for gap priority preference; or 2> 3> if the current musim-GapPriorityPreferenceList is different from the one indicated in the last transmission of the UEAssistanceInformation message including musim-GapPriorityPreferenceList and the timer T346h is not running: 3> 4> initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide the musim-GapPriorityPreferenceList and/or musim-GapKeepPreference; 3> 4> start or restart the timer T346h with the timer value set to the musim-GapProhibitTimer. /***********************Omit the Unrelated part*******************************************/

---Second Change #### 5.7.4.3 Actions related to transmission of UEAssistanceInformation message /***********************Omit the Unrelated part*******************************************/ 1> if transmission of the UEAssistanceInformation message is initiated to provide MUSIM assistance information according to 5.7.4.2 or 5.3.5.3: 2> if the UE has a preference for MUSIM periodic gap(s): 3> include musim-GapPreferenceList with an entry for each periodic gap the UE prefers to be configured; 4> set musim-GapLength and musim-GapRepetitionAndOffset in the musim-GapInfo IE to the values of the length and the repetition/offset of the gap(s), respectively, the UE prefers to be configured with; 4> if UE has a preference for MUSIM gap priority: 5> include the musim-GapPriorityPreferenceList the UE prefers to be configured; 5> if the UE has preference to keep all collided MUSIM gaps for periodic MUSIM gap(s): 6> include the musim-GapKeepPreference; 2> if the UE has a preference for MUSIM aperiodic gap: 3> include the field musim-GapPreferenceList, with one entry for the aperiodic gap the UE prefers to be configured; 4> include musim-GapLength in the musim-GapInfo IE and set it to the values of the length of the gap the UE prefers to be configured with; 4> optionally include musim-Starting-SFN-AndSubframe in the musim-GapInfo IE and set it to the starting SFN/subframe of the gap the UE prefers to be configured with; 2> if the UE has no longer preference for the periodic/aperiodic gaps: 3> do not include musim-GapPreferenceList in the musim-Assistance IE; 2> if UE has a preference to leave RRC_CONNECTED state: 3> set musim-PreferredRRC-State to the preferred RRC state. 2> if UE has a preference for MUSIM gap priority: 3> include the musim-GapPriorityPreferenceList the UE prefers to be configured; 3> if the UE has preference to keep all collided MUSIM gaps for periodic MUSIM gap(s): 4> include the musim-GapKeepPreference; /***********************Omit the Unrelated part*******************************************/