3GPP TSG RAN WG2 Meeting #125                                                            R2-2401837

# Athens, Greece, February 26th – March 1st, 2024

Agenda item:	7.5.3.3

Source:	Intel Corporation, CATT, Fujitsu, Ericsson, Canon, Apple, InterDigital, Futurewei, Huawei, HiSilicon, ZTE, vivo, NTT DOCOMO, MediaTek Inc., Nokia, Nokia Shanghai Bell

Title:	PDCP SN Gap Reporting

Document for:	Discussion and decision

# Introduction

The following aims to provide a summary of companies view on PDCP SN Gap notification in Rel-18 considering RAN2#125 TDocs [1]-[14] trying to suggest a consolidated view between the supportive companies. It also considers the related previous email discussion [AT124][019][XR] PDCP discard (CATT)[15].

# Summary

## Support

Support or not of PDCP SN Gap notification/report

- Support [1][3][4][5][6][7][8][9][10][11][12][13][14][15] – 15 companies.

- No support [2] – 5 companies.

Many contributions justify the benefit to enable this mechanism considering also the XR related discard enhancements.

## Network configurability

Allow network to configure a UE when PDCP SN Gap notification is used [1][5][13] – 5 companies.

- The receiver should be able to configure in advance to limit the load of Discard notifications [13].

- Pre-configure the detail condition for sending the discard notification [13].

Tie the notification of PDCP SN gap to whether outOfOrderDelivery is not configured [3].

## Notification of PDCP SN Gap

The notification of PDCP SN Gap could be enabled via the following mechanisms:

- PDCP control PDU [1][3][5][6][7][8][9][10][11][12] – 11 companies.

  - Bitmap kind of information [1][3][5][6][7][9][10][12] – 10 companies.

    - It could reuse the PDCP status report (SR) approach [1][3].

    - The byte aligned bitmap to indicate the discarded PDUs should include a value 1 for discarded PDUs and 0 for a PDU that is not yet discarded [5].

    - It provides information of SN discarded [6].

    - The PDCP receiver considers the last SN before packet discard and the next SN after packet discard as in-sequence (i.e., without considering the SN gap) [10].

    - The PDCP entity triggers the PDCP SDU discard report when: (1) he PDCP entity discards SDU(s) which have not been transmitted (for UM DRBs) or acknowledged (for AM DRBs), due to the expiry of PDCP discard timer; and (2) there is a buffered SDU associated with an SN higher than the SN of the discarded SDU(s) [12].

    - Possible TPs are included e.g. in [5], [12] and in previous [AT124][019] email discussion [15].

  - Range kind of information [1][9][11] – 3 companies.

    - Indicate the first COUNT of discarded PDCP SDU and number of discarded PDCP SDU, e.g. reusing the RLC status report (SR) approach, i.e.  [1][9][11].

    - Indicate the first and last COUNT of discarded PDCP SDU [9][11].

Possible TPs were included in previous [AT124][019] email discussion [15].

- PDCP data PDU header [4] – 2 companies.

- COUNT value of consecutive PDUs immediately preceding this PDU expected not to be received by receiver [4].

  - Not integrity-protection for this new indication (of discarded PDUs in the PDCP Data-PDU header like the same indication in a PDCP control PDU would not be) [4].

Possible TPs were included in previous [AT124][019] email discussion [15].

There is a larger support to enable a control based approach (instead a data header based).

# Proposal for discussion

## Support

- To define a mechanism for PDCP Transmitter to report to PDCP Receiver about the gap on the PDCP SN (i.e., transmitting PDCP entity can inform the receiving PDCP entity about the discarded SDUs).

## Network configurability

- To agree that the usage of a PDCP SN gap report is under network control (i.e. network configures UE whether/when PDCP SN gap report can be used).

  - To confirm that the usage of a PDCP SN gap reporting is dependent or applicable only when outOfOrderDelivery is not configured.

## Notification of PDCP SN Gap

- To agree on PDCP control PDU approach for transmitter to provide PDCP SN Gap reporting to receiver.

  - To discuss whether to enable PDCP SN Gap reporting via: option (A.1) bitmap kind of information, or option (A.2) range kind of information.

  - To discuss whether/which rules needs to be defined in PDCP transmitter entity to trigger PDCP SDU discard report considering e.g. (1) the PDCP entity discards SDU(s) which have not been transmitted (for UM DRBs) or acknowledged (for AM DRBs), due to the expiry of PDCP discard timer; and (2) there is a buffered SDU associated with an SN higher than the SN of the discarded SDU(s), as well as, related TPs included in R2-2401420, R2-2400748 and R2-2313923.

  - To consider the related TPs included in R2-2401420, R2-2400748 and R2-2313923.

## UE capabilities for the notification of PDCP SN Gap

- To discuss whether to define a new UE capability to indicate the support of PDCP SN Gap reporting. If so, to discuss whether UE supporting PDCP SN Gap reporting shall also support pdu-SetDiscard-r18 and/or psi-BasedDiscard-r18.

# Conclusion

The proposals captured are the following:

Proposal 1.	To define a mechanism for PDCP Transmitter to report to PDCP Receiver about the gap on the PDCP SN (i.e., transmitting PDCP entity can inform the receiving PDCP entity about the discarded SDUs).

Proposal 2.	To agree that the usage of a PDCP SN gap report is under network control (i.e. network configures UE whether/when PDCP SN gap report can be used).

Proposal 2.1.	To confirm that the usage of a PDCP SN gap reporting is dependent or applicable only when outOfOrderDelivery is not configured.

Proposal 3.	To agree on PDCP control PDU approach for transmitter to provide PDCP SN Gap reporting to receiver.

Proposal 3.1.	To discuss whether to enable PDCP SN Gap reporting via: option (A.1) bitmap kind of information, or option (A.2) range kind of information.

Proposal 3.2.	To discuss whether/which rules needs to be defined in PDCP transmitter entity to trigger PDCP SDU discard report considering e.g. (1) the PDCP entity discards SDU(s) which have not been transmitted (for UM DRBs) or acknowledged (for AM DRBs), due to the expiry of PDCP discard timer; and (2) there is a buffered SDU associated with an SN higher than the SN of the discarded SDU(s), as well as, related TPs included in R2-2401420, R2-2400748 and R2-2313923.

Proposal 3.3.	To consider the related TPs included in R2-2401420, R2-2400748 and R2-2313923.

Proposal 4.	To discuss whether to define a new UE capability to indicate the support of PDCP SN Gap reporting. If so, to discuss whether UE supporting PDCP SN Gap reporting shall also support pdu-SetDiscard-r18 and/or psi-BasedDiscard-r18.

# References

- R2-2400390	PDCP SN Gap Notification	Intel Corporation.

- R2-2400440	Need for PDCP discard notifications to receiving PDCP entity	LG Electronics, Xiaomi, NEC, Oppo, Samsung.

- R2-2400452	Discussion on PDCP discard notification to receiver	vivo.

- R2-2400478	PDCP Discarding Issues	Nokia, Nokia Shanghai Bell.

- R2-2400748	PDCP discard notification for XR	ZTE Corporation, Sanechips, Futurewei, Canon.

- R2-2400797	Indication of PDCP SN Gaps	Ericsson.

- R2-2400834	Discussion on SN gap issue	CANON Research Centre France, CATT.

- R2-2400845	PDCP and discard operation	InterDigital.

- R2-2400902	PDCP discard operation	MediaTek Inc.

- R2-2400926	Views on PDCP Discard Notification for Rel-18 XR	Apple.

- R2-2401326	On PDCP Discard Notification for XR	Google Inc.

- R2-2401420	Discussion on receiving window update for PDCP discard	Huawei, HiSilicon.

- R2-2401443	Discussion on PDCP discard notification	NTT DOCOMO INC.

- R2-2401448	Remaining issues red to PDCP discard	Sony.

- R2-2313923	Report of [AT124][019] PDCP discard (CATT)	CATT.