3GPP TSG-RAN WG2 Meeting #125bis	R2-2402524

# Changsha, China, April 15th – April 19th, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 4670 | rev | - | Current version: | 16.16.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on combination of HST and RRM relaxation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_HST, NR_UE_pow_sav-Core |  |  |  |  |  | Date: |  |  | 2024-04-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | RAN2 received a reply LS on combination of HST and RRM relaxation (R4-2403532). RAN4 extensively discussed the applicability of RRM requirements when relaxed measurement criterion is configured to a UE configured with highSpeedMeasFlag-r16 or highSpeedMeasInterFreq-r17 and agreed that: • If the lowMobilityEvaluation criterion is configured and fulfilled, it is up to UE implementation whether to apply the existing FR1 HST RRM requirements or relaxed RRM requirements corresponding to lowMobilityEvaluation, and no additional RAN4 requirement to be defined. According to the current field description of highSpeedMeasFlag, the UE shall apply the enhanced RRM requirements to support high speed up to 500 km/h if the field is present. This needs to be updated according to RAN4 agreement. As CRs in RAN4 were agreed from Rel-16, the corresponding correction in RAN2 also needs to start from Rel-16. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In the field description of highSpeedMeasFlag, clarify when the field is present and the lowMobilityEvaluation-r16 is configured and fulfilled, it is up to UE implementation whether to apply high speed RRM requirements or the relaxed RRM measurement requirements. Impact Analysis Impacted 5G architecture options: NR SA Impacted functionality: HST, RRM Relaxation Inter-operability: 1. If the NW is implemented according to the CR but the UE is not then UE will always apply high speed RRM requirements that is not aligned with agreement. 2. If the UE is implemented according to the CR but the NW is not then there is no inter-operability issue. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | When both HST flag and low mobility relaxation are enabled, UE measurement behavior would still be unclear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | Y |  | Other core specifications |  |  |  | TS/TR 38.304 CR#0395 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

CHANGE START

6.3.1	System information blocks

<Irrelevant Texts Omitted>

–	HighSpeedConfig

The IE HighSpeedConfig is used to configure parameters for high speed scenarios.

HighSpeedConfig information element

-- ASN1START

-- TAG-HIGHSPEEDCONFIG-START

HighSpeedConfig-r16 ::=  SEQUENCE {

highSpeedMeasFlag-r16    ENUMERATED {true}        OPTIONAL,   -- Cond SpCellOnly

highSpeedDemodFlag-r16   ENUMERATED {true}        OPTIONAL,   -- Need R

...

}

-- TAG-HIGHSPEEDCONFIG-STOP

-- ASN1STOP

HighSpeedConfig field descriptions

highSpeedMeasFlag If the field is present and the lowMobilityEvaluation-r16 is configured and the corresponding criterion is fulfilled, it is up to UE implementation whether to apply high speed RRM requirements or the relaxed RRM measurement requirements as specified in TS 38.133 [14]. For other cases: If the field is present and UE supports measurementEnhancement-r16, the UE shall apply the enhanced intra-NR and inter-RAT EUTRAN RRM requirements to support high speed up to 500 km/h as specified in TS 38.133 [14]. If the field is present and UE supports intraNR-MeasurementEnhancement-r16, the UE shall apply enhanced intra-NR RRM requirement to support high speed up to 500 km/h as specified in TS 38.133 [14]. If the field is present and UE supports interRAT-MeasurementEnhancement-r16, the UE shall apply enhanced inter-RAT EUTRAN RRM requirement to support high speed up to 500 km/h as specified in TS 38.133 [14]. This parameter only applies to the serving frequency of SpCell.

highSpeedDemodFlag If the field is present and UE supports demodulationEnhancement-r16, the UE shall apply the enhanced demodulation processing for HST-SFN joint transmission scheme with velocity up to 500km/h as specified in TS 38.101-4 [59]. This parameter only applies to SpCell.

| Conditional Presence | Explanation |
| --- | --- |
| SpCellOnly | The field is optionally present, Need R, in the ServingCellConfigCommonSIB or in the ServingCellConfigCommon of an SpCell. It is absent otherwise. |

CHANGE END