3GPP TSG-RAN WG2 Meeting#125bis		R2-2402534

Changsha, China, Apr. 15th – 19th, 2024

Title:	[Draft] LS on ra-ssb-OccasionMaskIndex handling for CFRA with Msg1 repetition

Response to:

Release:	Release 18

Work Item:	NR_cov_enh2-Core

Source:	China Telecom [to be RAN2]

To:	RAN1

Cc:

Contact person:

Name:	Pei LIN

E-mail Address:	linp@chinatelecom.cn

Attachments:	none

1	Overall description

In RAN2#125bis meeting, RAN2 has discussed how to handle the ra-ssb-OccasionMaskIndex for CFRA with Msg1 repetition and agreed to support ra-ssb-OccasionMaskIndex for msg1 repetitions.

Regarding how to use ra-ssb-OccasionMaskIndex for Msg1 repetition, RAN2 reached the following agreements:

- UE identifies the allowed set(s) of PRACH occasions based on ra-ssb-OccasionMaskIndex.

- Update the definition of ra-ssb-OccasionMaskIndex to indicate the allowed PRACH occasion set(s) of selected SSB for CFRA with Msg1 repetition.

- The maximum index of PRACH occasion set(s) is 8. For the case there are more than 8 sets of PRACH occasions of the selected SSB within the time period, the indexing of PRACH occasion sets indicated by the mask index value is reset. In addition, the indexing of PRACH occasion sets indicated by the mask index value is reset every time period.

In addition, RAN2 updated the values of ra-ssb-OccasionMaskIndex for CRFA with Msg1 repetition as follows.

| ra-ssb-OccasionMaskIndex for CRFA with Msg1 repetition | Allowed set(s) of PRACH occasions of SSB |
| --- | --- |
| 0 | All |
| 1 | PRACH occasion set index 1 |
| 2 | PRACH occasion set index 2 |
| 3 | PRACH occasion set index 3 |
| 4 | PRACH occasion set index 4 |
| 5 | PRACH occasion set index 5 |
| 6 | PRACH occasion set index 6 |
| 7 | PRACH occasion set index 7 |
| 8 | PRACH occasion set index 8 |
| 9 | Every even PRACH occasion set |
| 10 | Every odd PRACH occasion set |
| 11 | Reserved |
| 12 | Reserved |
| 13 | Reserved |
| 14 | Reserved |
| 15 | Reserved |

2	Actions

To RAN1

ACTION: 	RAN2 respectfully asks RAN1 to take into consideration above RAN2 agreements on handling of ra-ssb-OccasionMaskIndex for CFRA with Msg1 repetition to introduce PRACH occasion set index accordingly.

3	Dates of next TSG-RAN WG2 meetings

TSG-RAN WG2 Meeting #126	20 – 24 May                       	Fukuoka, Japan

TSG-RAN WG2 Meeting #127	19 – 23 August                       	Maastricht, Netherlands