3GPP TSG-RAN WG2 Meeting #125bis	R2-2402553

Changsha, China,  April 15th–19th, 2024

Agenda Item:	8.3.5

Source:	CMCC

Title:	Discussion on common Evaluation Methodology and Simulation Assumption

WID/SID:				FS_NR_AIML_Mob

Document for:	Discussion

# 1 Introduction

In RAN#102 meeting, the Study on AI/ML for mobility in NR was approved [1] with the following objectives:

The study will focus on mobility enhancement in RRC_CONNECTED mode over air interface by following existing mobility framework, i.e., handover decision is always made in network side. Mobility use cases focus on standalone NR PCell change. UE-side and network-side AI/ML model can be both considered, respectively. Study and evaluate potential benefits and gains of AI/ML aided mobility for network triggered L3-based handover, considering the following aspects: - AI/ML based RRM measurement and event prediction, - Cell-level measurement prediction including intra and inter-frequency (UE sided and NW sided model) [RAN2] - Inter-cell Beam-level measurement prediction for L3 Mobility (UE sided and NW sided model) [RAN2] - HO failure/RLF prediction (UE sided model) [RAN2] - Measurement events prediction (UE sided model) [RAN2] - Study the need/benefits of any other UE assistance information for the network side model [RAN2] - The evaluation of the AI/ML aided mobility benefits should consider HO performance KPIs (e.g., Ping-pong HO, HOF/RLF, Time of stay, Handover interruption, prediction accuracy, and measurement reduction) etc.) and complexity tradeoffs [RAN2] - NOTE: Simulation assumption and methodology can leverage TR 38.901, 38.843 and 36.839. And leave the detail discussion to RAN2 - Potential AI mobility specific enhancement should be based on the Rel19 AI/ML-air interface WID general framework (e.g. LCM, performance monitoring etc) [RAN2] - NOTE: This would only be treated after sufficient progress is made in the Rel-19 AI/ML air interface WID - Potential specification impacts of AI/ML aided mobility [RAN2] - Evaluate testability, interoperability, and impacts on RRM requirements and performance [RAN4] - NOTE 1: RAN1/3 work can be triggered via LS - NOTE 2: RAN4 scope/work can be defined and confirmed by RAN#105 after some RAN2 discussions (within the RAN4 pre-allocated TUs) NOTE 3: To avoid duplicate study with “AI/ML for NG-RAN” led by RAN3 NOTE 4: Two-sided model is not included

In this contribution, we would discuss on the key issues of evaluation methodology, simulation assumption for AI/ML for mobility in NR and provide our proposals.

# 2 Discussion

2.1 Key issues for common Evaluation Methodology

Common understanding for the evaluation of AI/ML for mobility

As the analysis on the study purposes of AI/ML for mobility in NR [2], we think the study and the related evaluation should focus on the measurement reduction for mobility and the benefits for HO performance through AI/ML model. So the evaluation scenarios or the simulation assumption which are not directly affect the prediction results of AI/ML for mobility should be as simple and uniform as possible. And the specific inputs or the simulation assumption which are different from the baseline or potentially impact on the outputs should be reported by companies.

Proposal 1: RAN2 takes following common understanding in consideration for the evaluation of AI/ML for mobility:

- The evaluation scenarios and the simulation assumption should be as simple and uniform as possible.

- The specific inputs which are different from the baseline input of AI/ML model and potentially impact on the outputs should be reported by companies.

- The specific simulation assumption which potentially impact on the outputs should be reported by companies.

Dataset for Training/Inference and Channel Model

In order to simulation calibration/alignment and performance results analysis, we prefer to reuse the Synthetic dataset generated according to the statistical channel models in TR 38.901(e.g. UMa channel) for model training and inference as the baseline. The dataset is generated by a system level simulator (SLS) based on 3GPP simulation methodology. And the Field data and the public dataset agreed by 3GPP can be used as the optional dataset if the Synthetic dataset cannot generate some specific scenarios (e.g., HOF scenario, etc.)

Proposal 2: To reuse the Synthetic dataset generated according to the statistical channel models in TR 38.901(e.g. UMa channel) for model training and inference as the baseline, and the Field data and public dataset agreed by 3GPP as the optional dataset if the SLS cannot generate the expected scenarios.

UE Trajectory Model

Regarding to the UE trajectory model in simulation, there are three options in TR 38.843. And we think the legacy options for UE Trajectory Model can be used as the baseline in the case that Synthetic dataset is used for model training and inference. For the Field data and the public dataset, the UE Trajectory should be decided based on the actual situation with the dataset.

Proposal 3: The legacy options for UE Trajectory Model in TR 38.843 can be used as the baseline in the case that Synthetic dataset is used for model training and inference.

Proposal 4: For the Field data and the public dataset, the UE Trajectory should be decided based on the actual situation with the dataset.

2.2 Simulation assumptions

Based on the above analysis, we would like to provide the baseline System Level Simulation assumptions and simulation assumptions for Measurement Event for AI/ML for mobility evaluation.

Table 1: The baseline System Level Simulation assumptions for AI/ML for mobility evaluation@ FR1

| Parameter |  | Value |
| --- | --- | --- |
| Frequency Range |  | FR1 @2GHz |
| Inter-BS distance |  | 500m ISD, 2-tier model with wrap-around (7 sites, 3 sectors/cells per site) |
| Channel model |  | UMa with distance-dependent LoS probability function defined in Table 7.4.2-1 in TR 38.901. |
| UE Speed |  | - 30km/h - 60km/h - 90km/h - 120km/h |
| UE distribution |  | 10 UEs per sector/cell 100% outdoor |
| Antenna setup and port layouts at gNB |  | Companies need to report which option(s) are used between - 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ - 16 ports: (8,4,2,1,1,2,4), (dH,dV) = (0.5, 0.8)λ |
| Antenna setup and port layouts at UE |  | 4RX: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ for (rank 1-4) 2RX: (1,1,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ for (rank 1,2) |
| BS Tx power |  | 41 dBm for 10MHz, 44dBm for 20MHz, 47dBm for 40MHz |
| BS antenna height |  | 25m |
| UE antenna height & gain |  | Follow TR36.873 |
| UE receiver noise figure |  | 9dB |
| Numerology | Slot/non-slot | 14 OFDM symbol slot |
|  | SCS | 15kHz for 2GHz |
| Simulation bandwidth |  | 10 MHz for 15kHz as a baseline |
| UE measurements/reports |  | Periodicity of time instance for each measurement/report: 40ms |
| Scenario |  | Urban Macro |
| Measurement window |  | 5*UE measurements/reports |
| Prediction window |  | 1,2,4*UE measurements/reports |
| Top-K （Top-K beam-level RSRP are averaged to derive cell-level RSRP） |  | 1~5 |

Table 2: The baseline System Level Simulation assumptions for AI/ML for mobility evaluation@ FR2

| Parameter | Value |
| --- | --- |
| Frequency Range | FR2 @ 30 GHz; SCS: 120 kHz |
| Deployment | 200m ISD, 2-tier model with wrap-around (7 sites, 3 sectors/cells per site) |
| Channel model | UMa with distance-dependent LoS probability function defined in Table 7.4.2-1 in TR 38.901. |
| System BW | 80MHz |
| UE Speed | - 30km/h - 60km/h - 90km/h - 120km/h |
| UE distribution | 10 UEs per sector/cell 100% outdoor |
| Transmission Power | Maximum Power and Maximum EIRP for base station and UE as given by corresponding scenario in 38.802 (Table A.2.1-1 and Table A.2.1-2) |
| BS Antenna Configuration | Antenna setup and port layouts at gNB: (4, 8, 2, 1, 1, 1, 1), (dV, dH) = (0.5, 0.5) λ Number of BS beams: 64 downlink Tx beams (max number of available beams) at NW side. |
| BS Antenna radiation pattern | TR 38.802 Table A.2.1-6, Table A.2.1-7 |
| UE Antenna Configuration | Antenna setup and port layouts at UE: (1, 4, 2, 1, 2, 1, 1), 2 panels (left, right) Number of UE beams: 4 downlink Rx beams (max number of available beams) per UE panel at UE side. |
| UE Antenna radiation pattern | TR 38.802 Table A.2.1-8, Table A.2.1-10 |
| Inter-panel calibration for UE | Ideal, non-ideal following 38.802 (optional) |
| BS Tx Power | 40 dBm (baseline) Other values (e.g., 34 dBm) not precluded |
| Maximum UE Tx Power | 23 dBm |
| BS receiver Noise Figure | 7 dB |
| UE receiver Noise Figure | 10 dB |
| Inter site distance | 200 m |
| BS Antenna height | 25 m |
| UE Antenna height | 1.5 m |
| Car penetration Loss | 38.901, sec 7.4.3.2: μ = 9 dB, σp = 5 dB |
| UE measurements/reports | Periodicity of time instance for each measurement/report: 40ms |
| Scenario | Dense Urban (macro-layer only, TR 38.913) is the basic scenario for dataset generation and performance evaluation. |
| Spatial consistency | - Procedure A in TR38.901 - Procedure B in TR38.901 |
| Measurement window | 5*UE measurements/reports |
| Prediction window | 1,2,4*UE measurements/reports |
| Top-K （Top-K beam-level RSRP are averaged to derive cell-level RSRP） | 1~5 |

Table 3: The Baseline simulation assumptions for Measurement Event A3

| Parameter | Value |
| --- | --- |
| L3 measurement filter | K = 4,1,0 |
| Handover criteria | Event A3, RSRP |
| a3-offset | 2dB |
| A3 TTT | 320ms 40ms |
| A3 hysteresis | 1dB |
| HO preparation time | 50ms |
| HO execution time | 40ms |
| RLM | L1 measurement period: 20ms Qin sliding window length：100ms Qout sliding window length: 200ms Qin threshold: -6dB Qout threshold: -8dB N310: 1 N311: 1 T310: 1s |
| MTS (Minimum time of stay) | 1s |

Proposal 5: RAN2 considers above System Level Simulation assumptions for FR1/FR2 and simulation assumptions for Measurement Event as the baseline simulation assumptions.

# 3 Conclusion

In this paper, we discuss on the key issues of common evaluation methodology, simulation assumption for AI/ML for mobility in NR, and we have the following proposals:

Proposal 1: RAN2 takes following common understanding in consideration for the evaluation of AI/ML for mobility:

- The evaluation scenarios and the simulation assumption should be as simple and uniform as possible.

- The specific inputs which are different from the baseline input of AI/ML model and potentially impact on the outputs should be reported by companies.

- The specific simulation assumption which potentially impact on the outputs should be reported by companies.

Proposal 2: To reuse the Synthetic dataset generated according to the statistical channel models in TR 38.901(e.g. UMa channel) for model training and inference as the baseline, and the Field data and public dataset agreed by 3GPP as the optional dataset if the SLS cannot generate the expected scenarios.

Proposal 3: The legacy options for UE Trajectory Model in TR 38.843 can be used as the baseline in the case that Synthetic dataset is used for model training and inference.

Proposal 4: For the Field data and the public dataset, the UE Trajectory should be decided based on the actual situation with the dataset.

Proposal 5: RAN2 considers above System Level Simulation assumptions for FR1/FR2 and simulation assumptions for Measurement Event as the baseline simulation assumptions.

# Reference

- RP-234055, Study on Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR, CMCC

- R2-2402552, Initial consideration on RRM measurement prediction, CMCC